3GPP TSG-RAN WG4 Meeting #101-e                 	R4-2201722

Electronic Meeting, 17th – 25th, January,  2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on the reference point for UE transmit timing requirement

Agenda item:	6.23.2.2

Document for:	Discussion

Introduction

In this paper, we continue to discuss the remaining issues for UE transmit timing requirement.

Discussion

- 2.1	The reference point

From the RAN4 #100e meeting, two options are most discussed for the definition of reference point for UE UL timing requirement:

- Option 1: The downlink timing is defined as the time, when the first detected path in time of the corresponding downlink frame from the reference cell [arrives/is received] at the UE antenna.

- Option 2: The downlink timing is defined as the time, when the first path in time of the corresponding downlink frame from the reference cell [arrives/is received] at the UE antenna

The concern for option 1 is about the testability and it indicates that the DL timing detection error is not included in Te , which contradicts to the RAN4 agreement that Te does include such error component [1]. Although option 2 was proposed to fix the above problems, but if the reference point is based upon the first path then Te should have included the margin for the channel first tap detection error. But to our understandings, the current Te design does not include such margin.

We see that neither one of the options can precisely capture Te and choosing any one of them will introduce a misinterpretation of Te. Considering the current wording works well in Rel-15/16 and there will not be any TA enhancement in Rel-17, we think option 2 by adding the term “at the UE antenna” to the current wordings can be compromised.

TP for Rel-15/16/17 38.133 7.1.2

“The downlink timing is defined as the time when the first detected path in time of the corresponding downlink frame from the reference cell is received at the UE antenna”

Conclusion

In this paper we further discuss the definition for the reference point of UE transmit timing requirement. We have the following proposals:

- TP for Rel-15/16/17 38.133 7.1.2

“The downlink timing is defined as the time when the first detected path in time of the corresponding downlink frame from the reference cell is received at the UE antenna”.

References

[1] R4-2105850, Reply LS on UE transmit timing error, RAN4