3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201784

E-meeting, January 17-25, 2022

Agenda item:	6.6.5.3

Source:	Intel Corporation

Title:	Discussion on SDR requirements for DL 1024QAM in FR1

Document for:	Discussion

# Introduction

During the RAN4#101-e meeting the following was agreed for SDR requirements [1]:

- Define SDR requirements for 1024 QAM with rank 1 and rank 2. - Add the MCS indexes for SDR test with 1024QAM based on Table 2. - Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, where - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj) - Simulation parameters: TS38.101-4 Table 5.5A-1 Table 2 MCS indexes for SDR test with 1024QAM vLayersj; Qmj; fj; MCSupperboundj; MCSpracticalj; MCSreqj. 1; 10; 1; 25; TBD; TBD. 1; 10; 0.8; 21; TBD. 1; 10; 0.75; 19; TBD. 1; 10; 0.4; 9; TBD. 2; 10; 1; 25; TBD; TBD. 2; 10; 0.8; 21; TBD. 2; 10; 0.75; 19; TBD. 2; 10; 0.4; 9; TBD.

In this paper we discuss the values for MCSpracticalj and corresponding MCSreqj.

# Discussion

As it was captured in the WF [1], the MCS indexes for 1024QAM SDR tests can be defined as the minimum between MCSupperboundj and MCSpracticalj. MCSupperboundj is the higher MCS according to UE capability calculated by Sec. 2.1 of R4-1812165. Its values for 1024QAM were agreed during the RAN4#101-e meeting. MCSpracticalj is the highest practical MCS considering feasible SNR levels. To define its values the link level simulations need to be conducted.

In Figure 1 we illustrate link level results for scenarios with 1024QAM. The corresponding simulation assumptions are shown in Table 1. In Table 2 we provide summary of simulation results at the 85% throughput point. For analysis we pick MCS equal to MCSupperboundj for scaling factor 1, which is MCS25 for both Rank 1 and Rank 2, and compare the results with MCS24 and MCS23 to check which MCS corresponds to more feasible SNR level.

Table 1. Test parameters for SDR test

| Parameters | Values |  |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz |  |
| MCS | MCS23 MCS24 MCS25 |  |
| Rank | 1, 2 |  |
| Antenna configuration | 1x2, 2x2 with ULA Low |  |
| Propagation channel | AWGN |  |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 0 ms |  |
| Number of HARQ processes | 4 | 8 |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 85% of maximum throughput |  |

| Figure 1. 1024QAM simulation results |  |
| --- | --- |

Table 2. Summary of SDR simulation results (SNR @ 85% of Max T-put)

|  | Rank 1 | Rank 2 |
| --- | --- | --- |
| MCS23 | 28.2 | 41.4 |
| MCS24 | 31.3 | N/A |
| MCS25 | 34.1 | N/A |

Based on the results from Table 2 the following observation can be made:

Observation 1. For the scenario with Rank 1 and 2 Rx antennas SNR operating point is feasible for MCS23.

Observation 2: For the scenario with Rank 2 and 2 Rx antennas SNR operating point for MCS23 is very high and for MCS24 and MCS25 is even out of simulated SNR range.

Based on Observation 2 we recommend not to define 1024QAM SDR tests for the scenario with Rank 2 and 2 Rx antennas.

Proposal 1: RAN4 shall not define 1024QAM SDR tests for the scenario with Rank 2 and 2 Rx antennas

In Figure 2 the simulation results for Rank 2 with 4 Rx antennas are shown and the SNR values at the 85% throughput point for that scenario are shown in Table 3.

|  | Table 3. Summary of SDR simulation results for nRx=4 (SNR @ 85% of Max T-put) Rank 2. MCS23; 31.8. MCS24; 34.1. MCS25; 37.7. |
| --- | --- |

Observation 3: In the scenario with 4 Rx antennas SNR operating point for MCS23 is feasible even for Rank 2.

Proposal 2: For 1024QAM SDR tests with Rank 2 consider 4 Rx antennas

Based on Observations 1 and 3 we can define MCSpracticalj as MCS23 for Rank 1 and Rank 2

Proposal 3: MCS index for MCSpracticalj is 23 for Rank 1 and Rank 2. Therefore, the MCS indexes for 1024QAM SDR tests can be defined as it is shown in Table 4.

Table 4. MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 23 | 23 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# Conclusion

Observation 1. For the scenario with Rank 1 and 2 Rx antennas SNR operating point is feasible for MCS23.

Observation 2: For the scenario with Rank 2 and 2 Rx antennas SNR operating point for MCS23 is very high and for MCS24 and MCS25 is even out of simulated SNR range.

Proposal 1: RAN4 shall not define 1024QAM SDR tests for the scenario with Rank 2 and 2 Rx antennas

Observation 3: In the scenario with 4 Rx antennas SNR operating point for MCS23 is feasible even for Rank 2.

Proposal 2: For 1024QAM SDR tests with Rank 2 consider 4 Rx antennas

Proposal 3: MCS index for MCSpracticalj is 23 for Rank 1 and Rank 2. Therefore, the MCS indexes for 1024QAM SDR tests can be defined as it is shown in Table 4.

Table 4. MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 23 | 23 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# References

- R4-2120699, WF on DL1024QAM demodulation and CQI reporting requirements, Ericsson, RAN4 #101-e, Nov 1-11, 2021