3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201785

E-meeting, January 17-25, 2022

Agenda item:	6.13.6.2

Source:	Intel Corporation

Title:	Discussion on Satellite Access Node demodulation requirements for NR NTN

Document for:	Discussion

# Introduction

In the Rel-17 new WI introducing Solutions for NR to support non-terrestrial networks was approved [1].  The objective captured for the performance part WI is rather general:

- Specify necessary UE and network performance requirements for the specified enhancements

In this paper we discuss the scope for the network performance requirements.

# Discussion

Scope of requirements

NR NTN system has specific propagation conditions with less scatterers long propagation distance and satellite motion. NTN channel models for link level simulations are introduced in TR38.811: two TDL models, namely NTN-TDL-A and NTN-TDL-B are constructed to represent two different channel profiles for NLOS, while NTN-TDL-C and NTN-TDL-D are constructed for LOS.

Considering the specifics of the propagation conditions, the existing demodulation requirements can not be reused for NR NTN. New requirements for all the physical channels need to be defined for NR NTN

Proposal 1: RAN4 to define the requirements for PUSCH, PUCCH and PRACH for NTN-TDL channel models

For NTN systems the satellite motion causes additional Doppler shift. RAN1 assumption is that UE will compensate this frequency offset using the information about UE location (from GNSS) and satellite ephemeris (broadcasted in System Information). However, the accuracy of this autonomous pre-compensation may not be perfect so some shift will still take place. RAN4 need to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

In NR NTN the Satellite Access Node is a dedicated access node and it is not expected to operate in regular terrestrial network. Thus, there is no need to conduct the Rel-15/Rel-16 tests for NTN Satellite Access Node

Proposal 3: NTN Satellite Access Node is not required to pass legacy Rel-15/Rel-16 tests

PUSCH requirements

Our views on the scope of NR NTN PUSCH requirements and corresponding test parameters are shown in Table 2-1.

Table 2-1. Scope of PUSCH requirements

| Requirement |  | Parameter | Values |
| --- | --- | --- | --- |
| Requirements for PUSCH with transform precoding disabled | Fraction of maximum throughput 70% | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2, 4, 8 |
|  |  | Modulation order | QPSK, 16QAM 64QAM can be added based on the agreement in RF room |
|  |  | CBW / SCS | 15kHz SCS: 5MHz, 10MHz, 15MHz, 20MHz 30kHz SCS: 10MHz, 15MHz, 20MHz |
|  |  | Propagation conditions | Different combinations of NTN-TDL channels and FRC to be tested |
|  |  | PUSCH mapping type | A, B |
|  | Fraction of maximum throughput 30% | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | QPSK |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for PUSCH with transform precoding enabled |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2, 4, 8 |
|  |  | Modulation order | QPSK |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for UCI multiplexed on PUSCH |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | 16QAM |
|  |  | CBW / SCS | 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for UL timing adjustment |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | 16QAM |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | FFS |
|  |  | PUSCH mapping type | A, B |

Proposal 4: RAN4 to define PUSCH requirements for NR NTN according to the scope shown in Table 2-1.

PUCCH requirements

The PUCCH requirements for NR NTN need to be defined for all Formats and for the same test parameters as in Rel-15. The difference is in channel model (NTN TDL channel model should be used) and CBW set (40MHz and 100MHz are not supported in NR NTN)

Proposal 5: RAN4 to define requirements for PUCCH Formats 0-4 and for the same test parameters as for Rel-15 PUCCH requirements except propagation conditions (NTN TDL should be used) and CBW set. The CBW set for requirement definition should follow Table 2-2

Table 2-2. NTN CBW set to for requirement definition

| SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| 15 | Yes | Yes | Yes | Yes |
| 30 |  | Yes | Yes | Yes |

PRACH requirements

For NR NTN PRACH, Rel-15 requirements for false alarm probability and for missed detection in AWGN channel can be reused

Proposal 6: For NTN PRACH, RAN4 to reuse Rel-15 requirements for false alarm probability and missed detection in AWGN channel

As for fading channel, PRACH time error tolerance and missed detection requirements need to be defined for NTN TDL channel. The same set of test parameters as in Rel-15 can be used. The only parameter that may need further discussion is the set of PRACH preambles to be tested since in NR NTN big cell sizes are considered.

Proposal 7: RAN4 to define requirements for PRACH time error tolerance and missed detection for NTN TDL channel considering the same set of test parameters as in Rel-15 except the set of PRACH preambles to be tested.

Proposal 8: RAN4 to define the set of PRACH preambles to be tested for NR NTN.

# Conclusion

In this paper we discussed the scope for the network performance requirements. The following proposals were made:

Proposal 1: RAN4 to define the requirements for PUSCH, PUCCH and PRACH for NTN-TDL channel models

Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

Proposal 3: NTN Satellite Access Node is not required to pass legacy Rel-15/Rel-16 tests

Proposal 4: RAN4 to define PUSCH requirements for NR NTN according to the scope shown in Table below.

Scope of PUSCH requirements

| Requirement |  | Parameter | Values |
| --- | --- | --- | --- |
| Requirements for PUSCH with transform precoding disabled | Fraction of maximum throughput 70% | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2, 4, 8 |
|  |  | Modulation order | QPSK, 16QAM 64QAM can be added based on the agreement in RF room |
|  |  | CBW / SCS | 15kHz SCS: 5MHz, 10MHz, 15MHz, 20MHz 30kHz SCS: 10MHz, 15MHz, 20MHz |
|  |  | Propagation conditions | Different combinations of NTN-TDL channels and FRC to be tested |
|  |  | PUSCH mapping type | A, B |
|  | Fraction of maximum throughput 30% | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | QPSK |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for PUSCH with transform precoding enabled |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2, 4, 8 |
|  |  | Modulation order | QPSK |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for UCI multiplexed on PUSCH |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | 16QAM |
|  |  | CBW / SCS | 30 kHz / 10MHz |
|  |  | Propagation conditions | One NTN TDL channel to be chosen |
|  |  | PUSCH mapping type | A, B |
| Requirements for UL timing adjustment |  | Number of TX antennas | 1 |
|  |  | Number of RX antennas | 2 |
|  |  | Modulation order | 16QAM |
|  |  | CBW / SCS | 15 kHz / 5MHz 30 kHz / 10MHz |
|  |  | Propagation conditions | FFS |
|  |  | PUSCH mapping type | A, B |

Proposal 5: RAN4 to define requirements for PUCCH Formats 0-4 and for the same test parameters as for Rel-15 PUCCH requirements except propagation conditions (NTN TDL should be used) and CBW set. The CBW set for requirement definition should follow Table below

NTN CBW set to for requirement definition

| SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| 15 | Yes | Yes | Yes | Yes |
| 30 |  | Yes | Yes | Yes |

Proposal 6: For NTN PRACH, RAN4 to reuse Rel-15 requirements for false alarm probability and missed detection in AWGN channel

Proposal 7: RAN4 to define requirements for PRACH time error tolerance and missed detection for NTN TDL channel considering the same set of test parameters as in Rel-15 except the set of PRACH preambles to be tested.

Proposal 8: RAN4 to define the set of PRACH preambles to be tested for NR NTN

# References

- RP-210908, Solutions for NR to support non-terrestrial networks (NTN), Thales, RAN #91e, March 22 – 26th, 2021