3GPP TSG-RAN WG4 Meeting #101-bis-e  		                                  R4-2201839

Electronic Meeting, 17th-25th of January, 2022

Source:	THALES, Magister Solutions Ltd

Title:	TN-NTN Coexistence Results Updates

Type:	discussion

Document for:	discussion

Agenda Item:	6.13.2.3

Release:	Rel-17

# Introduction

R4-2115749	WF on NTN co-existence study

- Decision:		Approved.

R4-2115750	Simulation assumption for NTN co-existence study

- Decision:		Approved.

During RAN4#100-e, the following documents related to NTN coexistence have been approved:

For further details, please also consider the email discussion summary for [100-e][313] NTN_Solutions_Part2 (R4-2115785, noted).

The simulation calibration phase (Phase 0) almost finished (see for example R4-2115628), and since then companies provided updates on the calibration results for both NTN and TN.

RAN4#100-e Agreements:

- The updated summary of calibration results and assumptions will be captured in the new TR 38.863

- The calibration results indicate the consistency of most companies’ simulations. Therefore, calibration work has mostly been done for NTN coexistence. Companies can continue to contribute on calibration aspect over emails till Sep 30th.

- RAN4 start to discuss the simulation assumption and co-existence results for phase 1 as agreed in previous work plan, RAN4 will check the status in Nov 2021 RAN4 meeting with the target to conclude phase 1 co-existence study by Nov 2021.

Moreover, during RAN4#100-e meeting, the following agreements have been made on the 20th August GTW online meeting:

RAN4#101-e Agreements:

- RAN4 agreed no further effort on NTN to NTN (satellite to satellite) co-existence scenarios in Rel-17 NTN WI.

Further, following RAN4#101-e decision, it has been decided to remove NTN-NTN coexistence scenarios, as indicated below:

This contribution therefore presents simulation updated results for TN-NTN coexistence, i.e., focusing on the scenarios 1 to 6 from the table below.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

# Simulation assumptions

The simulation assumptions are defined in [1] and [2]. The simulation methodology states that NTN UEs should be dropped at the edge of TN clusters, with the new methodologies provided in [2] and [3], considering an isolation distance of NTN UEs from TN cells. In the figure below [3], isolation distance is defined as the distance between the blue-dotted line (which represents TN cell boarder) and the red line, the isolation distance is therefore about 2*ISD=1500 m or 1.5 km for the urban case.

As mentioned in [3], only the TN stations in the “central 19 TN cells (cluster)”, but not the TN stations in “wrap-around TN cells” are to be evaluated.

Similar as in contribution [4], most of the UEs are allocated to TN cells, while the UEs at the edge of the TN cells may select the NTN beam. The simulations are further performed by considering the following use cases/combinations:

Figure 2.1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode (e.g. n1)

Figure 2.2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

where the interference types and varied parameters as respect to each scenarios are represented below:

| Interference Type | Combination | Aggressor | Victim | Notes | Evaluation | Varied & Static parameters |
| --- | --- | --- | --- | --- | --- | --- |
| i1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | ACIR | NTN UE ACS varied TN gNB ACLR static |
| i2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | ACIR | NTN satellite ACS varied TN UE ACLR static |
| i3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | ACIR | NTN satellite ACLR varied TN UE ACS static |
| i4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | ACIR | NTN UE ACLR varied TN gNB ACS static |
| i5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | ACIR | NTN UE ACLR varied TN UE ACS static |
| i6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | ACIR | NTN satellite ACS varied TN gNB ACLR static |

The simulation parameters are considered from R4-2115750, with the following updates of Table 2.3-7 with two ACLR regions (ACLR1 and ACLR2) and improved ACS, and results from [4] have been therefore updated accordingly:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30 dB (ACLR1) 43 dB (ACLR2) | 37 |
|  | ACS | 33 dB | 28 |

Moreover, the simulation takes into account the simplified simulation method provided in [2] and [3], taking into account the activity factor and specified for Case 4 and Case 6. For all cases, results have now been provided in terms of ACIR (where 1/ACIR=1/ACLR_Tx + 1/ACS_Rx).

# Simulation results

# Scenario 1

Figure 3.1.1: LEO@600 (rural, urban)

Figure 3.1.2: LEO@1200 (rural, urban)

Figure 3.1.3: GEO (rural, urban)

Observation 1: In Urban scenario with TN DL aggressor and NTN DL victim, the average SINR of NTN UEs degrades up to 1.9 dB depending on NTN UE ACS which also decreases average throughput in the order of ~10% with low ACS. NTN cell edge may suffer additional losses with LEO satellite compared to average values.

Proposal 1: For Case 1, an ACIR of 15 dBs seems sufficient.

# Scenario 2

Figure 3.2.1: LEO@600 (rural, urban)

Figure 3.2.2: LEO@1200 (rural, urban)

Figure 3.2.3: GEO (rural, urban)

Observation 2: Link budget from omnidirectional UE to the satellite is very modest, and with TN power control and limitations to the number of scheduled users the TN UEs do not pose a threat to the satellite UL. However, a great number of non-power controlled UEs under a large satellite beam could theoretically cause noticeable interference.

Proposal 2: For Case 2, an ACIR of 25-30 dBs seems sufficient.

# Scenario 3

Figure 3.3.1: LEO@600 (rural, urban)

Figure 3.3.2: LEO@1200 (rural, urban)

Figure 3.3.3: GEO (rural, urban), 90 degrees elevation angle

Figure 3.3.4: GEO (rural, urban), 45 degrees elevation angle

Observation 3: Path loss from satellite to UEs with omnidirectional antennas is high, so leaked interference power to adjacent channel becomes even less. In Rural scenario the TN UE link budget is smaller, so slightly increased interference can be seen in the SINR with low satellite ACLR.

Proposal 3: For Case 3, an ACIR of 20 dBs seems sufficient.

# Scenario 4

Figure 3.4.1: LEO@600 (rural, urban)

Figure 3.4.2: LEO@1200 (rural, urban)

Figure 3.4.3: GEO (rural, urban)

Observation 4: The UEs that connect to the satellite have a very large path loss towards TN gNBs. When ACIR is reduced from the NTN UEs’ transmitted power, the NTN UL ACI cannot be heard at the TN gNBs.

Proposal 4: For Case 4, an ACIR of 10-15 dBs seems sufficient.

# Scenario 5

Figure 3.5.1: LEO@600 (rural, urban)

Figure 3.5.2: LEO@1200 (rural, urban)

Figure 3.5.3: GEO (rural, urban)

Observation 5: Under used simulation assumptions there is a low probability that UEs are close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE.

Proposal 5: For Case 5, an ACIR of 10-15 dBs seems sufficient.

# Scenario 6 (only rural)

Figure 3.6.1: LEO@600 (rural)

Figure 3.6.2: LEO@1200 (rural)

Figure 3.6.3: GEO (rural), 90 degrees elevation angle

Figure 3.6.4: GEO (rural), 45 degrees elevation angle

Observation 6: The antenna directivity for TN gNBs is not low enough, and with high transmission power (and depending on the density of gNBs/number of gNBs in a beam) the TN gNBs will interfere with adjacent channel NTN UL transmissions with a low link budget. This situation is very particular, for a specific scenario, for a specific KPI, and the resulted requirement can be considered as for the worst case scenario. On the other hand, this could be a requirement integrated for an identified specific class of satellite access node, if further needed.

Proposal 6: For Case 6, an ACIR of 35-40 dBs seems sufficient.

# Proposals

Following all previous results for Scenario 1 to Scenario 6 and also previous observations, we can reach at the following proposal for ACIR values:

Proposal 1: For NTN coexistence simulations in Case 1, an ACIR of 15 dBs seems sufficient.

Proposal 2: For NTN coexistence simulations in Case 2, an ACIR of 25-30 dBs seems sufficient.

Proposal 3: For NTN coexistence simulations in Case 3, an ACIR of 20 dBs seems sufficient.

Proposal 4: For NTN coexistence simulations in Case 4, an ACIR of 10-15 dBs seems sufficient.

Proposal 5: For NTN coexistence simulations in Case 5, an ACIR of 10-15 dBs seems sufficient.

Proposal 6: For NTN coexistence simulations in Case 6, an ACIR of 35-40 dBs seems sufficient.

Which are collaborating/justifying some of the previous proposals for NTN UE and Satellite ACLR and ACS values:

Proposal 7. RAN4 shall consider for NTN UE same ACLR and ACS parameter values as for TN UE.

Proposal 8. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR.

Proposal 9. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS.

Proposal 10. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR when TN FDD.

Proposal 11. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACS when TN FDD.

Proposal 12. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS when TN TDD.

Proposal 13. RAN4 may consider to separate NTN requirements based on the TN type (FDD or TDD).

Proposal 14. RAN4 could consider taking the average value of Satellite Access Node ACS between Case 2 and Case 6 (rural) scenarios (see R4-2201842).

Proposal 15. RAN4 could consider taking the average value of NTN UE ACLR between Case 4 and Case 5 scenarios.

# Conclusions

Observation 1: Scenario 1. In Urban scenario with TN DL aggressor and NTN DL victim, the average SINR of NTN UEs degrades up to 1.9 dB depending on NTN UE ACS which also decreases average throughput in the order of ~10% with low ACS. NTN cell edge may suffer additional losses with LEO satellite compared to average values.

Observation 2: Scenario 2. Link budget from omnidirectional UE to the satellite is very modest, and with TN power control and limitations to the number of scheduled users the TN UEs do not pose a threat to the satellite UL. However, a great number of non-power controlled UEs under a large satellite beam could theoretically cause noticeable interference.

Observation 3: Scenario 3. Path loss from satellite to UEs with omnidirectional antennas is high, so leaked interference power to adjacent channel becomes even less. In Rural scenario the TN UE link budget is smaller, so slightly increased interference can be seen in the SINR with low satellite ACLR.

Observation 4: Scenario 4. The UEs that connect to the satellite have a very large path loss towards TN gNBs. When ACIR is reduced from the NTN UEs’ transmitted power, the NTN UL ACI cannot be heard at the TN gNBs.

Observation 5: Scenario 5. Under used simulation assumptions there is a low probability that UEs are close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE.

Observation 6: Scenario 6. The antenna directivity for TN gNBs is not low enough, and with high transmission power (and depending on the density of gNBs/number of gNBs in a beam) the TN gNBs will interfere with adjacent channel NTN UL transmissions with a low link budget. This situation is very particular, for a specific scenario, for a specific KPI, and the resulted requirement can be considered as for the worst case scenario. On the other hand, this could be a requirement integrated for an identified specific class of satellite access node, if further needed.

Following all previous results for Scenario 1 to Scenario 6 and also previous observations, we can reach at the following proposal for ACIR values:

Proposal 1: For NTN coexistence simulations in Case 1, an ACIR of 15 dBs seems sufficient.

Proposal 2: For NTN coexistence simulations in Case 2, an ACIR of 25-30 dBs seems sufficient.

Proposal 3: For NTN coexistence simulations in Case 3, an ACIR of 20 dBs seems sufficient.

Proposal 4: For NTN coexistence simulations in Case 4, an ACIR of 10-15 dBs seems sufficient.

Proposal 5: For NTN coexistence simulations in Case 5, an ACIR of 10-15 dBs seems sufficient.

Proposal 6: For NTN coexistence simulations in Case 6, an ACIR of 35-40 dBs seems sufficient.

Which are collaborating/justifying some of the previous proposals for NTN UE and Satellite ACLR and ACS values:

Proposal 7. RAN4 shall consider for NTN UE same ACLR and ACS parameter values as for TN UE.

Proposal 8. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR.

Proposal 9. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS.

Proposal 10. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR when TN FDD.

Proposal 11. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACS when TN FDD.

Proposal 12. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS when TN TDD.

Proposal 13. RAN4 may consider to separate NTN requirements based on the TN type (FDD or TDD).

Proposal 14. RAN4 could consider taking the average value of Satellite Access Node ACS between Case 2 and Case 6 (rural) scenarios (see R4-2201842).

Proposal 15. RAN4 could consider taking the average value of NTN UE ACLR between Case 4 and Case 5 scenarios.

# References

- R4-2115750, Simulation assumptions for NTN co-existence study, 3GPP RAN4#100-e.

- R4-2120671, Simulation assumptions for NTN co-existence, 3GPP RAN4#101-e.

- R4-2120670, WF on [310] NTN_Solutions_Part2, 3GPP RAN4#101-e.

- R4-2120628, TN-NTN Coexistence Results for Phase 1, 3GPP RAN4#101-e, THALES.

END