3GPP TGS-RAN WG4 Meeting #101-Bis-e	R4-2201924
Electronic Meeting, January 17 – 25, 2022

Agenda item:	6.16.2

Source:	Intel Corporation

Title:	Views on channelization for 52.6 to 71 GHz

Document for:	Discussion

# 1	Introduction

This paper continues the channelization discussion in 60 GHz.

# 2	Discussion

In RAN4#100-e and RAN4#101-e, the following agreements were captured in the GTW session regarding channelization:

[Agreement]: For channel raster and sync raster, use Option 1C and Option 1D as starting point to seek the compromised solution. Channelization Option 1C (no IEEE 802.11ad/ay alignment and floating channelization) and Option 1D (hybrid between IEEE and no IEEE alignment with fixed channelization) will be evaluated in the next RAN4 meeting based on: - Number of sync raster entries and cell searching complexity - Support of CA in this frequency range - Whether flexibility is needed for minimum channel bandwidth, and how much it benefits system performance - Co-existence with IEEE channels (find out whether there is similar activity in IEEE) - Whether to consider and how to ensure future proof for adding new frequency bands or channel bandwidth - Other aspects are not precluded If no agreement is reached in the next RAN4 meeting, different channelizations will be considered for licensed and unlicensed bands

Option 1C and Option 1D are listed below:

- Option 1C: No IEEE 802.11ad/ay alignment and floating channelization

- Option 1D: Hybrid between IEEE and no IEEE alignment with fixed channelization depending on max spectrum utilization and better coexistence

Channelization has been discussed in RAN4 for a couple of meetings with little progress. The first main issue is whether 60 GHz NR channelization will be aligned with IEEE 802.11ad/ay channels. The second issue is whether 3GPP will specify different channelizations for licensed and unlicensed bands.

Earlier in our discussions, there was a clear preference to have separate channelization for licensed band and unlicensed band, respectively. In RAN4 #98Bis-e, RAN4 agreed to seek a unified raster design for both licensed and unlicensed bands. This was suggested by the Chair during GTW session, but no conclusion was reached. While we believe Option 1D addresses most of the open issues and is quite good as a compromised solution, if RAN4 cannot further narrow down between Option 1C and Option 1D, then we think the better approach is to have separate channelization designs for licensed and unlicensed bands, respectively.

A few reasons why it might be beneficial for RAN4 to focus on concluding the design for unlicensed in Rel-17, and work on licensed channelization later include that licensed band is unlikely to be defined in Rel-17, therefore applying floating channelization that may be beneficial for licensed is that such over-design is completely unnecessary for unlicensed operation. In fact, having more granular choice of the channels can potentially lead to higher fragmentation usage of the band. It is no surprise unlicensed operation in 2.4 GHz is notorious for bad coexistence and has difficulties operating with interferers that overlap fractionally. Additionally, it is not even clear whether 100 MHz, 200 MHz, 400 MHz, 800 MHz, 1600 MHz, and 2000 MHz channel bandwidths are going to be useful for future licensed bands. For example, the licensed operation in 66 to 71 GHz may require frequency blocks in units of 250 MHz. We think this might also be a likely outcome since 250 MHz frequency blocks are already being leveraged in E-band (71 to 86 GHz) for lightly licensed operations in many countries (such as Canada, South Africa, United Kingdom, etc) around the world [5]. Designing some optimization for that work only for agreed channel bandwidths for unlicensed may result in complete waste, as it is unclear it can be used for channel bandwidths required for future licensed operation in 66 to 71 GHz.

Furthermore, based on regulatory survey conducted for bands above 52.6 GHz, no country currently has any plan for licensed operation between 57 to 66 GHz. The only potential usage for licensed seems to be from 66 to 71 GHz. Therefore, applying a floating channel raster on the entirety of spectrum between 57 to 71 GHz seems to simply create complexity and in fact is counterproductive to coexistence.

Observation 1: There was no agreement to seek a unified raster design for both licensed and unlicensed bands.

Observation 2: It is unclear if future licensed bands will require entirely different channel bandwidth sizes, e.g., 250 MHz, and in such case, it is unclear if potential optimizations conducted for 100/200/400/800/1600/2000 MHz aimed for licensed operation will actually be optimal or even functional.

Observation 3: The potential possibility for licensed operation only exists between 66 to 71 GHz, and there is no need to apply licensed operation optimization on the entire band between 57 to 71 GHz.

Proposal 1:  RAN4 to consider supporting separate raster design for licensed and unlicensed bands.

Based on the discussions from RAN4 #101-e, we provide updated channelization proposal that provides sufficient flexibility to maximize spectrum usage for various regulatory domains, minimal cell search complexity, and minimal signalling efforts for CORESET#0 RB offset, which may determine the supported useful CORESET#0 configurations that could be supported for various deployment scenarios.

To provide flexibility in deployment, we define non-overlapping sets of 100 MHz channel bandwidth (CBW) which are used as a building block for wider CBW as shown in Figure 1. 400 MHz, 800 MHz, 1600 MHz and 2000 MHz CBW are defined by selecting contiguous 4, 8, 16, and 20 channels of 100 MHz respectively. To minimize the number of required RB offsets, the synchronization raster was chosen such that the SSB closest to the center of each 100 MHz channel is selected. In order to generally achieve RB offset 0 for majority of the cases, synchronization raster was chosen such that SSB are selected closest to the center of each 100 MHz channel bandwidth (CBW). These selected synchronization raster entries are chosen as valid entries for 120 kHz. From the subset of synchronization raster entries (selected for each 100 MHz CBW), the first raster instance among valid SSB candidate positions within the 400 MHz CBW were selected for valid synchronization raster for 480 kHz. This results in raster entries for 480 kHz to be a subset of the raster entries for 120 kHz.

Figure 1. Supported 100 MHz channels in 57-71 GHz. The dotted line represents the OCW of the 100 MHz channel with the SSB placed in the center of the channel.  The black lines represent the 802.11 ad/ay channel boundaries.

In order to support better coexistence and maximize the spectrum usage, the larger CBW are defined in 2 sets:

- Set A: CBW align with 802.11 ad/ay channel boundaries to improve coexistence. Here align with channel boundaries means the CBW do not cross or overlap with 802.11ad 802.11ay channel boundaries. In figure 2, we see 4 sets of 100 MHz CBW are selected such that the center of the channels is the center frequency for 400MHz with 120kHz, 480 kHz and 960 kHz.

- Set B: CBW are selected that would maximize the spectrum usage. Set B is optimized for 59-64 GHz band allowing 12 non-overlapping channels of 400 MHz as shown in Figure 3, instead of 10 channels as shown in Figure 2

Figure 2. Supported Set A 400 MHz channels in 57-71 GHz which are optimized for coexistence. The large, dotted block represents the OCW of the 400 MHz channel consisting of four 100 MHz channels.

Figure 3. Supported Set B 400 MHz channels in 59-64 GHz which are optimized for spectrum utilization. The large, dotted block represents the OCW of the 400 MHz channel consisting of four 100 MHz channels (only the optimized channels between 59-64 GHz are depicted).

For larger CBW such as 800MHz, 1600MHz and 2000MHz, a similar design process is followed where contiguous blocks of 400 MHz channels are selected.

Table 1. Number of channels for each (CBW, SCS) combination

| SCS | CBW |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 kHz | 138 | 38 | N/A | N/A | N/A |
| 480 kHz | N/A |  | 31* | 11 | N/A |
| 960 kHz | N/A |  |  |  | 6 |

*Note: 2 raster entries selected for 800 MHz are not a subset of the entries for 400 MHz. This trade off ensures minimizing the required RB offsets (RB offset [4] for 48 PRB and RB offset [52] for 96 PRB for mux 1; and RB offset [48] for 48 PRB for mux 3).

In Table 1, the total UE SSB search complexity is 138 for 120 kHz and 40 for 480 kHz = 178 raster entries for initial access. This is significantly lesser than the target value of 665 specified in the WID. We can consider separate floating raster design for the licensed operation between 66-71 GHz. This channelization design allows the support of 0 RB offset for all cases, except 48 PRB case for 120 kHz (RB offset 14).

The NR-ARFCN and SS raster entries are defined for the n263 band in Table 5.4.2.3-2 and Table 5.4.3.3-2 in TS 38-104 as shown in the text proposal below:

= = = = = = = = = = = = = = = = = = = = =start of text proposal= = = = = = = = = = = = = = = = = = = = = = = = = =

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR2

| NR operating band | ΔFRaster (kHz) | Uplink and Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n2631 | 120 | 2563347– <2> – 2794995 |
|  | 480 | 2565843– <8> – 2792499 |
|  | 960 | 2565843– <16> – 2792499 |
| NOTE 1: Applicable NR-ARFCN for band n263 for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} for 400 MHz channel bandwidth, NREF = {2571643, 2578363, 2585083, 2591803, 2598523, 2606923, 2613643, 2620363, 2627083, 2633803, 2642203, 2648923, 2655643, 2662363, 2669083, 2679163, 2685883, 2692603, 2699323, 2706043, 2714443, 2721163, 2727883, 2734603, 2741323, 2751403, 2758123, 2764843, 2771563, 2778283, 2786683, 2600203, 2640523, 2647243, 2653963, 2660683, 2667403, 2674123} for 800 MHz channel bandwidth, NREF = {2575003, 2588443, 2603563, 2617003, 2630443, 2645563, 2659003, 2672443, 2689243, 2702683, 2717803, 2731243, 2744683, 2761483, 2774923, 2788363, 2581723, 2595163, 2610283, 2623723, 2638843, 2652283, 2665723, 2682523, 2695963, 2711083, 2724523, 2737963, 2754763, 2768203, 2781643} for 1600 MHz channel bandwidth, NREF = {2581723, 2623723, 2652283, 2695963, 2724523, 2768203, 2610283, 2637163, 2664043, 2753083, 2781643} for 2000 MHz channel bandwidth, NREF = {2585083, 2620363, 2655643, 2692603, 2727883, 2764843} |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2)

| NR operating band | SS Block SCS | SS Block pattern (note) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n2632 | 120 kHz | Case D | 24153 – <1> – 24960 |
|  | 480 kHz | Case F | 24155– < 1 > – 24958 |
|  | 960 kHz | Case G | 24158– < 1 > – 24954 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213 [10]. NOTE 2: The following GSCN are allowed for operation in band n263: for 120 kHz, GSCN = {24157 + 6*N – floor((N-2)/6) - 1, N=0:137}. for 480 kHz, GSCN = {24180, 24186, 24203, 24227, 24250, 24273, 24279, 24285, 24303, 24326, 24332, 24349, 24373, 24378, 24396, 24402, 24419, 24425, 24431, 24443, 24448, 24466, 24472, 24489, 24495, 24513, 24518, 24536, 24553, 24577, 24583, 24600, 24623, 24647, 24653, 24676, 24682, 24699, 24723, 24746, 24769, 24781, 24804, 24828, 24833, 24851, 24874, 24880, 24898, 24921, 24927}. for 960 kHz, not applicable SS raster entries exist for PCell and PScell. |  |  |  |

= = = = = = = = = = = = = = = = = = = = =end of text proposal= = = = = = = = = = = = = = = = = = = = = = = = = =

Analysis on floating channelization for licensed band

If separate channelizations are supported for licensed and unlicensed bands, we can consider floating channelization in licensed band only (for example 66-71 GHz) with the channel raster grid based on SCS. The total number of ARFCN between 66-71 GHz are 83334 and number of GSCNs are 289. For example, the channel center frequencies (ARFCN raster) can be subsampled with granularity equal to the SCS. This results in a step size = {2, 8, 16} for SCS {120, 480, 960} kHz respectively for the ARFCN entries.  The step size for GSCN entries can be 1. The GSCN raster entries supported in the unlicensed band between 66-71 GHz can be avoided for use in the licensed band operation.

Support of valid GSCN with step size of 1 for 66-71 GHz would also allow potential support for arbitrary channel bandwidth sizes, such as 250 MHz or 500 MHz, instead of being limited to channel bandwidth that are defined for unlicensed operation. The total number of raster entries required for each {SCS, BW} combination is summarized in Table 2.

Table 2. Total number of {ARFCN, GSCN} raster entries required for floating channelization in licensed band (assuming 86.4% spectrum utilization)

| SCS/BW | 100 MHz | 400MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
| 120 kHz | 40834 ARFCN, 235 GSCN | 38334 ARFCN, 221 GSCN* | - | - | - |
| 480 kHz | - | 9584 ARFCN, 223 GSCN | 8750 ARFCN, 202 GSCN+ | 7084 ARFCN, 164 GSCN+ |  |
| Note*: 221 GSCN entries for 120kHz with 400 MHz are completely subset of 235 GSCN entries of 100 MHz. Therefore, UE does not need to perform a separate search for 221 GSCN possible for 400 MHz. Note+: GSCN entries for 480 kHz with 800 or 1600 MHz are completely subset of 223 GSCN entries of 400 MHz. Therefore, UE does not need to perform a separate search for these GSCN. |  |  |  |  |  |

Since the GSCN entries for 120 kHz and 480 kHz cases with our proposed 1D is 138 and 40, respectively. Therefore, the total GSCN raster entries with step size of 1 in this floating design between 66 to 71 GHz in licensed band along with Option 1-D in unlicensed band will still be lower than the target 665 entries specified in the WID.

Proposal 2: Consider separate GSCN usage between unlicensed and licensed band operation.

# 3	Conclusions

In this paper, we discussed three channelization proposal options for FR2-2. The following observations and proposals were made:

Observation 1: There was no agreement to seek a unified raster design for both licensed and unlicensed bands.

Observation 2: It is unclear if future licensed bands will require entirely different channel bandwidth sizes, e.g., 250 MHz, and in such case, it is unclear if potential optimizations conducted for 100/200/400/800/1600/2000 MHz aimed for licensed operation will actually be optimal or even functional.

Observation 3: The potential possibility for licensed operation only exists between 66 to 71 GHz, and there is no need to apply licensed operation optimization on the entire band between 57 to 71 GHz.

Proposal 1:  RAN4 to consider supporting separate raster design for licensed and unlicensed bands.

Proposal 2: Consider separate GSCN usage between unlicensed and licensed band operation.

# 4	References

- TR38.807 V16.0.0, Study on requirements for NR beyond 52.6 GHz

- R4-2118781,	“52.6-71 GHz System Parameters,”	Ericsson