3GPP TSG-RAN WG4 Meeting #101-bis-e 	R4-2201939

Online, 17 - 25 Jan 2021

Source: 	Huawei

Title: 	eIAB Timing error between MT TX and DU TX

Agenda Item:	6.17.2.2

Document for:	Discussion

# Introduction

I the WF on Rel 17 eIAB the following agreement open issue was raised:

Case#6 timing: Timing error between own MT TX and DU TX

Agreement:

Baseline assumption: Introducing RF requirements for Timing error between own MT TX and DU TX should be defined for case#6 timing

- Pending on further checking whether existing cell phase sync requirements already cover above timing error

- Further check the timing error tolerance between the parent IAB-DU node and child IAB-MT

With above baseline assumption confirmed FFS on:

-reference condition for this requirement

-error requirement with below Alts

- Alt1: 3us

- Alt2: cyclic prefix length of largest supported SCS

- Alt3: value as SCS dependent

This paper gives our view on the baseline assumption issue.

# Discussion

The baseline for the agreement in the WF was based on a majority view of 3 to 1 companies and is pending further checks on if cell phase synch requirement already cover this requirement.

Having checked the cell synchronisation requirement we believe that this covers the additional timing error.

For the timing error between the MT Tx and the DU Tx there are 2 possible factors which could contribute.

The existing cell phase sync requirements (3us)

The TAE

Compared to the cell sync requirement the additional time alignment error is small so can be ignored.

As such the existing cell sync requirements cover this and there is no need to define a new RF TAE requirement.