3GPP TSG-RAN WG4 Meeting#101-bis-e	R4-2201975

Online, 17th Jan – 25th Jan, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on Inter-cell Interference CQI Reporting Tests

Agenda item:	6.12.2.1.2

Document for:	Discussion

- Introduction

In RAN4#101-e, WF [1] was agreed and it had some open issues. In this paper, we provide our views on those open issues for inter-cell interference CSI reporting scenarios and corresponding simulation results.

- Simulation Results

Based on simulation assumptions agreed in [1], we provide our simulation results below.

Table 1: Simulation results for FDD CQI reporting under Inter-cell Interference Scenario

| Antenna Configuration | SINR (dB) | Throughput Ratio | BLER |
| --- | --- | --- | --- |
| 2x2 | -1 | 2.51 | 9.99% |
|  | -2 | 2.73 | 13.14% |
|  | -3 | 2.99 | 16.11% |
| 2x4 | -2 | 4.35 | 0.39% |
|  | -3 | 4.76 | 0.93% |
|  | -4 | 4.94 | 4.86% |
| 1x2 | -1 | 2.44 | 8.49% |
|  | -2 | 2.71 | 11.79% |
|  | -3 | 2.92 | 13.68% |
| 1x4 | -2 | 4.58 | 2.22% |
|  | -3 | 5.37 | 2.84% |
|  | -4 | 4.92 | 3.59% |

- Test Scope and Setup

Antenna Configuration

Based on the simulation results, there is not much difference in performance with 1Tx v/s 2Tx. Therefore, we propose the following.

Proposal 1: Define CQI reporting tests for only 1Tx or only 2Tx based on simulation alignment.

Test Metric

Based on above simulation results, we propose the following.

Proposal 2: Use following parameters for defining FDD CQI reporting tests.

- SINR = -1dB for 2Rx, -4dB for 4Rx

- Throughput Ratio = 1.5 for 2Rx, 3.0 for 4Rx

- BLER Threshold X = 1%

- Conclusions

This paper provides our views on open issues for inter-cell interference CSI reporting scenarios in RAN4. Following has been proposed.

Proposal 1: Define CQI reporting tests for only 1Tx or only 2Tx based on simulation alignment.

Proposal 2: Use following parameters for defining FDD CQI reporting tests.

- SINR = -1dB for 2Rx, -4dB for 4Rx

- Throughput Ratio = 1.5 for 2Rx, 3.0 for 4Rx

- BLER Threshold X = 1%

# References

[1] R4-2120708, “WF on CSI requirements for inter-cell interference MMSE-IRC”, 3GPP TSG RAN WG4 Meeting #101-e, Online, Nov 1 - 12, 2021.