3GPP TSG-RAN WG4 Meeting RAN4#101-bis-e     				R4-2201990 Online

January 17 – 25, 2022

Agenda Item:	7.1.2.2

Source:	Lenovo, Motorola Mobility

Title:	Issues with MIMO EVM Measurement Using the Pseudo-Inverse

Document for:  Approval

- Introduction

In the technical report [1], the pseudo-inverse is used to define and measure the EVM for two-layer transmissions with FR2. Similarly, it has been proposed in [2] that the pseudo-inverse be used to define and measure EVM for FR2.

In [3], some issues were identified with the use of the pseudo-inverse and these issues are also addressed in this contribution.  As discussed below, if the effective channel matrix has full rank, the pseudo-inverse is equivalent to the inverse.  In the case that the channel does not have full rank, the pseudo-inverse in [1,2] cannot be defined or computed as proposed. Finally, if the effective channel does not have full rank, the MIMO layers cannot be separated using the pseudo-inverse. In this contribution, is proposed that the inverse be used instead of the pseudo-inverse.

A text proposal for Section 5.2.3.1.1.2 of TR 38.884-120 replacing the pseudo-inverse with the inverse in the definition of EVM for Method 1 MIMO equalization is provided in the Appendix.

- EVM for Two-Layer Transmission

In [1,2], it has been proposed that the pseudo-inverse be used in the definition and measurement of EVM for two-layer transmission.  Following the notation in [2], the effective channel matrix is given by

H=HP= h0,0h0,1h1,0h1,1

where H is the 2x2 channel matrix and P is the precoding matrix.  It is then proposed that the zero-forcing MIMO equalizer is defined using the pseudo-inverse as

GZF=H+=HHH-1HH.

A few observations can be made on the use of the pseudo-inverse for measuring EVM.

Observation 1:	If a square matrix H has full rank, then H+=H-1, and the pseudo-inverse is not needed.

To see this, note that if the matrix has full rank so that H-1 exists, then the pseudo-inverse can be expressed as

H+=HHH-1HH=H-1HH-1HH=H-1

Observation 2:	If a square matrix H does not have full rank, then HHH does not have full rank and the inverse HHH-1 does not exist.  As a result, if H does not have full rank, then the pseudo-inverse cannot be defined or computed as

H+=HHH-1HH.

So, if the pseudo-inverse H+ were to be used, it would be necessary to define it in some other manner such as in terms of the singular value decomposition of H.

Observation 3:  	If the square matrix H does not have full rank, then it is not possible to separate the MIMO layers using the pseudo-inverse or any other linear receiver and the EVM requirement will be failed.

With respect to Observation 3, we note that if H does not have full rank, then multiplication by the pseudo-inverse does not yield the identity matrix; that is

H+H ≠I,

and as a result, the pseudo-inverse H+ cannot be used to separate the MIMO layers. To demonstrate, let y denote the signal received given by

y=H x+n

where x is the transmitted data vector, n is the transmitter noise vector, and H has dimension 2x2.  Furthermore, let the singular value decomposition of H be given by

H=U Σ VH,

where U and V are unitary matrices and Σ is a diagonal matrix for which the diagonal elements are real and non-negative.  In [3], it is shown that if

GZF=H+,

and H does not have full rank, then the resulting data estimate x is given by

x=H+y=v1 v1Hx+n

=v1* x v1+v1* n v1

where v1 is the right singular vector belonging to V for which the corresponding singular value is non-zero.  Thus, in the case that H does not have full rank, the expected value of the estimate is given by

Ex|x=v1H x v1

which is the projection of the data vector x onto the right singular vector v1.

In [3], it is shown that even if there is no transmitter noise so that n=0, the EVM for the estimate

x=H+y

when H does not have full rank is no less than 71%, so in this case the EVM requirement fails.

Observation 4: 	If H does not have full rank and GZF=H+ is used, the resulting EVM will be no less than 71%.

In summary, if the square matrix H does not have full rank, the pseudo-inverse cannot be defined and computed as

H+=HHH-1HH

because HHH-1 does not exist.  Furthermore, if H does not have full rank, the two layers of data cannot be separated with the pseudo-inverse or any other linear receiver. If the pseudo-inverse is used in this case, the EVM will be no less than 71%.

As the result of the above, we have the following proposal.

Proposal 1:  	If H has full rank, then GZF=H-1.  If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed.

In any test, the effective channel matrix H should have full rank.  However, if necessary, a threshold can be set on the condition number of the matrix H (the ratio of the largest eigenvalue to that of the smallest eigenvalue), such that if the threshold is exceeded, the test is failed.

A text proposal for Section 5.2.3.1.1.2 of TR38.884-120 implementing Proposal 1 for Method 1 MIMO equalization is given in the Appendix below.

Proposal 2:	Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-120 in the Appendix.

- Conclusions

# In [1,2], it has been proposed that the pseudo-inverse be used in the definition and measurement of EVM for two-layer transmission.  However, in this contribution, it has been observed that if the channel matrix H does not have full rank, then HHH does not have full rank and the pseudo-inverse cannot be defined as

# H+=HHH-1HH

# Furthermore, it has been shown, that if H does not have full rank, then the MIMO layers cannot be separated using the pseudo-inverse or any other linear receiver. Based on these observations, we have the following proposal.

Proposal 1:  	If H has full rank, then GZF=H-1.  If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed.

In any test, the effective channel matrix H should have full rank.  However, if necessary, a threshold can be set on the condition number of the matrix H (the ratio of the largest eigenvalue to that of the smallest eigenvalue), such that if the threshold is exceeded, the test is failed.

A text proposal for Section 5.2.3.1.1.2 of TR38.884-120 implementing Proposal 1 for Method 1 MIMO equalization is given in the Appendix below.

Proposal 2:	Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-120 in the Appendix.

# References

- R4-2107113, “Discussion on FR1 UL MIMO EVM measurements,” Rohde and Schwartz, 3GPP TSG-RAN WG4 Meeting #98-bis-e.

- R4-2119555, “Issues with FR2 MIMO EVM Measurement Using the Pseudo-Inverse,” 3GPP TSG-RAN WG4 Meeting #101-e.

- R4-2120768 TR38.884-120 Study on enhanced test methods for FR2 NR UEs, TSG RAN4 #101-e,

Appendix:  Text Proposal for TR 38.884-120

###### < start of changes >

###### 5.2.3.1.1.2	Method 1 MIMO Equalization

The MIMO equalization is based only on reference signals (DMRS) without using any data symbols. In order to obtain comparable EVM results independent of the number of DMRS symbols per slot, only the first DMRS symbol in each slot is used.

Estimation of effective 2x2 channel matrix is a well known procedure if reference signals use different subcarriers, e.g. in case of DMRS antenna ports 0 and 2. In case that same subcarriers are used, e.g. DMRS antenna ports 0 and 1, a channel decomposition is necessary taking advantage of the orthogonal codes wf and wt and assuming identical channel coefficients for adjacent subcarriers of same CDM group.

Effective channel including the precoding matrix P is:

H=HP= h0,0h0,1h1,0h1,1

with

hn,ν=ynrν*|rν|2

where y denotes the received symbol on port index n and r the reference signal for layer index ν.

Since reference signals of a specific layer are transmitted only on subcarriers of one CDM group channel, interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients are calculated as the inverse of the effective channel matrix, in general:

GZF=H-1H+=(HHH)-1HH

###### 5.2.3.1.1.3	Method 1 Maximum Ratio Combining

###### < end of changes >