3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2202038

# Electronic Meeting, 17-25 January 2022

Source: 	Skyworks Solutions, Inc.

Title: 	MSD for DC_20A-38A_n8A

Agenda Item:	5.15.1	EN-DC requirements without FR2 band	[DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

This proposes MSD test points for DC_20A-38A-n8A.

# Discussion

## 2UL DC_38_n8 Coexistence Analysis

Table 1 shows that B20 MSD may occur due to IMD3 due to dual uplink transmission of band 38 and band n8.

Table 1 Harmonic and IMD analysis for 2UL DC_38_n8

|  | FUL_x_low | FUL_x_high | FUL_y_low | FUL_y_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570.0 | 2620.0 | 880.0 | 915.0 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 4400 | 4575 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1740 | 1655 | 3450 | 3535 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4225 | 4360 | 860 | 740 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6155 | 4330 | 4450 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6795 | 6980 | 20 | 175 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3310 | 3480 | 6900 | 7070 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8775 | 5210 | 5365 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1090 | 900 | 9600 | 9365 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2395 | 2600 | 6100 | 5880 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6090 | 6280 | 11160 | 11395 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7780 | 7985 | 9470 | 9690 |

Observation 1: B20 MSD may occur due to IMD3 of dual uplink transmission of band 38 and band n8. Even though a B38 filter may not exhibit  rejection performance as well as that of a band 7 filter, we propose to adopt B20 MSD levels from previously agreed DC_7A-20A_n8A in Table 7.3B.2.3.5.2-1.

## 2UL DC_20_n8 Coexistence Analysis

Table 2 shows that B38 MSD may occur due to IMD3 due to dual uplink transmission of band 20 and band n8.

Table 2 Harmonic and IMD analysis for 2UL DC_20_n8

|  | FUL_x_low | FUL_x_high | FUL_y_low | FUL_y_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832.0 | 862.0 | 880.0 | 915.0 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

Observation 2: B38 MSD may occur due to IMD3 of dual uplink transmission of band 20 and band n8. We propose to adopt B38 MSD levels from previously agreed DC_7A-20A_n8A in Table 7.3B.2.3.5.2-1.

Based on observation 1 and 2, we make the following MSD proposal for DC_20-38_n8.

Proposal: Adopt the following MSD test points for DC_20-38_n8

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-38_n8A | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 17.4 | IMD3 |
|  | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |
| DC_20A-38_n8A | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 38 | 2610 | 5 | 25 | 2610 | 21.1 | IMD3 |

# Conclusions

Proposal: Adopt the following MSD test points for DC_20-38_n8

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-38_n8A | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 17.4 | IMD3 |
|  | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |
| DC_20A-38_n8A | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 38 | 2610 | 5 | 25 | 2610 | 21.1 | IMD3 |

# References

[1]