3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202173

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, Rogers

Title:	TP for TR 37.717-41-11: Including band combinations DC_2-5-7-66_n78

Agenda item:			5.17.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-41-11 to add DC_2A-5A-7A-66A_n78A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_2-5-7-66_n78

### 5.1.X.1	Configuration for DC

Table 5.1.X.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-7A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A DC_66A_n78A | CA_2A-5A-7A-66A | n78A |

### 5.1.X.2		∆TIB and ∆RIB values

For DC_2-5-7-66_n78, the TIB,c and RIB,c values are given in the tables below.

The values are based on the values for DC_2-7-12-66_n78, but where low band 12 is replaced with low band 5, as the filter loss of these two low bands are supposed to be similar.

,

Table 5.1.X.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n78 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.6 |

Table 5.1.X.2-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7-66_n78 | 2 | 0.2 |
|  | 5 | 0 |
|  | 7 | 0.2 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |

### 5.1.x.3	REFSENS requirements

MSD requirements are covered in lower order combinations.

---End of changes---

# Reference

[1] RP-213251, “Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1U (1DL/1UL)” Nokia