3GPP TSG-RAN WG4 Meeting # 101-e-Bis 	R4-2202203

# Electronic Meeting, January 17-25, 2022

Agenda item:			5.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [101-bis-e][103] NR_6 GHz_licensed

Document for:	Information

Introduction

This email thread discuss the band definition for 6GHz licensed band. The contributions are in agenda 5.3, which includes:

- Topic #1: General aspects

- Topic #2: System parameters

- Topic #3: UE RF requirements

- Topic #4: BS RF requirements

Due to the limited time for this meeting, moderator suggests not to discuss the contributions related to performance part, contributions for IAB specs (currently it is not included in the WID), and R4-2201454 which is maintenance CR for TR 38.921.

Topic #1: General aspects

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201330 | Ericsson | Observation: In the scope of the licensed 6Ghz band WI, RAN4 should only specify and introduce a new band covering the 6 425-7 125 MHz frequency range. |
| R4-2201503 | Huawei, HiSilicon | Proposal 1: It is proposed to include the 6425-7125 MHz band into the specification once the RF requirements are completed, without the need to wait the completion of the range 5925-7125 MHz. Proposal 2: It is proposed to discuss the work split at RAN4#101-e-bis |
| R4-2201987 | MediaTek (Chengdu) Inc. | It is proposed to send an LS to RCC to request more information in relation to the points raised - With which type of 5G NR system configuration does RCC consider that coexistence with fixed and other services would be facilitated? - The recommendation gives freedom for national administrations to restrict the use of frequency blocks, but it is unclear as to what kind of restrictions may be required or applicable. - Article 21 of the Radio Regulations seems to allow quite a broad range of Tx power, and RCC has indicated that these requirements are applicable for base stations and user equipment. This suggests that a very high UE power class could be applicable for 6GHz operation. |
| R4-2200436 | Apple | Proposal: We kindly request to discuss further presented co-existence issues for the upper 6GHz licensed band. |
| R4-2201231 | CATT | Proposal 1: RF requirements for 6GHz licensed band should be specified based on RCC regulatory requirements but the use of this band should not be limited to RCC countries. Such RF requirements can be applied to other countries or regions if they are willing to use. Proposal 2: No RF requirement for co-existence with FS, FSS and SOS within the same band is defined. The co-existence of IMT and other system within the same band can leave up to administrations or using site engineering solutions. |
| R4-2201545 | Skyworks Solutions Inc. | Observation 1: The RCC recommendation points at the 6425-7075 MHz range for coexistence, but it seem related to measurements over the ocean measurements while the 7075-7250 MHz range is more general. Observation 2: If coexistence with adjacent services above the 7125 MHz upper bound are discussed up to 7250 MHz it is not the cases for adjacent services below 6425 MHz. Observation 3: Although only RR 5.458 is cited in [3] based on article 5 of [4], there are other services and regulations applicable to the range: - Radio astronomy in 6650-6675.2 MHz (RR 5.149) - Standard frequency and time signal-satellite service Earth-to-space transmissions at 6427 MHz (RR 5.440) - Use of the bands 6 725-7 025 MHz (Earth-to-space) by fixed satellite service (RR 5.441) - There may to be an omission of the fact that 7100-7155 MHz and 7190-7235 MHz are also allocated to the space operation service (Earth-to-space) on a primary basis in the Russian Federation (RR 5.459). - It is important to better understand this, at least for the first range that is in-band for 6425-7125MHz Observation 4: To manage coexistence with other services based on [3]: - Use of reference blocks may be restricted. It is unclear if creation of guard bands is implied by using 10MHz extensions and also how this may be compatible with (“Administrations may restrict the use of frequency blocks, including within the 6425-6525 MHz and 7100-7125 MHz frequency bands, in order to ensure compatibility with stations in FS, FSS, SOS, SRS and EESS”). - It is unclear if the creation of guard bands is implied by using 10MHz extensions and also how this may be compatible with (“the necessary bandwidth for 5G-NR/IMT-2020 systems in this frequency band is not less than 100 MHz per operator”) unless non-contiguous CA is implied. Observation 5: In general, we observe that it would be useful that the coexistence with other services is better understood, including which mitigation techniques are within the scope of the RAN4 work or local regulations: - Use of guard bands to known incumbent services. - Use of NS and A-MPR, if required. Observation 6: 43 dBm TRP is applicable to BS with peak EIRP depending on geographical separation or separation angle to satellite services. Observation 7: Interfering services require additional study to further understand which ones of the NR UE receiver blocking and selectivity requirements are sufficient, if any. Observation 8: The general NR spurious emission requirement is adequate. |
| R4-2201855 | Qualcomm Incorporated | Proposal: Send an LS to RCC to clarify what (if any) additional spurious emission requirements are needed to ensure compatibility with adjacent services in the band. Further clarify whether emission requirements or other necessary restrictions will be forthcoming from individual administrations. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 – general aspects

Issue 1-1-1: Scope

The WI covers two frequency ranges, and the RAN4 work starts for one of them, i.e. 6425-7125 MHz, which is according to RCC Recommendation 1/21. The range 5925-7125 MHz is still on hold unless there are regulatory requirements are available for the frequency range.

- Proposals:

  - It is proposed to include a new band into the specification once the RF requirements covering the 6 425-7125 MHz frequency range are completed, without the need to wait the completion of the range 5925-7125 MHz.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 1-1-2: Work split

There are quite a few draft CRs submitted in this meeting. As usual it is proposed to take big CR approach for the WI.

- Proposals:

| TS No. | Remarks | Work split |
| --- | --- | --- |
| 38.101-1 | Core UE part | Huawei |
| 38.133 | Core UE part | Ericsson |
| 38.104 | Core BS part | ZTE |
| 36.104 | Core BS part | ZTE |
| 37.104 | Core BS part | Nokia |
| 37.105 | Core BS part | CATT |
| 37.145-1 | Perf. BS part | Huawei |
| 37.145-2 | Perf. BS part | Huawei |
| 38.141-1 | Perf. BS part | CATT |
| 38.141-2 | Perf. BS part | Ericsson |
| 36.141 | Perf. BS part | ZTE |
| 37.141 | Perf. BS part | Nokia |
| 38.174 | Core BS part | ZTE |
| 38.176-1 | Perf. BS part | CATT |
| 38.176-1 | Perf. BS part |  |

- Recommended WF

  - Discuss and approve the work split

Sub-topic 1-2 – Study on RCC Recommendation

Regarding the RCC Recommendation 1/21, some contributions have been submitted in this meeting to seek the further clarifications on the harmonized technical conditions. Meanwhile many companies believe the band definition work can be done and also some contributions were provide the study on BS and UE RF requirements according to RCC Recommendation 1/21. Hence it is suggested to have more discussion on the aspect,

Issue 1-2-1: Clarification on RCC Recommendation 1/21

- Proposals:

  - Proposal 1: To discuss further presented co-existence issues in R4-2200436

  - Proposal 2: Send an LS to RCC for more clarification

  - Proposal 3: No RF requirement for co-existence with FS, FSS and SOS within the same band is defined

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-2: Whether need to define additional UE RF requirements in 3GPP to ensure compatibility with adjacent services in the band according to RCC Recommendation 1/21?

- Proposals:

  - Option 1: yes

  - Option 2: no

- Recommended WF

  - TBA based on 1st round discussion

Companies views’ collection for 1st round

Open issues

Collection of comments:

To Sub-topic 1-1 – general aspects

| Company | Comments |
| --- | --- |
| Company A | Issue 1-1-1: Comment Issue 1-1-2: Comment |
| ZTE | Issue 1-1-1: Agree with proposal Issue 1-1-2: Fine with current work split, please also include the impacts on IAB spec and repeater spec due to coexistence issues. ZTE is also interested in both IAB spec (38.174, 38.176-1, 38.176-2 ) and repeater spec. |
| Huawei | Issue 1-1-1: Agree Issue 1-1-2: Repeater spec are not available yet in our understanding and hence can be considered later. |
| CMCC | Issue 1-1-1: we are OK with the proposal |
| CATT | Issue 1-1-1: Agree with the proposal Issue 1-1-2: CATT also contributed IAB spec CRs. |
| Ericsson | Issue 1-1-1: Agree with this proposal, the 6425-7125 band should be introduced as requested by RCC. Issue 1-1-2: Fine with the proposed work split, IAB specs should also be considered indeed. |
| Skyworks | Issue 1-1-1: We agree that given there are only RCC requirements available only the 6425-7125MHz range can be introduce once requirements are complete. |
| Apple | Issue 1-1-1: Since the RRC Recommendation mentions 6945-7125MHz range and the WI was resumed based on that document, the considered frequency range should be 6945-7125MHz. Issue 1-1-2: We are generally Ok with the proposal, but since 38.101-1 is the UE specification we would welcome seeing one of the UE OEM/chipset companies for this specification. |
| Xiaomi | Issue 1-1-1 agree the proposal |
| Qualcomm | Issue 1-1-1 We are ok to define requirements for 6425 – 7125 MHz first if the requirements can be completed. |

| Samsung | Issue 1-1-2 No comment to the work split. But since the IAB specifications are not in the WI as pointed by moderator, in addition there is no RAN-P meeting to update the WID, we believe the intention is to provide CRs in May meeting for maintenance of IAB with further review on missing operating bands (such as n96, n97, n98, n99, n100, n101, n102, 103 and n104 to see whether they are needed to be updated) for co-existence. WF can be considered under this agenda to capture the agreement on issue with related work split to avoid double effort on CR. |
| --- | --- |

| Broadcom | Issue 1-1-1: This WI should only consider the 6425 – 7125 MHz as it is based on the RCC recommendation only. |
| --- | --- |
| CHTTL | Issue 1-1-1: Agree with the proposal. |
| Huawei | Issue 1-1-2: To Apple: Huawei is also a UE and Chipset vendor. Of course other company who want to share the work load is welcome. |
| China Unicom | Issue 1-1-1: Agree with the proposal. |

To 1-2 – Study on RCC Recommendation

| Company | Comments |
| --- | --- |
| Company A | Issue 1-2-1: Comment Issue 1-2-2: Comment |
| ZTE | We need to discuss the RF requirement based on the LS information from RCC. In addition, based on RCC recommendation, the coexistence with fixed service, the fixed-satellite service (Earth-to-space, space-to-Earth) and the space operation service (space-to-Earth) has been mentioned and considered. |
| Huawei | Issue 1-2-1: Regarding co-existence issues mentioned in R4-2200436, protection of passive service in 7075-7250 MHz, co-existence with other radio systems and cross-border co-existence have been considered in the recommendation and the harmonized technical conditions also present in the Annex of RCC recommendation 1/21. For co-existence with unlicensed L6G, from the ECC decision, it is clearly stated that the usage of unlicensed L6G is based on non-protected basis. We think according to RCC recommendation 1/21, if there is no specific protection requirements then no need for 3GPP to define additional requirements, i.e. in these cases the co-existence is ensured by deployment measures. Regarding some companies’ question on recommendation 2) for the restriction of frequency blocks. It means no operation at all in certain blocks to ensure co-existence. Hence we support option 3 no additional RF requirements are needed to be defined in 3GPP. To MTK R4-2201987 on the type of 5G NR system configuration, one question for clarification, does it ask which type of Base stations are considered? Issue 1-2-2: Option 2, see comment above |
| CMCC | Issue 1-2-1: Clarification on RCC Recommendation 1/21 option 3 is preferred. According to RCC recommendation, the co-existence issue between other services are already been considered and no additional requirement is listed in the recommendation considering some engineering solution may be used. So there is no RF requirement for co-existence with other services in the same band. Issue 1-2-2: Whether need to define additional UE RF requirements in 3GPP to ensure compatibility with adjacent services in the band according to RCC Recommendation 1/21? Option 2. Since there is no additional UE RF requirement listed in RCC recommendation, it’s suggested no additional UE RF requirements. |
| CATT | Issue 1-2-1: We support proposal 3 as dicussed in our paper. Issue 1-2-2: Comment If there is no UE requirement in RCC recommendation, then we can leave this to each administration. |
| Ericsson | Issue 1-2-1; We don’t think we need to send a LS to RCC to complete the specification of the 6425-7125 MHz band. RCC has clearly indicated they have considered coexistence with the incumbent services (FS, FSS and SOS), the RCC Recommendation has taken those into account. There is no reason to challenge this. Option 3 then. Issue 1-2-2: We shall follow RCC Recommendation, not challenging their decision. |
| MediaTek | Issue 1-2-1: Companies here are stating that coexistence has been considered, however RCC seems to have left full flexibility to national administrations on how to fulfil that. Therefore we believe it would be good to try to get more detail on what sort of restrictions on the radio configuration they have in mind and in which domain. We hope this answers Huawei’s question. Also regarding Huawei response that: It means no operation at all in certain blocks to ensure co-existence, we appreciate your view on this, but this is not clear from the documentation provided. Therefore, we believe that an LS to RCC would help. Proposal 2 is preferred. Issue 1-2-2: We think it is premature to agree one way or another due to the lack of information provided about the UE requirements. |
| Skyworks | Issue 1-2-1: We need to discuss concerns raised in R4-2200436 but also the observations raised in our contribution R4-2201545 where we at least point to some aspects not described in RCC recommendations but also about how co-existence with existing services must be managed. Based on the discussion an LS for further clarifications may/may not be needed. Issue 1-2-2: At least we need to clarify if and how the RCC related 6425-7125MHz band may coexist with band n102 (5925-6425MHz) |
| Apple | Issue 1-2-1: We support discussing further presented co-existence issues and welcome sending an LS to RCC to clarify several open issues. @Huawei: The RCC Recommendation clearly mentions that local Administrations should ensure co-existence with other systems within the band and out-of-band. However, the RCC Recommendation does not mention whether it will be accomplished only by means of deployments. As for the co-existence with the lower 6G services, the EC Decision states that “When introducing new applications into the 5 945-6 425 MHz frequency band or into adjacent frequency bands after the entry into force of this Decision, Member States shall not adopt technical and operational conditions applicable to any new application that unduly restrict the continued use of WAS/RLAN in the 5 945-6 425 MHz frequency band in accordance with this Decision”. And since the RCC countries are also CEPT members, we fail to see how the RCC Recommendation addresses this point even for the lower 6GHz services in the RCC countries, not mentioning the cross-border scenarios. Maybe RCC can clarify how they accounted or will account for the CEPT decisions. Issue 1-2-2: Whether we need to define additional UE RF requirements is the secondary issue, which is dependent on the outcome of issue 1-2-1. |
| Qualcomm | Issue 1-2-1 Prefer proposal 2. According to our reading of the LS from RCC, coexistence is a fundamental aspect of enabling this spectrum. In the RCC Recommendation 1/21 document it states that administrations may restrict the use of frequency blocks, but doesn’t indicate how that restriction might be implemented. It states that compatibility should be ensured with FS, FSS, SOS, SRS, and EESS. Thus, it is a reasonable interpretation that individual administrations may impose emission requirements for example in which case the 3GPP specifications would not meet their needs if no such emission requirements are defined in 3GPP. Sending an LS to RCC seems to be the most straightforward way to resolve this ambiguity, though we are open to discussion of other alternatives. Issue 1-2-2 Depends on resolution of issue 1-2-1 |
| Broadcom | Similar position as Apple and Qualcomm on both Issue 1-2-1 and 1-2-2. |
| vivo | Issue 1-1-1: Support the proposal |
| CHTTL | Issue 1-2-1 Share the similar view as CMCC and option 3 is preferred. |
| Huawei | Issue 1-2-1 To MediaTek, If my understand your question correctly, according to recommendation 3), there is no restriction on base station classes in general. |
| China Unicom | Issue 1-2-1: Option 3. Issue 1-2-2: Option 2. No need to define additional UE RF requirements in 3GPP. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Scope | - Proposals: - It is proposed to include a new band into the specification once the RF requirements covering the 6 425-7125 MHz frequency range are completed, without the need to wait the completion of the range 5925-7125 MHz. Recommendations for 2nd round: Based on 1st round discussion, the proposal is agreeable and can be captured in WF. No further discussion is needed for 2nd round. |
| Issue 1-1-2: Work split | Recommendations for 2nd round: Based on 1st round discussion, companies are generally ok with work split. There are proposals to consider IAB specs. And Apple have a comment on 38.101-1. Moderator suggests to further check whether the following work split is agreeable for 2nd round. TS No.; Remarks; Leading company. 38.101-1; Core UE part; [Huawei]. 38.133; Core UE part; Ericsson. 38.104; Core BS part; ZTE. 36.104; Core BS part; ZTE. 37.104; Core BS part; Nokia. 37.105; Core BS part; CATT. 37.145-1; Perf. BS part; Huawei. 37.145-2; Perf. BS part; Huawei. 38.141-1; Perf. BS part; CATT. 38.141-2; Perf. BS part; Ericsson. 36.141; Perf. BS part; ZTE. 37.141; Perf. BS part; Nokia. 38.174; Core BS part; CATT. 38.176-1; Perf. BS part; Nokia. 38.176-2; Perf. BS part; Ericsson. |
| Issue 1-2-1: Clarification on RCC Recommendation 1/21 | - Proposals: - Proposal 1: To discuss further presented co-existence issues in R4-2200436 and R4-2201545) (Apple, Skyworks) - Proposal 2: Send an LS to RCC for more clarification (MediaTek, Apple, Qualcomm, Broadcom) - Proposal 3: No need for 3GPP to define additional UE RF requirements (ZTE, Huawei, CMCC, CATT, Ericsson, CHTTL, China Unicom ) Recommendations for 2nd round: Based on 1st round discussion, some companies propose to further discussion on the issues and send an LS to RCC for further clarification on the co-existence conditions. While others think the co-existence has been considered in RCC recommendation and shall not challenge their decision. It is proposed to be further discussed for 2nd round. |
| Issue 1-2-2: | Recommendations for 2nd round: Based on 1st round discussion, we can focus on issue 1-2-1 firstly and no further discussion for 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on general aspects for 6GHz licensed band | Huawei, HiSilicon |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #2: System parameters

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200149 | CATT | Discussion on system parameters for 6GHz licensed band |
| R4-2201304 | Xiaomi | Discussion on system parameters for 6G license band |
| R4-2201447 | ZTE Corporation | Discussion on system parameters for 6425-7125MHz |
| R4-2201504 | Huawei, HiSilicon | System parameters for 6GHz NR licensed band |
| R4-2201545 | Skyworks Solutions Inc. | 6GHz licensed band attributes based on RCC recommendations |
| R4-2201855 | Qualcomm Incorporated | Proposal: RAN4 to decide whether the defined 6 GHz licensed band should be specific to 6425 – 7125 MHz with possible NS, or a more general band 5925 – 7125 MHz with specific NS requirements for countries allowing only the subset. Proposal: Channel bandwidths for this band as 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |
| R4-2201824 | ZTE Corporation | draft CR on introduction of 6GHz system parameters for 38.101-1 |
| R4-2201825 | ZTE Corporation | draft CR on introduction of 6GHz system parameters for 38.104 |
| R4-2201330 | Ericsson | Based on the above information, we would propose to specify at least the following channel BWs for this new 6GHz licensed band: 60, 70, 80, 90 and 100 MHz. And other channel BWs are not precluded. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 – Band definition

Issue 2-1-1: Frequency range

Most contributions propose to define the new band covering the range according to the RCC Recommendation 1/21. At the same time R4-2201855 propose to discuss whether it could be defined as the entire range.

- Proposals:

  - Option 1: Define a new band covering the 6425-7125 MHz frequency range

  - Option 2: A more general band 5925 – 7125 MHz with specific NS requirements for countries allowing only the subset

- Recommended WF

  - Option 1 as this follows the WID scope.

Issue 2-1-2: Band number

The new band should defined as the next available band number, considering,

- n100: RMR 900 MHz.

- n101: RMR 1900MHz.

- n102: unlicensed 6GHz band for Europe.

- 103: Upper 700MHz band?

- Proposals:

  - Option 1: Defining a new band as n103

  - Option 2: Defining a new band as n104

- Recommended WF

  - Further check which one is the next available band number

Sub-topic 2-2 – Channel arrangment

Issue 2-2-1: channel bandwidth

- Proposals:

  - Option 1: 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 MHz

  - Option 2:  20, 30, 40, 50, 60, 70, 80, 90, 100 MHz

  - Option 3: 20,25,30,40,50, 60,70,80,90, 100MHz FFS for 35MHz and 45MHz

  - Option 4: 60, 70, 80, 90, 100 MHz, other channel BWs are not precluded

- Recommended WF

  - TBA based on 1st round discussion

Issue 2-2-2: Channel raster

- Proposals:

  - To define the applicable NR-ARFCN as the Table below.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nx | 15 | 828334 – <1> –875000 | 828334 – <1> –875000 |
|  | 30 | 828334 – <2> –875000 | 828334 – <2> –875000 |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 2-2-3: Synchronization raster

The sync raster entries is related to the assumption of minimum channel bandwidth and the choice of step size.

- Proposals: to define the applicable SS raster entries as,

  - Option 1:

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nx | 30 kHz | Case C | 9881 – <1> – 10360 |

  - Option 2:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nx | 30 kHz | Case C | 9884 – <4> – 10360 |

  - Option 3:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n104] | 15 kHz | Case A | 16068 – <1> – 17805 |
|  | 30 kHz | Case C | 16074 – <1> – 17799 |

- Recommended WF

  - TBA based on 1st round discussion

Companies views’ collection for 1st round

Open issues

Collection of comments:

To Sub-topic 2-1 – Band definition

| Company | Comments |
| --- | --- |
| Company A | Issue 2-1-1: Comment Issue 2-1-2: Comment |
| ZTE | Issue 2-1-1: Support the option 1 Issue 2-1-2: Both option 1 and option 2 is fine for us, if 103 has been reserved for LTE band, then we are fine to go with option 2. |
| Huawei | Issue 2-1-1: Option 1 to follow WID scope Issue 2-1-2: Option 2, it seems 103 is to be used for upper 700MHz band |
| Xiaomi | Issue 2-1-1: Support the option 1 Issue 2-1-2: we are fine for both options, if n103 was assigned to other band, n104 is OK |
| CMCC | Issue 2-1-1: Option 1 is preferred. Issue 2-1-2: Band number Recommended WF is OK |
| CATT | Issue 2-1-1: Support option 1. Issue 2-1-2: We are OK with both options. Further check which band number is available. |
| Ericsson | Issue 2-1-1: option 1, 3GPP has not received any information from any Regulatory Body about a full 5925 – 7125 MHz licensed band. Issue 2-1-2: option 2. To avoid any confusion, we recommend to use n104 number for the new licensed band (if 103 number is confirmed for the LTE upper 700MHz band). Band number n103 might be interpreted as the LTE refarmed band 103… |
| MediaTek | Issue 2-1-1: Option 1 is ok, with clarification in the spec (as agreed in RP#94-e) that this band only applies for RCC countries. |
| Skyworks | Issue 2-1-1: Option 1: only RCC related spectrum is in scope Issue 2-1-2: Can be decided when introduced in the spec but should be > n103. |
| Apple | Issue 2-1-1: The RCC Recommendation mentions 6425-7125MHz range and the 3GPP should proceed accordingly. And the new band applies only to the RCC countries. Issue 2-1-2: This is just a number, which is not relevant at this point. We can find a suitable number once the core part completion level is high enough, i.e. we cannot see much value in discussing which band number it can or shall be. |
| Qualcomm | Issue 2-1-1 We would prefer to avoid defining multiple bands and wanted to be a bit forward thinking in defining the band over the entire 1.2 GHz frequency range, recognizing that only the upper 700 MHz would have meaningful requirements at this time. But it seems that we are alone in that view, so we are ok to define the band only over the upper 700 MHz from 6425 – 7125 MHz Issue 2-1-2 We prefer Band n106 since there is already Band n96 for the 6 GHz frequency range. Otherwise, we can wait for the next number in line; there is no need to assign it now. |
| Broadcom | Issue 2-1-1: New band shall apply to RCC countries only and should only cover the 6425-7125MHz range as mentioned in the RCC Recommendation. Issue 2-1-2: This discussion can take place in a later stage. No need to assign a new number now. |
| vivo | Issue 2-1-1: Option 1 Issue 2-1-2: Can be decided later. |

To Sub-topic 2-2 –Channel arrangement

| Company | Comments |
| --- | --- |
| Company A | Issue 2-2-1: Comment Issue 2-2-2: Comment Issue 2-2-3: Comment |
| ZTE | Issue 2-2-1: Based on the outcome of study phase for ITU-R reply, the minimum channel bandwidth could be 20MHz. Issue 2-2-2: Fine with NR-ARFCN in the table. Issue 2-2-3: We need to agree on the minimum channel bandwidth firstly, then we could derive the sync raster easily. |
| Huawei | Issue 2-2-1: Our preference is to consider 20 MHz as minimum channel bandwidth, and We can further discussion on option 2 and 4 Issue 2-2-2: Comment agree Issue 2-2-3: Comment Our preference is to consider 20 MHz as minimum channel bandwidth, so we support option 2. |
| Xiaomi | Issue 2-2-1: prefer Option2 and 1, consider 20 or 10 MHz as minimum channel bandwidth. Issue 2-2-2: we are OK with the proposal Issue 2-2-3: Depends on issue 2-2-1. |
| CMCC | Issue 2-2-1: channel bandwidth Option 2 is preferred, according to RCC recommendation 1_21, it seems 20MHz is the minimum frequency allocation unit and increase with multiple of 10MHz. Issue 2-2-2: Channel raster Proposal is OK for us. Issue 2-2-3: Synchronization raster Could someone give more clarification about the reason of step size 4? What’s the internal relation between step size 4 and minimum 20MHz bandwidth? |
| CATT | Issue 2-2-1: We prefer option 2 based on the interpretation of RCC recommendation. Issue 2-2-2: Support the proposal. Issue 2-2-3: Decide the minimum bandwidth first. |
| Ericsson | Issue 2-2-1: According to RCC Recommendation, the minimum block size is 20MHz, and a block could be increase by multiples of 10MHz. Based on this information, options 1 and 3 are not correct.We have proposed option 4 but could also accept option 2. Issue 2-2-2: agree with the proposal Issue 2-2-3: the proposed GSCN range in option 3 is a mistake from our side. Option 1 should provide full flexibility with 20MHz channel B, option 1 is our preference. |
| MediaTek | Issue 2-2-1: Option 1 and 3 do not seem to align with RCC recommendation. Issue 2-2-2: As the practical channel raster is 5MHz, we wonder if we really need a 15kHz granularity of ARFCN. Further analysis seems warranted here. Issue 2-2-3: The appropriate GSCN raster needs further analysis in our view, and should consider the minimum channel BW, the 5MHz minimum channel raster granularity requested, and the SCS agreed for the SSB. Initially it seems that a much larger GSCN raster granularity than <1> or <4> could be possible. Current proposal of <1> would lead to roughly 486 raster entries per SSB. Minimizing unnecessary cell search is an important consideration from a UE perspective. |
| Nokia | Issue 2-2-1: Option 2 can be considered. If there is a need for additional channel BWs in the future, they can be added later. |
| Skyworks | Issue 2-2-1: we proposed option 2 but depending whether 10MHz extension can be covered by contiguous CA or not then 10MHz may be needed. In any case 35 and 45MHz do not comply with 10MHz extension and 5MHz raster in RCC recommendation. If contiguous UL CA is not needed (hard to interpret if the RCC recommendation of no less than 100MHz per operator will be followed and applies to contiguous and/or non contiguous spectrum. In our view since there may be restricted blocks anywhere due to coexistence with existing services, we may need more flexibility than 60, 70, 80, 90, 100 MHz. Issue 2-2-2: with the large band and the way blocks are defined by RCC, a 5MHz raster is sufficient. Should a larger step size be considered to reduce time to find channels? Issue 2-2-3: I tought this can only have 30Khz based case, again it might be useful to increase the step size. |
| Apple | Issue 2-2-1: The RCC Recommendation mentions 20MHz as the minimum block, so this can be considered as the starting point. Other channel bandwidths in multiples of 20MHz are not precluded, e.g. 40, 60, 80, 100MHz. It is not clear whether we would need e.g. 35, 45, 90MHz. Issue 2-2-2: Ok as a principle, but we need to wait for the RCC feedback on whether they will define/enforce the guard band between the lower 6GHz services and the upper 6GHz licensed deployments. In other words, at least the starting N_ref should be in square brackets. Issue 2-2-3: It is a bit premature to discuss this now. We should conclude first on the channel raster points. |
| Qualcomm | Issue 2-2-1 Although we proposed 10 MHz, this was an incorrect reading of the RCC recommendation. We are fine with Option 2 Issue 2-2-2 The RCC recommendation indicates that channel blocks are accessible with 5 MHz resolution. So there does not appear to be a need for a finer step size. Issue 2-2-3 Needs further discussion including the step size and SCS. |
| Broadcom | Issue 2-2-1: We support Option 2. Issue 2-2-2: A decision may require RCC feedback as previously indicated by Apple. |
| vivo | Issue 2-2-1: 20MHz is the minimum. We support Option 2. Issue 2-2-2: Agree with the proposal. |
| Huawei | Issue 2-2-2 To Skyworks and Qualcomm, The proposal is to reuse existing approach for FR1 and also FR2. It has no impact to UE searching time. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Frequency range | - Proposals: - Option 1: Define a new band covering the 6425-7125 MHz frequency range - Option 2: A more general band 5925 – 7125 MHz with specific NS requirements for countries allowing only the subset Recommendations for 2nd round: Based on 1st round discussion, Option 1 is agreeable and can be captured in WF. No further discussion is needed for 2nd round. |
| Issue 2-1-2: Band number | Recommendations for 2nd round: Based on 1st round discussion, most companies can go with n104, while a few companies comment that it can be decided later. It is suggested to use the next number in line and can be decided later. No further discussion is needed for 2nd round. |
| Issue 2-2-1: channel bandwidth | - Proposals: - Option 1: 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 MHz - Option 2: 20, 30, 40, 50, 60, 70, 80, 90, 100 MHz - Option 3: 20,25,30,40,50, 60,70,80,90, 100MHz FFS for 35MHz and 45MHz - Option 4: 60, 70, 80, 90, 100 MHz, other channel BWs are not precluded Recommendations for 2nd round: Based on 1st round discussion, most companies support to consider 20 MHz as minimum channel bandwidth. It is proposed to further check whether option 2 is agreeable for 2nd round. |
| Issue 2-2-2: Channel raster | Recommendations for 2nd round: Based on 1st round discussion, most companies support the proposal which follows existing approach for FR1 and FR2. 3 companies propose to consider 5MHz step and 2 companies think the need to wait for RCC feedback. It is proposed to further discuss for 2nd round. |
| Issue 2-2-3: Synchronization raster | Recommendations for 2nd round: Based on 1st round discussion, it will depend on the minimum channel bandwidth. It is proposed to further discuss the proposal including step size and SCS for 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on System parameters for 6GHz licensed band | ZTE |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #3: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200150 | CATT | Discussion on UE RF requirements for 6GHz licensed band |
| R4-2201305 | Xiaomi | Discussion on UE RF requirements for 6G license band |
| R4-2201332 | Ericsson | 6GHz licensed band - UE requirements |
| R4-2201448 | ZTE Corporation | Discussion on UE RF requirements for 6425-7125MHz |
| R4-2201505 | Huawei, HiSilicon, CMCC, China Unicom, OPPO | UE RF requirements |
| R4-2201506 | Huawei, HiSilicon,CMCC, China Telecom, China Unicom, OPPO | Draft CR for 38.101-1: 6GHz NR licensed band |
| R4-2201826 | ZTE Corporation | draft CR on introduction of 6GHz UE RF requirements for 38.101-1 |
| R4-2201855 | Qualcomm Incorporated | 6 GHz licensed band |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 – TX requirements

Issue 3-1-1: Maximum output power

- Proposals:

  - Option 1: Specify PC3 and PC2 power classes for this band, with the default power class as PC3. The tolerance is +2/-3 dB

  - Option 2: PC 3 with the tolerance +2/-3 dB

- Recommended WF

  - Discuss whether option 1 is acceptable since it also cover option 2.

Issue 3-1-2: MPR

MPR requirements are defined as band agnostic, and relative channel BW for this new band is less than 4%

- Proposals:

  - Existing FR1 MPR requirement can apply to 6425-7125MHz and no ∆MPR will be needed

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-3: Output power dynamics

- Proposals:

  - Requirements specified in TS 38.101-1 sub-clause 6.3 apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-4: Transmit signal quality

- Proposals:

  - Requirements specified in TS 38.101-1 sub-clause 6.4 apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-5: Occupied bandwidth

- Proposals:

  - Requirement specified in TS 38.101-1 sub-clause 6.5.1 shall also apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-6: Spectral emission mask

- Proposals:

  - To adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-7: ACLR

- Proposals:

  - According to the SI TR clause 7.1.3, it is proposed to adopt 26 dB ACLR for 6.425 - 7.125 GHz band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-8: Spurious emission-general

- Proposals:

  - Requirements specified in TS 38.101-1 sub-clause 6.5.3.1 apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Note: the additional spurious emission pending the discussion in Issue 1-2-1.

Issue 3-1-9: Transmit intermodulation

- Proposals:

  - Requirements specified in TS 38.101-1 sub-clause 6.5.4 apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-1-10: UL MIMO

- Proposals: UL MIMO should be supported for 6425-7125MHz

  - Option 1: yes

  - Option 2: no

- Recommended WF

  - TBA based on 1st round discussion

Sub-topic 3-2 – RX requirements

Issue 3-2-1: Reference sensitivity

According to the SI TR clause 7.2.1, a noise figure in the [9, 13] dB interval was agreed for reporting to ITU WP5D sharing studies. In the meeting companies provide evaluations and proposal for reference sensitivity.

- Proposals:

  - Option 1: it is proposed to define the same REFSENS as n78 and n79 for the new band

| nx | 15 | 20, 30, 40, 50 | [-92.5] + 10log10(NRB/106) | TDD |
| --- | --- | --- | --- | --- |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | [-92.8] + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | [-93.1] + 10log10(NRB/24) |  |

  - Option 2: Adopt 10.5 dB NF for 6GHz NR band.

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| nX | 15 | 20, 30, 40, 50 | [-92.2] + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | [-92.4] + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | [-92.6] + 10log10(NRB/24) |  |

Option 3: FFS

- Recommended WF

  - TBA based on 1st round discussion

Issue 3-2-2: Maximum input level

- Proposals:

  - Requirements specified in TS 38.101-1 sub-clause 6.3 apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-2-3: ACS

- Proposals:

  - According to the SI TR clause 7.2.4, it is proposed to adopt 32 dBc adjacent channel selectivity (ACS) for 6.425 - 7.125 GHz band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-2-4: blocking

- Proposals:

  - UE blocking is specified conventionally without the need to improve blocking beyond the standard requirements.

  - Requirement specified in TS 38.101-1 sub-clause 7.6 for frequency above than 3300 MHz shall also apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposals are agreeable

Issue 3-2-5: Intermodulation

- Proposals:

  - Requirement specified in TS 38.101-1 sub-clause 7.8 shall also apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 3-2-6: Spurious emissions

- Proposals:

  - Requirement specified in TS 38.101-1 sub-clause 7.9 shall also apply for the new 6GHz licensed band.

- Recommended WF

  - Discuss whether the proposal is agreeable

Companies views’ collection for 1st round

Open issues

Collection of comments:

To Sub-topic 3-1 – TX requirements

| Company | Comments |
| --- | --- |
| Nokia | Issue 3-1-1: Comment Issue 3-1-2: Proposal is agreeable Issue 3-1-3: Proposal is agreeable Issue 3-1-4: Proposal is agreeable Issue 3-1-5: Proposal is agreeable Issue 3-1-6: Proposal is agreeable Issue 3-1-7: Proposal is agreeable Issue 3-1-8: Proposal is agreeable Issue 3-1-9: Proposal is agreeable Issue 3-1-10: Option 1 |
| ZTE | Issue 3-1-1: Our original proposal is for PC3, if the majority of companies are interested in PC2, we are also fine with that. Issue 3-1-2: The proposal is fine for us. Issue 3-1-3: The proposal is fine for us. Issue 3-1-4: The proposal is fine for us. Issue 3-1-5: The proposal is fine for us. Issue 3-1-6: In general, we are fine with that, however this should also be discussed together with channel bandwidth. Issue 3-1-7: The proposal is fine for us. Issue 3-1-8: The proposal is also fine for us. The spurious emission in TR 38.921 and TS 38.101-1 should be same. Issue 3-1-9: The proposal is fine for us. Issue 3-1-10: UL MIMO is necessary for this high band to improve the experienced throughput. |
| Huawei | Issue 3-1-1: Option 1 which is the same as C band. Issue 3-1-2: The proposal is agreeable Issue 3-1-3: The proposal is agreeable Issue 3-1-4: The Proposal is agreeable Issue 3-1-5: The proposal is agreeable Issue 3-1-6: The proposal is agreeable Issue 3-1-7: The proposal is agreeable Issue 3-1-8: The proposal is agreeable Issue 3-1-9: The proposal is agreeable Issue 3-1-10: Ok to Option 1 |
| Xiaomi | Issue 3-1-1: Prefer Option 2, PC2 can be requested by the Operator. Issue 3-1-2: we are OK with the proposal Issue 3-1-3: we are OK with the proposal Issue 3-1-4: we are OK with the proposal Issue 3-1-5: we are OK with the proposal Issue 3-1-6: we are OK with the proposal Issue 3-1-7: we are OK with the proposal Issue 3-1-8: we are OK with the proposal Issue 3-1-9: we are OK with the proposal Issue 3-1-10: Option 1 is OK |
| CATT | Issue 3-1-1: OK with option 1. Issue 3-1-2: OK with the proposal. Issue 3-1-3: OK with the proposal. Issue 3-1-4: OK with the proposal. Issue 3-1-5: OK with the proposal. Issue 3-1-6: OK with the proposal. Issue 3-1-7: OK with the proposal. Issue 3-1-8: OK with the proposal. Issue 3-1-9: OK with the proposal. Issue 3-1-10: OK with option 1. |
| Ericsson | Issue 3-1-1: Option 2, as we don’t have much time and RCC doesn’t mention any specific need here. Issue 3-1-2: Agree with the proposal Issue 3-1-3: Agree with the proposal Issue 3-1-4: Agree with the proposal Issue 3-1-5: Agree with the proposal Issue 3-1-6: Agree with the proposal, this has been studied in the 6-10GHz SI. Issue 3-1-7: Agree with the proposal, this has been studied in the 6-10GHz SI. Issue 3-1-8: Agree with the proposal. Issue 3-1-9: Agree with the proposal. Issue 3-1-10: Option 1 |
| MediaTek | In line with our previous comment, for Tx output power and other emissions requirements we would best to confirm once we have consulted further with RCC. |
| Skyworks | Issue 3-1-1: option 1 is OK for us but it should be further clarified if both PC3 and PC2 are based on 1Tx or 2TX or both. Issue 3-1-2: It is not possible to agree to reuse existing MPR if SEM/ACLR is changed. ACLR and SEM needs to be agreed first. We have to be consistent. Issue 3-1-6: This requires agreement and the if agreed an MPR study is needed Issue 3-1-7: This requires agreement and if agreed an MPR study is needed, in any case NRU uses 27dB for PC5 and we need here ACLR for PC3 and PC2, 26dBm looks strange and MPR may still be limited by new SEM if agreed. Issue 3-1-10: if we are not against UL MIMO (and TxD?) support, MPR needs to be re-assessed for new ACLR/SEM if any but also 10dB antenna isolation may need discussion. I note here that n46 and n96 are bands noted for UL MIMO but there is no associated requirement or work. |
| Apple | Issue 3-1-1 Maximum output power That should be clarified by RCC which maximum output power the local Administrations assume and/or are willing to see. The assumption of the maximum output power can depend on other assumptions and requirements for co-existence with incumbents within the band and outside the band. Issue 3-1-2 to 3-1-10 Tx requirements: Most proposals look reasonable, but it is a bit early to make these decisions because we need to understand first the maximum output power and related co-existence requirements. As explained by Skyworks, most parameters are dependent on each other. Issue 3-1-10: Ok as a principle to have UL MIMO with the understanding that it is an optional feature. |
| Qualcomm | Issue 3-1-1: Assuming no further contradicting information from the regulators, we support option 1. Issue 3-1-2: We suggest a bit more study may be beneficial. The transceiver and PA characteristic up to 7 GHz need to be checked as well as different ACLR, SEM, and spurious emissions being proposed. Issue 3-1-6: Need to check Issue 3-1-7: If MPR is preserved (issue 3-1-2), then what is the benefit of relaxing the ACLR? This may need a bit more consideration. Issue 3-1-10: UL MIMO can be added later if needed. For now, the focus should be on single Tx uplink. |
| vivo | Issue 3-1-1 Maximum output power Prefer Option 2. |
| Huawei | Issue 3-1-2: Existing MPR is defined as band agnostic. The ACLR and SEM agreed in the SI in TR 38.921 is relaxed from existing FR1 requirement, and we think there is no need to tighten the MPR for high frequency, which will have the benefit to support high power UE. |

To Sub-topic 3-2 – RX requirements

| Company | Comments |
| --- | --- |
| Nokia | Issue 3-2-1: Option 1 Issue 3-2-2: Proposal is agreeable Issue 3-2-3: Proposal is agreeable Issue 3-2-4: Proposals are agreeable Issue 3-2-5: Proposal is agreeable Issue 3-2-6: Proposal is agreeable |
| ZTE | Issue 3-2-1: Option 1 as baseline, whether this could be further retuning which could be further discussed. Issue 3-2-2: Proposal is fine for us. Issue 3-2-3: The proposal is fine for us to follow the SI agreement. Issue 3-2-4: The proposal is fine for us. Issue 3-2-5: The proposal is fine for us. Issue 3-2-6: The proposal is fine for us. |
| Huawei | Issue 3-2-1: Option 1 Issue 3-2-2: The proposal is agreeable Issue 3-2-3: The proposal is agreeable Issue 3-2-4: The proposals are agreeable Issue 3-2-5: The proposal is agreeable Issue 3-2-6: The proposal is agreeable |
| Xiaomi | Issue 3-2-1: Option 2, the value need modify according to the discussion of issue 2-2-1: channel bandwidth Issue 3-2-2: Proposal is agreeable Issue 3-2-3: Proposal is agreeable Issue 3-2-4: Proposals are agreeable Issue 3-2-5: Proposal is agreeable Issue 3-2-6: Proposal is agreeable |
| CMCC | Issue 3-2-1: Reference sensitivity Option 1 is preferred Issue 3-2-2: Maximum input level Proposal is agreeable Issue 3-2-3: ACS Proposal is agreeable Issue 3-2-4: blocking Proposal is agreeable Issue 3-2-5: Intermodulation Proposal is agreeable Issue 3-2-6: Spurious emissions Proposal is agreeable |
| CATT | Issue 3-2-1: Option 1 Issue 3-2-2: OK with the proposal. Issue 3-2-3: OK with the proposal. Issue 3-2-4: OK with the proposal. Issue 3-2-5: OK with the proposal. Issue 3-2-6: OK with the proposal. |
| Ericsson | Issue 3-2-1: The key is to agree on the UE NF for 6GHz, which shall be in [9, 13]dB range. Then, we could easily conclude on REFSENS. Options 1 or 2 are agreeable. Issue 3-2-2: Agree with the proposal Issue 3-2-3: Agree with the proposal, this has been studied in the 6-10GHz SI. Issue 3-2-4: Agree with the proposal, this has been studied in the 6-10GHz SI. Issue 3-2-5: Agree with the proposal Issue 3-2-6: Agree with the proposal. |
| MediaTek | Issue 3-2-1: More analysis is required on Rx sensitivity values, like with any new band. In line with our previous comment, we also feel that blocking requirements would be important to confirm once we have consulted further with RCC. |
| Skyworks | Issue 3-2-1: for n79 is not applicable another 2GHz above, n96 REFSENS for 20MHz and 15kHz is -89.2dBm and even if we agree that it is normal to discuss a different value for this licensed band, it can’t just be 3dB better either. So for us it is FFS especially unless we have a clear view on Tx/RX filtering needed, we cannot agree on an NF value. Issue 3-2-3: it is unclear to us how the ACLR/ACS is decided if we do not agree on ACLR for PC3 and PC2 first. |
| Apple | Issue 3-2-1 to 3-2-6 Rx requirements: Too early to conclude. Somewhat similar to the band n102 discussion, there will be UEs supporting band n96 in the full frequency range of 5925-7125MHz, so a deeper analysis is needed for the Rx requirements with an assumption that the same HW components will be used support band n96 and a new upper band. |
| Qualcomm | Issue 3-2-1: Option 3. Noise figure and refsens were extensively discussed during the work on Band n96 (overlapping frequency range with this band). We are not necessarily suggesting that the same refsens as Band n96 should apply here, but we’re not yet convinced on option1 or option 2 either. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Maximum output power | - Proposals: - Option 1: Specify PC3 and PC2 power classes for this band, with the default power class as PC3. The tolerance is +2/-3 dB (ZTE, Huawei, CATT, Skyworks, Qualcomm, - Option 2: PC 3 with the tolerance +2/-3 dB (ZTE, Xiaomi, Ericsson, Vivo) Recommendations for 2nd round: At 1st round discussion, most companies show preference on the two options, and 2 companies think it need to further confirm with RCC. It is proposed to further discussion for 2nd round. |
| Issue 3-1-2: MPR | - Proposals: - Existing FR1 MPR requirement can apply to 6425-7125MHz and no ∆MPR will be needed (Nokia, Xiaomi, CATT, ZTE, Huwei, Ericsson) Recommendations for 2nd round: Based on 1st round discussion, some companies support the proposal. While two companies require more study together with ACLR and SEM, 2 companies think it related to RCC Rec discussion. It is proposed to further discussion for 2nd round. |
| Issue 3-1-3 to issue 3-1-5 | Recommendations for 2nd round: Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further check whether the proposals can be agreed for 2nd round. |
| Issue 3-1-6: Spectral emission mask | - Proposals: - To adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz. (Nokia, Xiaomi, CATT, ZTE, Huwei, Ericsson) Recommendations for 2nd round: Based on 1st round discussion, some companies support the proposal. While a few companies require further check. It is proposed to further discussion for 2nd round. |
| Issue 3-1-7: ACLR | - Proposals: - According to the SI TR clause 7.1.3, it is proposed to adopt 26 dB ACLR for 6.425 - 7.125 GHz band. (Nokia, Xiaomi, CATT, ZTE, Huwei, Ericsson) Recommendations for 2nd round: Based on 1st round discussion, some companies support the proposal. While a few companies require further check. It is proposed to further discussion for 2nd round. |
| Issue 3-1-8 and Issue 3-1-9 | Recommendations for 2nd round: Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further check whether the proposals can be agreed for 2nd round. |
| Issue 3-1-10: UL MIMO | - Proposals: UL MIMO should be supported for 6425-7125MHz - Option 1: yes (Nokia, Xiaomi, CATT, ZTE, Huwei, Ericsson, Apple) - Option 2: focus on single TX uplink firstly (Qualcomm) Based on 1st round discussion, most companies are ok to support UL MIMO. It is proposed to further discussion for 2nd round. |
| Issue 3-2-1: Reference sensitivity | - Proposals: - Option 1: it is proposed to define the same REFSENS as n78 and n79 for the new band (Nokia, ZTE, Huawei, CMCC, CATT) - Option 2: Adopt 10.5 dB NF for 6GHz NR band. (Xiaomi) - Option 3: FFS (MediaTek, Skyworks, Apple, Qualcomm) Recommendations for 2nd round: More study/discussion is needed, It is proposed to further discussion on the WF for 2nd round. |
| Issue 3-2-2: Maximum input level | - Proposals: - Requirements specified in TS 38.101-1 sub-clause 6.3 apply for the new 6GHz licensed band. (Nokia, ZTE, Huawei, Xiaomi, Ericsson, CMCC, CATT) Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further check whether the proposals can be agreed for 2nd round. |
| Issue 3-2-3: ACS | - Proposals: - According to the SI TR clause 7.2.4, it is proposed to adopt 32 dBc adjacent channel selectivity (ACS) for 6.425 - 7.125 GHz band. (Nokia, ZTE, Huawei, Xiaomi, Ericsson, CMCC, CATT) Based on 1st round discussion, most companies are ok with the proposals. It is proposed to further check whether the proposals can be agreed for 2nd round. |
| Issue 3-2-4: blocking | - Proposals: - UE blocking is specified conventionally without the need to improve blocking beyond the standard requirements. - Requirement specified in TS 38.101-1 sub-clause 7.6 for frequency above than 3300 MHz shall also apply for the new 6GHz licensed band. Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further discuss the proposals for 2nd round. |
| Issue 3-2-5: Intermodulation | - Proposals: - Requirement specified in TS 38.101-1 sub-clause 7.8 shall also apply for the new 6GHz licensed band. Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further check whether the proposal can be agreed for 2nd round. |
| Issue 3-2-6: Spurious emissions | - Proposals: - Requirement specified in TS 38.101-1 sub-clause 7.9 shall also apply for the new 6GHz licensed band. Based on 1st round discussion, companies who made comments are ok with the proposals. It is proposed to further check whether the proposal can be agreed for 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on UE RF requirements for 6GHz licensed band | Ericsson |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #4: BS RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201331 | Ericsson | 6GHz licensed band - BS requirements |
| R4-2200153 | CATT | Analysis on BS requirements for operation in 6GHz band |
| R4-2200154 | CATT | draft CR on introduction of 6GHz licensed band for 38.104 |
| R4-2200155 | CATT | draft CR on introduction of 6GHz licensed band for 38.141-1 |
| R4-2200156 | CATT | draft CR on introduction of 6GHz licensed band for 38.141-2 |
| R4-2200157 | CATT | draft CR on introduction of 6GHz licensed band for 37.104 |
| R4-2200158 | CATT | draft CR on introduction of 6GHz licensed band for 37.105 |
| R4-2200159 | CATT | draft CR on introduction of 6GHz licensed band for 37.141 |
| R4-2200160 | CATT | draft CR on introduction of 6GHz licensed band for 37.145-1 |
| R4-2200161 | CATT | draft CR on introduction of 6GHz licensed band for 37.141-2 |
| R4-2200480 | CATT | draft CR on introduction of 6GHz licensed band for 38.174 |
| R4-2200481 | CATT | draft CR on introduction of 6GHz licensed band for 38.176-1 |
| R4-2200482 | CATT | draft CR on introduction of 6GHz licensed band for 38.176-2 |
| R4-2201449 | ZTE Corporation | Discussion on BS RF requirements for 6425-7125MHz |
| R4-2201450 | ZTE Corporation | draft CR to TS38.104 the introduction of 6425-7125MHz |
| R4-2201451 | ZTE Corporation | draft CR to TS36.104 the introduction of coexistence requirements of licensed band 6425-7125MHz |
| R4-2201452 | ZTE Corporation | draft CR to TS36.141 the introduction of coexistence requirements of licensed band 6425-7125MHz |
| R4-2201507 | Huawei, HiSilicon, China Unicom, CMCC | BS RF requirements |
| R4-2201508 | Huawei, HiSilicon | Draft CR for 38.104: 6GHz NR licensed band |
| R4-2201827 | ZTE Corporation | draft CR to TS 38.174 introduction of 6GHz coexistence requirement in IAB spec |
| R4-2201828 | ZTE Corporation | draft CR to TS 38.176-1 on introduction of coexistence requirement for 6GHz |
| R4-2201829 | ZTE Corporation | draft CR to TS 38.176-1 on introduction of coexistence requirement for 6GHz |
| R4-2201453 | ZTE Corporation | Discussion on MR/LA BS UEM requirements for 6425-7125MHz and 10-10.5GHz |
| R4-2201454 | ZTE Corporation | draft CR to TR38.921 MR and LA BS requirements for 6425-7125MHz and 10-10.5GHz |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1 – 38.104 RF requirements

Issue 4-1-1: BS type

- Proposals:

  - Option 1: Specify BS type 1-H and type 1-O for the 6GHz licensed band

  - Option 2: Specify BS type 1-C, BS type 1-H and type 1-O for the 6GHz licensed band

- Recommended WF

  - TBA based on 1st round discussion.

Issue 4-1-2: Conducted Tx requirements

Several contributions with similar proposals are submitted for the topic.

- Proposals: WF for conducted Tx requirements is proposed as in below table

| Requirement | WF |
| --- | --- |
| BS output power | Requirement specified in TS 38.104 sub-clause 6.2.3 shall also apply for the new 6GHz licensed band. |
| RE power control dynamic range | Requirement specified in TS 38.104 sub-clause 6.3.2 shall also apply for the new 6GHz licensed band. |
| Total power dynamic range | Requirement specified in TS 38.104 sub-clause 6.3.3 shall also apply for the new 6GHz licensed band. |
| Transmit OFF power | Requirement specified in TS 38.104 sub-clause 6.4.1 shall also apply for the new 6GHz licensed band. |
| Transient period | Requirement specified in TS 38.104 sub-clause 6.4.2 shall also apply for the new 6GHz licensed band. |
| Frequency error | Requirement specified in TS 38.104 sub-clause 6.5.1 shall also apply for the new 6GHz licensed band. |
| Modulation quality | Requirement specified in TS 38.104 sub-clause 6.5.2 shall also apply for the new 6GHz licensed band. |
| Time alignment error | Requirement specified in TS 38.104 sub-clause 6.5.3 shall also apply for the new 6GHz licensed band. |
| Occupied bandwidth | Requirement specified in TS 38.104 sub-clause 6.6.2 shall also apply for the new 6GHz licensed band. |
| ACLR | Based on the coexistence study, an ACLR value of 38dB shall apply for the band. See TR 38.921 sub-clause 6.1.3 |
| OBUE | For Wide BS, the OBUE mask for 6425-7125MHz in TR 38.921 sub-clause 6.1.2 shall apply. For Medium range and Local area BS, See Issue 4-1-3 For ΔfOBUE requirement, see Issue 4-1-7 |
| Transmit spurious | The requirement in sub-clause 6.1.4 of TR 38.921 shall apply for licensed band 6425-7125MHz. |
| Transmitter intermodulation | Requirement specified in TS 38.104 sub-clause 6.7.3 shall also apply for the new 6GHz licensed band. |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 4-1-3: OBUE for Medium range and Local area BS

It seems R4-2200153 and R4-2201453 made the same mask proposal.

- Proposals:

- Table 1a. MR BS UEM limit values for 6425-7125MHz, 31< Prated,x  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | Prated,x-53dB-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | Prated,x - 60dB | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

- Table 1b. MR BS UEM limit values for 6425-7125MHz, Prated,x  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -22dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -29 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

- Table 2. LA BS UEM limit values for 6425-7125MHz,

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -30dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -37 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -37 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 4-1-4: Conducted Rx requirements

Several contributions with similar proposals are submitted for the topic.

- Proposals: WF for conducted Rx requirements is proposed as in below table

| Requirement | WF |
| --- | --- |
| Reference sensitivity | Reference sensitivity requirements in TS 38.104 sub-clause 7.2.2 shall be updated considering the agreed Noise Figure (6 dB for Wide Area BS, 11 dB for Medium Range BS and 14 dB for Local Area BS). |
| Dynamic range | Dynamic range requirements in TS 38.104 sub-clause 7.3.2 shall be updated considering this agreed Noise Figure. |
| ACS | Based on TR 38.921 sub-clause 6.2.4, an ACS value of 42dB shall be specified. |
| In-band blocking | The in-band blocking has been agreed in TR 38.921 sub-clause 6.2.3 shall be specified |
| Out-of-band blocking | Requirement specified in TS 38.104 sub-clause 7.5.2 shall also apply for the new 6GHz licensed band. For ΔfOOB requirement, see Issue 4-1-8A new NOTE proposed in R4-2201508 is to be discussed |
| Receiver spurious | Requirement specified in TS 38.104 sub-clause 7.6.4 shall also apply for the new 6GHz licensed band. |
| Receiver intermodulation | Requirement specified in TS 38.104 sub-clause 7.7.2 shall also apply for the new 6GHz licensed band. |
| In-channel selectivity | Requirement specified in TS 38.104 sub-clause 7.8.2 shall also apply for the new 6GHz licensed band. |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 4-1-5: OTA Tx requirements

- Proposals: WF for OTA Tx requirements is proposed as in below table

| Requirement | Suggested limits |
| --- | --- |
| Radiated transmit power | Requirement specified in TS 38.104 sub-clause 9.2.2 shall also apply for the new 6GHz licensed band. |
| OTA BS output power | Requirement specified in TS 38.104 sub-clause 9.3.2 shall also apply for the new 6GHz licensed band. |
| OTA RE power control dynamic range | Requirement specified in TS 38.104 sub-clause 9.4.2.2 shall also apply for the new 6GHz licensed band. |
| OTA total power dynamic range | Requirement specified in TS 38.104 sub-clause 9.4.3.2 shall also apply for the new 6GHz licensed band. |
| OTA transmit OFF power | Requirement specified in TS 38.104 sub-clause 9.5.2.2 shall also apply for the new 6GHz licensed band. |
| OTA transient period | Requirement specified in TS 38.104 sub-clause 9.5.3.2 shall also apply for the new 6GHz licensed band. |
| OTA frequency error | Requirement specified in TS 38.104 sub-clause 9.6.1.2 shall also apply for the new 6GHz licensed band. |
| OTA modulation quality | Requirement specified in TS 38.104 sub-clause 9.6.2.2 shall also apply for the new 6GHz licensed band. |
| OTA time alignment error | Requirement specified in TS 38.104 sub-clause 9.6.3.2 shall also apply for the new 6GHz licensed band. |
| OTA occupied bandwidth | Requirement specified in TS 38.104 sub-clause 9.7.2.2 shall also apply for the new 6GHz licensed band. |
| OTA ACLR | The requirements are referring to conducted requirement. No change is needed. |
| OTA OBUE | The requirements are referring to conducted requirement. No change is needed. |
| OTA transmit spurious | The requirements are referring to conducted requirement. No change is needed. |
| OTA transmitter intermodulation | Requirement specified in TS 38.104 sub-clause 9.8.2 shall also apply for the new 6GHz licensed band. |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 4-1-6: OTA Rx requirements

- Proposals: WF for OTA Rx requirements is proposed as in below table

| Requirement | Suggested limits |
| --- | --- |
| OTA sensitivity | Requirement specified in TS 38.104 clause 10.2.1 shall also apply for the new 6GHz licensed band. |
| OTA reference sensitivity | The requirements are referring to conducted requirement. No change is needed. |
| OTA dynamic range | The requirements are referring to conducted requirement. No change is needed. |
| OTA ACS | The requirements are referring to conducted requirement. No change is needed. |
| OTA in-band blocking | The requirements are referring to conducted requirement. No change is needed. |
| OTA out of band blocking | Requirement specified in TS 38.104 sub-clause 10.6.2 shall also apply for the new 6GHz licensed band. |
| OTA receiver spurious | Requirement specified in TS 38.104 sub-clause 10.7.2 shall also apply for the new 6GHz licensed band. |
| OTA receiver intermodulation | Requirement specified in TS 38.104 sub-clause 10.8.2 shall also apply for the new 6GHz licensed band. |
| OTA in-channel selectivity | This requirement is not depending on the considered frequency range. |

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 4-1-7: ΔfOBUE

- Proposals:

  - Option 1: defineΔfOBUE = 100 MHz for the 6GHz licensed band

  - Option 2: defineΔfOBUE = 100 MHz for BS type 1-H and type 1-O, and  ΔfOBUE = 40 MHz for BS type 1-C

- Recommended WF

  - TBA based on 1st round discussion

Issue 4-1-8: ΔfOOB

- Proposals:

  - Option 1: define ΔfOOB = 100 MHz for the 6GHz licensed band

  - Option 2: define ΔfOOB = 100 MHz for BS type 1-H and type 1-O, and  ΔfOOB = 60 MHz for BS type 1-C

- Recommended WF

  - TBA based on 1st round discussion

Sub-topic 4-2 – other core RF requirements

Issue 4-2-1: TS 37.104

- Proposals: draft CR in R4-2200157

- Recommended WF

  - Comments collection on the draft CR

Issue 4-2-2: TS 37.105

- Proposals: draft CR in R4-2200158

- Recommended WF

  - Comments collection on the draft CR

Issue 4-2-3: TS 36.104

- Proposals: draft CR in R4-2201451

- Recommended WF

  - Comments collection on the draft CR

Companies views’ collection for 1st round

Open issues

Collection of comments:

To Sub-topic 4-1 – 38.104 RF requirements

| Company | Comments |
| --- | --- |
| Company A | Issue 4-1-1: Comment Issue 4-1-2: Comment Issue 4-1-3: Comment Issue 4-1-4: Comment Issue 4-1-5: Comment Issue 4-1-6: Comment Issue 4-1-7: Comment Issue 4-1-8: Comment |
| ZTE | Issue 4-1-1: For BS class, currently we don’t see the necessity to preclude BS type 1-C, in addition, there is no harm to define it in spec since BS type 1-H requirement is also defined based on basic limit of BS type 1-C. Issue 4-1-2: Fine with proposals from Ericssson which is also aligned with ours. Issue 4-1-3: We support the proposals in Issue 4-1-3 to define MR and LA BS requirements; Issue 4-1-4: Narrow band related blocking and intermodulation requirement is not applicable for licensed 6GHz band; it should be clarified firstly. In channel selectivity requirement is also related with noise figure, therefore we think that this requirement should also been updated for licensed 6GHz similar as refersense and dynamic range requirements; Issue 4-1-5: Similar comments as conducted part, ACLR, OBUE and Fobue requirement should be updated for licensed 6GHz band; Issue 4-1-6: Similar comments as conducted Rx requirements |
| Nokia | Issue 4-1-1: Support option 2; see no need to exclude BS type 1-C which is applicable for n96. Issue 4-1-2: Ok with proposals in general, but OBUE and transmit spurious need to be agreed together with ΔfOBUE. Issue 4-1-3: Ok with proposals in general, but they need to be agreed together with ΔfOBUE. Issue 4-1-4: Narrow band blocking and intermodulation requirements are not applicable; wanted and interfering signal levels for ICS should be updated with 1dB higher NFs. Issue 4-1-5: Same comment as on issue 4-1-2. Issue 4-1-6: Same comment as on issue 4-1-4. Issue 4-1-7: Propose to define ΔfOBUE similar to band n102 at least for MR and LA BS, i.e., 40 MHz for BS type 1-C and type 1-H (ref. R4-2201082). Issue 4-1-8: Propose to define ΔfOOB similar to band n102 at least for MR and LA BS, i.e., 60 MHz for BS type 1-C and type 1-H (ref. R4-2201082). |
| Huawei | Issue 4-1-1: Option2, at least for LA and MR BS, type 1-C will be needed. Issue 4-1-2: The proposal is agreeable Issue 4-1-3: The proposal is agreeable Issue 4-1-4: Agree with ZTE and Nokia on narrow blocking, intermodulation and ICS requirements. Issue 4-1-5: The proposal is agreeable Issue 4-1-6: Same comment as issue 4-1-4. Issue 4-1-7: Agree to define ΔfOBUE = 100 MHz for BS type 1-H and type 1-O, open to discuss ΔfOBUE for BS type 1-C Issue 4-1-8: Agree to define ΔfOOB = 100 MHz for BS type 1-H and type 1-O, open to discuss ΔfOBUE for BS type 1-C |
| CATT | Issue 4-1-1: Option 2 Issue 4-1-2: The proposal is agreeable. Issue 4-1-3: The proposal is agreeable; Issue 4-1-4: Agree that Narrow band related blocking and intermodulation requirement is not applicable for licensed 6GHz band; in channel selectivity needs to be updated. Issue 4-1-5: Similar comments as conducted part, ACLR, OBUE and FOBUE requirement should be updated for licensed 6GHz band; Issue 4-1-6: Similar comments as conducted Rx requirements Issue 4-1-7: Fine with ΔfOBUE = 100 MHz for BS type 1-H and type 1-O. FFS for BS type 1-C. Issue 4-2-1/-2/-3: can be discussed further after work split. |
| Ericsson | Issue 4-1-1: Option 1, there should not be any BS type 1-C for this frequency range. Issue 4-1-2: Agree with the proposal, this has already been studied in the 6-10GHz SI. Issue 4-1-3: Agree with the proposal Issue 4-1-4: Agree with the proposal, this has already been studied in the 6-10GHz SI. Issue 4-1-5: Agree with the proposal. Issue 4-1-6: Agree with the proposal. Issue 4-1-7: Option 1, we should not need to specify 1-C. Issue 4-1-8: Option 1, we should not need to specify 1-C. |
| Skyworks | We do not intend to discuss BS requirements but we need to make sure that UE and BS requirements are consistent and captured simultaneously. Especially system requirements. in our view none of the R4-2200157, R4-2200158, R4-2201451, R4-2200154, R4-2201450, R4-221508 can be approved without requirements agreed for both the UE and BS. |
| Apple | Same view as Skyworks. Since most of the system parameters should be consistent for both sides, some of the BS related agreements should be aligned with the corresponding agreements for the UE side. |

To 4.2.2	Sub-topic 4-2 – other core RF requirements

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200157 | ZTE: the band number might be updated, other part seem okay for us |
|  | Nokia: focus on CR for TS 38.104 first, CR leading company may be different. |
|  | Huawei: we are ok to focus on TS 38.104 firstly in this meeting |
| R4-2200158 | ZTE: similar comments as previous one. |
|  | Nokia: focus on CR for TS 38.104 first, CR leading company may be different. |
|  | Huawei: we are ok to focus on TS 38.104 firstly in this meeting |
| R4-2201451 | Nokia: focus on CR for TS 38.104 first, CR leading company may be different. |
|  | Huawei: we are ok to focus on TS 38.104 firstly in this meeting |
|  | Ericsson: We have some concerns with this CR as there is no separator highlighted the skipped subclaused which don’t have to be udpated. Also, it seems this draft CR is copied from the draft CR on TS 38.104, keeping the tracked changes, this is confusing and hard to properly review. |
| R4-2200154 | Nokia: - 38 dB ACLR would be used also for other bands than n103, so should not define a n103 specific ACLR table, - Table 6.6.4.2.3-1 should not be changed, - it should be clarified that Table 6.6.5.2.1-1 does not apply to n103, - why n103 is added for n96 in Table 6.6.5.2.4-1 but there are no exceptions for other requirements? - it should be clarified that Table 7.2.2-2 does not apply to n103, - receiver dynamic range requirements should also be updated, - ACS is relaxed by 4dB not 3dB, - receiver spurious emission requirements should also be specified. |
|  | Ericsson: Before finalizing this CR, we should better agree on the main open issues listed in this summary |
| R4-2201450 | Nokia: - in Table 5.3.5-1, is 25/35/45MHz CHBW needed? - 38 dB ACLR would be used also for other bands than n103, so should not define a n103 specific ACLR table - table references in clauses 6.6.3.3 and 6.6.3.4 should also be updated - statement about measurement bandwidth scaling should also apply for Note 1 in Table 6.6.4.2.2.1-3 - it should be clarified that Table 6.6.5.2.1-1 does not apply to n103 - clause 6.6.5.2.2 should also be updated - 'could' is not clear wording on 1dB refsens relaxation - OOBB requirements should also be specified. |
|  | Ericsson: Same comments as for R4-2201451 above |
| R4-221508 | Nokia: - NC ACLR and CACLR should also be specified; missing notes in OBUE table - it should be clarified that Table 6.6.5.2.1-1 does not apply to n103 - clause 6.6.5.2.2 should also be updated - why n103 is added for n96 in Table 6.6.5.2.4-1 but there are no exceptions for other requirements? - receiver dynamic range requirements should also be updated - ACS is relaxed by 4dB not 3dB - receiver spurious emission requirements should also be specified. Huawei: Thanks Nokia for the comments, I think the proposal and comments will be considered in the running CR. |
|  | Ericsson: Before finalizing this CR, we should better agree on the main open issues listed in this summary (GSCN step size, mistake in the ACLR requirement, the update on the Tx spurious is missing, …) |
| R4-2200155, R4-2200156 | Keysight: these are for BS conformance test spec, which should wait for Core to be done and also MU needs some study. These should be postponed to future meeting. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1-1: BS type | - Proposals: - Option 1: Specify BS type 1-H and type 1-O for the 6GHz licensed band (Ericsson) - Option 2: Specify BS type 1-C, BS type 1-H and type 1-O for the 6GHz licensed band (Nokia, Huawei, CATT Recommendations for 2nd round: At 1st round discussion, companies show preference on the two options. It is proposed to further discussion for 2nd round. |
| Issue 4-1-2 and Issue 4-1-5: Tx requirements | Recommendations for 2nd round: Based on 1st round discussion, the proposal is agreeable, with the clarification that OBUE and spurious need to be agreed together with ΔfOBUE. The update will be checked for 2nd round. |
| Issue 4-1-3: OBUE for Medium range and Local area BS | Recommendations for 2nd round: Based on 1st round discussion, the proposal is agreeable, with the clarification that OBUE need to be agreed together with ΔfOBUE. The agreement will be captured in the WF and no further discussion is needed for 2nd round. |
| Issue 4-1-4 and Issue 4-1-6: Rx requirements | Recommendations for 2nd round: Based on 1st round discussion, the proposal is agreeable, with the update that narrow band blocking and intermodulation requirements are not applicable, wanted and interfering signal levels for ICS should be updated with 1dB higher NF. The update will be checked for 2nd round. |
| Issue 4-1-7: ΔfOBUE | - Proposals: - Option 1: define ΔfOBUE = 100 MHz for BS type 1-H and type 1-O, no need for type 1-C (Ericsson) - Option 2: define ΔfOBUE = 100 MHz for BS type 1-H and type 1-O, and ΔfOBUE = 40 MHz for BS type 1-C (ZTE) - Option 3: at least for MR and LA BS, 40 MHz for BS type 1-C and type 1-H (Nokia) - Option 4: define ΔfOBUE = 100 MHz for BS type 1-H and type 1-O, FFS for type 1-C (Huawei, CATT) Recommendations for 2nd round: At 1st round discussion, companies show preference on the options. It is proposed to further discussion for 2nd round. |
| Issue 4-1-8: ΔfOOB | - Proposals: - Option 1: define ΔfOOB = 100 MHz for BS type 1-H and type 1-O, no need for type 1-C (Ericsson) - Option 2: define ΔfOOB = 100 MHz for BS type 1-H and type 1-O, and ΔfOOB = 60 MHz for BS type 1-C (ZTE) - Option 3: at least for MR and LA BS, i.e., 40 MHz for BS type 1-C and type 1-H (Nokia) - Option 4: define ΔfOOB = 100 MHz for BS type 1-H and type 1-O, FFS for type 1-C (Huawei, CATT) Recommendations for 2nd round: At 1st round discussion, companies show preference on the options. It is proposed to further discussion for 2nd round. |
| Sub-topic 4-2 | Recommendations for 2nd round: Focus on the WF discussion for 2nd round, no further discussion on the CRs and the leading company will take these contributions into consideration for next meeting. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on BS RF requirements for 6GHz licensed band | CATT |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general aspects for 6GHz licensed band | Huawei, HiSilicon |  |
| WF on system parameters for 6GHz licensed band | ZTE |  |
| WF on UE RF requirements for 6GHz licensed band | Ericsson |  |
| WF on BS RF requirements for 6GHz licensed band | CATT |  |

Existing tdocs

All contributions under agenda 5.3 can be noted.

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents