3GPP TSG-RAN WG4 Meeting #101-Bis-e  						        				R4-2202270

Electronic Meeting, Jan. 17-25, 2022

Agenda item:			6.9.3

Source:	Samsung

Title:	WF on UE RF requirement for FR2 HST

Document for:	Approval

# 1 Background

The WF is focused UE RF requirement on the topic of Rel-17 NR support for high speed train (HST) scenario in FR2 (AI 6.9.3) in RAN4 #101-bis-e, with the email thread identifier “[101-bis-e][123] NR_HST_FR2_enh”. In previous RAN4 meeting, based on the approved WF [R4-2120066], the following agreement and conclusion were made on UE RF core requirement for FR2 HST UE:

- WF1: UE RF requirement framework and Power Class - For power class, the following agreement is achieved in GTW (Thursday, 4th Nov): - Proposal 1: Introduce new power class for FR2 HST UE, by numbering as UE power class 6 and specifying UE type as: UE Power class; UE type. 1; Fixed wireless access (FWA) UE. 2; Vehicular UE. 3; Handheld UE. 4; High power non-handheld UE. 5; Fixed wireless access (FWA) UE. 6; High Speed Train Roof-Mounted UE. Agreement: the proposal 1 is agreed. - For UE RF requirement framework, the following agreement is achieved in GTW (Thursday, 4th Nov): Agreement: - The unified RF requirements for FR2 HST UE are defined except spherical coverage - FFS on spherical coverage requirements - Option 1: use the union of the largest spherical coverage of theta and phi to define the unified requirements - Option 2: The unified RF requirement for FR2 HST UE is defined based on one particular scenario requiring the largest spherical coverage. - For UE RF requirement framework, the following agreement is achieved in GTW (Friday, 12th Nov): Agreement: The assumption that UE has two panels, i.e., back-to-back panels, will be used to derive spherical coverage requirements. - Further discuss whether one panel based spherical coverage requirement will be specified - FFS on whether to mandate two panels. - WF2: Minimum Peak EIRP - In RAN4#99-e, the following agreement is reached [R4-2107861]: - WF3: Minimum Peak EIRP - Minimum peak EIRP requirement for FR2 HST UE: - RAN4 adopt 30.x dBm (similar to PC5) as baseline. - The baseline could be further discussed if technical issue identified.. - In this meeting, the following confirmation is reached: - Minimum peak EIRP requirement for FR2 HST UE: - Keep existing agreement from RAN4#99-e - WF3: Spherical coverage - The following agreement is achieved in GTW (Thursday, 11th Nov): Agreement: - Directions of antenna panels: - Boresight directions for forward and backward panels shall be declared by UE vendors. - FFS whether the limitation on boresight directions is needed - Coordination system to be used for requirement definition: - Option-1: absolute coordination system: - Option 2: relative coordination system (relative to the claimed boresight direction) - Spherical coverage x%-tile point per panel - Azimuth angle (i.e., phi) range to cover: - Option-1: [-45, +45] degree relative to absolute coordination system - Option-2: [-25, +25] degree relative to UE declared boresight direction - Other options are not precluded - Elevation angle (i.e., theta) range to cover: - Option-1: [45, 90] degree relative to absolute coordination system - Option-2: [-10, +10] degree relative to UE declared boresight direction - WF4: Beam correspondence for FR2 HST UE - In this meeting, the following agreement is reached: - For FR2 HST UE, the beam correspondence support can be summarized in the following table: FR2 Power Class; Rel-15 BC Feature beamCorrespondenceWithoutUL-BeamSweeping; Rel-16 SSB based enhanced BC beamCorrespondenceSSB-based-r16; Rel-16 CSI-RS based enhanced BC beamCorrespondenceCSI-RS-based-r16; Requirement Applicability for (1) Minimum peak EIRP, spherical coverage requirement (2) BC Tolerance requirement; Side condition. FR2 HST UE (PC X); Supported (Mandatory); Supported (Mandatory); Not Supported; Meet (1) w/o UL beam sweeping BC Tolerance req. (2) is met implicitly; Side condition for SSB based enh. BC (CSI-RS not provided). Supported; Side condition for CSI-RS based enh. BC (weak SSB). - WF5: RX Requirement for FR2 HST UE - In this meeting, the following agreement is reached: - For FR2 HST UE, RAN4 adopt REFSENS requirement as PC5, that is Operating band; REFSENS (dBm) / Channel bandwidth. 50 MHz; 100 MHz; 200 MHz; 400 MHz. n257; -92.6; -89.6; -86.6; -83.6. n258; -92.8; -89.8; -86.8; -83.8. N261; -92.6; -89.6; -86.6; -83.6. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4.

# 2 Way Forward

### Sub-Topic 2-1 Spherical Coverage Requirement and Framework

Issue 2-1-1: Spherical coverage requirement framework – Requirement for one panel

- The following agreement achieved in GTW session (19th Jan), captured Chairman Notes and copied here for information:

Agreement: RAN4 shall not define core requirement for one-panel based spherical coverage requirement.

Issue 2-1-2: Spherical coverage requirement – Coordination system

- The following agreement achieved in GTW session (19th Jan), captured Chairman Notes and copied here for information:

Agreement: use the absolution coordination system as well as Qualcomm proposals below as baseline - The minimum EIRP measured over the spherical coverage evaluation areas specified below is defined as the spherical coverage requirement and is found in Table 6.2.1.6-3 below. UE spherical coverage evaluation areas are found in Table 6.2.1.6-3a below, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex J.1. The requirement is verified with the test metric of EIRP (Link= Spherical coverage grid, Meas=Link angle). Table 6.2.1.6-3: UE spherical coverage for power class 6 Operating band; Min EIRP over UE spherical coverage evaluation areas (dBm). n257; TBD. n258; TBD. n259; TBD. NOTE 1: Minimum EIRP over UE spherical coverage evaluation areas is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1.. Table 6.2.1.6-3a: UE spherical coverage evaluation areas for power class 6 θ range (degree); ϕ range (degree). Area-1; 90 to (90-theta_elev); -phi_az to + phi_az. Area-2; 90 to (90-theta_elev); 180-phi_az to 180+ phi_az. NOTE 1: When testing power class 6 UEs, DUT orientation can be determined according to the UE spherical coverage evaluation areas, not necessarily following default alignment in Figure J.1-2 or positioning guidelines in clause J.3. NOTE 2: High speed train deployment is expected to be w.r.t. the reference coordination system: θ = 90 (degree) corresponds to the ground plane the train is running on, and ϕ= 0 or 180 with θ = 90 are the train track directions..

Issue 2-1-3: Spherical coverage requirement framework - Coverage region

- The following agreement achieved in GTW session (19th Jan), captured Chairman Notes and copied here for information:

Agreement: network signaling is provided to configure UE to follow enhanced RRM requirement Set 2.

Issue 2-1-4: Spherical coverage requirement – Coverage Region and x%-tile

- [Moderator] Based on email discussion, companies’ understanding on the required spherical coverage region are converged, while the following proposals are provided by identifying the required azimuth and elevation region respectively.

- Way forward:

  - Use below region (provided in spherical coordination system) as the baseline for UE spherical evaluation areas:

    - Azimuth angle range (relative to 0 and 180 degree in spherical coordination system):

      - [[-37.5]degrees, [+37.5]degrees]

    - Elevation angle:

      - [-15degrees, +15degrees] w.r.t. UE claimed boresight direction

        - i.e., [90degrees, 90-30degrees] in spherical coordination system by assuming 15degree upper-tilted for panel’s boresight.

    - Companies are encouraged to provide evaluation on EIRP spherical coverage requirement over the above baseline for UE spherical evaluation areas.

Issue 2-1-5: Spherical coverage requirement - EIRP drop

- [Moderator] Based on agreement achived in GTW session, P3 needs no more discussion, while P1 and P2 can be further discussed, while very likely the EIRP drop can be further checked and evaluated only after the spherical coverage range is concluded in Issue 2-1-4.

- Way Forward:

  - FFS the spherical coverage requirement:

    - FFS Set EIRP drop requirement to keep received power at gNB stable.

    - FFS EIRP drop requirement for HST is -15dB.

### Sub-Topic 2-2 Beam Correspondence for FR2 HST UE

Issue 2-2-1: Side Condition for FR2 Power Class 6 UE Beam Correspondence Requirement

- The following agreement achieved in GTW session (19th Jan), captured Chairman Notes and copied here for information:

Agreement: - After RAN4 obtained PC6 EIS spherical coverage requirement, the side conditions for beam correspondence requirement can be derived according by: - Minimum SSB_RP = EIS spherical coverage(PC6, n259, 50MHz) - 10*log10(nrofRBs x 12) – SNR(at Refsens) + SSB Ês/Iot + ΔMBS

# References

[1] R4-2202223, “Email discussion summary for [101-bis-e][123] NR_HST_FR2”, Samsung

[2] R4-2202323, “Email discussion summary for [101-bis-e][123] NR_HST_FR2”, Samsung