3GPP TSG-RAN WG4 Meeting # 101-bis-e 											     R4-2202287

Electronic Meeting, 17th – 25th January, 2022

Source: 	Huawei, HiSilicon, []

Title: 	WF on improvements to MSD table

Agenda Item:	5.27.1

Document for:	Approval

Background

Based on the approved WF R4-2119878 [1], companies provided their views on the applicability and scope in this meeting.

For MSD due to cross-band isolation, there are some remaining issues to be addressed, e.g. interference type category and the MSD exception table format.

For MSD due to harmonic interference /harmonic mixing, there are some remaining issues to be addressed, e.g. FFS is Tx / Rx harmonic mixing tables should be kept separate, and if PC2 and PC3 MSD should also be kept in separate tables.

In this meeting, companies provides some contributions [2][3][4] to further discuss these remaining issues.

Way forward on Applicability and Scope

P1: The combos introduced in R17 are allowed to be discussed for improvements to MSD table.

FFS the impact on ENDC combos for R17.

FFS how to implement these changes.

P2: Reduce or restrict the number of test points for a given MSD type and a given band combination

FFS the number of test points.

Way forward on MSD due to cross band isolation

P3: Option 1a: The specific necessity of all these types (“>ACLR2”, “ACLR1/ACLR2” and “C-IM3/C-IM5”) can be discussed on a case-by-case basis.

Note: These types are defined in WF R4-2119878.

P4: For MSD test configurations due to cross band isolation, the proposed guidelines can be way forward.

Guideline A: The UL/DL carrier frequencies shall be carefully selected so as to minimize the frequency distance that separates the UL aggressor carrier from the DL victim’s carrier.

P5: How to specify the MSD due to cross band isolation (“>ACLR2”) for Rel-17 band combinations.

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1720 | 20 | 15 | 100 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n18 | n28 | 817.5 | 5 | 15 | 18 (RBstart=0) | 800.5 | 5 | 4.5 | >ACLR2 |
| n34 | n3 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| NOTE X: When the victim DL bands are FDD bands, the UL RB allocation of victim FDD bands shouldn’t be configured |  |  |  |  |  |  |  |  |  |

Way forward on MSD due to Harmonics

P6: FFS on whether Tx / Rx harmonic mixing tables can be merged in a common table

P7: PC2 and PC3 MSD should be merged in the common table.

P8: candidate options for the format of MSD table due to harmonics

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| nX | nY | TBD | 5 | 15 | 25 (RBstart=0) | TBD | 5 | 23.5 | UL2/DL1 |

Option 2:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| nX | nY | 5 | 15 | 25 (RBstart=0) | 10 | 23.5 | Note1 | UL2/DL1 |
| Note 1: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

- References

[1]	R4-2119878	WF on how to improve MSD tables in Rel-17, Skyworks Solutions Inc.

[2] R4-2200463	MSD table improvements, Nokia, Nokia Shanghai Bell

[2] R4-2201251	Discussion on simplifying extended MSD table, Huawei, HiSilicon

[3] R4-2202040	Discussion on NR-CA and EN-DC MSD Table Simplifications, Skyworks Solutions Inc.