3GPP TSG-RAN WG4 Meeting # 101bis-e 													R4-2202316

Electronic Meeting, 17 – 25 January 2022

Agenda item:			5.37, 5.38, 5.39, 8.7, 8.9

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101bis-e][116] LTE_NR_Other_WI

Document for:	Information

# Introduction

In this email thread, the following agenda items are discussed.

5.37 Additional NR bands for UL-MIMO
5.38 DL interruption for band combo dynamic Tx Switching
5.39 Simultaneous Rx/Tx band combination
8.7 Additional LTE bands for UE Cat M1/2, NB1/2
8.9 R17 Additional enhancements for NB-IoT and LTE-MTC

# Topic #1: Additional NR bands for UL-MIMO

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201759 | Ligado Networks | Draft CR for n24 and n99 UL-MIMO PC3 Includes n24 and corresponding SUL band n99 as bands supporting UL MIMO, UL MIMO MOP for n24 and n99 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Open issues and candidate options before e-meeting:

## Companies views’ collection for 1st round

### Open issues

No open issues, please comment in the CR section (1.3.2) directly.

| Company | Comments |
| --- | --- |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201759 Draft CR for n24 and n99 UL-MIMO PC3 | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic# 1 | Tentative agreements: No comments are received on the draft CR. Recommend to be endorsed. Candidate options: Recommendations for 2nd round: The topic is closed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201759 Draft CR for n24 and n99 UL-MIMO PC3 | To be endorsed. |

# Topic #2: Downlink interruption for band combinations to conduct dynamic Tx Switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200770 | China Telecom | Draft CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching <Not available> |

## Companies views’ collection for 1st round

### Open issues

No open issues to be discussed.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | China Telecom: Given no inputs for this WI, the draft CR of R4-2200770 can be withdrawn. |

## Summary for 1st round

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200770 | To be withdrawn. |

The topic is closed.

## Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Simultaneous Rx/Tx band combination

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200566 | NTT DOCOMO INC. | Observation 1: Simultaneous RxTx capability signalling for per-band-pair per band combination was newly introduced from Rel-15 RAN2 specification TS 38.306 and TS 38.331 according to RAN4 request [2][3]. Observation 2: Based on the current description of TS 38.101-1 and TS 38.101-3, if it is mandatory for UE(s) to support simultaneous RxTx of band combination Band 1+Band 2, it may be unclear whether it is mandatory to support simultaneous RxTx of a band pair Band 1+Band 2 within a higher order band combination such as Band 1+ Band 2+ Band 3. Observation 3: Applicability rule of mandatory simultaneous RxTx for higher order band combinations is needed only when per-band-pair signalling is indicated in the higher order band combinations in order to align with RAN2 signalling design and avoid the impact on legacy UEs. Proposal 1: Clarify the applicability of mandatory simultaneous RxTx for band pairs included in higher order band combinations by adding the following description to relevant notes as shown in Table 2.2-2 in this paper: - Mandatory simultaneous RxTx capability also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration and the field of simultaneousRxTxInterBandENDCPer-band-pair is included in the higher order EN-DC configuration. Proposal 2: Changes in proposal 1 should apply to TS 38.101-1 and TS 38.101-3 from Rel-15. - Some wording should be modified such as EN-DC and NR CA, and simultaneousRxTxInterBandENDCPer-band-pair and simultaneousRxTxInterBandCAPer-band-pair. |
| R4-2200354 | SoftBank Corp. | Observation: Some band combinations have already specified as the simultaneous Rx/Tx capability is mandatory support. The maximum MSD value in those band combinations is 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. Proposal: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. If there are some difficulties to define those values from the implementation perspective, they should be clarified and considered when defining the threshold. |
| R4-2201067 | Samsung | Observation1: Simultaneous Rx/Tx capability is optional means that for one band combination it depends on UE’s claim supporting simultaneous Rx/Tx, otherwise, the capability is not present Observation 2: If simultaneous Rx/Tx capability is not present means worse MSD performance than a threshold, it may conflict with potential “Low MSD” capability which will be discussed in Rel-18 Proposal 1: It is proposed to consider specify mandatory simultaneous Rx/Tx capability case by case for FR1 FDD-TDD band combinations at least in Rel-17, i.e. whether a FDD-TDD band combination mandatory support simultaneous Rx/Tx is based on operator’s request. |
| R4-2201230 | Xiaomi | Proposal 1: MSD threshold could be decided from some system level simulation. |
| R4-2201340 | ZTE Corporation | Observation 1: For a FDD-TDD inter-band NR CA band combination, inter-band NR CA operation can not workable if simultaneous Rx/Tx operation is not supported. Observation 2: Due to actual MSD can be 20 dB or more better than the MSD in the specs, actual MSD could be smaller than the threshold in case of the MSD defined in the spec larger than the MSD threshold. Observation 3: The performance would be affected if changing the simultaneous Rx/Tx capability from mandatory to optional. Proposal: The threshold value should be higher than 32.5dB. |
| R4-2201954 | Huawei, HiSilicon | Observation 1: MSD for FDD-TDD band combination is defined for simultaneous Rx/Tx operation. Observation 2: Though the MSD for some band combination could be very large, whether to enable a band combination is a choice of deployment decision Proposal 1: For FDD-TDD band combination for FR1 with specified MSD, simultaneous Rx/Tx is the default capability and no need to further discuss the MSD threshold. |
| R4-2201955 | Huawei, HiSilicon | TP for TR 38.839: Principles for simultaneous Rx/Tx capability |
| R4-2200567 | MediaTek Beijing Inc. | Proposal: Add NOTE to clarify the minimum requirements apply only when there is non-simultaneous Rx/Tx operation for CA_n257-n259 and CA_n258-n260, as CA_n260-n261. |
| R4-2201341 | ZTE Corporation | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat F) A note is added in table 5.2A.2-1 to indicate the information of simultaneous Rx/Tx capability for the existing FR2 band combinations of CA_n260-n261. |
| R4-2201342 | ZTE Corporation | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat A) |
| R4-2201343 | ZTE Corporation | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA CA_n257-n259 and CA_n258-n260 (Cat F) To indicate same note as in R4-2119950 for simultaneous Rx/Tx capability for the existing FR2 band combinations CA_n257-n259 and CA_n258-n260. |
| R4-2201956 | Huawei, HiSilicon | Observation 1: Non-simultaneous Rx/Tx operation would have strong limitation for network deployment for FR2 CA inter-band combinations Observation 2: There are no detailed analysis for the MSD for specific band combinations, especially for the far separated FR2 band combinations Observation 3: The simultaneous Rx/Tx capability is optionally supported for FR2 TDD-TDD band combinations. If UE does not support simultaneous Rx/Tx operation for the band combination, it does not need to report the capability. Proposal 1: For FR2 TDD-TDD band combinations, especially those with large frequency separation, leave the capability to UE implementation, and no need to have the note with non-simultaneous Rx/Tx operation to limit the network deployment flexibility. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination

Sub-topic description: The discussion on MSD threshold is continued and a new issue is raised by DoCoMo in R4-2200566.

Open issues and candidate options before e-meeting:

Issue 3-1-1: MSD threshold

Issue 3-1-2: Applicability of mandatory simultaneous RxTx for band pairs included in higher order band combinations

### Sub-topic 3-2 Simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD band combination

Sub-topic description: The same issue as in last meeting is presented in the contribution papers.

Open issues and candidate options before e-meeting:

Please comment in the CR/TP section directly.

## Companies views’ collection for 1st round

### Open issues

Issue 3-1-1: MSD threshold

For FR1+FR1 FDD-TDD band combinations whose MSD is larger than a threshold (value FFS), further discuss whether simultaneous Rx/Tx can be changed to optional, otherwise, the simultaneous Rx/Tx capability is mandatory support.

- Proposals

  - Option 1: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. If there are some difficulties to define those values from the implementation perspective, they should be clarified and considered when defining the threshold.

  - Option 2: The threshold value should be higher than 32.5dB.

  - Option 3: MSD threshold could be decided from some system level simulation.

  - Option 4: It is proposed to consider specify mandatory simultaneous Rx/Tx capability case by case for FR1 FDD-TDD band combinations at least in Rel-17, i.e. whether a FDD-TDD band combination mandatory support simultaneous Rx/Tx is based on operator’s request.

  - Option 5: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Samsung | Our proposal as option4 is based on our observation that “If simultaneous Rx/Tx capability is not present means worse MSD performance than a threshold, it may conflict with potential “Low MSD” capability which will be discussed in Rel-18”. I would like to hear opinions of experts on this. If indeed there may be a conflict or some relationship, then it would be better to discuss the threshold in Rel-18. |
| ZTE | Either option 1 or option 2. If there were no agreements in the end, we propose not to define the MSD threshold for FDD-TDD band combs, instead mandatory simultaneous Rx/Tx capability to all the FDD-TDD band combs. In addition, it may need discuss how to support FDD-TDD CA if simultaneous Rx/Tx is not supported. |
| SoftBank | Support option 2 but we are fine with option 1 considering the discussion in the last meeting. |
| CHTTL | Support option 2 but also fine with option 1. |
| Huawei, HiSilicon | Ok with either option 1 or option 2. Alternatively, since FDD-TDD band combination for FR1 with specified MSD, simultaneous Rx/Tx is the default capability, whether to have a MSD threshold may not be that important. If indeed there are some FDD-TDD band combinations with MSD cannot support simultaneous Rx/Tx operation (so far not identified yet), they can be indicated with a NOTE in the spec. |
| Qualcomm | We prefer Option 5. Field/deployment data from operators or infra-vendors would be very useful to decide on what the threshold should be. Option 3 will be very time consuming and unlikely to lead to any conclusion. Option 1 or 2 seem arbitrary, any technical reason to choose any of these values? |
| Apple | Option 4 seems to be reasonable as it includes case by case study of CA combinations. In the case of very high MSD it could hinder DL demodulation and therefore prevent signal reception. This needs to be checked before mandatory Rx/Tx is defined. |
| Xiaomi | Considering the MSD threshold is hard to decide and the limited time in R17, the option 4 which seems a traditional way is acceptable for us. |

Issue 3-1-2: Applicability of mandatory simultaneous RxTx for band pairs included in higher order band combinations

- Proposals

- Proposal 1: adding the following description to relevant notes as shown in Table 2.2-2 in R4-2200566:

Mandatory simultaneous RxTx capability also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration and the field of simultaneousRxTxInterBandENDCPer-band-pair is included in the higher order EN-DC configuration.

- Proposal 2: Changes in proposal 1 should apply to TS 38.101-1 and TS 38.101-3 from Rel-15

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| SoftBank | Support both proposal 1 and 2. |
| CHTTL | In the paper, it mentions if it is mandatory to support simultaneous RxTx of band combination Band 1+Band 2, it is also mandatory to support simultaneous RxTx of the band pair Band 1+Band 2 within a higher band combination such as Band 1+ Band 2+ Band 3. So probably there is no need to add the signalling aspect in RAN4? Ex: the following sentence is enough? Mandatory simultaneous RxTx capability also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration |
| Huawei, HiSilicon | Support the proposals. Also ok with the suggestion by CHTTL. |
| Qualcomm | We are fine with the proposals |
| DOCOMO | To CHTTL, thank you very much for the comment. The reason why we put the text of “the field of simultaneousRxTxInterBandENDCPer-band-pair is included in the higher order EN-DC configuration” is that if UE cannot support per band pair signalling newly introduced in RAN2, the UE cannot say that band pair of bands 1+2 supports simultaneous RxTx if the UE function does not support simultaneous RxTx for other band pairs such as band pair of bands 1+3 and bands 2+3. This is because the UE can only indicate per band combination capability, so UE has no choice but to indicate “not-supported” for simultaneous RxTx for the band combination 1+2+3, which means that any band pairs cannot support simultaneous RxTx according to RAN2 specification. |
| Ericsson | A formal comment first: in our understanding, a table is ‘self-contained’ and its notes can only refer to the table itself, not to any contents of another table (the three-band etc in this case). Therefore, the proposed note must (unfortunately) be repeated in all the tables where the normative requirement applies. Then a technical comment: according to the new capability simultaneousRxTxInterBandCAPer-band-pair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair [...] The UE does not include this field if the UE supports simultaneous transmission and reception for all band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. The UE shall consistently set the bits which correspond to the same band pair. the UE must support simultaneous RxTx for all band pairs to include the simultaneousRxTxInterBandCA. But is this needed if the CA configuration contains an intra-band contiguous or non-contiguous CA configuration (band pair) for TDD for which simultaneous RxTx can ‘never’ be supported? The UE should be able to indicate simultaneousRxTxInterBandCA in this case. The same scenario for DC. This does not imply that TDD carriers of the intra-band pair must be collocated, but the GP should be set such that simultaneous RxTx never occurs. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201955 TP for TR 38.839: Principles for simultaneous Rx/Tx capability | MediaTek: About Huawei’s proposal “For FR2+FR2 TDD-TDD band combination, the simultaneous Rx/Tx capability should be studied case by case: Currently, we add NOTE to existed band combinations about no support simultaneous Tx/Rx, however, as proposed/commented last meeting, we think it is also fine to make the NOTE as general principle for FR2+FR2. So, we think it’s too early to agree the CR’s proposal. |
|  | Huawei, HiSilicon: The TP just captures the previous agreements. For FR2 TDD-TDD, even there are some discussion undergoing, it does not break the general principle that the band combinations should be studied case by case, similar to that for FR1 TDD-TDD. Curious to see “too early to agree the proposal”, those proposals were already agreed before actually. |
| R4-2201341 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat F) | ZTE: This draft CR was endorsed in last RAN4 meeting. Unfortunately, it was not implemented to the specification. Therefore, the draft CR is re-submitted. |
|  | MediaTek: Support. Thanks for ZTE’s effort. |
|  | Huawei, HiSilicon: Since this one was endorsed in last meeting, we are ok with it. But this is the bis meeting, re-submission of the draft CR still cannot make it to be implemented in the spec. ZTE2: Before the meeting, we have asked for MCC’s guidance. I was told i can re-submit it in this meeting. |
| R4-2201342 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat A) | ZTE: This draft CR was endorsed in last RAN4 meeting. Unfortunately, it was not implemented to the specification. Therefore, the draft CR is re-submitted. |
|  | MediaTek: Support. Thanks for ZTE’s effort. |
|  | Huawei, HiSilicon: see above comments. ZTE2: Before the meeting, we have asked for MCC’s guidance. I was told i can re-submit it in this meeting. |
| R4-2201343 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA CA_n257-n259 and CA_n258-n260 (Cat F) | ZTE: Agree. This draft CR is aligned with the previous agreements. |
|  | Rohde & Schwarz: Agree with the reason for change and support the CR. |
|  | MediaTek: Support. Thanks for ZTE’s effort. |
|  | Verizon: agree the draft CR! It is reasonable to have a consistent requirement. Huawei, HiSilicon: We didn’t see technical analysis of possible REFSENS degradation for these band combinations under simultaneous Rx/Tx operation. We are not against the note itself and the applicability for some band combinations, but case by case study means at least some study is needed. We think the RAN4 specification should be built with technical inputs rather than just presenting the proposal. We expect to see some basic analysis on MSD in next meeting and then to close the discussion. ZTE2. To huawei, some discussion/analysis happened in #100 meeting. |
|  | Apple: Thanks for drafting the CR. The “Reason for change” mentions that R4-2119950 was endorsed but not implemented. As it was not implemented and note 1 is not provided to CA_n260-n261 with this CR it might remain missing. We would like to get a clarification on how the note is finally applied to this combination. ZTE2. To apple, this draft CR is try to avoid to add the note as in endorsed in 2119950, otherwise overlapping will happen. Actually we have asked for MCC’s guidance before the meeting, but i didn’t get clear guidance on how to treat it, except for the guidance that i can re-submit 2119950 in this meeting. Therefore, we try to descriple the situation in the CR cover, such as ...to avoid duplication, we draft this CR on top of the endorsed CR R4-2119950... |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 3-1 & 3-2 | Issue 3-1-1: MSD threshold Four companies are OK with either opt 1 or 2; three companies support option 4 and one company proposes option 5 (more field/deployment data). No clear consensus is reached yet. Issue 3-1-2: Applicability of mandatory simultaneous Rx/Tx for band pairs included in higher order band combinations The majority view seems to support the two proposals, with some comments on the wording or corner cases. Consensus is likely to be reached after further discussions. Tentative agreements: Candidate options: Recommendations for 2nd round: For issue 3-1-1, assign a WF to the WI rapporteur to drive the discussion on MSD threshold. Companies are encouraged to reach a compromise. For issue 3-1-2, assign a WF to the proponent company (NTT Docomo) to capture the potential agreement. Discuss the above WFs in the 2nd round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201955 TP for TR 38.839: Principles for simultaneous Rx/Tx capability | The rapporteur has clarified that the TP only captures previous agreements. From moderator’s perspective, if new agreement is reached, the TR can be updated accordingly. With the above clarification, can the TP be approved? |
| R4-2201341 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat F) | To be endorsed? |
| R4-2201342 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat A) | To be endorsed? |
| R4-2201343 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA CA_n257-n259 and CA_n258-n260 (Cat F) | One company asks for more technical analysis and another company seeks further clarification. And the reply from the proponent shows uncertainty (“Actually we have asked for MCC’s guidance before the meeting, but i didn’t get clear guidance on how to treat it”). Postponed to the next meeting? |

## Discussion on 2nd round (if applicable)

WF on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combinations

| Company | Comments |
| --- | --- |
| SoftBank | Thanks Huawei for the WF. In the WF, it seems not clear that how to treat the band combinations which can support the simultaneous Rx/Tx operation. Considering the above observations, which is the correct understanding? a) the support of simultaneous Rx/Tx operation is mandatory for all FR1 FDD-TDD band combinations since there is no identified combination which cannot support the simultaneous Rx/Tx operation. b) the support of simultaneous Rx/Tx operation is mandatory for FR1 FDD-TDD band combinations if operator requests. |
| Huawei | Thanks SoftBank for the clarification question. Our understanding is a). For a FR1 FDD-TDD band combination with MSD, simultaneous Rx/Tx operation can always be supposed by default, unless there is an identified FDD-TDD band combination which cannot support such operation. If it does have this kind of combinations, the number must be very limited. To just indicate these band combinations in the spec would require much less standard effort. |
| Samsung | We think the threshold could be discussed in Rel-18, but if all companies think there is no need to discuss it again, then we can compromise. |
| Apple | To our understanding when a UE uses simultaneousRxTxInterBandENDCPerBandPair then it signals the band pairs which can be supported with simultaneous Rx/Tx. We would like to have a clarification what happens regarding proposal 1 if a UE signals simultaneousRxTxInterBandENDCPerBandPair but does not indicate any band pair supported with simultaneous Rx/Tx. Especially if the lower order combination is mandatory. Will it be possible for a UE to indicate in this way that it cannot support simultaneous Rx/Tx for the higher order combination? |
| Huawei | To Apple’s question, this one question seems related to the other WF under discussion in the agenda. But we also would like to share our view here. According to the agreed WF in R4-2107843, it clarifies the issue we have in RAN4 at that time, i.e. - E.g. Band combination A+B+C, - A+B+C does not support simultaneous Rx/Tx - A+B and A+C supports simultaneous Rx-Tx but B+C does not support simultaneous Rx-Tx Based on the WF, and LS was sent to RAN2, and then the capability simultaneousRxTxInterBandENDCPerBandPair is introduced. There should be no ambiguity for the signaling in RAN2 spec. |
| Apple | Many thanks to Huawei for the answer and detailed explanation. Sorry for the wrong post. Got confused to due to similar names the WFs, thought that the upload did not go through and reuploaded the question with the wrong WF. |
| Qualcomm | This WF proposal seems to make the support of simultaneous Rx-Tx mandatory all FDD-TDD combinations so it’s not agreeable to us. Beyond a certain MSD value, there is no point in making this feature mandatory. |
| Apple | Similar to Qualcomm and Samsung we would also prefer to define a specific threshold MSD. |
| Huawei | To Qualcomm and Apple, just for clarification of the capability for this case. Based on the discussion in these two meetings, in our understanding: FR1 FDD-TDD band combination with MSD by default can support simultaneous Rx/Tx operation, but UE can still optionally choose to report the capability. |

WF on simultaneous Rx/Tx capability for band pairs included in higher order band combinations

| Company | Comments |
| --- | --- |
| DOCOMO | I uploaded the initial draft WF in inbox (here) Based on CHTTL comments, [] part is set as FFS. We replied an answer in the first round, and we would like to check if we can remove []. Based on Ericsson comments, we prepared two options to capture the principle in the specifications, and clarify that simultaneous RxTx capability apples for inter-band configuration, not for carries within intra-band configuration. |
| SoftBank | Support the proposals in the WF. Regarding proposal 3, either option is fine with us. |
| Apple | To our understanding when a UE uses simultaneousRxTxInterBandENDCPerBandPair then it signals the band pairs which can be supported with simultaneous Rx/Tx. We would like to have a clarification what happens regarding proposal 1 if a UE signals simultaneousRxTxInterBandENDCPerBandPair but does not indicate any band pair supported with simultaneous Rx/Tx. Especially if the lower order combination is mandatory. Will it be possible for a UE to indicate in this way that it cannot support simultaneous Rx/Tx for the higher order combination? |
| DOCOMO | Based on the further feedback from CHTTL, as a proponent, we are OK to remove the signalling aspect from our original proposal. We would like to check if other companies are also OK with this change. At least Huawei seems OK based on 1st round comments. To Apple: Thank you for your question. >Especially if the lower order combination is mandatory. Will it be possible for a UE to indicate in this way that it cannot support simultaneous Rx/Tx for the higher order combination? No. If we agree proposal 1, it means that if band combination A+B is mandatory simultaneous RxTx capability, A+B band pairs is also mandatory simultaneous RxTx capability under higher order band combinations such as A+B+C. So, UE should indicate simultaneousRxTxInterBandENDCPerBandPair to say A+B band pair can be supported with simultaneous RxTx under higher order band combinations. |
| Qualcomm | The text proposed by CHTTL is much better than Proposal 1 so this should be used in the specifications. |

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201955 TP for TR 38.839: Principles for simultaneous Rx/Tx capability | MediaTek: To Huawei, thanks. We are okay to apply the general principle for FR2+FR2. It is reasonable to further update it if RAN4 can achieve further agreements. Moderator: This TP has been approved in the 1st round. Please refer to the decisions in Section 7. |
| R4-2201341 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat F) | MediaTek: As ZTE’s comment, we think this CR is agreeable. Moderator: This draft CR has been endorsed in the 1st round. Please refer to the decisions in Section 7. |
| R4-2201342 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA (Cat A) | MediaTek: As ZTE’s comment, we think this CR is agreeable. Moderator: This draft CR has been endorsed in the 1st round. Please refer to the decisions in Section 7. |
| R4-2201343 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA CA_n257-n259 and CA_n258-n260 (Cat F) | MediaTek: To Huawei: RAN4 already agreed it, and we just implement it by CR this meeting. Let me capture the content for your reference (R4-2114959): Issue 1: Whether or not mark current CA configurations as no support for SimultaneousRxTx operation. - Option 1: Agree to mark current CA configurations as no support for SimultaneousRxTx operation. - Option 2: Not agree to mark current CA configurations as no support for SimultaneousRxTx operation. Agreements: Option 1 is agreeable. Apple: Thanks to ZTE for the clarification. It is fine for us. Moderator: The conclusion for this draft CR in the 1st round is to be postponed to the next meeting. And no objection was received before the deadline. Please refer to the decisions in Section 7. |

## Summary for 2nd round (if applicable)

WF on simultaneous Rx/Tx capability for band pairs included in higher order band combinations is agreeable. WF on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combinations is noted. The decisions on other tdocs remain unchanged as the 1st round. Note that if one wishes to discuss additional issues/topics, please inform the moderator first. Do not add them by oneself.

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

# Topic #4: Additional LTE bands for UE Cat M1/2, NB1/2

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201716 | Ericsson | Table 6.2.4E-x: A-MPR for "NS_27" for Cat-M1 with sub-PRB allocation Channel BW; Parameter; Region. 5 MHz; (NB_index, SCstart); (0, ≤4); (0, ≤72); (3, >=48); (3, >=60); (0, ≤14); (0, ≤72); (3, >=48); (3, >=60). Lcsc; 2,3; 2,3; 2,3; 2,3; 6; 6; 6; 6. A-MPR [dB]; ≤3; ≤1; ≤1; ≤3; ≤2; ≤1; ≤1; ≤2. Note 1: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex.. Proposal: Consider above A-MPR table for the sub-PRB allocation for “NS-27” |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1 A-MPR for LTE Band 48

Sub-topic description: A-MPR for LTE band 48 NS_27 Cat-M1 UE.

Open issues and candidate options before e-meeting:

Issue 4-1-1: A-MPR for NS_27

## Companies views’ collection for 1st round

### Open issues

Issue 4-1-1: A-MPR for NS_27

Discuss the A-MPR requirements as proposed in R4-2201716, duplicated below.

| Channel BW | Parameter | Region |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | (NB_index, SCstart) | (0, ≤4) | (0, ≤72) | (3, >=48) | (3, >=60) | (0, ≤14) | (0, ≤72) | (3, >=48) | (3, >=60) |
|  | Lcsc | 2,3 | 2,3 | 2,3 | 2,3 | 6 | 6 | 6 | 6 |
|  | A-MPR [dB] | ≤3 | ≤1 | ≤1 | ≤3 | ≤2 | ≤1 | ≤1 | ≤2 |
| Note 1: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |  |  |  |

- Proposals

  - Option 1: Agree with the proposal

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 4-1 | No objection is received on the proposed A-MPR for NS_27. Tentative agreements: The proposed A-MPR for NS_27 is agreed. Candidate options: Recommendations for 2nd round: The sub-topic is closed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #5: NB-IoT 16QAM

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200415 | Nokia, Nokia Shanghai Bell | Proposal: There is no need to differentiate the NB-IoT carrier power with 16QAM and the NB-IoT carrier with QPSK. One declaration is applied when configured for 16QAM/QPSK transmissions. |
| R4-2201714 | Ericsson | Observation#1: There should be no coverage impact on legacy NB-IoT device due to the 16QAM introduction. Observation#2: new feature will be specified starting from Rel-17 and there is no NBC (non-backward compatibility issue for it. Observation#3: For new equipment, it is better to have one NB-IoT declared power to support both 16QMA and QPSK. Observation#4: Declaration on the NB-IoT carrier to support 16QAM would be fine for legacy equipment. - Observation#5: It is not necessary for RAN4 to discuss how to support the 16QAM in legacy BS as it is a NBC feature. |
| R4-2201831 | Huawei,HiSilicon | Observation 1: When upgrading, legacy equipment may use lower power for 16QAM transmissions. Observation 2: The coverage of QPSK should be maintained, regardless of the transmission power for 16QAM. Proposal 1: Further discuss the two options in the WF, and find the best solution that can maintain the QPSK coverage while meet the demand for 16QAM. |
| R4-2201832 | Huawei,HiSilicon | Draft CR to TS 36.101 The EVM requirement for 16QAM NB-IoT DL is added. |
| R4-2201833 | Huawei,HiSilicon | Draft CR to TS 36.141 The EVM requirement for 16QAM NB-IoT DL is added. |

## Open issues summary

### Sub-topic 5-1 BS RF Requirements

Sub-topic description: The impact to BS RF requirements in support of 16QAM are discussed here.

Open issues and candidate options before e-meeting:

Issue 5-1-1: Declaration of rated output power for 16QAM DL

## Companies views’ collection for 1st round

### Open issues

Issue 5-1-1: Declaration of rated output power for 16QAM DL

Discuss whether to allow manufacturers to declare different rated output power for NB-IoT 16QAM transmission in standalone mode

- Proposals

  - Option 1: One declaration is applied when configured for 16QAM/QPSK transmissions.

  - Option 2: Up to two rated output power declarations may be made.

  - Option 3: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 1; We see no need for option 2, as we stated in our paper, a suitable design for BS with NB-IoT operation in standalone mode should maintain the QPSK coverage while meet the demand for 16QAM; we also provide some suggestions for legacy BS handling in our paper without the need to have two rated output power declarations. |
| Ericsson | Option 1. Agree with Nokia. If the legacy BS is upgraded to a “Rel-17” and support 16QAM NB-IoT UE, it will not roll-back to legacy node so seems the need of option 2 is not there in practice. |
| Huawei | Thanks Nokia and Ericsson for the good discussions and constructive suggestions. We also share the view that the QPSK coverage should not be affected. Option 1 is acceptable. Hence no spec changes are needed. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201832 Draft CR to TS 36.101 | Nokia: This draft CR is to TS 36.104 but not TS 36.101. |
|  | Huawei: Thanks Nokia for pointing out the typo in the moderator’s summary. The CR cover sheet is correct. |
| R4-2201833 Draft CR to TS 36.141 | Nokia: The number of NPDSCH bits should be double from 304 to 608 and from 200 to 400 for 16QAM in clause 6.1.4.5; we propose to have an agreement to share the CR drafting workload among participating companies in February RAN4#102 meeting, Nokia volunteer to provide the CR to TS 36.141. |
|  | Huawei: For Tx power or EVM measurement, the coding rate doesn’t really matter. But we’re OK to adopt the changes proposed by Nokia. Since we already prepared the draft CR, we’re happy to continue with the formal CR. Maybe we can co-source? |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | Issue 5-1-1: It seems that the consensus is option 1. Tentative agreements: Option 1 is adopted, i.e. one declaration is applied when configured for 16QAM/QPSK transmissions. Candidate options: Recommendations for 2nd round: The sub-topic is closed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201832 Draft CR to TS 36.104 | To be endorsed. |
| R4-2201833 Draft CR to TS 36.141 | To be revised. |

## Discussion on 2nd round (if applicable)

### Open issues

| Company | Comments |
| --- | --- |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2201833 Draft CR to TS 36.141 | Nokia: Comments and suggestions made directly on draft CR. |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

The final version of the revised draft CR to TS 36.141 is agreeable. The proponent companies should bring formal CRs to the next meeting.

# Topic #6: LTE-MTC Additional Enhancements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201287 | Sony | Observation A: The power class for NB-IoT is defined for a single-subcarrier transmission. Observation B: The NB-IoT MPR specification allows the NB-IoT UE to transmit at a lower power level than the power level used for single-subcarrier transmission. Observation C: eMTC and NB-IoT can be implemented with the same RF front-end. The NB-IoT MPR values for full-PRB transmission relative to sub-PRB transmission are equally applicable to eMTC. Proposal A: The MPR framework allows a sub-PRB capable UE to apply a power reduction of full-PRB, PRACH, PUCCH and SRS relative to the power of a 2-tone sub-PRB PUSCH transmission. Proposal B: For an eMTC UE capable of sub-PRB transmission: - The eMTC power class is defined for a 2-of-3 sub-PRB transmission - The eMTC MPR specification allows the eMTC UE to transmit at a lower power level than the power level used for a 2-of-3 sub-PRB transmission - The power reduction for eMTC UEs for full-PRB transmissions is the same as the power reduction allowed for NB-IoT UEs for full-PRB transmission |
| R4-2201715 | Ericsson | Observation#1: Reducing the full-PRB transmission power generally is against the UE rated power definition. Proposal-1: Follow the framework of NR pi/2 BPSK power boosting if RAN4 decides that there is an overall gain from the subPRB boosting. Observation#2: If RAN4 decided for the subPRB power boosting, it will be possible to boost power for 2 out 3 tone subPRB transmission thanks to low PAPR characteristic. Proposal-2: Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission. Proposal-3: RAN4 could discuss the feasibility only in Rel-17 in remaining two RAN4 meetings. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 6-1 Feasibility study on max power reduction for PRACH, PUCCH, and full-PRB PUSCH

Sub-topic description: The feasibility study is continued. Effort in two different directions remains.

Open issues and candidate options before e-meeting:

Issue 6-1-1: Feasibility study on max power reduction

## Companies views’ collection for 1st round

### Open issues

Issue 6-1-1: Feasibility study on max power reduction

- Proposals

  - Option 1: Enable power boosting for sub-PRB PUSCH from a lower PC

    - Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission

  - Option 2: Define full power transmission for 2-of-3 sub-PRB and allow power reduction for full-PRB PUSCH, PRACH and PUCCH

    - Add supplementary MPR for full PRB transmissions for UE CAT-M1 PC3 and PC5

  - Option 3: Conclude the feasibility study without specification impact in Rel-17

  - Option 4: Remove the objective from the WI

  - Option 5: others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Option 2 seems more complex than option 1. E.g the A-MPR/MPR simulation assumption, how PA calibrated, how to specify the A-MPR for different bands of different NS values for such new UE power class. Whether it will have more impact on other RAN spec as the power class is a parameter used a parameter for S-criteria compensation in 36.304. Pcompensation; If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: if PPowerClass is 14 dBm: max(PEMAX1 –(PPowerClass – Poffset), 0) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) For IAB-MT, Pcompensation is set to 0.. Seems option 1 impact less, at least there is no need to re-do all A-MPR simulation for full-RB allocation. Maybe for now focusing the feasibility study as only one meeting left. |
| Sony | Our preference is option 2. Option 2 achieves the work item objective (which states twice that *power reduction* of full-PRB channels is the objective rather than power boosting of sub-PRB transmissions). Option 2 is the approach that is taken for NB-IoT in section 6.2.3F of TS36.101. It is clear that this approach works, more so given that there are many implementations that have a common RF for NB-IoT and eMTC. While option 1 seems to achieve a similar goal to the work item objective, we think this would entail more work than option 2. Option 1 is not really consistent with the WID. Option 3 would pre-suppose that power reduction of full-PRB is not feasible, since the work item objective is to “study and if found feasible specify”. If power reduction of full-PRB is feasible (as is evident from the NB-IoT specification), then the work item mandates that it will be specified. The only way that RAN4 would not specify power reduction of full-PRB is if it were found to be unfeasible (and it is clear that full-PRB power reduction is feasible). Option 4 would have to be discussed in RAN plenary rather than RAN4. Summary: Support option 2. Other options are not consistent with the WID. |
| Huawei | Both option 1 and 2 require significant effort. For option 1, new MPR/A-MPR need to be determined for boosted sub-PRB. Simulation/measurement data would be needed. For option 2, it’s effectively asking for a big relaxation on Tx power, since the current spec doesn’t allow MPR for sub-PRB transmissions of Cat-M1 UEs. Again, measurement/simulation data would be needed to support such a request as per the convention in 3GPP. And it’s not clearly how to use the proposed S-MPR. Given that only one meeting is left for the WI, the situation would lead us to option 3, i.e. two options are proposed, but no consensus is reached for the feasibility of either one. |
| Qualcomm | Option 4, this should be brought up to plenary. There has been a lot of discussion on this and so far not much technical discussions. It seems very difficult that we would be able to conclude the discussion on time. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 6-1 | Issue 6-1-1: Views are much diverged. Four companies expressed support for four different options, respectively. Little progress is achieved. Tentative agreements: Candidate options: Recommendations for 2nd round: Companies are encouraged to continue the discussions and narrow down the options. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Open issues

Issue 6-1-1: Feasibility study on max power reduction

- Proposals

  - Option 1: Enable power boosting for sub-PRB PUSCH from a lower PC

    - Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission

  - Option 2: Define full power transmission for 2-of-3 sub-PRB and allow power reduction for full-PRB PUSCH, PRACH and PUCCH

    - Add supplementary MPR for full PRB transmissions for UE CAT-M1 PC3 and PC5

  - Option 3: Conclude the feasibility study without specification impact in Rel-17

  - Option 4: Remove the objective from the WI

  - Option 5: others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Sierra Wireless | We support either option 1 or option 2. However, we prefer option 2 as it meets the WI objectives better and would require less simulation work than option 1. |
| Sony | Our preference is option 2, as stated in the first round. However, we can compromise on option 1. We think the end result will be the same whether we go with option 2 or option 1. We would appreciate concrete references to the methodologies and PA models that companies have used with option 1 (these methodologies and PA models must have been used in other MPR-related work in this meeting and in previous meetings). We appreciate the feedback from other companies in round 1 and respectfully have the following quick responses. Ericsson> We don’t envisage a change to the power class and expect there to be no change to 36.304. For option 2, reducing the power of full PRB transmissions will lead to lower ACLR and hence we think that the A-MPR specifications will not need updating for option 2. Huawei> The proposal in option 2 is to support MPR for full PRB (the proposal is not about supporting MPR for subPRB transmissions). Yes, option 2 is a proposal about reduction of TX power for full PRB transmissions. The S-MPR proposal is to add an “S-MPR” term to the definition of UE transmit power. We think that this would minimize RAN4 specification changes and would be cleaner than introducing multiple new tables for this feature. Qualcomm> In one way or another, this objective is going to be discussed at RAN plenary anyway (as the WI will finish soon anyway and the completion of the WI will be discussed at RANP). We would like RAN4 to avoid spending time on mulling whether to work on this objective or not and to actually get on with the technical work. Hence, we would like to focus on doing the work associated with option 1 (or preferably option 2). We would be OK to compromise to option 1 and look forward to constructive dialogue on option 1. |
| Huawei | Thanks Sony for the clarification. Indeed, your proposal is about extra MPR for full-PRB transmissions. Since the values are higher than existing MPR and they could impact PRACH, PUCCH, naturally concerns on cell coverage would raise. As per normal 3GPP procedure, simulation/measurement data is needed to justify the changes. Given the limited time, I’m not optimistic about the outcome of either option 1 or 2. We may end up with option 3 or 4. |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Two companies expressed continued interest in option 1 and 2. Another company remains concerned about the limited time for completion. No consensus is reached in this meeting.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combinations | Huawei, HiSilicon |  |
| WF on simultaneous Rx/Tx capability for band pairs included in higher order band combinations | NTT Docomo |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2201759 | Draft CR for n24 and n99 UL-MIMO PC3 | Ligado Networks | To be endorsed |  |
| R4-2200770 | Draft CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching | China Telecom | To be withdrawn |  |
| R4-2200566 | Clarification on per-band-pair simultaneous RxTx capability | NTT DOCOMO INC. | To be noted |  |
| R4-2200354 | Discussion on the simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination | SoftBank Corp. | To be noted |  |
| R4-2201067 | Discussion on simultaneous RxTx capability for FR1 FDD-TDD band combination | Samsung | To be noted |  |
| R4-2201230 | Discussion on principle for simultaneous Rx Tx band combinations for CA, SUL, MR-DC and NR-DC | Xiaomi | To be noted |  |
| R4-2201340 | Simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination | ZTE Corporation | To be noted |  |
| R4-2201954 | Further consideration on the MSD principle for FR1 FDD-TDD band combination | Huawei, HiSilicon | To be noted |  |
| R4-2201955 | TP for TR 38.839: Principles for simultaneous RxTx capability | Huawei, HiSilicon | To be approved |  |
| R4-2200567 | View on FR2 simultaneous Tx/Rx discussion | MediaTek Beijing Inc. | To be noted |  |
| R4-2201341 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | To be endorsed |  |
| R4-2201342 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | To be endorsed |  |
| R4-2201343 | Draft CR to TS38.101-2[R17] On Simultaneous RxTx capability for FR2 inter-band CA_n257-n259 and CA_n258-n260 | ZTE Corporation | To be postponed |  |
| R4-2201956 | Further consideration on the simultaneous Rx/Tx capability for FR2 TDD-TDD band combination | Huawei, HiSilicon | To be noted |  |
| R4-2201716 | AMPR simulation results for Cat-M1 for B48 | Ericsson | To be agreed |  |
| R4-2200415 | Proposals on BS RF requirements for support of 16QAM in NB-IoT | Nokia, Nokia Shanghai Bell | To be noted |  |
| R4-2201714 | BS RF impact analysis on R17 NB_IoT | Ericsson | To be noted |  |
| R4-2201831 | Remaining issues for NB-IoT 16QAM BS RF requirements | Huawei,HiSilicon | To be noted |  |
| R4-2201832 | Draft CR to TS36104 Addition of NB-IoT 16QAM | Huawei,HiSilicon | To be endorsed |  |
| R4-2201833 | Draft CR to TS36141 Addition of NB-IoT 16QAM | Huawei,HiSilicon | To be revised | To Chair: please add Nokia, Nokia Shanghai Bell to the source companies. |
| R4-2201287 | On max power reduction for PRACH, PUCCH, and full-PRB PUSCH | Sony | To be noted |  |
| R4-2201715 | RF impact analysis on R17 eMTC WID | Ericsson | To be noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4- 2202296 | Draft CR to TS 36.141: Addition of NB-IoT 16QAM | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | To be endorsed |  |
| R4-2202294 | WF on simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combinations | Huawei, HiSilicon | To be noted |  |
| R4-2202295 | WF on simultaneous Rx/Tx capability for band pairs included in higher order band combinations | NTT Docomo | To be agreed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents