3GPP TSG-RAN WG4 Meeting # 101-bis-e												R4-2202337

Electronic Meeting, 17 – 25 January, 2022

Agenda item:			8.8

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101-bis-e][137] LTE_Upper_700MHz

Document for:	Information

# Introduction

This summary covers agenda items 8.8 for the LTE_upper_700MHz_A work item. The following topics were identified:

- General aspects

- RF requirements

- TPs to TR 36.779

- Draft CRs

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Agree on the work-split for second round and next meeting.

  - Agree on the technical aspects for BS RF (co-ex, co-location requirements)

  - Collect comments to the TPs and draft CRs as preparation to the 2nd round corrections.

- 2nd round:

  - Aim to approve TPs to TR 36.779.

  - Aim to Endorse draft CRs to the affected specifications.

  - Identify remaining open issues (see section 2.5) to be clarified for the next RAN4#102-e meeting in February, considering the work-split and capture them in the WF.

# Topic #1: General aspects

In this topic we handle the updated TR, work-plan for the remaining meetings, as well as the work-split.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200766 | Puloli | TR 36.779: Introduction of upper 700MHz A block E-UTRA band for the US, v0.0.3, Rel-17 Moderator: v 0.0.3 of TR 36.779 is published based on this contribution. No need for discussion. To be Noted. |
| R4-2200765 | Puloli | Work plan for LTE_upper_700MHz_A WI Proposal: It is proposed to approve the work plan for LTE_upper_700MHz_A WI, as follows: - 3GPP RAN4#101-bis-e: - Complete the discussion on coexistence / co-location issues - Agree the TR skeleton - Discuss and agree relevant text proposals on regulatory framework, co-existence, band/channel numbering - Analysis of UE specific changes to TS 36.101 - Analysis of BS specific changes to TS 36.104 - Endorsement of draft CRs to TS 36.101 and TS 36.104 - 3GPP RAN4#102-e - Agree any pending relevant TP’s to TR - Agree full set of CRs. |
| R4-2202024 | Huawei | Further discussion on the BS RF requirements Proposal 2: agree on the work-spilt for implementation of related requirements in affected BS RF specifications |

## Open issues summary

### Sub-topic 1-1: Work-plan

Based on the work-plan in R4-2200765 the following is proposed:

- Proposals:

  - Option 1: By default the work-plan based on R4-2200765 is proposed to be approved. In case of any modification proposals from companies, please comment on sub-topic 1-1 in section 1.3.1.

- Recommended WF: Work-plan is proposed to be approved based on R4-2200765.

### Sub-topic 1-2: Work-split

Based on the contributions submitted to this meeting, the work-split is proposed to be applied for modifications implementation in the submitted TPs to TR, as well as in the draft CRs:

- Proposals

  - Option 1: work-split based on contribution submitted to this meeting. Allocation to the remaining specifications to be agreed this meeting. Volunteering companies are welcome to feed in, correct and comment.

| TS No. | Remarks |
| --- | --- |
| 36.101 | [Puloli] |
| 36.104 | [Huawei] |
| 36.133 | TBD |
| 37.104 | [Baicells] |
| 37.105 | [Baicells] |
| 36.141 | TBD |
| 37.141 | TBD |
| 37.145-1 | TBD |
| 37.145-2 | TBD |
| 38.101-1 | TBD |
| 38.104 | [Baicells] |
| 38.141-1 | TBD |
| 38.141-2 | TBD |

  - Option 2: Other

- Recommended WF: Option 1 as baseline (for the 2nd round, as well as for next meeting), subject to modifications.

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1: Work-plan

| Company | Comments |
| --- | --- |
| Huawei | Slight modifications proposed to account for any other spec besides TS 36.101/104: - 3GPP RAN4#101-bis-e: - Complete the discussion on coexistence / co-location issues - Agree the TR skeleton - Discuss and agree relevant text proposals on regulatory framework, co-existence, band/channel numbering - Analysis of UE specific changes to TS 36.101 - Analysis of BS specific changes to TS 36.104 - Endorsement of draft CRs to TS 36.101 and TS 36.104, and any other affected specification. - 3GPP RAN4#102-e - Agree any pending relevant TP’s to TR - Agree full set of formal of CRs. |
| Puloli | Agree with Huawei |
| Baicells | OK with Huawei’s update based on co-sourced contribution. |

Sub topic 1-2: Work-split

| Company | Comments |
| --- | --- |
| Huawei | Regarding the remaining specs the following is suggested: - UE specs: as Puloli covers the 36.101 UE spec, it may be easiest to assign UE specs to one company, - RRM spec 36.133: Huawei can sync internally with RRM team to cover it. - BS conformance testing specs (): Based on past experience with eh BS conformance specs, Huawei can volunteer to cover them all in necessary, depending on interest from other companies. Of course, other interested companies are welcome to feed in below. TS No.; Remarks. 36.101; [Puloli]. 36.104; [Huawei]. 36.133; [Huawei, TBD]. 37.104; [Baicells]. 37.105; [Baicells]. 36.141; [Huawei, TBD]. 37.141; [Huawei, TBD]. 37.145-1; [Huawei, TBD]. 37.145-2; [Huawei, TBD]. 38.101-1; TBD. 38.104; [Baicells]. 38.141-1; [Huawei, TBD]. 38.141-2; [Huawei, TBD]. |
| Puloli | Agree with Huawei that Puloli covers UE side, including both 36.101 and 38.101 TS No.; Remarks. 36.101; [Puloli]. 36.104; [Huawei]. 36.133; [Huawei, TBD]. 37.104; [Baicells]. 37.105; [Baicells]. 36.141; [Huawei, TBD]. 37.141; [Huawei, TBD]. 37.145-1; [Huawei, TBD]. 37.145-2; [Huawei, TBD]. 38.101-1; [Puloli]. 38.104; [Baicells]. 38.141-1; [Huawei, TBD]. 38.141-2; [Huawei, TBD]. |
| Baicells | As co-rapporteur, Baicells would like to provide all draft CRs to BS and related conformance testing specifications especially on 36/37 series. TS No.; Remarks. 36.101; [Puloli]. 36.104; [Baicells, Huawei]. 36.133; [Huawei, TBD]. 37.104; [Baicells]. 37.105; [Baicells]. 36.141; [Baicells, Huawei, TBD]. 37.141; [Baicells, Huawei, TBD]. 37.145-1; [Baicells, Huawei, TBD]. 37.145-2; [Baicells, Huawei, TBD]. 38.101-1; TBD. 38.104; [Baicells]. 38.141-1; [Huawei, TBD]. 38.141-2; [Huawei, TBD]. |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

Work-plan and the work-split were discussed in topic #1.

|  | Status summary |
| --- | --- |
| Sub topic 1-1: Work-plan | Candidate options: Small adjustment of the work-plan in R4-2200765 was proposed. Recommendations for 2nd round: work-plan to be revised based on comments. |
| Sub topic 1-2: Work-split | Candidate options: Work-plan inputs were collected. Recommendations for 2nd round: Based on the inputs received, the following work-split is proposed for the final set of CRs: TS No.; Remarks. 36.101; Puloli. 36.104; Baicells. 36.133; Huawei. 37.104; Baicells. 37.105; Baicells. 36.141; Baicells. 37.141; Baicells. 37.145-1; Huawei. 37.145-2; Huawei. 38.101-1; Puloli. 38.104; Baicells. 38.141-1; Huawei. 38.141-2; Huawei. Work-split is proposed to be captured in the WF. No further discussion on sub-topic 1-2 needed in the second round. |

## Discussion on 2nd round

### CRs/TPs comments collection

| Tdoc number | Comments collection |
| --- | --- |
| Revision of R4-2200765 (work-plan) | Huawei: ok |
|  | Baicells:OK |

# Topic #2: RF requirements

In this topic only one discussion paper is included for the BS RF requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2202024 | Huawei | Further discussion on the BS RF requirements Proposal 1: Define co-location and co-existence requirements for the new 700MHz NAM band in the EUTRA BS specifications. |

## Open issues summary

### Sub-topic 2-1: BS co-location and co-existence requirements

- Proposals

  - Option 1: Define co-location and co-existence requirements for the new 700MHz NAM band in the EUTRA BS specifications. (R4-2202024, Huawei)

  - Option 2: Other

- Recommended WF

  - Option 1

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1: BS co-location and co-existence requirements

| Company | Comments |
| --- | --- |
| Huawei | Option 1 |
| Puloli | Option 1 |
| Baicells | Unclear about the intention to agree on this proposal because the impacted specs in lastest WID RP-212917 already include the co-location and co-existence requirements. It may be efficient to focus discussion on TP and draftCR. |
| Nokia | While co-existence and co-location requirements can be introduced, certain exceptions need to be taken into account for legacy bands (Band 13, 14). It should be also clarified if HNB as well as MR/LA operation is considered for this band especially it is for NB-IoT only. |

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub topic 2-1 | Candidate options: Proceed with the definition of the co-location and co-existence requirements. As clarification to the WID comment: co-location/co-ex requirements were put in the WID in [] as there was no consensus at that time, whether or not they were applicable. The usual decision process in RAN4 is like: discussion –> decision –> TPs/CRs. Due to short duration of this WI, that decision taking process was not fully followed. Now the situation has changed and it is common understanding that the co-location/co-ex requirements are applicable. Therefore, RAN4 can proceed with related TPs/CRs. Recommendations for 2nd round: As the conclusion has been reached that the co-location/co-ex requirements are applicable to the set of specifications listed in the WID, the following is proposed for the second round: - Continue the discussion on set of legacy bands that should be excluded from the newly introduced co-location/co-ex requirements (new sub-topic 2-2). Consider bands 13 and 14 as starting point. - Continue the discussion on the applicable BS classes (new sub-topic 2-3): clarify whether or not requirements for HNB, Medium Range, and or Local Area BS class are needed. - Proceed with the related draft CRs, subject to the above discussions in sub-topic 2-2 and 2-3. |

## Discussion on 2nd round

### Open issues

Sub topic 2-2: legacy bands to be excluded from co-location/co-ex requirements

Continue the discussion on set of legacy bands that should be excluded from the newly introduced co-location/co-ex requirements (new sub-topic 2-2). Consider bands 13 and 14 as starting point.

| Company | Comments |
| --- | --- |
| Nokia | Exception depends on co-ex requirement and cannot be treated in a generic way. E.g. how Band 13/14 BS is supposed to meet -52dBm/1MHz requirement in the 757-758 MHz frequency range? |
| Huawei | Agree with Nokia that for the bands covering the same frequency range as band 103, there is a need to consider exceptions. As we have multiple related draft CRs under discussion, for sake of progress it is proposed to proceed with those draft CRs and capture in the WF something like: “Any required band exceptions from the co-ex requirements are to be completed by RAN4#102-e meeting. Interested companies are encouraged to provide related analyses. Related modifications to the draft CRs are to follow the agreed work-split and are to be implemented during RAN4#102-e meeting, if any”. |
| Baicells | Band 103 is adjacent to Band 13/14. There is Note 1 under additional spurious emission requirement as below. According to our understanding with this note the spurious emission is not to be applied for B13/14 in generic way. We believe that's why there is no explicit exception note in table 6.6.4.3.1-1 between band 13 and band 14 which may also fall in the 10MHz range immediately outside DL band of each other. NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.4.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Table 5.5-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. |

Sub topic 2-3: BS classes

Continue the discussion on the applicable BS classes (new sub-topic 2-3): clarify whether or not requirements for HNB, Medium Range, and or Local Area BS class are needed.

| Company | Comments |
| --- | --- |
| Huawei | This topic was not clarified so far. The default approach may be to follow the same arrangement as for the other NB-IoT bands for US. As we have multiple related draft CRs under discussion, for sake of progress it is proposed to proceed with those draft CRs and capture in the WF something like: “BS classes considered for band 103 implementation are to be confirmed by RAN4#102-e meeting. Interested companies are encouraged to provide related analyses. Related modifications to the draft CRs are to follow the agreed work-split and are to be implemented during RAN4#102-e meeting, if any”. |
| Baicells | All BS classes are needed for different NB-IoT deployment scenarios. |
| Puloli | Agree with Baicells that all BS classes are needed for industrial IoT deployment scenarios |

Sub topic 2-4: UEtoUE co-ex

If needed, provide further inputs to the discussion on the UEtoUE co-existence. In particular, consider bands 24 and 30.

| Company | Comments |
| --- | --- |
| Huawei | We would suggest to keep it FFS till next meeting. As we have multiple related draft CRs under discussion, for sake of progress it is proposed to proceed with those draft CRs and capture in the WF something like: “Any potential UEtoUE co-ex aspects are to be concluded by RAN4#102-e meeting. Interested companies are encouraged to provide related analyses. Related modifications to the draft CRs are to follow the agreed work-split and are to be implemented during RAN4#102-e meeting, if any”. |
| Puloli | Band 24 and 30 are added to the band 103 protected band as Nokia suggested. |

# Topic #3: TPs to TR 36.779

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200771 | Puloli | TP to TR 36.779: Frequency band arrangements and regulatory background TP covering the following sections: - 5.1 Frequency band and channel bandwidth - 5.2 Adjacent 3GPP bands - 5.3 Nearby non-3GPP Services - 5.4 FCC regulatory requirements |
| R4-2200772 | Puloli | TP to TR 36.762: Operating band, channel bandwidths, channel numbering Moderator: - Wrong TR number on the cover page. - According to RAN4 practices, it is unnecessary and discouraged to repeat the spec content in the TR. Therefore, for this TP to TR, it is suggested to: - Based on section 3 content: reuse it as input to draft CR to TS 36.101 - Based on section 6 content: modify and simplify it to focus on related background, motivation, decisions, and to use it as TP to TR 36.779. |
| R4-2200774 | Puloli | TP to TR 36.762: Analysis on UE requirements for upper 700MHz A block in the US Moderator: - Wrong TR number on the cover page. - Same comments as to R4-2200772. |
| R4-2200461 | Baicells | TP to TR 36.779 on BS aspect issues Moderator: tdoc not available |
| R4-2201071 | Baicells | TP to TR 36.779 on BS aspect issues Moderator: According to RAN4 practices, it is unnecessary and discouraged to repeat the spec content in the TR. Therefore, for this TP, it is suggested to modify and simplify it in the revision to focus on related background, motivation, decisions. |

## Open issues summary

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200771 | Huawei: some text refinements are proposed in the revision in: R4-2200771 TP to TR 36.779 on Frequency band arrangements and regulatory background_ms.docx Similar as comments last time, the AAT > 305m does not seem to be relevant deployment, or? |
|  | Puloli: Agree with Huawei’s revision. The “AAT > 305m” is captured from FCC requirements. This may be removed if it is confusing. |
|  | Baicells: We think it is better to keep the regulation description same as FCC requirements. |
| R4-2200772 | Huawei: revision needed to fix the TR number. Some text refinements are proposed in the revision in: R4-2200772 TP to TR 36.779 on Operating band, channel bandwidths, channel numbering_ms.docx |
|  | Puloli: Agree with Huawei’ revision |
|  | Company C |
| R4-2200774 | Huawei: some text refinements are proposed in the revision in XX: R4-2200774 TP to TR 36.779 on Analysis on UE requirements for upper 700MHz A block_ms.docx |
|  | Puloli: Agree with Huawei’ revision |
| R4-2201071 | Huawei: simplify by removing spec content from TR. Instead add elaborated text to reflect the technical decision. Possibly list the affected RF requirements (no TS copy-paste). |
|  | Baicells: Thanks to comment shared beforehand from Moderator. We understand the recommendation. However, it is believed this kind of exception is allowed for TR to introduce specific band as this would facilitate the understanding. Since the band# is just confirmed in last Friday, anyway, this TP should be revised to address this. We are open for other technical comment and willing to revise this according to agreement. Nokia: how about exceptions for other bands for co-ex? is HNB considered for this band especially it is for NB-IoT? how about LA and MR? Band 102 is already reserved |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200771 | to be revised |
| R4-2200772 | to be revised |
| R4-2200774 | to be revised |
| R4-2201071 | to be revised |

## Discussion on 2nd round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2200771 | Huawei: looks ok. |
|  | Baicells:OKwith current version. |
| Revision of R4-2200772 | Huawei: looks ok. |
|  | Baicells:OKwith current version. |
| Revision of R4-2200774 | Huawei: looks ok. |
|  | Baicells:OKwith current version. |
| Revision of R4-2201071 | Huawei: formal comment: tdoc name shall not be modified during the meeting, unless this is coordinated with RAN4 secretary. Couple of corrections were uploaded in the revision. |
|  | Baicells: OK with the corrections from Huawei. As for the tdoc name, the reserved name is TP to TR 36.779 on Analysis on BS requirements for upper 700MHz A block, but a wrong version was submitted. We are OK to use previous name if cannot be corrected. |
|  | Puloli: Looks OK |

# Topic #4: Draft CRs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200004 | Puloli | Introduction of upper 700MHz A block into TS 36.101 Moderator: tdoc not available. |
| R4-2200790 | Puloli | CR to TS 36.101: Introduction of upper 700MHz A block into TS 36.101 Moderator: - According to the RAN4 rules, we should follow the “draft CR” approach to collect all the modificaitons to the spec, before triggering formal CR. Therefore this CR will be proposed to be marked as “not pursued”. - Content of this tdoc is proposed to be used as baseline for the draft CR to TS 36.101 during this meeting. |
| R4-2200457 | Baicells | DraftCR to 36.104: introduction of LTE upper 700MHz band Moderator: there is another draft CR to TS 36.104 in R4-2202025. Works-split to be applied before proceeding. |
| R4-2200458 | Baicells | DraftCR to 37.104: introduction of LTE upper 700MHz band |
| R4-2200459 | Baicells | DraftCR to 37.105: introduction of LTE upper 700MHz band |
| R4-2200460 | Baicells | DraftCR to 38.104: introduction of LTE upper 700MHz band |
| R4-2202025 | Huawei | Draft CR to TS 36.104: exemplary implementation of LTE_upper_700MHz_A band Moderator: there is another draft CR to TS 36.104 in R4-2200457. Works-split to be applied before proceeding. |

## Open issues summary

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200790 | Huawei: work-split to be agreed first, before we dive into draft CRs corrections during the second round. This is formal CR – this is not allowed at this stage. New tdoc number needs to be assigned for the “draft CR”. Correct [102] to band 103. Other than that, the technical content seems ok, but we would like to have more time also during the second round for cross-check. |
|  | Puloli: This tdoc was originally started by preparing a formal CR when R4-2200004 (withdrawn) had been reserved. A new tdoc (R4-2200790) for the draft CR was used as formal CR is not appropriate / allowed for this meeting. The CR number will be removed to reflect the fact that it is a draft CR; together with other changes advised by Huawei. |
|  | Baicells: The band# should update which is applicable to our draft as well. Furthermore, we are open to discuss if the alignment between 36.101 and 36.104 should be considered and if the new NB-IoT standalone band should be introduced in table 5.6.1-1. Btw, according to our understanding of RAN4 leadership guideline in R4-2200003, it seems formal CR is not strictly precluded for spectrum related WI. Hence it would fine for us if we can continue discuss the revision to this CR during this meeting as this quarter is already the end of this WI. |
|  | Nokia: 102 is already taken by NR. There is an agreement that same number can be used for LTE and NR when frequency range matches exactly. Hence unique band number is needed for this one. UEtoUE co-ex this new band need to protect also bands 24 and 30. |
|  | Puloli: @Nokia The reasons why Band 24 and 30 were not included are given in #2 and #3; respectively: - Band # 103 was just assigned for this band. Puloli will update document with the number assigned - UEtoUE co-ex for Band 24: the 2nd harmonics for band 103 is 1574 - 1576 MHz, the DL frequency for band 24 is 1525 – 1559 MHz. Based on Note 2 of Table 6.6.3.2-1, with the additional “1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission”, and the range is 1573 – 1577 MHz. It seems that Band 24 is not applicable. - UEtoUE co-ex for Band 30: the 3nd harmonics for band 103 is 2361 - 2364 MHz. Based on Note 2 of Table 6.6.3.2-1, with the additional “1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission”, and the range is 2360 – 2365 MHz. The DL frequency for band 30 is 2350 – 2360 MHz. It seems that Band 30 is not applicable either; though they are adjacent. |
| R4-2200457 | Huawei: work-split to be agreed first, before we dive into draft CRs corrections during the second round. Depending on the work-split arrangement: As there are two draft CRs to TS 36.104, modifications from one of them are to be considered in the revised version of the other one. Therefore, draft CR to be revised. |
|  | Puloli: Agree with Huawei |
|  | Baicells: As mentioned in topic 1-2 we would like to take our version as baseline. And welcome technical comment. We do believe this version can be technically endorsed directly since all necessary updates have been addressed correctly. Nokia: how about exceptions for other bands for co-ex? is HNB considered for this band especially it is for NB-IoT? how about LA and MR? Band 102 is already reserved |
| R4-2200458 | Huawei: Correct [102] to band 103. Draft CR to be revised. For Table 7.5.2-1: there is note 2 on band 13. We would like to check during the second round on the background past decision behind the band 13 exclusion. This approach shall be aligned among the BS specs (it was noted that for 37.105 there is no such Band 13 exception applicable) |
|  | Baicells: Will also check the note 2 on Band 13. Nokia: how about exceptions for other bands (Band 13, 14) for co-ex? |
| R4-2200459 | Huawei: Correct [102] to band 103. Draft CR to be revised. Same technical comment as to R4-2200458 above. |
|  | Puloli: Band 103 needs to be added to Table 9.7.6.4.3.2-3 |
|  | Baicells: Will check and revise the draft CR. |
| R4-2200460 | Huawei: Correct [102] to band 103. Draft CR to be revised. Nokia: how about exceptions for other bands (Band 13, 14) for co-ex? |
| R4-2202025 | Huawei: Correct [102] to band 103. Draft CR to be revised. Refer to the work-split decision. |
|  | Puloli: Band 103 needs to be added to Table 6.6.4.3.1-3 |
|  | Baicells: - unnecessary update to 6.6.3.1 and 6.6.3.2.1 which don't apply for NB-IoT standalone. - FCC regulation to protection to 700M public spectrum is missing. - unnecessary update to receiver blocking requirements. Nokia: why cat b option 1 is considered? how about exceptions for other bands for co-ex? is HNB considered for this band especially it is for NB-IoT? how about LA and MR? for blocking there are comments on further updates but it is not clear how they would be updated since it is for stand-alone operation only. |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200790 | To be noted; new tdoc for related Draft CR to be allocated. |
| R4-2200457 | To be revised; any applicable comment to R4-2202025 shall be also considered. |
| R4-2200458 | To be revised |
| R4-2200459 | To be revised |
| R4-2200460 | To be revised |
| R4-2202025 | To be noted |

## Discussion on 2nd round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| dCR for TS 36.101 | Huawei: tdoc name corrected; I will inform MCC and coordinate. |
| Revision of R4-2200457 | Baicells: At Table 7.6.2.1-1, Table 7.6.2.1-2 and Table 7.6.2.1-3 in 36.104, there are same Note 1 related to Band 13. “Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz.” For Band 103, the frequency range of 10 MHz immediately outside any of the supported uplink operating band is 777MHz-798MHz, there is no DL operations at this range, so Band 103 just follow the generic way, no need to update. |
| Revision of R4-2200458 | Align with 36.104, no need to update the note 2 at Table 7.5.2-1. |
| Revision of R4-2200459 | Baicells: 1. Response to Puloli: Band 103 needs to be added to Table 9.7.6.4.3.2-3 =>Baicells: Table 9.7.6.4.3.2-3 applies to the AAS BS operation. In section 4 of 37.105, there is a description that “NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation is not supported by AAS BS. ”, so no need to add band 103 in Table 9.7.6.4.3.2-3. 2. Align with 36.104, no need to update the note 2 at Table 7.5.2.2-1, Table 10.6.2.2-1, Table 10.6.3.2-1 and Table 10.6.4.2-1. |
| Revision of R4-2200460 |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| Draft CR to TS 36.101: Introduction of upper 700MHz A block into TS 36.101 | Puloli | CR content in R4-2200790 as baseline |
| WF on the remaining RF issues for upper 700MHz A block E-UTRA band | Huawei | Placeholder for remaining issues to be discussed and arranged next meeting, plus work-split. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200766 | TR 36.779: Introduction of upper 700MHz A block E-UTRA band for the US, v0.0.3, Rel-17 | Puloli | Noted |  |
| R4-2200765 | Work plan for LTE_upper_700MHz_A WI | Puloli | Revised | Moderator: Please update the work-plan based on comments. Additionally, it is suggested to include the agree work-split into the same revised tdoc. |
| R4-2200771 | TP to TR 36.779: Frequency band arrangements and regulatory background | Puloli | Revised | Moderator: for all the revisions, consider band number 103. |
| R4-2200772 | TP to TR 36.762: Operating band, channel bandwidths, channel numbering | Puloli | Revised |  |
| R4-2200004 | Introduction of upper 700MHz A block into TS 36.101 | Puloli | Withdrawn | It was not uploaded. |
| R4-2200774 | TP to TR 36.762: Analysis on UE requirements for upper 700MHz A block in the US | Puloli | Revised |  |
| R4-2200790 | CR to TS 36.101: Introduction of upper 700MHz A block into TS 36.101 | Puloli | Not Pursued |  |
| R4-2200457 | DraftCR to 36.104: introduction of LTE upper 700MHz band | Baicells | Revised | Any applicable comment to R4-2202025 shall be also considered. |
| R4-2200458 | DraftCR to 37.104: introduction of LTE upper 700MHz band | Baicells | Revised |  |
| R4-2200459 | DraftCR to 37.105: introduction of LTE upper 700MHz band | Baicells | Revised |  |
| R4-2200460 | DraftCR to 38.104: introduction of LTE upper 700MHz band | Baicells | Revised |  |
| R4-2200461 | TP to TR 36.779 on BS aspect issues | Baicells | Withdrawn | It was not uploaded. |
| R4-2201071 | TP to TR 36.779 on BS aspect issues | Baicells | Revised |  |
| R4-2202024 | Further discussion on the BS RF requirements | Huawei | Noted |  |
| R4-2202025 | Draft CR to TS 36.104: exemplary implementation of LTE_upper_700MHz_A band | Huawei | Noted |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202389 | Draft CR to TS 36.101: Introduction of upper 700MHz A block into TS 36.101 | Puloli | Agreeable | Tdoc name correction to be coordinated with MCC. |
| R4-2202388 | WF on the remaining RF issues for upper 700MHz A block E-UTRA band | Huawei | Agreeable |  |
| R4-2202390 | Work plan for LTE_upper_700MHz_A WI | Puloli | Agreeable |  |
| R4-2202391 | TP to TR 36.779: Frequency band arrangements and regulatory background | Puloli | Agreeable |  |
| R4-2202392 | TP to TR 36.762: Operating band, channel bandwidths, channel numbering | Puloli | Agreeable | Tdoc name correction (TR number) to be coordinated with MCC. |
| R4-2202393 | TP to TR 36.762: Analysis on UE requirements for upper 700MHz A block in the US | Puloli | Agreeable | Tdoc name correction (TR number) to be coordinated with MCC. |
| R4-2202394 | DraftCR to 36.104: introduction of LTE upper 700MHz band | Baicells | Agreeable |  |
| R4-2202395 | DraftCR to 37.104: introduction of LTE upper 700MHz band | Baicells | Agreeable |  |
| R4-2202396 | DraftCR to 37.105: introduction of LTE upper 700MHz band | Baicells | Agreeable |  |
| R4-2202397 | DraftCR to 38.104: introduction of LTE upper 700MHz band | Baicells | Agreeable |  |
| R4-2202398 | TP to TR 36.779 on BS aspect issues | Baicells | Agreeable |  |

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Michal Szydelko | Michal.szydelko@gmail.com |
| Baicells | Xiang Yun | yunxiang@baicells.com |
| Puloli | Heng Pan | heng.pan@puloli.com |