3GPP TSG-RAN WG4 Meeting # 101-bis-e												R4-2202576

Electronic Meeting, 17 – 25 January, 2022

Agenda item:			6.23

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [101-bis-e][225] NR_IIOT_URLLC_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

This email discussion covers following agenda item in the RAN4#101 e-meeting:

6.23	Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

* Incoming LS from RAN1: R1-2112834 LS on propagation delay compensation

6.23.1	General	[NR_IIOT_URLLC_enh-Core]

6.23.2	RRM core requirements	[NR_IIOT_URLLC_enh-Core]

6.23.2.1	Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

6.23.2.2	Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

6.23.2.3	Others	[NR_IIOT_URLLC_enh-Core]

Based on the company contributions in this meeting there will be a discussion on each of the topics:

- Propagation delay compensation enhancements

  - LS

- Reference point for Te requirements

  - CR

  - LS

  - Side condition discussion

  - test case update

During the discussion in RAN4#101 meeting a number of agreements were reached related to the topics under discussion (R4-2120335). These are listed next and at each topic discussion, to further facilitate the discussion and agreements:

- Topic #1: Propagation delay compensation enhancements

Sub-topic 1-1, Issue 1-1:

- Agreements

  - There is no consensus in RAN4 whether it is feasible to support a smaller value than the current Te assuming the existing conditions in TS 38.133 for Te requirement

Sub-topic 1-2, Issue 1-2:

- Agreements

  - It is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under assumption of using TRS (or other RS used for Te estimation) instead of SSB

    - Smaller Te can be achieved when TRS (or other RS) bandwidth is larger than SSB bandwidth

    - Smaller Te can be achieved for UE is operating in RRC_CONNECTED mode

    - FFS whether a smaller Te can be achieved for the first transmission in the DRX cycle

      - Further check with RAN1 if presence of TRS (or other RS) can be guaranteed during the DRX OFF

      - Support of Smaller Te can be defined as an optional Rel-17 capability and should not apply to all UEs

  - Agreements

    - It is feasible to introduce enhanced TA command indication granularity

    - It is RAN4 understanding that

      - Enhanced granularity will apply for UE supporting PDC

      - This agreement does not impact UL timing accuracy requirement

    - TA command indication granularity can be reduced to 64Tc for both 15kHz SCS and 30kHz SCS.

      - RAN4 will further discuss whether there are any specific conditions to use improved TA command indication granularity

  - Issue 1-6-2: Channel model

    - Use the same channel model as used in the test case: LOS/AWGN channel

  - Issue 1-6-3: SINR

    - Moderator suggest to at least use same SINR as used in the test case. Other SINR levels are not prohibited.

  - Issue 1-8-1: Applicability rules for new TA command indication granularity

    - New TA command indication granularity is only applicable to UEs supporting PDC and it is understood that granularity and accuracy are not the same. Hence, this agreement does not impact UL timing requirement.

  - Issue 1-8-2: Conditions for evaluating the TA command indication granularity

    - Conditions for evaluating the TA command indication granularity, use at least 15KHz SCS and 30KHz SCS.

      - Note: this is not related to UL timing requirements.

- Topic #2: Reference point for Te requirements

  - Do not define UE timing error requirements side conditions (rely on the test case side conditions)

During the first round of discussions companies are invited to give their view on both Topic#1 and Topic#2s. Once agreements have been reached CR and LSs content can be discussed in detail.

Earlier agreed documents

RAN4#101:

- R4-2120335, WF on NR_IIOT_URLLC_enh_RRM

RAN4#100bis:

- R4-2115371, WF on NR_IIOT_URLLC_enh_RRM

RAN4#99:

- R4-2108368, WF on RRM for NR IIoT and URLLC,

# Topic #1: Propagation delay compensation enhancements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201168 | OPPO | Proposal 1: Reduced Te and enhanced TA command granularity should not be discussed in RAN4. Proposal 2: Reuse the existing Rx-Tx time difference measurement accuracy requirements for positioning to define the following requirements for RTT-based PDC: - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking: discuss simulation assumptions especially on CSI-RS configuration - UE Rx-Tx time difference measurement accuracy based on PRS: reuse the existing accuracy requirements for higher Es/Iot side condition (i.e. -3dB) in clause 10.1.25 - gNB Rx-Tx time difference measurement accuracy based on SRS: reuse the existing accuracy requirements for higher Es/Iot side condition (i.e. +3dB) in clause 13.2.2 |
| R4-2201368 | vivo | Proposal 1: No measurement period requirements are specified for RTT-based PDC. Proposal 2: Accuracy requirements based on measurements with one sample are specified for RTT-based PDC. Proposal 3: Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS. Proposal 4: Accuracy requirements of gNB Rx-Tx time difference measurement based on SRS in Rel-16 is reused for RTT-based PDC. Proposal 5: Accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17 for reduced number of samples can be reused as much as possible for RTT-based PDC. Proposal 6: Accuracy requirements of UE Rx-Tx time difference measurement based on CSI-RS for tracking would be derived based on simulation campaign. |
| R4-2201584 | Ericsson | - Already at SCS = 15 kHz, a BW of 44 PRB and Ês/Iot ≥ -13 dB will meet the control-to-control use case requirement of 145 ≤ Errorclock_sync ≤ 275 ns, using existing release-16 requirements for gNB and UE Rx-Tx time difference. PRS and SRS bandwidths are flexible and we can reach an RTT Errorclock_sync accuracy of 165 ns with the bandwidths and the requirements already specified in TS 38.133 for gNB Rx - Tx time difference and UE Rx – TX time difference. - Reuse UE Rx-Tx time difference measurement accuracy based on PRS and gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC . - Down prioritize RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. |
| R4-2201633 | Huawei, HiSilicon | Proposal 1: Measurement requirements for UE Rx-Tx is defined based on single-shot measurement. Proposal 2: No scheduling restriction or MG is required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP. Proposal 3: The existing reporting range for UE and gNB Rx-Tx is re-used for PDC. Proposal 4: UE and gNB Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel. Proposal 5: RAN4 to re-use the Rel-16 gNB Rx-Tx accuracy requirements at 3dB for PDC. Proposal 6: RAN4 to agree on simulation assumptions for UE Rx-Tx measurement based on Annex B. |
| R4-2201633 | Huawei, HiSilicon | Draft CR |
| R4-2201723 | Nokia | Proposal 1: The side conditions defined in TR 38.133 - 10.1.25.2 should also apply to the UE Rx-Tx time difference measurement accuracy requirements based on TRS. Proposal 2: For UE Rx-Tx time difference measurement accuracy requirements based on TRS for Rel-17 PDC RTT method, link simulation study is needed. Proposal 3: Simulation assumptions for UE Rx-Tx time difference measurement accuracy based on TRS for Rel-17 PDC RTT method: (Moderator: see tables 1 and 2 in R4-2201723) Proposal 4: Reuse the side conditions and the accuracy requirements respective to the serving cell defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method. The single-cell UE Rx-Tx time difference measurement accuracy requirements with PRS are (Moderator: see tables 3 and 4 in R4-2201723) Proposal 5: Reuse the test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC RTT-based method. Proposal 6: Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method. The single-cell gNB Rx-Tx time difference measurement accuracy requirements based on SRS are (Moderator: see table 5 in R4-2201723) Proposal 7: For Rel-17 single-cell PDC UE Rx-Tx report for both TRS based method or PRS based method, in case gNB side PD estimation is supported, the UE Rx-Tx measurement report value range should only be positive and designed for the maximum supported distance to the serving cell. Proposal 8: For Rel-17 single-cell PDC gNB Rx-Tx report, in case UE side PD estimation is supported, the gNB Rx-Tx measurement report value range can be negative and positive. A smaller range, e.g. equivalent to cyclic prefix length, should be sufficient since UL subframe transmissions received in gNB are time aligned with DL subframe. |
| R4-2201781 | MediaTek | Observation 1: There is no new PRS signal structure from RAN1 for the RTT-based method propagation delay compensation. Observation 2: The existing UE Rx-Tx time difference measurement accuracy for PRS covers the baseband only, where the RF part is still under discussion. Observation 3: The existing gNB Rx-Tx time difference measurement accuracy for SRS covers both the baseband and RF impediments. Observation 4: There is no UE Rx-Tx time difference measurement accuracy defined yet for TRS signals. Observation 5: Simulations are needed to estimate the baseband accuracy for the UE Rx-Tx time difference measurement accuracy using TRS signals. Also, we have the following proposals: Proposal 1: Support reusing the UE Rx-Tx time difference measurement accuracy of PRS for the RTT-based method PDC for both baseband and RF accuracy. Proposal 2: For the UE Rx-Tx time difference measurement accuracy of PRS, RAN4-URLLC shall wait for RAN4-positioning to finalise the accuracy requirements. Proposal 3: Support reusing the gNB Rx-Tx time difference measurement accuracy of SRS for the RTT-based method PDC in URLLC. Proposal 4: Support conducting simulations to find the accuracy of the baseband UE Rx-Tx time difference of TRS signal. Proposal 5: For the RF accuracy part of the UE Rx-Tx time difference using TRS, RAN4 to leverage the RF accuracy defined for PRS, yet frequency range and reference signal bandwidth shall be taken into consideration when RF accuracy defined for TRS signals. |

## Open issues summary

Based on the questions from RAN1 and the company contributions for this RAN4 meeting, following open issues have been identified for discussion:

- Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking? (Sub-topic 1-1)

- Reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC? (Sub-topic 1-2)

- Reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC? (Sub-topic 1-3)

- Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP (Sub-topic 1-4)

- MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP (Sub-topic 1-5)

- Measurement samples (sub-topic 1-6):

  - Measurement requirements for UE Rx-Tx is defined based on single-shot measurement

  - Accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17 for reduced number of samples can be reused as much as possible for RTT-based PDC

- Side conditions – UE (Sub-topic 1-7, Issue 1-7-1):

  - Reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method.

  - UE Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel

- Side conditions – gNB (Sub-topic 1-7, Issue 1-7-2):

  - Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method

  - gNB Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel

- Simulation assumptions (Sub-topic 1-8):

  - Simulation assumptions for UE Rx-Tx measurement based on Annex B of R4-2201633

  - Simulation assumptions for UE Rx-Tx time difference measurement accuracy based on TRS for Rel-17 PDC RTT method: tables 1 and 2 in R4-2201723

  - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking: discuss simulation assumptions especially on CSI-RS configuration

- Measurement accuracy requirements:

  - gNB (Sub-topic 1-9, Issue 1-9-1):

    - Re-use the Rel-16 gNB Rx-Tx accuracy requirements at 3dB for PDC

    - Reuse the existing serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method. The single-cell gNB Rx-Tx time difference measurement accuracy requirements based on SRS are: see table 5 in R4-2201723

  - UE (Sub-topic 1-9, Issue 1-9-2):

    - Reuse the accuracy requirements defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method. The single-cell UE Rx-Tx time difference measurement accuracy requirements with PRS are: see tables 3 and 4 in R4-2201723.

    - For the UE Rx-Tx time difference measurement accuracy of PRS, RAN4-URLLC shall wait for RAN4-positioning to finalise the accuracy requirements

    - Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS

- UE Rx-Tx measurement report value range (Sub-topic 1-10, Issue 1-10-1)

  - Use the existing reporting range for UE Rx-Tx is re-used for PDC?

  - For Rel-17 single-cell PDC UE Rx-Tx report for both TRS based method or PRS based method, in case gNB side PD estimation is supported, the UE Rx-Tx measurement report value range should only be positive and designed for the maximum supported distance to the serving cell.

- gNB Rx-Tx measurement report value range (Sub-topic 1-10, Issue 1-10-2)

  - For Rel-17 single-cell PDC gNB Rx-Tx report, in case UE side PD estimation is supported, the gNB Rx-Tx measurement report value range can be negative and positive. A smaller range, e.g. equivalent to cyclic prefix length, should be sufficient since UL subframe transmissions received in gNB are time aligned with DL subframe.

- Link simulations (Sub-topic 1-11):

  - For UE Rx-Tx time difference measurement accuracy requirements based on TRS for Rel-17 PDC RTT method, link simulation study is needed

  - Accuracy requirements of UE Rx-Tx time difference measurement based on CSI-RS for tracking would be derived based on simulation campaign

- Test case (Sub-topic 1-12):

  - Reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS based RTT method

### Sub-topic 1-1

Sub-topic description: Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking

Open issues and candidate options before e-meeting:

It has been proposed to down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. Is it this agreeable?

Issue 1-1-1: Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking?

- Proposals

  - Option 1: Yes, RAN4 should down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking.

  - Option 2: No, RAN4 should not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking.

- Recommended WF

  - Discussion needed

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Issue 1-1-1 | Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking |
| vivo | We support option 1. |
| Ericsson | We support option 1. |
| MediaTek | RAN4 shall define the UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. Support Option 2. |
| Nokia | Option 2. We don’t see the reason for the proposed down prioritization. In our view, specification of CSI-RS for tracking for RTT-based PDC should be of equal relevance or even higher relevance, seen that the majority of UEs should support CSI-RS for tracking kind of reference signal anyway, while PRS is mainly supported by UEs with positioning features. |
| Qualcomm | RAN1 asked RAN4 in the LS to specify measurement accuracy requirements based on both PRS and CSI-RS. We do not see a strong reason to deprioritize requirements for CSI-RS, although it will require more effort in RAN4 in comparison to PRS. Since RTT-based PDC is a stand-alone feature, independent of positioning, we understand that it would be beneficial to specify requirements based on a legacy signal like TRS, that does not have dependency on NR positioning. |
| Huawei | Option 2 There is no clear reason to de-prioritize TRS based RTT. |

### Sub-topic 1-2

Sub-topic description Reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC

Open issues and candidate options before e-meeting:

Proposal is to reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC. Is this agreeable?

Issue 1-2-1: Reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC?

- Proposals

  - Option 1: Yes, RAN4 should reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC

  - Option 2: No, RAN4 should not reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC

- Recommended WF

  - Discussion needed.

Sub topic 1-2

| Company | Comments |
| --- | --- |
| Issue 1-2-1 | Reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC? |
| vivo | Firstly, there is no accuracy requirements for TRS based UE Rx-Tx time difference. For PRS based UE Rx-Tx time difference, the accuracy requirements should be based on Rel-17 accuracy requirements, which would be defined with single shot measurement. So, we support option 2. |
| Ericsson | Option 1, for PRS from release 16 specification for RTT-based PDC. We agree with Vivo that there are no TRS based UE Rx-Tx time difference available to reuse. Moreover it is not clear that rel-17 will be mandatory. We are fine to reuse rel-17 values, if these are available in time for PDC to be defined in this WI and if they are mandatory. |
| MediaTek | RAN4 has no existing requirements of UE Rx-Tx time difference measurement accuracy based on TRS, hence the Options should be revised with removing the word TRS. Given that there is no signal structure for PRS from RAN1, hence we support Option 1 with re-writing it as: Option 1: Yes, RAN4 should reuse UE Rx-Tx time difference measurement accuracy based on TRS/ PRS from release 16 specification for RTT-based PDC. |
| Nokia | We think the UE Rx-Tx time difference measurement accuracy based on PRS from Rel-16 specification CAN be reused for RTT-based PDC. |
| XXXQualcomm | We favor option 2. First, RAN4 should discuss the measurement conditions/assumptions for RTT-based PDC. PDC is a different application and we think that new requirements should be derived from simulations based on conditions that are targeted to the application. Also note that Rel-16 UE Rx-Tx measurement accuracy requirements have not been completed yet, RF margins are still TBD. Finally, for RTT-based PDC using TRS, requirements cannot be leveraged directly from Rel-16. |
| Huawei | FFS, we need to first agree on the sample number and side condition (Es/Iot, propagation) for the UE Rx-Tx for PDC. If it is same as Rel-16 PRS accuracy requirements, then we agree we can reuse the Rel-16 requirements for PRS based UE Rx-Tx. For TRS based UE Rx-Tx, we anyway need to define new requirements. |

### Sub-topic 1-3

Sub-topic description Reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC

Open issues and candidate options before e-meeting:

It has been proposed to reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC. Is this agreeable?

Issue 1-3-1: Reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC?

- Proposals

  - Option 1: Yes, RAN4 can reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC

  - Option 2: No, RAN4 cannot reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC

- Recommended WF

  - Discussion needed.

Sub topic 1-3

| Company | Comments |
| --- | --- |
| Issue 1-3-1 | Reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC |
| vivo | Support option 1. |
| Ericsson | We support option 1. |
| MediaTek | Given that there is no signal structure for SRS from RAN1, hence we support Option 1. |
| Nokia | We support option 1. |
| Qualcomm | For gNB Rx-Tx time difference it may be possible to leverage the existing measurement accuracy requirements in Rel-16. We assume that for RTT-based PDC, Rel-16 SRS for positioning will be used. FFS if the side condition needs to be revisited. |
| Huawei | Fine with option 1. |

### Sub-topic 1-4

Sub-topic description Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Open issues and candidate options before e-meeting:

It has been proposed that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Issue 1-4-1: Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

- Proposals

  - Option 1: Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

  - Option 2: Scheduling restriction is required for UE Rx-Tx measurement

- Recommended WF

  - Discussion needed.

Sub topic 1-4

| Company | Comments |
| --- | --- |
| Issue 1-4-1 | Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP |
| vivo | Support option 1. |
| Ericsson | Support option 1. |
| MediaTek | Not clear to us. Need further study. |
| Nokia | We think option 1 is true in principle. But we need to further check the PRS/TRS assumptions. |
| Qualcomm | Assuming that the UE needs to measure PRS transmitted by the serving TRP, we can support option 1, subject to UE capability which may include a UE PRS processing capability for RTT-based PDC. |
| Huawei | Option 1. |

### Sub-topic 1-5

Sub-topic description MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Open issues and candidate options before e-meeting:

It has been proposed that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Issue 1-5-1: MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

- Proposals

  - Option 1: MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

  - Option 2: MG is required for UE Rx-Tx measurement

- Recommended WF

  - Discussion needed.

Sub topic 1-5

| Company | Comments |
| --- | --- |
| Issue 1-5-1 | MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP |
| vivo | Support option 1. |
| Ericsson | Support option 1. |
| MediaTek | Need further study. |
| Nokia | We think option 1 is true in principle. But we need to further check the PRS/TRS assumptions. |
| Qualcomm | Assuming that the UE needs to measure PRS transmitted by the serving TRP, we can support option 1, subject to UE capability which may include a UE PRS processing capability for RTT-based PDC. |
| Huawei | Option 1. |

### Sub-topic 1-6

Sub-topic description Measurement samples

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed different assumptions to be used for the number of measurement samples for deriving the accuracy requirements. RAN4 should discuss the proposed options and agree on the assumed number of samples used for deriving the requirements.

Issue 1-6-1: Measurement samples

- Proposals

  - Option 1: Measurement requirements for UE Rx-Tx is defined based on single-shot measurement

  - Option 2: Accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17 for reduced number of samples can be reused as much as possible for RTT-based PDC

- Recommended WF

  - Discussion is needed.

Sub topic 1-6

| Company | Comments |
| --- | --- |
| Issue 1-6-1 | Measurement samples |
| vivo | For option 2, we also think the accuracy should be based on 1 sample measurement. Therefore option 1 and 2 could be the same. |
| Ericsson | We prefer to reuse rel-16, if possible. It is not clear that rel-17 will be mandatory. We are fine to reuse rel-17 values, if these are available in time for PDC to be defined in this WI and if they are mandatory. |
| MediaTek | We prefer to reuse the existing requirements for UE Rx-Tx. |
| Nokia | We don’t think the measurement gap is needed for the PRS within the active BWP and therefore, with such assumption, we think it is up to gNB to configure the PRS resources (i.e. PRS in continues symbols) such that UE can measure 4 samples of PRS without any or without large interruption and to provide that the channel won’t change drastically during the 4 samples of PRS measurement. Therefore, we don’t think the single sample measurement is necessary and we think there is no issue of applying the current Rel-16 UE Rx-Tx PRS measurement accuracy requirement for RTT PDC. In fact, RAN1 has made the following agreement in RAN1#106bis-e Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that - The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy - The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC And no issue was raised for the measurement 4 samples of PRS. |
| Qualcomm | Our understanding is that RTT-based PDC is not necessarily a low-latency application so there may not be a justification for considering only single sample measurements. |
| Huawei | Support option 1 and option 2. It is noted that in Rel-17 ePOS WI, reduced sample number is being discussed with the goal to achieve similar accuracy as Rel-16 under better condition, and to us this is more relevant to PDC. |

### Sub-topic 1-7

Sub-topic description Side conditions

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed side conditions for UE and gNB. For each case companies have proposed two options. RAN4 should discuss the proposed options and agree on the side conditions for the UE and the gNB.

Issue 1-7-1: Side conditions - UE

- Proposals

  - Option 1: Reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method

  - Option 2: UE Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel

- Recommended WF

  - Discussion is needed.

Sub topic 1-7

| Company | Comments |
| --- | --- |
| Issue 1-7-1 | Side conditions - UE |
| Ericsson | Option 1. |
| MediaTek | If the requirements are reused, then we can reuse the side conditions as well. Support Option 1. |
| Nokia | Option 1. We think defining new side conditions on a different Es/Iot level and different channel is NOT necessary. |
| Qualcomm | RAN4 should agree on a few candidate Es/Iot values for the simulation assumptions. We support at least Es/Iot = -3 dB, which was used in Rel-16. |
| Huawei | Option 2. On option 1, in Rel-16 there are 4 sets of UE Rx-Tx requirements, for Es/Iot of -13dB and -3dB and for AWGN and NLOS fading channel, so which side conditions are proposed to be re-used? We can agree to option 1 if the intention is to reuse the condition of Es/Iot of -3dB and AWGN channel. |

Issue 1-7-2: Side conditions - gNB

- Proposals

  - Option 1: Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method

  - Option 2: gNB Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel

- Recommended WF

  - Discussion is needed.

Sub topic 1-7

| Company | Comments |
| --- | --- |
| Issue 1-7-2 | Side conditions - gNB |
| Ericsson | Option 1- We prefer to reuse rel-16 as much as possible. |
| MediaTek | If the requirements are reused, then we can reuse the side conditions as well. Support Option 1. |
| Nokia | Option 1 in principle. |
| Huawei | Option 2. On option 1, in Rel-16 there are 2 sets of gNB Rx-Tx requirements, for Es/Iot of -13dB and +3dB for AWGN channel, so which side conditions are proposed to be re-used? We can agree to option 1 if the intention is to reuse the condition of Es/Iot of 3dB, which is in fact same as option 2. |

### Sub-topic 1-8

Sub-topic description Simulation assumptions

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed simulation assumptions. RAN4 would need to discuss and agree on a common set of simulation assumptions.

Issue 1-8-1: Simulation assumptions

- Proposals

  - Option 1: Simulation assumptions for UE Rx-Tx measurement based on Annex B of R4-2201633

  - Option 2: Simulation assumptions for UE Rx-Tx time difference measurement accuracy based on TRS for Rel-17 PDC RTT method: tables 1 and 2 in R4-2201723

  - Option 3: UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking: discuss simulation assumptions especially on CSI-RS configuration

- Recommended WF

  - More discussion is needed.

Sub topic 1-8

| Company | Comments |
| --- | --- |
| Issue 1-8-1 | Simulation assumptions |
| vivo | Depends on outcome of issue 1-1-1. If TRS based UE Rx-Tx time difference measurement is deprioritized, then no simulation for TRS is needed. |
| Ericsson | We prefer to reuse existing rel-16 (or rel-17, if available in time and mandatory) to be able to finish PDC definition in time for rel-17. It will be difficult to agree simulation assumptions and align simulations in rel-17. |
| MediaTek | We support Option 3. The simulations needed only for CSI-RS for tracking. |
| Nokia | Option 2. Given the simulation setup for Rel-16 PRS in R4-2009127, we think most of the general parameters can be reused for the TRS link simulation assumption. The difference would be the cell layout since only the single serving cell is considered in PDC RTT-based method and the channel conditions can be adjusted to AWGN and TDL-A (30 ns delay spread, 5Hz) given that only these two channel assumptions are adopted in TR 38.133 - 10.1.25.2. For TRS configuration parameters, we think the periodicity, evaluation period and BW should be the same as the PRS settings in R4-2009127. Since the TRS frequency density is fixed as 3, we then only need to compare different TRS symbol numbers {2, 4}. |
| Qualcomm | Elements from both options 1 and 2 can be combined to get a complete set of simulation assumptions. The PRS and CRI-RS BW configurations in Tables B2 and B3 from option 1 can be used as the baseline for FR1. What about FR2 configurations? Number of samples: 1, 4 For side-conditions, see response to issue 1-7-1. For propagation conditions, AWGN can be the baseline. |
| Huawei | The options are not exclusive. We think we need to agree on some simulation assumption for the TRS based RTT, and it can be done by merging the proposals in option 1 and 2. |

### Sub-topic 1-9

Sub-topic description Measurement accuracy requirements

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed measurement accuracy requirements for both UE and gNB. For both gNB and UE, some measurement accuracy requirements to be agreed have been proposed. RAN4 would need to discuss and if possible, agree on the measurement accuracy requirements for gNB and UE.

Issue 1-9-1: Measurement accuracy requirements - gNB

- Proposals

  - Option 1: Re-use the Rel-16 gNB Rx-Tx accuracy requirements at 3dB for PDC

  - Option 2: Reuse the existing serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method. The single-cell gNB Rx-Tx time difference measurement accuracy requirements based on SRS are: see table 5 in R4-2201723

  - Option 3: Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS

- Recommended WF

  - More discussion is needed.

Sub topic 1-9

| Company | Comments |
| --- | --- |
| Issue 1-9-1 | Measurement accuracy requirements - gNB |
| vivo | At least option 3 should be fine. Option 1 and 2 seems being discussed in previous issues. |
| Ericsson | Option 1. Why limit to 15 and 30 kHz SCS? PRS and SRS Rx-Tx time difference requirements exist for all SCS and could be reused as long as requirements can be met. |
| MediaTek | We support reusing the existing serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 (Table 13.2.2.2-1 and Table 13.2.2.2-2) for Rel-17 PDC RTT method. We don’t understand the motivation to remove the SCS = 60kHz from URLLC. Besides, should we define the Rx-Tx time difference measurement accuracy for FR2? We suggest adding the issue in the sub-topics. |
| Nokia | Option 2. And attaching RAN1 agreement in below to justify of only considering SCS 15/30kHz Agreements: Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization. |
| Huawei | Support combining option 1 and 3. We have same view as Nokia on the applicable SCS. |

Issue 1-9-2: Measurement accuracy requirements - UE

- Proposals

  - 1: Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS

  - 2: Reuse the accuracy requirements defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method. The single-cell UE Rx-Tx time difference measurement accuracy requirements with PRS are: see tables 3 and 4 in R4-2201723

  - 3: For the UE Rx-Tx time difference measurement accuracy of PRS, RAN4-URLLC shall wait for RAN4-positioning to finalise the accuracy requirements

- Recommended WF

  - More discussion is needed. Moderator suggest to work directly on a detailed table.

Sub topic 1-9

| Company | Comments |
| --- | --- |
| Issue 1-9-2 | Measurement accuracy requirements - UE |
| vivo | At least option 1 should be fine. |
| Ericsson | Option 2, Reuse the accuracy requirements defined in TR 38.133-10.1.25.2.. We prefer to reuse existing requirements, for efficient work. Why limit to 15 and 30 kHz SCS? PRS and SRS Rx-Tx time difference requirements exist for all SCS and could be reused as long as requirements can be met. |
| MediaTek | Support reusing the accuracy requirements defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method. We don’t understand the motivation to remove the SCS = 60kHz from URLLC. Besides, should we define the Rx-Tx time difference measurement accuracy for FR2? We suggest adding the issue in the sub-topics. |
| Nokia | Option 2 and we think option 2 is consistent with option 1. We think the current Rel-16 PRS UE Rx-Tx time difference measurement accuracy requirement can be reused. Also see our comments in issue 1-6. Also see our comments in issue 1-9-1. |
| Qualcomm | The measurement accuracy requirements for RTT-based PDC should be based on updated simulation results according to a new set of simulation assumptions. Option 3 is a valid observation. There will be a dependency on the Rel-16 NR positioning requirements that we should leverage in this WI. |
| Huawei | Support option 1. For option 2 and 3, RAN4 should first agree on the sample number and side condition for UE Rx-Tx measurement for PDC. |

### Sub-topic 1-10

Sub-topic description Measurement report value range

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed measurement report value range for both UE and gNB Rx-Tx. RAN4 would need to discuss and if possible, agree on the measurement report value range for gNB and UE.

Issue 1-10-1: UE Rx-Tx measurement report value range

- Proposals

  - Option 1: Use the existing reporting range for UE Rx-Tx is re-used for PDC

  - Option 2: For Rel-17 single-cell PDC UE Rx-Tx report for both TRS based method or PRS based method, in case gNB side PD estimation is supported, the UE Rx-Tx measurement report value range should only be positive and designed for the maximum supported distance to the serving cell

  - Option 3: Other

- Recommended WF

  - More discussion is needed.

Sub topic 1-10

| Company | Comments |
| --- | --- |
| Issue 1-10-1 | UE Rx-Tx measurement report value range |
| vivo | Option 1. |
| Ericsson | Option 1. |
| MediaTek | Support Option 1. |
| Nokia | Option 2. From the above figure, it is our understanding that for the serving cell, the UE Rx-Tx reporting values can only be positive because of the timing advance. We think that is also the same understanding from many other companies based on R4-2002310. Therefore, we think the report value range can be reduced to half and only include the positive values. |
| Qualcomm | Option 1. |
| Huawei | Option 1 for simplicity. We can understand the technical rationale of option 2, but we do not think signaling overhead for RRC is a big issue. |

Issue 1-10-2: gNB Rx-Tx measurement report value range

- Proposals

  - Option 1: For Rel-17 single-cell PDC gNB Rx-Tx report, in case UE side PD estimation is supported, the gNB Rx-Tx measurement report value range can be negative and positive. A smaller range, e.g. equivalent to cyclic prefix length, should be sufficient since UL subframe transmissions received in gNB are time aligned with DL subframe

  - Option 2: The existing reporting range for gNB Rx-Tx is re-used for PDC

  - Option 3: Other

- Recommended WF

  - More discussion is needed.

Sub topic 1-10

| Company | Comments |
| --- | --- |
| Issue 1-10-2 | gNB Rx-Tx measurement report value range |
| vivo | Option 2. |
| Ericsson | Option 2. |
| MediaTek | Support Option 2. |
| Nokia | Option 1. As shown in the above figure, UL transmission timing is controlled by TA, the range of single-cell gNB Rx-Tx time difference should be smaller compared to the current reporting range in TR 38.133-13.2.1, i.e., measured DL and UL subframe boundaries are close in time. In practice, such timing difference should not be more than the cyclic prefix length. |
| Huawei | Option 2, same comment as Issue 1-10-1. |

### Sub-topic 1-11

Sub-topic description Link simulations

Open issues and candidate options before e-meeting:

Before the meeting it has been proposed that for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS for Rel-17 PDC RTT method, a link simulation study is needed. RAN4 should decide whether link simulations are needed to define UE Rx-Tx time difference measurement accuracy requirements based on TRS.

Issue 1-11-1: Link simulation study is needed for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS

- Proposals

  - Option 1: Yes, RAN4 need link simulation study for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS

  - Option 2: Other

- Recommended WF

  - More discussion is needed.

Sub topic 1-11

| Company | Comments |
| --- | --- |
| Issue 1-11-1 | Link simulation study is needed for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS |
| vivo | It depends on outcome of issue 1-1-1. |
| Ericsson | We prefer to prioritize UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate) and gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate) in rel-17. Then Link simulation study is needed for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS is not needed. |
| MediaTek | Support Option 1. |
| Nokia | We support option 1 |
| Qualcomm | Option 1 |
| Huawei | Option 1 |

### Sub-topic 1-12

Sub-topic description Test case

Open issues and candidate options before e-meeting:

Before the meeting it has been proposed reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS bases RTT method

Issue 1-12-1: reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS based RTT method

- Proposals

  - Option 1: Yes, RAN4 can reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS based RTT method

  - Option 2: Other

- Recommended WF

  - More discussion is needed.

Sub topic 1-12

| Company | Comments |
| --- | --- |
| Issue 1-12-1 | reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS based RTT method |
| Ericsson | It would be very good if we could reuse and existing test case. This is in line with our proposals to re-use existing core requirements. From that point of view we are positive to investigate if option 1 will work. |
| MediaTek | To further study if the test case is applicable for the single-cell or not. |
| Nokia | Option 1 in principle. But we are also fine to discuss other sub-topics first and then discuss the corresponding test cases. |
| Qualcomm | FFS |
| Huawei | Suggest FFS. Although option 1 makes good sense, it may be too early to discuss the test case at this stage. |

## Companies views’ collection for 1st round

### Open issues

Feedback form is listed separately under each Sub-topic.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201634 Huawei, Hisilicon | CR Title: CR on requirements for UE Rx-Tx measurement for PDC |
|  | Qualcomm: Suggest to postpone the CR until further progress. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | Tentative agreements: Yes. Agree that RAN4 will not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. Candidate options: - Option 1: Yes, RAN4 should down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. [vivo, Ericsson] - Option 2: No, RAN4 should not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. [MTK, Nokia, QC, HW] Based on the comments in 1st round and the majority view not to down prioritize, moderator propose to agree on option 2. Moderator would like to hear from proponents of option 1 if they can compromise? Recommendations for 2nd round: Agree that RAN4 will not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. 2nd round discussion needed. |
| Sub-topic 1-2 | Tentative agreements: No Candidate options: - Option 1: Yes, RAN4 should reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC - Option 2: No, RAN4 should not reuse UE Rx-Tx time difference measurement accuracy based on TRS/PRS from release 16 specification for RTT-based PDC No consensus on the proposal (option 1) based on the feedback in 1st round. Instead, moderator suggest that RAN4 will focus the discussion on agreeing on assumptions and conditions to be used for simulations. Recommendations for 2nd round: RAN4 will not agree option 1 or option 2. RAN4 will focus the discussion on agreeing on assumptions and conditions to be used for simulations. Issue is closed. 2nd round discussion is not needed. |
| Sub-topic 1-3 | Tentative agreements: Yes Agree that RAN4 can reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC Candidate options: - Option 1: Yes, RAN4 can reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC - Option 2: No, RAN4 cannot reuse gNB Rx-Tx time difference absolute accuracy based on SRS, from release 16 specification for RTT-based PDC Based on the 1st round discussion the majority of companies agree to go forward using option 1. Moderator propose to agree on option 1. Recommendations for 2nd round: Issue is closed. 2nd round discussion is not needed. |
| Sub-topic 1-4 | Tentative agreements: Almost agreement on option 1. Hence, following agreement is proposed for 2nd round: To progress the work RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP. This can be revisited in next meeting if companies find that such assumption is not feasible. Candidate options: - Option 1: Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP - Option 2: Scheduling restriction is required for UE Rx-Tx measurement Most companies can support the proposed option 1 while one company prefer to study further. Based on this moderator would propose to use option 1 as working assumption and continue the work based on that. Moderator would like to hear if MTK can agree to this? Recommendations for 2nd round: Confirm following agreement: To progress the work RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP. This can be revisited in next meeting if companies find that such assumption is not feasible. 2nd round discussion needed |
| Sub-topic 1-5 | Tentative agreements: Almost agreement on option 1. Hence, following agreement is proposed for 2nd round: To progress the work RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP. This can be revisited in next meeting if companies find that such assumption is not feasible. Candidate options: - Option 1: MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP - Option 2: MG is required for UE Rx-Tx measurement Most companies can support the proposed option 1 while one company prefer to study further. Based on this moderator would propose to use option 1 as working assumption and continue the work based on that. Moderator would like to hear if MTK can agree to this? Recommendations for 2nd round: To progress the work RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP. This can be revisited in next meeting if companies find that such assumption is not feasible. 2nd round discussion needed/Confirm agreement |
| Sub-topic 1-6 | Tentative agreements: No agreement on the proposal in option 1. However, there is support for option 2 and no company was against the principle proposed in option 2. It was raised whether Rel-16 or Rel-17 PRS should be used. Hence, moderator propose to agree on the principle of option 2 and discuss in 2nd round if Rel-16 or Rel-17 PRS should be used. Candidate options: - Option 1: Measurement requirements for UE Rx-Tx is defined based on single-shot measurement - Option 2: Accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17 for reduced number of samples can be reused as much as possible for RTT-based PDC No agreement among companies on the proposal in option 1. However, there is support for option 2 and no company was against the principle proposed in option 2. It was raised whether Rel-16 or Rel-17 PRS should be used. Hence, moderator propose to agree on the principle of option 2 and discuss in 2nd round if Rel-16 or Rel-17 PRS should be used. Recommendations for 2nd round: Discuss in 2nd round if Rel-16 or Rel-17 PRS should be used to derive the accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17. 2nd Round discussion needed. |
| Sub-topic 1-7 | Issue 1-7-1: Side conditions - UE Tentative agreements: Yes Conditional agreement on option 1. Condition 1: RAN4 should agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB. Condition 2: reuse the condition of Es/Iot of -3dB and AWGN channel Candidate options: - Option 1: Reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method - Option 2: UE Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel Moderator suggest to agree on option 1 with the condition that AWGN channel is assumed and RAN4 should agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB. Recommendations for 2nd round: Confirm following agreement: RAN4 agree to reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method 1 with the condition that AWGN channel is assumed and RAN4 will agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB. 2nd Round discussion needed. Issue 1-7-2: Side conditions – gNB Tentative agreements: Yes Conditional agreement on option 1. Condition 1: reuse the condition of Es/Iot of 3dB Candidate options: - Option 1: Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method - Option 2: gNB Rx-Tx accuracy requirements are defined based on Es/Iot ≥ 3dB and LOS (AWGN) channel Moderator suggest to agree on option 1 with the condition of Es/Iot of 3dB. Recommendations for 2nd round: Confirm following agreement: Reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method with the condition of Es/Iot of 3dB. 2nd Round discussion needed. |
| Sub-topic 1-8 | Tentative agreements: No Candidate options: - Option 1: Simulation assumptions for UE Rx-Tx measurement based on Annex B of R4-2201633 - Option 2: Simulation assumptions for UE Rx-Tx time difference measurement accuracy based on TRS for Rel-17 PDC RTT method: tables 1 and 2 in R4-2201723 - Option 3: UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking: discuss simulation assumptions especially on CSI-RS configuration No consensus among companies during the discussion. Options are not considered mutually exclusive. More discussion is needed and companies are encouraged to propose detailed simulation assumptions the the scenarios they see necessary. Recommendations for 2nd round: More discussion is needed. |
| Sub-topic 1-9 | Issue 1-9-1: Measurement accuracy requirements – gNB Tentative agreements: No Candidate options: - Option 1: Re-use the Rel-16 gNB Rx-Tx accuracy requirements at 3dB for PDC - Option 2: Reuse the existing serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method. The single-cell gNB Rx-Tx time difference measurement accuracy requirements based on SRS are: see table 5 in R4-2201723 - Option 3: Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS No consensus. Recommendations for 2nd round: More discussion is needed. Companies are invited to check if the options can be combined into a single agreeable proposal. 2nd round discussion needed More discussion needed. Issue 1-9-2: Measurement accuracy requirements - UE Tentative agreements: No Candidate options: - 1: Accuracy requirements for RTT-based PDC are specified for both 15kHz SCS and 30kHz SCS - 2: Reuse the accuracy requirements defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method. The single-cell UE Rx-Tx time difference measurement accuracy requirements with PRS are: see tables 3 and 4 in R4-2201723 - 3: For the UE Rx-Tx time difference measurement accuracy of PRS, RAN4-URLLC shall wait for RAN4-positioning to finalise the accuracy requirements No consensus. It was proposed to wait agreement on the simulation assumptions and simulation results. Hence, moderator suggest that RAN4 wait agreement on the simulation assumptions and simulation results. Recommendations for 2nd round: More discussion needed. 2nd round discussion needed . |
| Sub-topic 1-10 | Issue 1-10-1: UE Rx-Tx measurement report value range Tentative agreements: Almost Candidate options: - Option 1: Use the existing reporting range for UE Rx-Tx is re-used for PDC - Option 2: For Rel-17 single-cell PDC UE Rx-Tx report for both TRS based method or PRS based method, in case gNB side PD estimation is supported, the UE Rx-Tx measurement report value range should only be positive and designed for the maximum supported distance to the serving cell - Option 3: Other Most companies can agree to option 1. One company prefer option 2. Moderator suggest to agree on option 1 and ask Nokia if they can compromise to option 1? Recommendations for 2nd round: Confirm whether compromise agreement can be reached: RAN4 agree to use the existing reporting range for UE Rx-Tx is re-used for PDC. 2nd round discussion needed Issue 1-10-2: gNB Rx-Tx measurement report value range Tentative agreements: Almost Candidate options: - Option 1: For Rel-17 single-cell PDC gNB Rx-Tx report, in case UE side PD estimation is supported, the gNB Rx-Tx measurement report value range can be negative and positive. A smaller range, e.g. equivalent to cyclic prefix length, should be sufficient since UL subframe transmissions received in gNB are time aligned with DL subframe - Option 2: The existing reporting range for gNB Rx-Tx is re-used for PDC - Option 3: Other Most companies can agree to option 2. One company prefer option 1. Moderator suggest to agree on option 1 and ask Nokia if they can compromise to option 2? Recommendations for 2nd round: Confirm whether compromise agreement can be reached: RAN4 agree to use the existing reporting range for gNB Rx-Tx is re-used for PDC. 2nd round discussion needed |
| Sub-topic 1-11 | Tentative agreements: yes (conditioned 1-1-1) Agreement: RAN4 need link simulation study for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS Candidate options: - Option 1: Yes, RAN4 need link simulation study for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS - Option 2: Other Companies support option 1 but 1 company states that it depends on agreement in Issue 1-1-1. Moderator proposal is that RAN4 does not agree on the proposal in Issue 1-1-1 as there is no consensus on the proposal. Based on this moderator propose to agree on option 1: RAN4 need link simulation study for defining UE Rx-Tx time difference measurement accuracy requirements based on TRS Recommendations for 2nd round: no need for further discussion topic is closed |
| Sub-topic 1-12 | Tentative agreements: no Candidate options: - Option 1: Yes, RAN4 can reuse the current test case of cell 1 defined in TR 38.133 – A.6.7.15 for the single-cell UE Rx-Tx time difference measurement accuracy requirements for Rel-17 PDC PRS based RTT method - Option 2: Other No consensus. Moderator suggest companies to consider further. Recommendations for 2nd round: No need to discuss further in this meeting. Discussion is closed |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
|  | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2201634 CR on requirements for UE Rx-Tx measurement for PDC | to be revised (to capture possible agreement) |

## Discussion on 2nd round (if applicable)

### Sub-topic 1-13

2nd round of sub-topic 1-1.

Sub-topic description: Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking

Open issues and candidate options before e-meeting:

It has been proposed to down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. Is it this agreeable?

Issue 1-13-1: Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking?

After 1st Round:

- Option 1: Yes, RAN4 should down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. [vivo, Ericsson]

- Option 2: No, RAN4 should not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking. [MTK, Nokia, QC, HW]

Based on the comments in 1st round there is no consensus on down selection and the majority view is not to down prioritize. Hence, moderator propose to agree on option 2. Moderator would like to hear from proponents of option 1 if they can compromise?

- Recommended WF

  - Agree that RAN4 will not down prioritize of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking

Sub topic 1-13

| Company | Comments |
| --- | --- |
| Issue 1-13-1 | Down prioritization of RTT-based propagation delay compensation based on UE Rx-Tx time difference measurement accuracy using CSI-RS for tracking |

### Sub-topic 1-14

2nd round of sub-topic 1-4.

Sub-topic description Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Open issues and candidate options before e-meeting:

It has been proposed that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Issue 1-4-1: Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

After 1st Round:

Almost agreement on option 1. Hence, following agreement is proposed for 2nd round:

To progress the work RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

This can be revisited in next meeting if companies find that such assumption is not feasible.

Moderator would like to hear if MTK has any concerns?

- Recommended WF

  - RAN4 assume that scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

  - This can be revisited in next meeting if companies find that such assumption is not feasible.

Sub topic 1-14

| Company | Comments |
| --- | --- |
| Issue 1-14-1 | Scheduling restriction is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP |

### Sub-topic 1-15

2nd round of sub-topic 1-5.

Sub-topic description MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Open issues and candidate options before e-meeting:

It has been proposed that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP

Issue 1-5-1: MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

After 1st Round:

Almost agreement on option 1. Hence, following agreement is proposed for 2nd round:

To progress the work RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

This can be revisited in next meeting if companies find that such assumption is not feasible.

Moderator would like to hear if MTK has any concerns?

- Recommended WF

  - RAN4 assume that MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP.

  - This can be revisited in next meeting if companies find that such assumption is not feasible.

Sub topic 1-15

| Company | Comments |
| --- | --- |
| Issue 1-15-1 | MG is not required for UE Rx-Tx measurement, provided that UE is not required measure TRS/PRS outside the active BWP, and the SCS of the TRS/PRS is same as the active BWP |

### Sub-topic 1-16

2nd round of sub-topic 1-6.

Sub-topic description Measurement samples

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed different assumptions to be used for the number of measurement samples for deriving the accuracy requirements. RAN4 should discuss the proposed options and agree on the assumed number of samples used for deriving the requirements.

After 1st Round:

No agreement among companies on the proposal in option 1.

However, there is support for option 2 and no company was against the principle proposed in option 2. It was raised whether Rel-16 or Rel-17 PRS should be used. Hence, moderator propose to agree on the principle of option 2 and discuss in 2nd round if Rel-16 or Rel-17 PRS should be used.

- Recommended WF

  - Agree on the principle proposed in option 2.

  - Discuss if Rel-16 or Rel-17 PRS should be used to derive the accuracy requirements of UE Rx-Tx time difference measurement based on PRS in Rel-17.

Sub topic 1-16

| Company | Comments |
| --- | --- |
| Issue 1-16-1 | Measurement samples |

### Sub-topic 1-17

2nd round of sub-topic 1-7.

Sub-topic description Side conditions

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed side conditions for UE and gNB. For each case companies have proposed two options. RAN4 should discuss the proposed options and agree on the side conditions for the UE and the gNB.

After 1st Round:

Issue 1-7-1: Side conditions – UE

Conditional agreement on option 1.

Condition 1: RAN4 should agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB.

Condition 2: reuse the condition of Es/Iot of -3dB and AWGN channel

- Recommended WF

  - RAN4 agree to reuse the side conditions defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method 1 with the condition that AWGN channel is assumed and RAN4 will agree on a few candidate Es/Iot values for the simulation assumptions including at least Es/Iot = -3 dB.

Issue 1-7-2: Side conditions – gNB

Conditional agreement on option 1.

Condition 1: reuse the condition of Es/Iot of 3dB

- Recommended WF

  - RAN4 agree to reuse the existing side conditions and the serving cell gNB Rx-Tx time difference measurement accuracy requirements in TR 38.133 - 13.2.2 for Rel-17 PDC RTT method with the condition of Es/Iot of 3dB.

Sub topic 1-17

| Company | Comments |
| --- | --- |
| Issue 1-17-1 | Side conditions - UE |

| Company | Comments |
| --- | --- |
| Issue 1-17-2 | Side conditions - gNB |

### Sub-topic 1-18

2nd round of sub-topic 1-8

Sub-topic description Measurement accuracy requirements

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed measurement accuracy requirements for both UE and gNB. For both gNB and UE, some measurement accuracy requirements to be agreed have been proposed. RAN4 would need to discuss and if possible, agree on the measurement accuracy requirements for gNB and UE.

After 1st Round:

No consensus among companies during the discussion. Options are not considered mutually exclusive. More discussion is needed and companies are encouraged to propose detailed simulation assumptions the the scenarios they see necessary.

- Recommended WF

  - Continue the discussion.

Sub topic 1-18

| Company | Comments |
| --- | --- |
| Issue 1-18-1 | Measurement accuracy requirements |

### Sub-topic 1-19

2nd round of sub-topic 1-9

Sub-topic description Measurement accuracy requirements

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed measurement accuracy requirements for both UE and gNB. For both gNB and UE, some measurement accuracy requirements to be agreed have been proposed. RAN4 would need to discuss and if possible, agree on the measurement accuracy requirements for gNB and UE.

After 1st Round:

Issue 1-9-1: Measurement accuracy requirements – gNB

No consensus.

- Recommended WF

  - More discussion is needed. Companies are invited to check if the options can be combined into a single agreeable proposal.

Issue 1-9-2: Measurement accuracy requirements - UE

No consensus. It was proposed to wait agreement on the simulation assumptions and simulation results. Hence, moderator suggest that RAN4 continue discussion while waiting agreement on the simulation assumptions and simulation results

- Recommended WF

  - More discussion is needed. Companies are invited to check if the options can be combined into a single agreeable proposal.

Sub topic 1-19

| Company | Comments |
| --- | --- |
| Issue 1-19-1 | Measurement accuracy requirements – gNB |

| Company | Comments |
| --- | --- |
| Issue 1-19-2 | Measurement accuracy requirements - UE |

### Sub-topic 1-20

2nd round of sub-topic 1-10

Sub-topic description Measurement report value range

Open issues and candidate options before e-meeting:

Before the meeting companies have proposed measurement report value range for both UE and gNB Rx-Tx. RAN4 would need to discuss and if possible, agree on the measurement report value range for gNB and UE.

After 1st Round:

Issue 1-10-1: UE Rx-Tx measurement report value range

Most companies can agree to option 1. One company prefer option 2. Moderator suggest to agree on option 1 and ask Nokia if they have concern compromising to option 1?

- Recommended WF

  - RAN4 agree to use the existing reporting range for UE Rx-Tx is re-used for PDC .

Issue 1-10-2: gNB Rx-Tx measurement report value range

Most companies can agree to option 2. One company prefer option 1. Moderator suggest to agree on option 1 and ask Nokia if they have concern compromising to option 2?

- Recommended WF

  - RAN4 agree to use the existing reporting range for gNB Rx-Tx is re-used for PDC

Sub topic 1-20

| Company | Comments |
| --- | --- |
| Issue 1-20-1 | UE Rx-Tx measurement report value range |

| Company | Comments |
| --- | --- |
| Issue 1-20-2 | UE Rx-Tx measurement report value range |

# Topic #2: Reference point for Te requirements

Main technical topic to be discussed here is to discuss the question related to the UE transmit timing error from RAN and reply to the rest of the question in addition to what RAN4 has already been replied in R4-2105850.

Note that in RAN4#100 meeting following was agreed:

- Whether to include ‘antenna’ in the definition or not (Issue 3-1):

  - Agree on Option 1: Use ‘antenna’ in definition as proposed in the tentative TP

- Whether to use ‘detected’, ‘detectable’ or not mention either (Issue 3-2):

  - RAN4 down select among the three options and discontinue discussion related to Option 2 (use of ‘detectable’ in the text)

    - WF1: Continue the discussion whether to use ‘detected’ or not use ‘detected’ in the 38.133.

- Whether to include ‘Received’, ‘arrives’ or ‘true arrival’ in the definition (Issue 3-3):

  - RAN4 will not discuss ‘true arrival’ further.

Note that in RAN4#101 meeting following was agreed:

- Do not define UE timing error requirements side conditions (rely on the test case side conditions)

Hence, only issue left open from RAN4#100 meeting is:

- TP for downlink timing definition

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200528 | Intel | Draft CR |
| R4-2201369 | vivo | - Proposal 1: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna. - Proposal 2: The change of Te reference point definition is made from Rel-15. |
| R4-2201635 | Huawei, HiSilicon | - Proposal 1a: Do not mention ‘detected’ nor ‘detectable’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133. - Proposal 1b: Add a note in the requirements that the requirements may not apply in all conditions but shall apply under conditions used in the test cases. - Proposal 2: Use ‘arrives’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133. - Proposal 3: Update the definition of the “reference point” in clause 7.1.2 of 38.133 from Rel-15: “The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.” - Proposal 4: Send LS to inform RAN1 about the updated definition of the “reference point”. |
| R4-2201673 | Intel | Observation 1: Since there is no way for a UE to know the DL frame timing detection error, it has no choice but to simply use the perceived timing. Proposal 1: The interpretation of “the reference point” defined in section 7.1.2 in TS 38.133 for UE transmission timing is (NTA + NTA offset)*Tc ahead of the time when the first path (in time) from the reference cell arrives at the UE antenna. Proposal 2: Correct the spec with ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’ from Rel-15. Observation 2: Test equipment verify the UE transmit timing by comparing the received timing against the transmitted DL signal timing. Observation 3: Although there is no clear requirement specified for DL timing detection error, the UE needs to be good enough to cover detection error in Te to pass the tests. Proposal 3: Correct test case requirements with the wording ‘The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the time when the first path (in time) of DL SSB from the reference cell arrives at the UE antenna’. |
| R4-2201722 | Nokia, Nokia Shanghai Bell | Proposal 1: TP for Rel-15/16/17 38.133 7.1.2 “The downlink timing is defined as the time when the first detected path in time of the corresponding downlink frame from the reference cell is received at the UE antenna”. |
| R4-2201782 | MediaTek | Observation 1: Using the first detected path as reference point results in downlink frame timing detection error being not included in UE transmit timing error (i.e. Te). This contradicts the RAN4 common understanding, which is the downlink frame timing detection error is already included in UE transmit timing error. Observation 2: Using the true arrival time at UE as the reference point for UE transmit timing error (i.e. Te) results in the DL frame timing detection error being included in the Te, which aligns with RAN4 common understating. Observation 3: The UE is only capable of detecting the first detectable path for sufficiently high SINR, and hence the UE should follow the Te requirement for the SINR above that threshold only. |
| R4-2202013 | Ericsson | • Observation 1: The term first “detected path” (in time) in the definition of the reference point for timing error control requirement in section 7.1.2 in TS 38.133 would mean that the reference point is inside the UE i.e. at the UE baseband. • Observation 2: The purpose of the reference point is for interpretation, derivation, verification or testing of the core requirements. But the reference point inside the UE (i.e. at baseband) means that the reference point cannot be determined/estimated by the test system • Observation 3: It is agreed to include “UE antenna” will in the reference point definition. But UE antenna does not ‘detect’ rather receive signal, • Observation 4: Testing of Te is done under AWGN which has one path. Therefore, the path arriving at the UE antenna and detectected by the UE is the same. That’s why the problem has not been observed or will not be observed in the test. • Observation 5: In principle use of first “detected” path in the reference point definition creates ambiguity and in principle such definition (with detected path) also leaves core requirements “untestable”. • Proposal #1: The term “detected” is not included in the reference point definition, which is defined as follows: - The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna. |

## Open issues summary

Only aspect to discuss is the actual wording of the definition. This topic has now been discussed for several meetings Based on company views in RAN4#101 meeting, the outcome of the discussion as listed in the agreed WF was:

Issue 2-6-1: TP for downlink timing definition for Rel-17

- Proposals

  - Option 1: Clarifying by including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’

  - Option 2: Clarifying by removing ‘detected’ and including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’

  - Option 3: clarifying by removing ‘detected’ and use ‘arrives’ instead of ‘is received’ and including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.’

  - Option 4: Keep current text:

    - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell.’

Following TPs have been proposed for this meeting:

- The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna. (option 3 in WF)

- The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna (option 3 in WF)

- The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna. (option 1 in WF)

- The downlink timing is defined as the time when the first detected path in time of the corresponding downlink frame from the reference cell is received at the UE antenna (almost option 1 in WF)

- The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna. (Option 1 in WF)

### Sub-topic 2-1

Sub-topic description: TP for downlink timing definition

Open issues and candidate options before e-meeting:

This topic has now been discussed for several meetings Based on company views in RAN4#101 meeting, the outcome of the discussion as listed in the agreed WF is listed above. In this meeting 4 papers have been submitted with basically 2 different proposals: option 1 and option from the agreed WF in RAN4#101.

Moderator has combined TP 1 and TP 4 (=option 1 TP) and proponent of TP 4 (Nokia) should comment whether this is acceptable or not. [Moderator]: Nokia confirmed that they can agree to merging.

Issue 2-1-1: Whether to use TP option 1 or TP option 3 as the downlink timing definition text?

- Proposals

  - Option 1: Use TP option 1‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna’

  - Option 2: Use TP option 3 ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna’

- Recommended WF

  - More discussion is needed

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Issue 2-1-1 | Whether to use TP option 1 or TP option 3 as the downlink timing definition text? |
| vivo | Support option 1. The comparison between option 1 and option 3 are summarized in Table 1. Table 1. Comparison of definition of reference point of Te First detected path; First path. Requirement’s applicability; Static channel, LoS channel, Multipath fading channel; Static channel, LoS channel maybe. UE autonomous adjustment; UE can always meet autonomous adjustment requirements as it is based on UE first detected path.; UE could fail to meet autonomous adjustment requirements when in multipath-fading channel or when there is channel variation.. System impact; NW does not know the downlink detection error at UE side in fading channel.; NW does not know the downlink detection error at UE side in fading channel, as there are no applicable requirements.. Test design feasibility; Feasible in AWGN channel Tests has already been designed.; Feasible in AWGN channel Tests need to be revised.. RAN5 Test; Already implemented; New test implementation is needed.. |
| Intel | We support option 3. Sorry for the confusing proposal from our paper. We had typos in the paper. We meant first detected path (in time) and is received from but forget to apply the strike through to the removed parts. |
| Ericsson | We support option 2 (TP option 3). |
| Nokia | We prefer option 1 because we think both options cannot precisely capture Te and to compromise, option 1 is already implemented for Rel-15/16 UE. |
| Qualcomm | We continue to support option 2 as the reference for the purpose of evaluating the Te requirement, consistent with the LS response from RAN4. Admittedly, the Te requirement may not be met under all propagation conditions. |
| Huawei | Support option 2 (TP option 3) |

## Companies views’ collection for 1st round

### Open issues

Feedback form is listed separately under each Sub-topic.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2202014 (Agenda Item 4.1.6). Ericsson, Intel, Huawei, HiSilicon | CR Title: Correction to reference point defintion for UE timing in TS 38.133 |
|  | Company A |
|  | Company B |
| R4-2200528 Intel | CR Title: DraftCR to clarify timing reference point for UE UL timing test cases |
|  | Company A |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Whether to use TP option 1 or TP option 3 as the downlink timing definition text? Tentative agreements: No Candidate options: - Option 1: Use TP option 1‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna’ - Option 2: Use TP option 3 ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna’ From the 1st round discussion: Option 1: vivo, Nokia Option 2: Intel, Ericsson, Qualcomm, Huawei No consensus among companies. To progress the work moderator think RAN4 should discuss the technical aspect of defining the UE requirements and the accurate definition. Whether there is then a need to update test case in RAN4 or RAN5 should not impact the technical discussion. Considering the two comments during 1st round: - ‘we think both options cannot precisely capture Te’ - ‘the Te requirement may not be met under all propagation conditions’ Companies could consider following for 2nd round discussion: - RAN4 does not define how the UE selects the DL signal used for time tracking. - In a multipath environment UE may use strongest but may also use non-strongest. - RAN4 has only defined AWGN test under good signal conditions Hence, the UE implementation and how the UE selects the DL frame in a multipath environment is not defined by RAN4. And this leaves room for different implementation as should be the case with RAN4 requirements. This also means that different UEs, in a DL multipath environment, may not select same DL tab for tracking. However, as long as the UE behaviour is consistent and follow the Te requirements, it does not matter from a system point of view. Therefore, moderator try to provide a new possible compromise TP for discussion as it seems a bit fruitless to continue only discussing the existing two options. Moderator suggest to introduce following wording: ‘used by the UE to determine downlink timing’ to reflect above discussion. Companies should comment on the technical parts of the discussion and above instead of just stating which of the two existing TP options they support. Recommendations for 2nd round: Discuss the new TP option is agreeable: ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna’ Accounting the technical reasoning. Moderator assumes that different company positions do not change regarding TP option 1 and option 2. Only in case company position, please list it. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
|  | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2202014 Correction to reference point defintion for UE timing in TS 38.133 | to be revised |
| R4-2200528 DraftCR to clarify timing reference point for UE UL timing test cases | to be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 2-2

This is 2nd round of Sub-topic 2-1

Sub-topic description: TP for downlink timing definition

Open issues and candidate options before e-meeting:

This topic has now been discussed for several meetings Based on company views in RAN4#101 meeting, the outcome of the discussion as listed in the agreed WF is listed above. In this meeting 4 papers have been submitted with basically 2 different proposals: option 1 and option from the agreed WF in RAN4#101.

After 1st Round:

Candidate options:

  - Option 1: Use TP option 1‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna’

  - Option 2: Use TP option 3 ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna’

From the 1st round discussion:

Option 1: vivo, Nokia

Option 2: Intel, Ericsson, Qualcomm, Huawei

No consensus among companies.

- Recommended WF

  - Moderator suggest to introduce following wording: ‘used by the UE to determine downlink timing’ to reflect above discussion.

  - Companies should comment on the technical parts of the discussion and above instead of just stating which of the two existing TP options they support

  - Discuss the new TP option is agreeable:

    - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna’

  - Accounting the technical reasoning. Moderator assumes that different company positions do not change regarding TP option 1 and option 2. Only in case company position, please list it

Sub topic 2-2

| Company | Comments |
| --- | --- |
| Issue 2-2-1 | Whether the modified TP from moderator can be agreeable as the downlink timing definition text? |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR_IIOT_URLLC_enh_RRM | Nokia, Nokia Shanghai Bell | Capture agreements and WF from RAN4#101bis |
| LS on TA-based propagation delay compensation | ZZZ | To: RAN_1 (reply LS to R4-2108635). This LS will contain information related to PDC enhancements and reference point Te. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202014 | Correction to reference point defintion for UE timing in TS 38.133 | Ericsson, Intel, Huawei, HiSilicon | Revised | Pending meeting discussion |
| R4-2200528 | DraftCR to clarify timing reference point for UE UL timing test cases | Intel | Revised | Pending meeting discussion |
| R4-2201634 | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, Hisilicon | Revised | Pending meeting discussion |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-211xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-211xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-211xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Intel | Meng Zhang | Meng.zhang@intel.com |
| MediaTek | Waseem Ozan | Waseem.ozan@mediatek.com |
| Qualcomm | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)