3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2202594

Electronic Meeting, January 17-25, 2022

Agenda item:			6.9.4

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	WF on FR2 HST RRM (part 1)

Document for:	Approval

# Background

This contribution is to capture the agreements and the directions of further discussions for the email discussion of Rel-17 NR HST FR2 enhancements RRM core requirements in RAN4#101-bis-e, with the email thread identifier “[100-bis-e][204] NR_HST_FR2_RRM_1”.

This e-mail thread captures the e-mail discussions for the following sub-agenda items for FR2 HST RRM

- 6.9.4	RRM core requirements

  - 6.9.4.1	General

  - 6.9.4.2	Number of RX beams

  - 6.9.4.3	RRC Idle/Inactive and connected state mobility requirements

  - 6.9.4.5	Signalling characteristics requirements

  - 6.9.4.6	Measurement procedure requirements

The following WFs were approved previously:

- R4-2103679, WF on Rel-17 NR HST FR2 enhancements, RRM requirements, Nokia, Nokia Shanghai Bell, RAN4#98-e

- R4-2105794, WF on FR2 HST RRM requirements, Nokia, Nokia Shanghai Bell, RAN4#98-bis-e

- R4-2115334, WF on FR2 HST RRM requirements (part 1), Nokia, Nokia Shanghai Bell, RAN4#100-e

- R4-2115335, WF on FR2 HST RRM requirements (part 2), Samsung, RAN4#100-e

- R4-2120292, WF on FR2 HST RRM requirements (part 1), Nokia, Nokia Shanghai Bell, RAN4#101-e

- R4-2120416, WF on FR2 HST RRM requirements (part 2), Samsung, RAN4#101-e

The detailed discussions related to the WF can be found in the e-mail discussion summary for [100-bis-e][204] NR_HST_FR2_RRM_1 [R4-2202721].

# WF on Topic#1: General

## Sub-topic 1-3: Network signaling

Issue 1-1-1: Signaling of uni-/bi-directional operation

GtW Agreement: Introduce network assistance to inform UE on the FR2 HST deployment type (uni-directional or bi-directional)

Issue 1-1-2: Signaling to indicate set 1 or set 2 RRM requirement

Agreement: Include cell-specific signaling of Set 1 or Set 2 enhanced RRM requirement into the LS to RAN2.

Issue 1-1-3: Applicability of enhanced RRM requirements in TP

Way forward: - FFS, whether it should be clarified in the TR that 2Rx beam sweep based requirement (set 1) applies to the deployment scenario with Dmin > [10] m or Hdiff (height difference between train rooftop mounted CPE and RRH) > [10] m, performance degradation is expected. - The proponents are encouraged to bring a TP to the TR, where the discussion can focus on the TP.

Issue 1-1-4: Lightweight network assistance signaling

Way forward: Discuss further which NWA signaling is needed: - Option 1: Enable network assisted signaling of SSB index and order per RRH. - Option 2: The network assistance signaling of SSB configuration shall not be introduced in Rel-17. - Option 3: Introduce inter-RRH indication - Option 4: Other options are not precluded

## Sub-topic 1-2: Deployment-related issues

Issue 1-2-1: Train travelling opposite to the serving beam and upper DRX bound

Agreement: The DRX upper bound = 80 ms applies both to Sets 1 (Scenario-A) and 2 (Scenario-B). Way forward: The companies are encouraged to provide the analysis of Scenario-A where the train is travelling in the direction opposite to the serving beam orientation in the TR.

Issue 1-2-2: Two-side RRM deployment in Scenario-B

GtW agreement: No special consideration for two-side RRH deployment for RRM requirements definition

Issue 1-2-3: Change of RRH panel orientation in uni-directional deployments

Agreement: Do not define defined additional network signalling to identify to the UE a change of RRH panel orientation in uni-directional deployment, which is merged with Issue 1-1-4.

## Sub-topic 1-3: UE capabilities

Issue 1-3-1: Capability to support different RX beam sweeping number

Agreement: FR2 HST UE (power class 6 UE) shall mandatorily support both Set 1 and Set 2 enhanced RRM requirements, in terms of different RX beams (i.e., RX beam sweeping scaling factor) per UE.

Issue 1-3-2: UE feature list

Way forward: Companies are encouraged to check further UE feature needed for the support of enhanced RRM requirements for FR2 HST: - FFS, whether power class shall be used to identify the feature support - FFS, whether per-band type is necessary or per-UE type is enough

## Sub-topic 1-4: CR work split (informative)

| TS 38.133 section number | TS 38.133 section title | Responsible company |
| --- | --- | --- |
| BigCR editor |  | Nokia |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.1 | Cell Selection |  |
| 4.2 | Cell Re-selection | ZTE |
| 5 | SA: RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover | Ericsson |
| 6.2 | RRC Connection Mobility Control | Ericsson |
| 7 | Timing |  |
| 7.1 | UE transmit timing | Nokia |
| 7.2 | UE timer accuracy |  |
| 7.3 | Timing advance |  |
| 7.4 | Cell phase synchronization accuracy |  |
| 7.7 | deriveSSB-IndexFromCell tolerance |  |
| 8 | Signalling characteristics |  |
| 8.1 | Radio Link Monitoring | CATT |
| 8.5 | Link Recovery Procedures | CATT |
| 8.10 | Active TCI state switching delay | Samsung |
| 8.12 | Uplink spatial relation switch delay | Samsung |
| 9 | Measurement Procedure |  |
| 9.2 | NR intra-frequency measurements | Nokia |
| 9.5 | L1-RSRP measurements for Reporting | ZTE |
| 9.8 | L1-SINR measurements for Reporting | Huawei |
| 9.10 | CSI-RS based L3 measurements | Huawei |

# WF on Topic#2: Mobility and Measurement procedure

## Sub-topic 2-1: RRC CONNECTED and IDLE state mobility requirements

Issue 2-1-1: Handover

Agreement: No enhancement is needed in HO requirement.

Issue 2-1-2: RRC Re-establishment requirement to known cell

Agreement: Do not define RRC Re-establishment requirement to known cell for HST FR2.

Issue 2-1-3: RRC Re-establishment requirement to unknown cell

Agreement: Define RRC connection re-establishment requirement to unknown NR intra-frequency cell Tidentify_intra_NR = MAX (1000 ms, 10 x N1 x TSMTC) when SINR≥ -8dB, the value of N1 refers to agreed RX beam number.

Issue 2-1-4: IDEL/INACTIVE state Cell Re-selection

Agreement: Defined enhanced requirements for Cell reselection in IDLE/INACTIVE mode for DRX 320 ms in HST FR2 deployments: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: [When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraRAT-MeasurementEnhancement-r16].. N1 refers to the number of Rx beams and equals 2 for Set 1, and 6 for Set 2. Way forward: The companies are encouraged to check the Notes and to reformulate Note 2.

## Sub-topic 2-2: Measurement procedure requirements

Issue 2-2-3: Time period for PSS/SSS detection and Measurement period for intra-frequency measurements

Agreement: PSS/SSS detection Set 1: Table 1: Time period for PSS/SSS detection when [flag1] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80 ms; max(600ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 2: Time period for PSS/SSS detection when [flag2] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80ms; max(600ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. SSB measurement period Set 1: Table 3: Measurement period for intra-frequency measurements without gaps when [flag1] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80ms; max(400ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 4: Measurement period for intra-frequency measurements without gaps when [flag2] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Way forward: - Discuss the following options to define the scaling factors: - Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2 - Option 2: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 10 for Set 1 and 24 for Set 2 - Companies are encouraged to check, whether 600ms lower bound is makes sense when 80ms< DRX cycle≤ 320ms

Issue 2-2-4: L1-RSRP measurements for Reporting

Agreement: - For Set 1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NA)*TSSB). DRX cycle ≤ 80ms; max(TReport, ceil(M*P*NA*M2)*max(TDRX,TSSB)). Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. NA = 2 - For Set 2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NB)*TSSB). DRX cycle ≤ 80ms; max(TReport, ceil(M*P*NB*M2)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. NB = 6

Issue 2-2-5: Applicability of requirements to non-PC6 UE

Way forward: FFS, whether enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

# WF on Topic#3: Signaling characteristics

Issue 3-1-1: RLM/BFD evaluation period

Agreement: Define scaling factor N for no DRX and DRX cycle <=80 ms by following the number of RX beams per UE for Set 1 and Set 2: Set 1: 2 RX beams; (2) Set 2: 6 RX beams.

Issue 3-1-2: TCI switching delay

Way forward: FFS how to avoid inter-symbol interference during TCI switching for FR2 HST scenario.

Issue 3-1-3: Uplink Spatial Relation Switch Delay

Agreement: For the known conditions for spatial relation when associated with DL-RS, the requirement defined in Rel-16 shall be reused, and no standard impact is expected for Rel-17 FR2 HST.