3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2202657

E-meeting, January 17-25, 2022

Agenda item:	6.16.7

Source:	Qualcomm

Title:	WF on NR extension to 71 GHz RRM requirements (Part 1)

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 6.16.7) in RAN4#101-bis-e under email thread [101-e][216] NR_ext_to_71GHz_RRM_1. This email thread covers following sub-agenda items:

- AI 6.16.7.1	General requirements

- AI 6.16.7.2	Timing requirements

The details on the discussion can be found at the [216] thread discussion summary [1].

# Way forward

## General

Deployment scenarios

- Following the agreement from RAN4#101e, no RRM requirements for EN-DC/NE-DC with FR2-2 will be defined in Rel-17

Rx beam sweeping scaling factor

- Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class.

Terminology update for FR2

- For some relevant requirements not defined in FR2-2, the existing term (FR2) needs to be adjusted.

  - It is suggested to clearly point out that it is only applied to FR2-1 to avoid unnecessary confusion

Current CR split

- The following list of Draft CRs to be prepared by RAN4 were identified during RAN4#101e:

  - Draft CR for timing requirements for FR2-2 – UE transmit timing and Timing Advance – Nokia

  - Draft CR for timing requirements for FR2-2 – MRTD, MTTD - Qualcomm

  - Draft CR for interruption requirements for FR2-2 - vivo

  - Draft CR for active BWP switching delay requirements for FR2-2 - Huawei

  - Draft CR for general measurement requirements for FR2-2 - Ericsson

  - Draft CR for scheduling restrictions – Mediatek

  - Additional Draft CR(s) shall be considered based on the outcome of open issues

- Note: Additional Draft CRs required due to introducing RRM requirements with LBT in FR2-2, are being discussed in thread 217

## Timing requirements

### UE transmit timing error

Percentage of UL CP length Te can occupy for UL SCS of 480/960 kHz

- UE transmit timing error requirements for UL SCS of 480/960 kHz are defined under the following assumptions

  - SCS:

    - SSB SCS ≥ UL SCS

    - FFS if other SCS combinations shall be considered

  - At least one SSB is available at the UE during the last: 20ms, 40ms, 80ms

    - Note: If multiple set of requirements are defined, then the requirements will be defined for at most for 2 periodicities

  - Max delay spread:

    - [30ns] for 480kHz

    - [20ns] for 960kHz

- FFS if a single set or multiple sets of requirements need to be defined

- FFS how to design test case for UE transmit timing error requirements

### MRTD

Basic principles

- Define MRTD requirements in FR2-2 based on the following rules in:

  - For Async cases: MRTD = 0.5 slot

  - For sync cases: MRTD = TAE + propagation delay difference

- FFS whether and how to change the MRTD definition, so that it could be larger than 0.5 slot.

Propagation delay difference

- For FR2-2 and FR2-2, there is no propagation delay difference

MRTD for intra-band non-contiguous CA

- The MRTD requirement for intra-band non-contiguous NR CA need to continue waiting for the TAE conclusion from RF group.

MRTD for FR1 and FR2-2 inter-band CA

- The MRTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MRTD requirements for inter-band NR carrier aggregation, update Table 7.6.4-2 for FR2-2 as below:

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | TBD |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

MRTD for FR1 and FR2-2 NR DC - Synchronous

- The MRTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MRTD requirements for inter-band synchronous NR DC, update Table 7.6.6-1 for FR2-2 as below:

Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC

| Frequency Range |  | Maximum receive timing difference (µs) |
| --- | --- | --- |
| Cell in MCG | Cell in SCG |  |
| FR1 | FR1 | 33 |
| FR2-1 | FR2-1 | 8 |
| FR1 | FR2-1 | 33 |
| FR1 | FR2-2 | TBD |

MRTD for FR1 and FR2-2 NR DC – Asynchronous

- Reuse the existing MRTD requirements for FR1 and FR2-1, that is, half the slot length with respect to the larger SCS of the MCG and SGC cells.

| Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)} | Maximum receive timing difference (µs) |
| --- | --- |
| 480 | 15.625 |
| 960 | 7.8125 |

### MTTD

Basic principles

- FFS: Define MTTD requirements in FR2-2 based on the following rule:

  - MTTD = MRTD + (TA resolution error cc1 + TA adjustment accuracy cc1 + UL transmit error cc1) + (TA resolution error cc2 + TA adjustment accuracy cc2 + UL transmit error cc2)

MTTD for FR1 and FR2-2 inter-band CA

- The MTTD requirements for FR2 inter-band CA is only applicable for FR2-1. For the MTTD requirements for inter-band NR carrier aggregation, update Table 7.5.4-1 for FR2-2 as below:

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | TBD |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

MTTD for FR1 and FR2-2 NR DC – Synchronous

- The MTTD requirements for FR2 +FR2 inter-band CA is only applicable for FR2-1+FR2-1. For the MTTD requirements for inter-band synchronous NR DC, update Table 7.5.6-1 for FR2-2 as below:

Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR DC

| Frequency Range |  | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| Cell in MCG | Cell in SCG |  |
| FR1 | FR1 | 34.6 |
| FR2-1 | FR2-1 | 8.5 |
| FR1 | FR2-1 | 34.1 |
| FR1 | FR2-2 | TBD |

MTTD for FR1 and FR2-2 NR DC - Asynchronous

- The MTTD requirements for inter-band asynchronous NR DC shall the half of slot length. For the MTTD requirements for inter-band asynchronous NR DC, update Table 7.5.6-2 for FR2-2 as below:

Table 7.5.6-2 Maximum uplink transmission timing difference requirement for inter-band asynchronous NR DC

| Max {Sub-carrier spacing in Pcell (kHz), Sub-carrier spacing in PSCell (kHz)} | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| 15 | 500 |
| 30 | 250 |
| 60 | 125 |
| 120 | 62.5 |
| 480 | 15.625 |
| 960 | 7.8125 |

### deriveSSB-IndexFromCell

Assumptions on deriveSSB-IndexFromCell

- For 480 kHz SSB SCS

  - deriveSSB-IndexFromCell is always enabled by the network

- For 960 kHz SSB SCS

  - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled)

  - FFS: NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled

  - FFS: If deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified for 960kHz SCS

- FFS whether the deriveSSB_IndexFromCell is always enabled or not in unlicensed band in FR2-2.

  - Note: It’s not always enabled in FR1 NR-U.

Tolerance for frame boundary alignment

- Keep the existing requirements, i.e., 2 SSB symbols, for SCS up to 240kHz

- FFS: For 480kHz SCS:

  - Option 1a: 3 SSB symbols

  - Option 1b: 2 SSB symbols

- FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

  - Option 2a: 3 SSB symbols

  - Option 2b: 2 SSB symbols

- FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

  - Option 3a: 6 SSB symbols

  - Option 3b: 4 SSB symbols

- FFS: Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS

## Scheduling restrictions

Impact of beam switching time on scheduling restrictions

- Scheduling restrictions due to beam switching time need to continue waiting for the conclusion about beam switching time from RF session

- Revisit how to introduce the differentiation in scheduling restriction requirements, if there is RAN1 agreement on UE capability signaling on beam-switching time.

Scheduling restrictions

- Introduce scheduling restrictions for one symbol before and one symbol after the measurement resources (SSB, CSI-RS etc.) during L1 measurements for 480/960kHz SCS

- FFS: Introduce a total of K (due to synchronization error) + L (due to beam switching time) + M (due to propagation delay difference, TA adjustment etc) symbols scheduling restriction before and after SSB transmission –

  - For 480 kHz SCS,

    - Option 1: K=2

    - Option 2: K=3

  - For 960 kHz SCS,

    - K=3

  - L = 1 for 480/960kHz SCS

  - FFS: M for 480/960kHz SCS

    - Consider propagation delay difference of 3.33us considering FR2-2 coverage of 1km

## Measurement procedures

Channel model

- The channel model from RAN1 [38.808] should be used for FR2-2.

  - AWGN and TDL-A (5/10/20ns delay spread)

Cell detection

- Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2, considering the agreed channel model (from RAN1)

PBCH detection for SSB index acquisition

- FFS: PBCH detection for SSB index acquisition for 480/960 kHz SCS

SSB measurements

- SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency and inter-frequency measurements.

# References

- R4-2202733, Email discussion summary for [101-bis-e][216] NR_ext_to_71GHz_RRM_1, Qualcomm, RAN4 #101-bis-e, January 17-25, 2022