3GPP TSG-RAN WG4 Meeting # 101-bis-e 									R4-2202667

Electronic Meeting, Jan. 17th – 25th, 2022

Agenda item:			6.19.3

Source:	Samsung

Title:	WF on FeMIMO RRM requirements for inter-cell beam management

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM w.r.t. inter-cell beam management in RAN4 #101-bis-e meeting.

Way Forward on Inter-cell Beam Management

- Specification Structures for Inter-cell L1-RSRP Measurement

9.12 L1-RSRP measurements for a cell with PCI different from serving cell 9.12.1 Introduction 9.12.2 Requirement applicability 9.12.3 Measurement reporting requirements 9.12.4 L1-RSRP measurement requirements 9.12.5 Measurement restriction 9.12.6 Scheduling availability

- Requirements for inter-cell L1-RSRP measurement

  - For inter-cell L1-RSRP measurement performed outside or inside SMTC, the timing offset between a serving cell and a cell with different PCI is studied :

    - Option i : CP length with a single FFT assumption

    - Option ii : study the measurement behaviour if timing offset is more than CP

  - The same Rx beam assumption for serving cell and cell with different PCI outside SMTC

  - RAN4 will further study whether using same RX beam for L3 and L1 inside SMTC

  - L1-RSRP measurement requirement for cell with different PCI will be specified for known condition and unknown condition

  - Known condition for cell with different PCI shall include at least

    - RAN1 agreements for non-serving cell, i.e., same center frequency, SCS, SFN offset

    - Cell detectable condition (FFS : existing intra-frequency measurement can be reused)

    - Timing alignment between serving cell and cell with different PCI are within CP

    - FFS other conditions

  - RAN4 will further study the flow for L1-RSRP measurement, (e.g., whether the L3 measurement will be configured before L1 measurement configuration. )

  - RAN4 will further study the measurement requirements for L1-RSRP measurement taking the existing R15/16 measurement requirements as starting point

  - RAN4 will further study the sharing factor for the case that SSB for serving cell and cell with different PCI are fully overlapped

  - For FR1 and FR2, the requirements for L1-RSRP measurement of cell with different PCI will be specified for measurement within STMC and outside SMTC