3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2202668

Electronic Meeting, January 17 – 25, 2022

Title: 	WF on other RRM requirements for FeMIMO

Source: 	Huawei, HiSilicon

Agenda item:	6.19.3

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #101bis-e meeting.

Way-forward

- Issue: Requirement for TRP specific Beam Failure Recovery

- Agreements

  - In FR2, introduce sharing factor PTRP for BFD and CBD evaluation period when BFD-RS or CBD-RS from 2 TRPs are overlapped on the same symbol.

    - FFS: whether to introduce the sharing factor PTRP for FR1.

    - FFS: value and conditions to apply the sharing factor PTRP.

- Issue: QCL definition

- Agreements

  - For DL TCI chain, SSB associated with a different PCID is added as a source RS. DL TCI chain can apply for separate DL TCI state or joint TCI state switching.

  - FFS: whether SRS should be part of the QCL chain definition.

- Issue: BFR for CORESET with two activated TCI states

- Agreements

  - For a CORESET with two activated TCI states, UE evaluates the RLM/BFD based on single hypothetical PDCCH BLER for the CORESET.