3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202694

# Electronic Meeting, 17th – 25th January, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for interruptions due to RRM measurements on deactivated SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specify interruption requirements due to RRM measurements on deactivated SCG for NR-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify interruption requirements due to RRM measurements on deactivated PSCell and RRM measurements on deactivated SCC in deactivated SCG for NR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement is applied to interruptions due to RRM measurements on deactivated SCG for NR-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1, New clause 8.2.4.2.X. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change1>

#### 8.2.4.1	Introduction

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

measurements on the deactivated PSCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell.

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

# <End of change1>

# <Start of change2>

##### 8.2.4.2.X	Interruptions due to RRM measurements on deactivated SCG

Interruptions on PCell or activated SCell(s) due to measurements on the deactivated PSCell are allowed with up to [TBD] probability of missed ACK/NACK feadback.

|  |  |  | Nslotsubframe,μ |
| --- | --- | --- | --- |
|  |  |  | Nslotsubframe,μ |
|  |  |  | Nslotsubframe,μ |
|  |  |  | Nslotsubframe,μ |
| Nslotsubframe,μ |  |  |  |

Editor’s Note: whether the interruptions caused by measurements on deactivated PSCell follows SCell dormancy or deactivated SCC is FFS; whether to differentiate cases between either RLM/BM is configured on the deactivated PSCell or not is FFS; the rate of the ACK/NACK feedback loss is FFS in any of the mentioned cases.

Interruptions on PCell or activated SCell(s) due to measurements on the deactivated SCell(s) other than PSCell within the deactivated SCG shall meet requirements in clause 8.2.2.2.3.

|  |  |  | Nslotsubframe,μ |
| --- | --- | --- | --- |
|  |  |  | Nslotsubframe,μ |
|  |  |  | Nslotsubframe,μ |
|  |  |  | Nslotsubframe,μ |
| Nslotsubframe,μ |  |  |  |

# <End of change2>