3GPP TSG-RAN4 WG4 Meeting #101-bis-e	R4-2202761

# Electronic meeting, January 17 – January 25, 2022

# Title:	Reply LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Response to:	R1-2112802 (R4-2200044)

Release:	Rel-17

Work Item:	NR_redcap

Source:	RAN4

To:	RAN1

Cc:

# Contact Person:

# Name:	Qian Yang

# E-mail Address: qian9.yang@vivo.com

# 1. Overall Description:

RAN4 thanks RAN1 for the LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE. RAN4 discussed the working assumptions in the LS and would like to provide feedback as follows.

For working assumption,

For a separate initial DL BWP (if it does not include CD-SSB and for FR1 the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective

it is feasible from RAN4 perspective.

For working assumption,

For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and for FR1 the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.

RAN4 reached conclusion that it is feasible as described below.

For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0):

- A RedCap UE that supports FG 6-1a but NOT support CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support SSB based L3 measurement, but cannot support CSI-RS based L3 measurement.

- A RedCap UE that supports FG 6-1a and CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support both SSB based L3 measurement and CSI-RS based L3 measurement with associated SSB.

    - The existing CSI-RS L3 measurement requirements for 2RX UE are NOT applicable for Redcap 1 Rx UE.

    - RAN4 will not define CSI-RS L3 based measurement requirements for Redcap 1RX UE in Rel-17.

- For serving cell timing related requirements, RAN4 will not define requirements based on CSI-RS in Rel-17.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 feedback into account in the future work.

3. Reference:

- R1-2112802, LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #102-e		February 21 – March 3, 2022			Electronic Meeting