3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202764

Online Meeting, Jan 17 – 25, 2022

Agenda item:	8.9.4, 8.9.5

Source: 	Huawei, HiSilicon

Title: 		WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC

Document for:	Approval

- Introduction

This document contains the agreements and discussion in RAN4#101-bis-e in email thread [101bis-e][229] NB_IOTenh4_LTE_eMTC6_RRM.

- Topic#1 Neighbour cell measurements in connected mode before RLF or NB-IoT

Sub-topic 1-1 intra-frequency measurement requirement

Issue 1-1-1: Intra-frequency requirement when DRX is not configure

Agreement:

Tdetect_intra = 1400 ms

Tmeasure_intra = 800 ms for NRS-based measurement

Tmeasure_intra = 1600 ms for NSSS-based measurement

Issue 1-1-2: Intra-frequency requirement when DRX is configured

Agreement:

Tdetect_intra = 6 DRX cycles

Tmeasure_intra = 5 DRX cycle

Sub-topic 1-2 inter-frequency measurement requirement

Issue 1-2-1: General – How to formulate inter-frequency measurement

Agreement:

- Define inter-frequency measurement requirements based on number of available samples in available measurement occasions.

- Number of samples are determined based on agreed time in LS reply and sampling rate.

- UE can process multiple samples within in one measurement occasion at a time

- FFS：Add addition assumption on minimum length of measurement occasions to avoid high overhead which will be discussed in issue 1-2-3

Issue 1-2-2: General – How to formulate inter-frequency detection

Agreement:

- Define inter-frequency detection requirements based on number of available samples in available measurement occasions.

- Number of samples are determined based on agreed time in LS reply and NPSS/NSSS interval.

- UE can process multiple samples within in one measurement occasion at a time

- FFS: Add addition assumption on minimum length of measurement occasions to avoid high overhead which will be discussed in issue 1-2-3

Issue 1-2-3: Condition on inter-frequency measurement

Sub-1: Interruption due to data traffic

FFS:

Option 1: The measurement period requirements for inter-frequency cell detection and measurements in connected mode shall apply assuming no UL/DL scheduling for the UE during the measurement period.

Sub-2: Minimum length per occasion

FFS:

Option 1

Minimum length of occasion for measurements:  50 ms

Minimum length of occasion for cell detection:  200 ms

Option 2

Minimum length of occasion for cell detection is 21 ms.

Minimum length of occasion for NRS based measurement is 11 ms, and 21ms for NSSS based measurement.

Sub-3: Assumed sampling rate for measurement

Agreement:

- Assumed sampling rate for measurement is 20 ms for NRS and 40 ms for NSSS

Sub-4: Whether to have minimum spacing between occasions.

Agreement:

- No need to require a minimum spacing between measurement occasions

Issue 1-2-4: Detailed requirements for inter-frequency measurement

Agreement：

Requirements for inter-frequency measurement on a carrier different from serving carrier requirements is defined as:

Tmeasure_inter = ∑(i=1)N Min(5000,Ta,i )ms,

- where Ta,i is the interval between available measurement samples in measurement occasions according to the conditions for inter-frequency measurement. And UE is not required to monitor NRS and NSSS more frequent than once per 20ms and once per 40ms, where Ta,i ≥ 20 ms for NRS and Ta,i ≥ 40 ms for NSSS.

- N = 60 for NRS-based measurement and 40 for NSSS based measurement.

- UE will restart the measurement when the interval between two samples are larger than 5000 ms, and the delay requirements are extended accordingly.

- The requirements apply when the Tmeasure_inter is less than X seconds. X is FFS.

Issue 1-2-5: Detailed requirements for inter-frequency detection:

Agreement：

Requirements for inter-frequency detection on a carrier different from serving carrier requirements is defined as:

Tdetect_inter = ∑(i=1)N Min(5000,Ta,i )ms,

- where Ta,i is the interval between available measurement samples in measurement occasions according to the conditions for inter-frequency measurement. And UE is not required to monitor NPSS/NSSS more frequent than once per 40ms, where Ta,i ≥ 40 ms, and N = 70.

- UE will restart the detection when the interval between two samples are larger than 5000 ms, and the delay requirements are extended accordingly.

- The requirements apply when the Tdetect_inter is less than X seconds. X is FFS.

Sub-topic 1-3 Multi carrier operation

Issue 1-3-1: Requirements when UE is required to monitor multiple carriers

Agreement:

- Requirements for cell detection and measurement when UE is required to monitor multiple carriers are defined as:

  - Tdetect = Tdetect _intra+Nfreq* Tdetect_inter

  - Tmeasure = Tmeasure _intra+Nfreq* Tmeasure_inter

- Where Nfreq is number of inter-frequency layers to be measured according to the measurement capability.

Issue 1-3-2: Carriers for neighbour cell measurement

FFS:

Option 1:

The UE continues measuring on at least one neighbour cell detected in RRC_IDLE/INACTIVE modes at least once every 5 seconds after transition to RRC_CONNECTED state provided that it belongs to the serving carrier frequency and meets the side condition (Es/Iot >= -6 dB).

Sub-topic 1-4 Scenarios

Issue 1-4-1: Whether to deprioritize defining requirements when serving cell in enhanced coverage.

Agreement:

- Define CONNECTED mode neighbour cell measurement requirements when the serving cell is in enhanced coverage

Sub-topic 1-5 Additional triggering conditions

Issue 1-5-1: Whether to have additional triggering conditions

FFS:

Option 1: In addition to the already agreed triggering conditions, the UE shall initiate the neighbour cell measurements if K number of out-of-sync indications are detected in the cell.

Sub-topic 1-6 Indicate when UE needs to start inter-f measurements

Issue 1-6-1: Whether to indicate when UE needs to start inter-f measurements

Agreements:

It is within in RAN2 scope and no need to further discuss in RAN4

Topic#2 channel quality reporting table for 16-QAM in TS 36.133

Issue 2-1-1: Capture channel quality reporting table for 16-QAM in TS 36.133

Agreement:

- Capture the mapping table for channel quality reporting of 16-QAM in TS 36.133, and the corresponding performance requirements will be discussed in Demod session.

Issue 2-1-2: Capture core requirements of channel quality reporting for 16-QAM in TS 36.133

Agreement:

- The core requirements related to channel quality reporting for 16-QAM (e.g. measurement resource and/or period) will be discussed in Demod session, and the corresponding requirements can be captured in TS 36.133

Draft CR Work split

Work split

| Requirements | Section | Spec | Company |
| --- | --- | --- | --- |
| Connected mode intra-frequency neighbour cell measurement before RLF | 8.14.5 (new) 8.14.5.1 (new) | TS 36.133 | Huawei |
| Connected mode inter-frequency neighbour cell measurement before RLF | 8.14.5 (new) 8.14.5.2 (new) | TS 36.133 | Ericsson |
| Channel quality report |  | TS 36.133 | Ericsson |

3. References

[1] R4-2202746	Email discussion summary for [101bis-e][229] NB_IOTenh4_LTE_eMTC6_RRM.