3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2202974

Electronic Meeting, 17th  – 25th Jan, 2022

Agenda item:			6.17.1, 6.17.2

Source:	Moderator (Samsung)

Title:                       Email discussion summary for [101-bis-e][310] NR_eIAB

Document for:	Information

Introduction

According to discussion in previous meeting, RAN4 will continue discussion on timing enhancement and simultaneous operation of IAB node’s child and parent links within this meeting in RF session. Considering the Rel-17 completion is approaching, it’s recommended to converge on technical discussion in 1st round and to consider discussion on work split for CR draft in 2nd round.

In addition, there is a LS from RAN1 to inquire RAN4 regarding range of power control parameters. It targets RAN4 provide reply LS with answer to facilitate RAN1/2 signalling design.

Topic #1: RF requirement impact due to NR eIAB

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200936 | Samsung | Proposal: it’s suggested to update clause 4 in TS38.174 with below information - Requirement applicability rule for IAB node in simultaneous operation - Different declaration is allowed according to declaration specified in conformance specification - Timing error between IAB in case 6 timing applies for simultaneous transmission only. |
| R4-2201595 | Nokia, Nokia Shanghai Bell | Observation 1: RAN1 solutions described in [3] do not remove the need to provide clarifications to RAN4 minimum requirements on power imbalance. Proposal 1: Clarify in RAN4 core specifications that during simultaneous IAB-MT and IAB-DU operation, EVM and relative ACLR requirements are required to be met only when operating within manufacturer declared power imbalance. |
| R4-2201700 | Ericsson | Observation 1 Declaration of the Tx power of IAB-MT and IAB-DU for simultaneous transmission should avoid the EVM impact of wanted signal and control the interference from higher power into minimal. Observation 2 EVM could meet the EVM requirement in TS 38.174. Proposal-1: EVM requirement should not be impacted for the simultaneous IAB-MT and IAB-DU transmission. Observation 3 The same ACLR/CACLR requirement would apply for the same PSD configuration of the IAB-MT and IAB-DU simultaneous transmission. Observation 4 Relaxing the CACLR for the different PSD of IAB-MT and IAB-DU simultaneous operation may impact the coexisting performance Proposal-2: The same ACLR/CACLR requirement would apply to the mixed IAB-MT and IAB-DU carrier configuration with different PSD level. Observation 5 There is no other RF requirement for forward-compatibility of the FDM operation of the IAB-MT and IAB-DU except the local area IAB-MT ACLR for type 2-O operating in DL timeslot. - Proposal-3: Capture general statement of “the same multiple carrier TX requirement of either IAB-MT or IAB-MT in contiguous or non-contiguous operation will apply to IAB-MT and IAB-DU simultaneous transmission” in either core specification of TS 38.174 or testing specification of TS 38.176-1/2. |
| R4-2200839 | ZTE Corporation | Observation 1: Timing difference between parent node DL timing and child node DL timing will put impact on UL Rx of the parent node when child node operates in Case 6 timing. Proposal 1: The total impact of Te between parent DU DL timing and child DU DL timing and Te between DU DL timing and co-located MT UL timing need to be considered to ensure UL Rx timing of the parent DU. |
| R4-2200937 | Samsung | For case 1 timing: Observation 1: Parent IAB-DU and Child IAB-DU DL synchronization requirement has been discussed and confirmed to the same as legacy cell phase accuracy requirement for timing case#1. Observation 2: from co-existence purpose the legacy sync accuracy would be enough to guarantee the system performance. Observation 3: no OTA-S requirement has been introduced in Rel-16 considering the necessity and complexity. For case 6 timing RAN1 conclusion: Observation 4: To enable timing case#6 by OTA synchronization, the mechanism would be still based the same criteria baseline applied for timing case#1. Observation 5: Rel-16 OTA synchronization specification will be updated further on T_delta range dedicatedly for timing case#6 to correct potential misalignment between parent and child. For case 6 timing RAN4 requirement: Observation 6: PHY design would be available to enable the compensation on misalignment between parent and child node by child IAB. Proposal 1: whether additional timing misalignment between parent node and child node should be defined for timing case#6 should be decision in RRM session. Observation 6: 3us timing error between IAB-MT and IAB-DU for intra-node in case 6 timing would be enough range from co-existence perspective for functional verification. Observation 8: Timing error within IAB-MT and IAB-DU are relative accuracy between two interfaces with no directly impact on parent node reception. Observation 9: 3us TAE has already been supported in gNB and IAB-DU for inter-band /intra-band non-contiguous CA and no issue identified for intra-node transmission. Proposal 2: 3us timing error is applied to IAB-node operating in case 6 timing alignment. |
| R4-2201596 | Nokia, Nokia Shanghai Bell | Proposal 1: Consider IAB timing error between IAB-DU and IAB-MT as the average frame timing difference between any two transmissions on IAB-DU and IAB-MT on different transmit antenna connectors or different physical antenna ports. Proposal 2: Consider smaller value from 3us (Alt 1) and cyclic prefix length of largest supported SCS (Alt 2) for IAB timing error requirement between IAB-MT and IAB-DU transmissions for both SDM and FDM operation when case #6 timing is used. Proposal 3: Specify clearly new test configuration(s) and test model(s) to verify IAB timing error between IAB-DU and IAB-MT transmission for both SDM and FDM operation when case #6 timing is used. |
| R4-2201701 | Ericsson | Observation 1 The IAB-MT can transmit in DL time slot in one sector and IAB-DU simultaneously transmit in another sector within the same IAB node. Observation 2 The TAE between IAB-MT and IAB-DU covers the cases where an IAB-MT and IAB-DU are transmitting at the same radio unit or at different radio unit, as long as these different radio units belong to the same IAB-node. Observation 3 The transmission time error when an IAB-MT transmits at one radio unit alone in DL time slots needs to be included in investigation scope as one scenario to be investigated for Case-6 timing. Proposal-1: Cell phase sync requirement in TS 38.133 apply to the IAB-MT indirectly when IAB-MT transmitting in DL time slot for Case-6 timing. Observation 4 TAE between IAB-MT and IAB-DU DL timing will violate the cell phase sync requirement. Observation 5 TAE between IAB-MT and IAB-DU DL timing may take budget from the IAB-DU cell phase sync and increase the IAB hardware cost Observation 6 From parent IAB-DU receiving perspective, both Case-6 and Case-7 need to tolerate the timing uncertainty from IAB-MT. Observation 7 For the case of child IAB-MT synchronizing with co-located child IAB-DU, Parent IAB-DU needs to be aware about the TAE between its DL timing and the DL timing of child IAB-DU for case#6 timing operation. so the correct setting of the receiving timing on parent IAB-DU will be possible Observation 8 Parent IAB-DU set its receiving timing differently depending on the child IAB-MT synchronization implementation. Proposal-2: For shared hardware architecture, the parent IAB node should tolerate the maximum 3 us timing error uncertainty between its child IAB node and its own DL timing. Proposal-3: Send LS to RAN1 asking if parent IAB can tolerate the 3 us time error between child IAB-MT and parent IAB-DU DL timing with different SCS configuration without additional system overhead with below LS wording: Proposal-4: Cell phase sync requirement covers the Case#6 timing requirement and no need to specify additional TAE. |
| R4-2201939 | Huawei | Compared to the cell sync requirement the additional time alignment error is small so can be ignored. As such the existing cell sync requirements cover this and there is no need to define a new RF TAE requirement. |

Open issues summary

The agreed WF on simultaneous operation and timing case#6 enhancement is as below:

Simultaneous operation: RF requirement impact due to TX power imbalance between IAB-MT and IAB-DU

Agreement

Maintain existing RF requirements, further discuss on conformance test cases for declared Tx power (ZTE, Samsung, Ericsson)

- Including some clarification on requirements applicable rules in core specification for the limitation of meeting relative ACLR requirements at least, FFS for EVM requirements.

- Further work on detailed wording for such clarification

Case#6 timing:

Agreement:

Baseline assumption: Introducing RF requirements for Timing error between own MT TX and DU TX should be defined for case#6 timing

- Pending on further checking whether existing cell phase sync requirements already cover above timing error

- Further check the timing error tolerance between the parent IAB-DU node and child IAB-MT

With above baseline assumption confirmed FFS on:

-reference condition for this requirement

-error requirement with below Alts

- Alt1: 3us

- Alt2: cyclic prefix length of largest supported SCS

- Alt3: value as SCS dependent

Sub-topic 1-1:  Simultaneous operation

There are three contributions on this aspect with proposal aligned quite well with each other based on last meeting agreement as the legacy IAB RF requirement should not be impacted by simultaneous operation. And there are specific examples shown in two contributions regarding how to update specification. The other contribution also provides proposal on how to update specification. All the proposals seems not contradict to each other, for which comment can be collected during 1st round.

Issue 1-1: RAN4 RF specification impact due to Simultaneous MT TX/DU TX with imbalance power

- Proposals: Companies are encouraged to provide the comments/view on below options(e.g, whether all of them are agreeable, or only some of them could be acceptable)

  - Option 1: it’s suggested to update clause 4 in TS38.174 with below information(R4-2200936)

    - Requirement applicability rule for IAB node in simultaneous operation

    - Different declaration is allowed according to declaration specified in conformance specification

  - Option 2: Clarify in RAN4 core specifications that during simultaneous IAB-MT and IAB-DU operation, EVM and relative ACLR requirements are required to be met only when operating within manufacturer declared power imbalance.(R4-2201595)

  - Option 3: Capture general statement of “the same multiple carrier TX requirement of either IAB-MT or IAB-MT in contiguous or non-contiguous operation will apply to IAB-MT and IAB-DU simultaneous transmission” in core specification of TS 38.174 or testing specification of TS 38.176-1/2 (R4-2201700)

- Recommended WF

  - TBA

Sub-topic 1-2: timing enhancement

Issue 1-2-1: Timing error between intra-node MT TX and DU TX for case#6

- Proposals: Companies are encouraged to provide the comments/view on below options

  - Option 1: No need to define RF requirement since cell phase synchronization or TAE covers this(R4-2201701, R4-2201939)

  - Option 2: Timing error needs to be defined with below Alts on limitation

    - Alt1: 3µs as function verification (R4-2200937)

    - Alt2: requirement should ensure the parent node reception and/or intra-node performance, e.g. min(3us , 4.69 / (SCS/15 kHz) µs (R4-220839, R4-2201596)

- Recommended WF

  - TBA

Issue 1-2-2: Timing error between parent IAB DU and Child IAB for case#6

- Proposals: Companies are encouraged to provide the comments/view on below options

  - Option 1: whether additional timing misalignment between parent node and child node should be defined for timing case#6 should be decision in RRM session. (R4-2200937)

  - Option 2: (R4-2201701)

    - Cell phase sync requirement in TS 38.133 apply to the IAB-MT indirectly when IAB-MT transmitting in DL time slot for Case-6 timing.

    - For shared hardware architecture, the parent IAB node should tolerate the maximum 3 us timing error uncertainty between its child IAB node and its own DL timing.

- Recommended WF

  - TBA

Issue 1-2-3: Necessity on LS to RAN1

- Proposals: Companies are encouraged to provide the comments/view on below option

  - Option 1: Send LS to RAN1 asking if parent IAB can tolerate the 3 us time error between child IAB-MT and parent IAB-DU DL timing with different SCS configuration without additional system overhead (R4-2201701)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 1-1: simultaneous operation

| Company | Comments |
| --- | --- |
| Samsung | All three options should be agreeable and can be merged. If our understanding to option3 is correct, the general statement proposed to core spec is quite similar to the idea in Alt 2 of option 1(R4-220936) to include one dedicated sub-clause in clause 4 which can be used as baseline for further discussion. And we also pointed in our contribution based on the agreement on general clause, we can review further on whether other clarification needed under specific requirement such as proposal in option 2. |
| Ericsson | Option 3. Seems a generic statement on the RF requirement applicability for simultaneous MT and DU is fine in TS 38.174. For option 1, would declaration be better placed in conformance test specification? |
| Nokia, Nokia Shanghai Bell | We think option 1 and option 2 together is the best solution. Option 1 alone is not sufficient. For content in clause 4 a new clause for simultaneous operation is preferable. Having only an informative note, as discussed in R4-2200936, is not sufficient. We would be ok to state in the new clause that “requirements are valid for any setting unless otherwise stated”, only if it is then also stated otherwise for relative ACLR and EVM in the requirement clauses as suggested in R4-2201595. So far it has not been discussed whether the declared maximum power difference can be such that IAB-MT may not reach its full dynamic range. As IAB-MT is controlled by IAB-DU, it is necessary to make it clear that in case IAB-MT is commanded to operate in a manner which puts the power difference beyond what is declared, requirements may not be met. The fact that IAB-MT is controlled from parent-node is a clear behavior difference to normal base station operation and therefore the practice from base station specification cannot be followed fully. For option 3 it seems that IAB-MT requirement is proposed to be applied for IAB-DU which we think is not a good idea. |

Sub topic 1-2-1:  timing case#6 timing error of intra-node

| Company | Comments |
| --- | --- |
| Samsung | We agree that there is similarity among timing error of intra IAB node under case 6 timing, TAE and cell phase sync requirement. However, it’s clear that the TAE is defined for IAB-DU only and cell phase sync accuracy is between two nodes, in which the function of case 6 timing within IAB node can never be verified. And as it’s closed to the completion of Rel-17, it’s preferred to follow RAN4 legacy agreement to decide the timing error of this requirement with option2. We suggest to define 3us timing error as functional verification. And we also understand better performance is achievable which may not be necessary according to our analysis. But we are willing to compromise to reasonable level of majority view if any. |
| Ericsson | Option 1. The case#6 timing can be enabled with two IAB radio unit in two sectors. TS 38.174 only specify the RF requirement on single IAB radio unit. So IAB-MT transmitting in DL time slot will work irrespective there is TAE between IAB-MT and IAB-DU. Setting TAE only cause negative impact on holdover time performance and thus increase the IAB hardware cost. |
| Nokia, Nokia Shanghai Bell | We prefer option 2, alt2, but could be ok also with alt1. We don’t see how existing cell phase synchronization or TAE cover this. If we rely on them the timing error between IAB-MT and IAB-DU is left unverified, with no guarantees on what is implemented and whether co-existence concerns realize in the field. Therefore we see a new requirement and verification of the timing error is necessary. |
| ZTE | Option 2 and Alt 2 is preferred The timing difference between MT UL Tx timing and co-located DU DL Tx timing need to be discussed or assumed to ensure that the UL Rx timing of parent node can be guaranteed when child IAB node operates in case-6 timing. 3us timing difference is not enough, let us take 120 SCS as an example the duration of CP is about 0.6us which is much smaller than 3us. Therefore 3us timing difference between MT UL Tx timing and the co-located DU DL Tx timing is apparently not enough to guarantee the UL Rx timing of the parent node. |

Sub topic 1-2-2: Timing error between parent IAB and Child IAB

| Company | Comments |
| --- | --- |
| Samsung | It should be scope of RRM session as indicated in option 1. We also ask clarification in RRM thread [218]. |
| Ericsson | Cell phase sync is RRM requirement so decision in RRM session is needed. Parent IAB-DU receiver tolerance of the timing uncertainty from IAB-MT should be considered as parent IAB-DU receiving timing setting will be set differently when receiving child IAB-MT at case-6 timing mode compared to the case-1 timing mode. for case-6 timing, the IAB-DU receiver timing should be set with IAB-MT propagation delay + timing uncertainty relative to its own DL timing. While for case-1 receiver timing, it is set to align with its own DL timing. By the consensus on the tolerance timing uncertainty, it means at least IAB-MT timing will not violate the cell phase sync timing. We think this need to be discussed as companies want to set the different TAE between the IAB-MT/IAB-DU in issue 1-2-1 and whether it impact on the parent IAB-DU tolerance on timing uncertainty needs to be understood. |
| Nokia, Nokia Shanghai Bell | We slightly prefer option 1, though we rather think it could be agreed that this is not part of RF discussion without taking a stance on further RRM discussion. |
| ZTE | It is necessary to discuss the timing error between parent IAB DU Tx timing and child IAB DU Tx timing or a reasonable value rather than 3us need to be determined. |

Sub topic 1-2-3: necessity on RAN1 LS

| Company | Comments |
| --- | --- |
| Samsung | The “3 us time error between child IAB-MT and parent IAB-DU DL” is related to previous topic discussion, which we think should be RRM scope decision. Furthermore, according to our contribution, the mechanism of case 6 timing is agreed with update on Tdelta range to accommodate to the misalignment between parent and child node. 3us if confirmed in RRM session is still the maximum level which can be compensated with mechanism defined in RAN1. No matter overhead or not, there is already RAN1 decision. In summary, we discussed similar issue several meetings, even there is updated proposal for this meeting, still not convinced by the necessity of the LS. |
| Ericsson | Once the tolerance of the timing uncertainty would be agreed in issue 1-2-2, the next question is that how its tolerance would be designed and whether or not it has impact on RAN1 spec. One aspect to check is that whether this “new” receiving timing setting to tolerate the 3 us timing uncertainty could be implementation specific and has no RAN1 impact. Using the extend cyclic prefix to tolerate this seems not spectrum efficient. Normal CP could be used for proprietary implementation, this, however, may have multi-vendor interoperability issue, if one IAB using extend CP while another one has proprietary solution. But seems such detail of behavior/configuration discussion is beyond the RAN4 domain. |
| Nokia, Nokia Shanghai Bell | We do not see a need to send LS. Firstly, a reasonable configuration in the network can be assumed and RAN4 minimum requirements should not cover performance of every possible parameter combination. Secondly, RAN4 is the performance group having the understanding on the impacts. Thirdly, it is under our control whether we specify 3us or more stringent requirement. |
| ZTE | The timing tolerance should be determined in RAN4 group. Whether to send LS can be discussed after a consensus is reached within RAN4 |

CRs/TPs comments collection

N/A

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1: simultaneous operation | Tentative agreements: In general there is no objection to include clarification with new Sub-clause 4.XX for rel-17 enhancement IAB with requirement applicability and allowed dedicated declaration for simultaneous operation. And it would be necessary to review the other TX and RX requirement whether dedicated statement needed. Candidate options: According to contribution submitted in RAN4#101-bis, it’s suggested to discuss the tentative work split on draftCR to 38.174 on simultaneous operation as below from RF perspective: Clause# in TS38.174; Responsible company for draftCR; Note. 3; [Samsung]; If any new definition/symbol/abbreviation needed. 4; [Samsung]; The intention is to include new sub-clause 4.11 for RF. The impact to 4.7 if any belongs to RRM scope.. [5]; N/A; If any update identified, the proponent would be responsible for this update. 6; [Nokia]; Include the update on timing error if agreed.. [7]; N/A; If any update identified, the proponent would be responsible for this update. 9; [TBD]; Include the update on timing error if agreed.. [10]; N/A; If any update identified, the proponent would be responsible for this update. Recommendations for 2nd round: Discuss the work split based on WF for next meeting draftCR input |
| Sub-topic#1-2: timing enhancement | Tentative agreements: Four companies provide comment to issue 1-2-1 on intra-node timing error requirement during 1st round, and the majority view is to introduce the timing error for timing case6 with slight different preference on mismatch value. Four companies provide comment to issue 1-2-2, and it seems common understanding that parent node and child node timing alignment should be scope of RRM. And according to the comment in [218] it seems no common understanding that whether cell phase sync applied to timing case#6 or not. So it’s proposed capture option 1 as agreement with necessary refinement if possible. For issue 1-2-3 there is no consensus to provide LS to RAN1. Candidate options: Issue 1-2-1: check the possibility to define intra-node timing error for timing case#6 with Alt2 of as below: Alt2: requirement should ensure the parent node reception and/or intra-node performance, e.g. min(3us , 4.69 / (SCS/15 kHz) µs (R4-220839, R4-2201596) Issue 1-2-2: check the possibility to agree on option 1 with necessary refinement Issue 1-2-3: make decision in WF based on decision to above issues. Recommendations for 2nd round: Discuss this topic further in round 2 with WF. |

CRs/TPs

N/A

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #2: Range of power control parameters for RAN1 LS-in

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200938 | Discussion on range of power control parameters for Rel-17 eIAB | Observation 1: power adjustment indication to be exchanged between IAB-MT and its parent IAB agreed in RAN1 is for beam direction associating to sounding reference signal. Observation 2: RAN1 has got the conclusion on association with other aspects such as timing mode or PHR. Observation 3: What RAN1 inquire RAN4 is just about range for UL PSD and DL TX power adjustment. Observation 4: for IAB-DU the RE power control dynamic range is specified with maximum range of [-6, +4] dB compared to average RE power for FR1. Observation 5: There is no corresponding RF requirement defined for IAB-DU in FR2. Proposal 1:The desired/actual applied IAB-DU DL power adjustment indication should be able to accommodate up to 10dB power up/down range in step of 1dB with respect to specific beam(s) according to existing RF specification. Observation 6: for IAB-MT the power dynamic range is defined as 5dB and 10dB based on certain reference condition for Wide Area and Local Area class respectively for both FR1 and FR2. Proposal 2: The desired IAB-MT UL PSD range should be able to accommodate up to 10dB power up/down range, according to existing RF specification. Observation 7: the UL PSD range for UE is up to 91dB for FR1 and up to 71dB for FR2. Proposal 3: Decide further on whether UE UL PSD range should be provided for RAN1 reference. |
| R4-2201597 | Draft reply LS to RAN1 on Power control parameters | For DL Tx RAN4 does not specify minimum requirements for PSD adjustments, i.e. implementation with no DL Tx PSD adjustment capability are possible. RAN4 minimum requirements only cover power change due to change in the number of scheduled resource blocks, while PSD stays constant. This power change can be up to 24.4 dB. For UL Tx dynamic range RAN4 minimum requirements both in FR1 and FR2 are - At least 5 dB PSD range for wide area IAB-MT - At least 10 dB PSD range for local area IAB-MT For Rx the closest RAN4 minimum requirement for this scenario is in-channel selectivity, which is a measure of ability to receive wanted signal when there is a higher PSD interfering signal within the same channel. The requirement is not defined for IAB-MT and for IAB-DU the PSD difference can be - Up to 21.3 dB in FR1 - Up to 10.1 dB in FR2 |
| R4-2201702 | LS response on range of power control parameters for eIAB | - The PSD dynamic range limitation of the receiver RF requirements should be considered first. - Only MT/DU shared architecture of an IAB-node is valid in the context of the LS response. - PSD dynamic range for simultaneous receiving should be based on Rel-16 IAB receiver requirement. - Around 17 dB PSD range could be tolerated for the in-band operation of simultaneous IAB-MT and IAB-DU receiving. - Around 37 dB ~ 43 dB PSD range could be tolerated for the out-of-band operation of simultaneous WA IAB-MT and WA IAB-DU for type 1-H and 1-O in FR1 receiving depending on the SCS and channel BW. - Around 45 dB ~ 51 dB PSD range could be tolerated for the out-of-band operation of simultaneous LA IAB-MT and LA IAB-DU for type 1-H and 1-O in FR1 receiving depending on the SCS and channel BW. - Around 24 dB PSD range could be tolerated for the out-of-band operation of simultaneous WA/LA IAB-MT and WA/LA IAB-DU for type 2-O in FR2 receiving depending on the SCS and channel Desired DL TX power adjustment: Answer: The indication to the parent node for the desired DL Tx power reductoin/PSD range should not be larger than the Min ( PSD_Tx_parent, abs (PSD_Tot_range)) when PSD_Tot_range < 0. The PSD_Tot_range is the difference between measured PSD receiving signal difference and the allowed PSD range for simultaneous receiving: - Around 17 dB PSD range could be tolerated for the in-band operation - Around 37 dB ~ 43 dB PSD range could be tolerated for the out-of-band operation of simultaneous WA IAB-MT and WA IAB-DU for type 1-H and 1-O in FR1 receiving depending on the SCS and channel BW. - Around 45 dB ~ 51 dB PSD range could be tolerated for the out-of-band operation of simultaneous LA IAB-MT and LA IAB-DU for type 1-H and 1-O in FR1 receiving depending on the SCS and channel BW. - Around 24 dB PSD range could be tolerated for the out-of-band operation of simultaneous WA/LA IAB-MT and WA/LA IAB-DU for type 2-O in FR2 receiving depending on the SCS and channel P_Tx_parent is depending on the parent IAB-DU TX PSD dynamic range and relates to the answer of question#2. In case of this is not pre-known by IAB-node, this can be ignored. Actual applied DL Tx power adjustment Parent IAB-DU for type 1-H and 1-O could adjust the RE Tx DL power with -3 dB for 16QAM and 0 dB for 64 QAM when the average transmitted power is kept the same (Pmax,c,AC or Pmax,c,TABC or Pmax,c,EIRP) - Answer: Parent IAB-node for type 1-H and 1-O could adjust the RE Tx DL power with -6 dB for QPSk, -3 dB for 16QAM and 0 dB for 64 QAM when the average transmitted power is kept the same (Pmax,c,AC or Pmax,c,TABC or Pmax,c,EIRP) An IAB-node can indicate to a parent node a desired UL Tx PSD range - Answer: Child WA IAB-MT could adjust the UL TX PSD -5 dB and child LA IAB-MT could adjust the UL TX PSD -10dB. |

Open issues summary

It’s suggested to decide the requirement to be refrence for parameters requested by RAN1 in round 1. Then to discuss the wroding for reply LS in round 2 based on consensus of round 1.

There are there quantities can be communicated as agreed RAN1 and  RAN4 is requested to provide input on allowable range for:

- An IAB-node can indicate to a parent node a desired DL Tx power adjustment to reduce the received power level by the IAB-MT. The DL Tx power adjustment is applied only to PDSCH and its associated DMRS and PTRS. (To be discussed in issue 2-3)

- A parent node can indicate to an IAB-node an actual applied DL Tx power adjustment. (To be discussed in issue 2-1)

- An IAB-node can indicate to a parent node a desired UL Tx PSD range. (To be discussed in issue 2-2)

Sub-topic 2: range of power control parameters

Issue 2-1: Actual applied DL Tx power adjustment from parent IAB-DU to its child IAB-MT

This issue is intended to discuss the allowable range for quantity of “A parent node can indicate to an IAB-node an actual applied DL Tx power adjustment” according to RAN1 LS

- Proposals from companies: Companies are encouraged to provide the comments/view on below alternatives

  - Alt 1: Refer to IAB-DU RE power dynamic control range defined in 6.3.1.2 and 9.4.1.2 of TS38.174(R4-2200938, R4-2201702)

  - Alt 2: No DL PSD adjustment requirement defined. Refer to IAB-DU Total power dynamic range in 6.3.1.3 and 9.4.1.3 of TS38.174. (R4-2201597)

- Recommended WF

  - TBA

Issue 2-2: Desired UL Tx PSD range from IAB-MT to its parent IAB-DU

This issue is intended to discuss the allowable range for quantity of “An IAB-node can indicate to a parent node a desired UL Tx PSD range.” according to RAN1 LS.

- Proposals from companies: Companies are encouraged to provide the comments/view on below proposals

  - Proposal 1: Refer to IAB-MT total power dynamic range defined in 6.3.2 and 9.4.2 of TS38.174 (R4-2200938, R4-2201597, R4-2201702)

  - Proposal 2: Decide on whether to mention UE UL power PSD range for RAN1 reference. (R4-2200938)

- Recommended WF

  - TBA

Issue 2-3: Desired DL Tx power adjustment sent from IAB-MT to its parent IAB-DU

This issue is intended to discuss the allowable range for quantity of “An IAB-node can indicate to a parent node a desired DL Tx power adjustment to reduce the received power level by the IAB-MT. The DL Tx power adjustment is applied only to PDSCH and its associated DMRS and PTRS.” according to RAN1 LS.

- Proposals from companies: Companies are encouraged to provide the comments/view on below alternatives, especially please share the understanding or clarification to the requirement relevant to the parameter asked by RAN1.

  - Alt 1: Refer to IAB-DU RE power dynamic control range defined in 6.3.1.2 and 9.4.1.2 of TS38.174 since this is constrained by IAB-DU DL power control parameter(R4-2200938)

  - Alt 2: Range to be derived by IAB receiver requirement since this is constrained by IAB-MT receiver RX PSD range

    - Alt 2-1: Derived by In-channel selectivity defined in 7.8 and 10.9 of TS38.174(R4-2201597)

    - Alt 2-2: (R4-2201702)

      - Derived by I-channel selectivity as defined in 7.8 and 10.9 of TS38.174 for in-band operation

      - Derived by ACS as defined in 7.4.1 and 10.5.1 of TS38.174 for out-of-band operation

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 2-1: Actual applied DL power adjustment from parent IAB-DU to its child IAB-MT

| Company | Comments |
| --- | --- |
| Samsung | Support Alt1 as the question from RAN1 is to ask the PSD range rather than the power difference due to RB allocation. And we believe total 10dB should be taken into account the case that parent node DL is power down from the maximum boosting level. |
| Ericsson | Alt 1 condition is that to keep the averaged power constant. If all RB PSD intends to be lowered, it will not work, so basically this Alt 1 means the case where IAB-MT traffic and other NR UE traffic is multiplexed and averaged power is kept the same. Alt 2 could only be referred if the answer should be valid for all IAB-DU type. There is no RE dynamic for IAB-DU type 2-O. |
| Nokia, Nokia Shanghai Bell | We would be also ok to refer to RE power dynamic control range, given that we also convey the limitations it has for average power and different modulations. However, average power can still be reduced using narrower RB allocation, though naturally this impacts capacity. |

Issue 2-2: Desired UL PSD range from IAB-MT to its parent IAB-DU

| Company | Comments |
| --- | --- |
| Samsung | As proponent we support proposal 1. For proposal 2, this is related whether the forward compatibility should be considered and whether IAB-MT to be equipped with more UE functionality. Anyway, it’s fine to follow majority view. |
| Ericsson | P1 is fine. For P2, if it is not verifiable, how network can trust whatever it reports? |
| Nokia, Nokia Shanghai Bell | We support proposal 1. |

Issue 2-3: Desired DL power adjustment sent from IAB-MT to its parent IAB-DU

| Company | Comments |
| --- | --- |
| Samsung | Our understanding is the Alt1 as there is below RAN1 agreement: The indicated desired/provided DL TX power adjustment is in terms of a relative offset to a CSI-RS TX power that is RRC configured. TCI state ID and RS ID (SSB ID and/or CSI-RS ID) is used to indicate IAB-MT’s DL beam for the desired/provided DL TX power adjustment indication by the IAB-node/the parent-node. In case the desired/provided DL TX power adjustment indication does not include information about the associated IAB-MT’s DL beams, the adjustment is applied to all MT’s DL beams. Hence the question for both desired and actual provided DL TX power adjustment is with respect to certain wanted signal only. And these two parameter should be identical range. Receiver dynamic range may have impact on this discussion as the smaller one can be considered. However, there is no direct receiver dynamic range of wanted signal for IAB-MT and IAB-DU. For receiver requirement of ICS and ACS, the range derived here is the delta of wanted signal and interference signal, which it’s understood to be assumed as the signals to be received simultaneously. However, in this case the receiver should still be able to operate properly to fulfil the RF requirement. Not quite understand how to apply this range as desired DL adjustment information to parent node. |
| Ericsson | We think the point of RX PSD is missed from LS questions, without it, it would not be meaningful to start DL/UL power adjustment as IAB node does not have clue what amount to be adjusted. We are fine to send a question to RAN1 to confirm this without reaching consensus in this meeting. |
| Nokia, Nokia Shanghai Bell | We prefer Alt 2-1. In our view it makes sense that IAB-MT can request DL Tx power adjustment based what the IAB-MT receiver can tolerate. In case the request is beyond what DL Tx can be adjusted, it is still very relevant information for parent node. It should be mentioned in the reply, that IAB-MT in-channel selectivity requirement is not specified, the values are taking from IAB-DU, and are relative difference between two signals. We don’t quite see how out-of-band operation is relevant here. Rather Rx dynamic range could be the reference if we consider maximum power that could be received, but this has the same issue as ICS being not specified for IAB-MT. |

CRs/TPs comments collection

N/A

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2 | Tentative agreements: Issue 2-1: Actual applied DL Tx power adjustment from parent IAB-DU to its child IAB-MT Alt 1 is agreeable as : Refer to IAB-DU RE power dynamic control range defined in 6.3.1.2 and 9.4.1.2 of TS38.174 Issue 2-2: Desired UL Tx PSD range from IAB-MT to its parent IAB-DU Proposal 1 is agreeable as: Refer to IAB-MT total power dynamic range defined in 6.3.2 and 9.4.2 of TS38.174 Candidate options: Issue 2-3: Desired DL Tx power adjustment sent from IAB-MT to its parent IAB-DU No consensus on this issue further discussion is needed for the alternatives: - Alt 1: Refer to IAB-DU RE power dynamic control range defined in 6.3.1.2 and 9.4.1.2 of TS38.174 since this is constrained by IAB-DU DL power control parameter(R4-2200938) - Alt 2: Range to be derived by IAB receiver requirement since this is constrained by IAB-MT receiver RX PSD range - Alt 2-1: Derived by In-channel selectivity defined in 7.8 and 10.9 of TS38.174(R4-2201597) - Alt 2-2: (R4-2201702) - Derived by I-channel selectivity as defined in 7.8 and 10.9 of TS38.174 for in-band operation - Derived by ACS as defined in 7.4.1 and 10.5.1 of TS38.174 for out-of-band operation Recommendations for 2nd round: Discuss further based on reply LS in 2nd round. |

CRs/TPs

N/A

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on work split for eIAB RF impact to TS38.174 | Samsung |  |
| WF on timing enhancement for eIAB | Nokia |  |
| Reply LS on range of power control parameters | Samsung | To: RAN_1; Cc: RAN_2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200839 | Discussion on timing issues for simultaneous operation of IAB | ZTE Corporation | Noted |  |
| R4-2200936 | Specification impact due to IAB simultaneous operation | Samsung | Noted |  |
| R4-2200937 | Discussion on timing enhancement | Samsung | Noted |  |
| R4-2200938 | Discussion on range of power control parameters for Rel-17 eIAB | Samsung | Noted |  |
| R4-2201595 | RF requirements due to Tx power imbalance between IAB-MT and IAB-DU | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2201596 | Discussion on timing error for eIAB | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2201597 | Draft reply LS to RAN1 on Power control parameters | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2201700 | RF core specification impact for Simultaneous operation of DU and MT | Ericsson | Noted |  |
| R4-2201701 | IAB MT /DU case 6 timing | Ericsson | Noted |  |
| R4-2201702 | LS response on range of power control parameters for eIAB | Ericsson | Noted |  |
| R4-2201939 | eIAB Timing error between MT TX and DU TX | Huawei | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Yankun Li | Yankun.li@samsung.com |
| Nokia | Toni Lahteensuo | Toni.Lahteensuo@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)