3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-2203007

E-meeting, 17 – 25 January, 2022

Agenda item:	6.12.2.2

Source:	Intel Corporation

Title:	TP to TR 38.833: Link level simulation results for Inter-user interference suppression for MU-MIMO

Document for:	Approval

# Introduction

This TP provides the update “Link level simulation results” section for Inter-user interference suppression for MU-MIMO. This document was approved in RAN4 #101-e meeting and will be updated in the RAN4 #101-bis-e meeting based on updated results submitted by companies.

# Reference

- RP-211135, Revised WID on Further enhancement on NR demodulation performance, China Telecom, RAN #92e, June 2021.

START OF TEXT PROPOSAL

### 4.3.2	Link level simulation results

In this sub-clause, link level simulation results from different companies are collected for analysis of PDSCH performance in scenario with inter-user MU-MIMO interference. The link level analysis of PDSCH performance is performed under assumptions from sub-clause 4.3.1.

The detailed simulation results from different companies are provided in the attached file ‘Attachment 1 - R4-2119048_Summary of simulation results for intra cell inter-user MMSE-IRC receiver’ and [R4-2200993].

Table 4.3.2-1 provide the summary of simulation results from different companies for scenarios with 2 Tx antenna and TDL-A channel model.

Table 4.3.2-2 provide the summary of simulation results from different companies for scenarios with 2 Tx antenna and TDL-C channel model.

Table 4.3.2-3 provide the summary of simulation results from different companies for scenarios with 4 Tx antenna and TDL-A channel model.

Table 4.3.2-4 provide the summary of simulation results from different companies for scenarios with 4 Tx antenna and TDL-C channel model.

Table 4.3.2-1: Summary of simulation results from different companies for 2 Tx case and TDL-A channel

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  |  |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-MRC |  | MMSE-IRC |  |  |
| #CDM | #Rx | MCS | PMI | Span | Avg | Span | Avg |  |
| 1 | 2 | 4 | Rand | 1.6 | 2.3 | 1.5 | 1.9 | 0.4 |
|  |  |  | Orthog | 1.9 | 1.6 | 1.6 | 1.3 | 0.3 |
|  |  | 13 | Rand | N/A | Inf | 2.4 | 12.7 | Inf |
|  |  |  | Orthog | N/A | Inf | 2.3 | 11.1 | Inf |
|  | 4 | 4 | Rand | 1.4 | -1.7 | 1.0 | -1.4 | -0.3 |
|  |  |  | Orthog | 1.8 | -2.5 | 1.0 | -2.2 | -0.3 |
|  |  | 13 | Rand | No results |  | 1.7 | 7.3 |  |
|  |  |  | Orthog | No results |  | 1.4 | 6.0 |  |
| 2 | 2 | 4 | Rand | 2.3 | 2.0 | 2.5 | 1.7 | 0.3 |
|  |  |  | Orthog | 2.5 | 1.4 | 2.6 | 1.1 | 0.3 |
|  |  | 13 | Rand | N/A | Inf | 2.1 | 12.5 | Inf |
|  |  |  | Orthog | N/A | Inf | 2.3 | 11.0 | Inf |
|  | 4 | 4 | Rand | 2.5 | -2.1 | 2.6 | -2.1 | 0.0 |
|  |  |  | Orthog | 1.9 | -4.0 | 2.0 | -4.1 | 0.1 |
|  |  | 13 | Rand | No results |  | No results |  |  |
|  |  |  | Orthog | No results |  | No results |  |  |

Table 4.3.2-2: Summary of simulation results from different companies for 2 Tx case and TDL-C channel

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  |  |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-MRC |  | MMSE-IRC |  |  |
| #CDM | #Rx | MCS | PMI | Span | Avg | Span | Avg |  |
| 1 | 2 | 4 | Rand | 0.9 | 2.9 | 0.9 | 2.6 | 0.4 |
|  |  |  | Orthog | 1.4 | 2.1 | 1.3 | 1.8 | 0.4 |
|  |  | 13 | Rand | N/A | Inf | [2.0] | [14.9] | Inf |
|  |  |  | Orthog | N/A | Inf | [1.0] | [13.0] | Inf |
|  | 4 | 4 | Rand | 1.3 | -1.0 | 1.0 | -0.9 | -0.1 |
|  |  |  | Orthog | 1.8 | -1.9 | 0.7 | -1.6 | -0.3 |
|  |  | 13 | Rand | N/A | Inf | [1.4] | [9.1] | Inf |
|  |  |  | Orthog | [4.1] | [11.1] | [1.0] | [6.9] | [4.2] |
| 2 | 2 | 4 | Rand | 1.8 | 2.6 | 2.2 | 2.0 | 0.6 |
|  |  |  | Orthog | 2.5 | 1.7 | 2.8 | 1.3 | 0.4 |
|  |  | 13 | Rand | N/A | Inf | [2.0] | [14.7] | Inf |
|  |  |  | Orthog | N/A | Inf | [2.1] | [13.1] | Inf |
|  | 4 | 4 | Rand | 2.1 | -1.5 | 2.1 | -1.6 | 0.1 |
|  |  |  | Orthog | No results |  | No results |  |  |
|  |  | 13 | Rand | N/A | Inf | [2.0] | [8.7] | Inf |
|  |  |  | Orthog | [3.7] | [11.2] | [1.9] | [6.8] | [4.4] |

Table 4.3.2-3: Summary of simulation results from different companies for 4 Tx case and TDL-A channel

| Parameters |  |  |  |  | SNR for 70% of max T-put, [dB] |  |  |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MMSE-MRC |  | MMSE-IRC |  |  |
| #Rx | SUE Rank | IUE Rank | SUE MCS | PMI | Span | Avg | Span | Avg |  |
| 4 | 2 | 1 | 13 | Rand | No results |  | 3.1 | 10.4 |  |
|  |  |  |  | Orthog | No results |  | No results |  |  |
|  |  |  | 19 | Rand | N/A | Inf | 1.8 | 16.8 | Inf |
|  |  |  |  | Orthog | N/A | Inf | 0.9 | 13.4 | Inf |
|  |  | 2 | 13 | Rand | N/A | Inf | [3.2] | [16.6] | Inf |
|  |  |  |  | Orthog | N/A | Inf | [2.9] | [12.7] | Inf |
|  |  |  | 19 | Rand | No results |  | No results |  |  |
|  |  |  |  | Orthog | N/A | Inf | 2.2 | 19.3 | Inf |

Table 4.3.2-4: Summary of simulation results from different companies for 4 Tx case and TDL-C channel

| Parameters |  |  |  |  | SNR for 70% of max T-put, [dB] |  |  |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MMSE-MRC |  | MMSE-IRC |  |  |
| #Rx | SUE Rank | IUE Rank | SUE MCS | PMI | Span | Avg | Span | Avg |  |
| 4 | 2 | 1 | 13 | Rand | N/A | Inf | 2.0 | 11.8 | Inf |
|  |  |  |  | Orthog | No results |  | No results |  |  |
|  |  |  | 19 | Rand | N/A | Inf | 0.7 | 20.9 | Inf |
|  |  |  |  | Orthog | N/A | Inf | 2.2 | 17.2 | Inf |
|  |  | 2 | 13 | Rand | No results |  | No results |  |  |
|  |  |  |  | Orthog | N/A | Inf | 2.1 | 15.2 | Inf |
|  |  |  | 19 | Rand | N/A | Inf | N/A | Inf | N/A |
|  |  |  |  | Orthog | N/A | Inf | N/A | Inf | N/A |

END OF TEXT PROPOSAL