3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2203011

Electronic Meeting, 17th – 25th January 2022

Title: 	Way Forward on MMSE-IRC for intra-cell inter-user interference

Source: 	Huawei, HiSilicon

Agenda item:	6.12.2.2

Document for:	Discussion

Inter-user interference modelling

PMI matrix selection for Co-scheduled UE for 2TX and 4TX

- Background

- Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE - Option 3: Use option 2 for rank 1+1 and option 1 for rank 2+2.

- Agreement

- Agree with option 3.i.e. Use random PMI selection for rank 1+1 and orthogonal PMI selection for rank 2+2

DMRS ports for case with rank 1+1

- Background

- Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Option 3: Variable DMRS port mapping during the test.

- Agreement

- DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

Number of CDM groups without data configuration for case with rank 1+1

- Background

- Depends on issue with DMRS ports for case with rank 1+1. If same CDM groups is used, set number of CDM groups without data to 1, if different CDM groups are used, set number of CDM groups without data to 2, otherwise, FFS.

- Agreement

- Set CDM groups without data to 1 for paired UEs for case with rank 1+1

DMRS scrambling ID for target UE and co-scheduled UE

- Background

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2: Same scrambling ID for all cases - Option 3: Configure variable scrambling ID during the test.

- Proposals

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Option 2: Same scrambling ID for all cases

- Option 3: Configure variable scrambling ID during the test.

Simulation assumptions

Companies can provide the simulation results in next meeting based on the assumptions in Table 2-1:

Table 2-1: Simulation assumptions for MU-MIMO

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD and TDD |  |
| TDD pattern |  |  | 7D1S2U |  |
| MCS |  |  | 13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R ULA Low and 2T4R ULA Low for case with rank1+1 and 4T4R ULA Low (Note 1) for case with rank 2+1 and rank 2+2 |  |
| Number of CDM groups without data |  |  | 1 for case with rank 1+1 and 2 for case with rank 2+2 |  |
| HARQ process number |  |  | 4 for FDD and 8 for TDD |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of Co-scheduled UE is not equal to any column of precoding matrix of Target UE - For case with rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For rank 1+1: TDLC300-100 For rank 2+2: TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

UE feature list

UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

- Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

- Option 2: Mandatory with or without UE capability signaling

- Option 3: Optional with or without UE capability and applicable from Rel-17

- Option 4: Optional without UE capability and applicable from Rel-15

CR split

| Section | Company |
| --- | --- |
| General and applicability section | ZTE |
| PDSCH requirements – 2 Rx | Intel |
| PDSCH requirements – 4 Rx | China Telecom |
| Annex A: FRC | Intel |
| Annex B: MU-MIMO Beamforming Model | Huawei |