3GPP TSG-RAN WG4 Meeting # 101-bis-e                                           R4-2203023

# Electronic Meeting, January 17-25, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on Repeater Specifications

Agenda Item:	6.5.1

Document for:	Approval

# Background

This Way forward contains the agreement and issues for further discussion on Repeater Specifications based on 1st round summary in [1].

# Way forward

Below presented agreements and way forward details are capture for Repeaters Specifications:

- Environmental Requirements

- The environmental requirements are removed from the repeater core specification TS 36.106 and will be defined in the test specifications TS 38.115-1 and TS 38.115-2.

- Reference points

- To consider the conducted reference points for Repeater Type 1-C as shown in Figure 1, and Radiated reference points for Repeater Type 2-O as shown in Figure 2:

Figure 1: Conducted reference points for Repeater Type 1-C

Figure 2: Radiated reference points for Repeater Type 2-O

- Regional requirements

- RAN4 to identify and discuss regional requirements for the repeaters. Consider at least the following:

    - Operating bands

    - OBUE

    - Spurious emissions

- Beam declarations

- It is agreed to use the similar OTA beam declarations for the repeater as the BS.

- Input conditions

- For OTA requirements the input condition must be a specified power and direction

  - Details FFS for each individual test case.

- Tentatively the above can be reworded as:

OTA requirements are applicable and tested with an input signal coming from a single direction with a specified or declared power level. The requirement may be applied based on one input direction or applied separately for each of several input directions. The number of input directions specified for each OTA requirement is FFS

- Output Power

- It is agreed that the directional capability of the DL receive antenna is covered by the UL Tx declarations (and visa-versa).

- Directional capabilities

- It is agreed to keep the option to have 2 output power declarations for FBW greater than 6%.

- Output Power Declaration

- Keep the option to have 2 output power declarations for FBW greater than 6%.

- Output Power Accuracy

- It is agreed that EIRP and TRP have accuracy requirements same as the BS.

# Reference

[1] R4-2202957, Email discussion summary for [101-bis-e][301] NR_Repeater_General, Moderator (Qualcomm)