3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2203035

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			NTN - WF on Rx RF requirements for SAN type 1-O

Agenda item:		6.13.3.4

Document for:		Approval

# Rx RF requirements for SAN type 1-O

## Declaration

Agreed baseline:

Manufacturer declarations would be further discussed in the performance part but the following OSDD and OTA REFSENS RoAoA declaration should be considered as starting point:

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 |  | FR1 |  | conformance directions |
| OTA sensitivity |  | Directional | Minimum EIS |  | OSDD |  | 5 |
| OTA reference sensitivity |  | Directional | OTA REFSENS |  | OTA REFSENS RoAoA |  | 5 |
| OTA Dynamic range |  | Directional | OTA REFSENS |  | OTA REFSENS RoAoA |  | 1 |
| OTA adjacent channel selectivity |  | Directional | minSENS |  | minSENS RoAoA |  | 1 |
| OTA in-band blocking |  | Directional | OTA REFSENS and minSENS |  | OTA REFSENS RoAoA and minSENS RoAoA |  | 5 |
| OTA out-of-band blocking | General requirement | Directional | minSENS |  | minSENS RoAoA |  | 1 |
| OTA receiver spurious emissions |  | TRP | See clause 7.7 |  | N/A |  | See annex I |
| OTA in-channel selectivity |  | Directional | minSENS |  | minSENS RoAoA |  | 1 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: The compliance direction for co-location blocking is applicable for the wanted signal only but not the interfering signal. |  |  |  |  |  |  |  |

## OTA sensitivity

Agreement:

OTA sensitivity shall be specified based on manufacturer declaration.

## OTA REFSENS

Way Forward:

Following option should be further evaluated for the next meeting:

- Option 1: OTA REFSENS shall not be specified

- Option 2: OTA REFSENS shall be specified using the same limits than 1-H ones, adjusted with NTN ΔOTAREFSENS.

## OTA Dynamic Range

Agreement:

OTA dynamic range shall be specified using the same limits than the SAN 1-H ones, adjusted with NTN ΔOTAREFSENS.

## OTA ACS

Agreement:

OTA ACS shall be specified based on the SAN EISminSENS adjusted with the SAN ΔminSENS using the same limits than the 1-H ones.

## OTA Blocking

Agreement:

OTA general blocking shall be specified using the same limits value than the SAN 1-H ones but:

- Based on the SAN EISREFSENS and adjusted with the SAN ΔOTAREFSENS for the interferer (pending on OTA REFSENS decision).

- Based on the SAN EISminSENS  and adjusted with the SAN ΔminSENS for the interferer.

## OTA Out-of-band blocking

Agreement:

OTA out of band blocking shall be specified based on the SAN EISminSENS using the same limits than the SAN 1-H ones.

## OTA receiver spurious emission

Agreement:

OTA receiver spurious emission shall be specified using the same limits as the SAN 1-H ones.

## OTA receiver intermodulation

Agreement:

OTA receiver intermodulation will not be specified.

## OTA In-channel selectivity

Way forward:

Following option should be further evaluated for the next meeting:

- Option 1: OTA In-channel selectivity shall be specified using the same limits as 1-H ones but adjusted with the SAN ΔminSENS  for both wanted signal and interfere values.

- Option 2: OTA In-channel selectivity will not be specified.

# References

- R4-2221321, NTN - Satellite Node Access - OTA Rx requirements, Ericsson

- R4-2201471, Discussion on conducted RF requirements from satellite network perspective, ZTE

- R4-2201820, Discussion on Satellite Access Node conducted RF requirements: Rx Huawei