3GPP TSG-RAN WG4 Meeting # 101-bis-e                           R4-2203036

Electronic Meeting, 17–25 January, 2022

Source: 	Huawei, HiSilicon

Title: 	WF on UE RF requirement for NTN UE

Agenda Item:	6.13.4

Document for:	Approval

Background

In RAN4#101bis-e, companies provides some contributions [1~13] to further discuss NTN UE requirements. Transmitter and receiver requirements for handheld NTN UE were discussed in the 1st round discussion generally and some agreements were reached. In this way forward, we’d like to capture all the agreements in current paper.

Way forward for UE RF requirements

- 64QAM

Include 64QAM (DL and UL ) for NTN satellite UE as optional feature with granularity [per UE]

- Minimum output power

Option 2: Introduce Minimum output power requirements, further discuss whether relaxed value needed or not based on existing TN UE requirements

- General spurious emission

Option 1: NR UE’s general spurious requirements from TS 38.101-1 are reused for NTN.

- Spurious emission for UE co-existence（n255/n256）

Agreement: Specify spurious coexistence limit of [-50dBm] for NTN UE.

FFS on TN bands should be considered as the possible exception.

- Number of Rx antenna

Option 1: Regarding handheld UE size limitation of low- and mid-bands, to adopt UE’s 2RX as reference assumption for diversity gain.

- FRC

Option 1: the FRC for TN UE REFSENS can be reused for NTN UE

- NF, SNR, IM diversity gain

Option 1: The requirements of NF, SNR, IM, diversity gain for TN UE can be reused for NTN UE on same frequency

- REFSENS for n255

Option 1:  to specify the REFSENS for band n 255

REFSENSE of n255

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |

- Maximum input level

Define maximum input level requirements for NTN UE. Further discuss the relaxation compared to TN requirements.

- NBB and IBB for band n255/n256

Option 1: For frequency lower than 2.7GHz, there are potential TN interferers. To consider reusing TN UE NBB and IBB requirements for NTN UE as assumption.

- Rx intermodulation characteristics

Option 1: To assume the same interfering signals for guaranteeing TN and NTN UE performance, TN wide-band intermodulation parameters in Table 7.8.2-1 of TS 38.101-1 could be reused for NTN.

- References

[1]	R4-2200073	Further discussion on UE Tx RF requirements for NTN, CATT

[2] R4-2200331	Discussion on UE TX requirements for NTN, Mediatek India Technology Pvt.

[3] R4-2200783	Considerations on NTN UE Tx requirements, Qualcomm Incorporated

[4] R4-2201310	Discussion on UE Tx requirements for satellite access, Huawei, HiSilicon

[5] R4-2201322	NTN - UE - Tx requirements, Ericsson

[6] R4-2201472	Discussion on NTN UE Tx RF requirements, ZTE Corporation

[7] R4-2200074	Further discussion on UE Rx RF requirements for NTN, CATT

[8] R4-2200332	Discussion on UE RX requirements for NTN, Mediatek India Technology Pvt.

[9] R4-2200784	Considerations on NTN UE Rx requirements, Qualcomm Incorporated

[10] R4-2201224	Discussion on Maximum input level for NTN UE, Xiaomi

[11] R4-2201311	Discussion on UE Rx requirements for satellite access, Huawei, HiSilicon

[12] R4-2201323	NTN - UE - Rx requirements, Ericsson

[13] R4-2201473	Discussion on NTN UE Rx RF requirements, ZTE Corporation