3GPP TSG-RAN WG4 Meeting # 101-bis-e												     R4-2203079

Electronic Meeting, January 17 – 25, 2022

Agenda item:			7.1

Source:	Intel Corporation

Title:	WF on OTA test methods for FR2-2

Document for:	Approval

# Introduction

This Way Forward covers discussions on OTA test methods for 52.6 - 71 GHz (i.e., Objective 7 of the Study on Enhanced Test Methods in FR2)

# Topics

## Maintenance of Objectives 1 - 6

### TP on applicability of release independence

TP R4-2200452 introduces a fourth applicability category for the applicability of release independence to FR2 test methodology enhancements. Content proposed in TP is found below:

- When the enhanced test methodology applies to a UE feature supported only from a specific release, the test method becomes applicable only from that release onwards.

Agreement: Approve R4-2200452

## UE types

### Vehicular UE

Agreement:

- Avoid developing a standardized ground plane for FR2 vehicular UEs as part of this SI.

- Consider the optional ground plane designed and manufactured by the OEM an integral part of the FR2 vehicular UE DUT submitted for conformance testing.

- Consider battery and AC/DC powered operation acceptable for FR2 vehicular UEs and request manufacturers to provide proper guidance on cable routings.

### Handheld UE

Worst-case antenna array size

Core requirement discussions converged on an 8-element antenna array as the assumption used to define requirements for PC3 in FR2-2 [1].

Candidate options:

- 8x2 array

- other

Agreement: The worst-case antenna assumption for testability and MU assessment of handheld UEs in FR2-2 is [8 x2]. Single UE antenna element pattern parameters, similar to Table 5.2.3.3-1, need to be finalized in RAN4#102-e.

## Test methodology for UE RF

### Permitted test methods

Candidate options:

- Proposal: Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size.

Agreement: Approve the proposal

### Enhanced test methodology

Candidate options:

- Proposal: Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2. Objective 3 discussions should be postponed until core requirements are discussed. Lastly, we should further discuss the remaining objectives

Agreement: Approve the proposal and extend applicability of Objective 2 and Objective 5 solutions to FR2-2

## Test methodology for RRM

### Reusing FR2-1 RRM test methodology

Candidate option:

- Proposal 1: All test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 are applicable for FR2-2, except for Noc derivation (TR 38.810 - Clause 6.2.1.4.3) and Maximum SNR derivation (TR 38.810 - Annex B.2)

Agreement: Add test methods in TS 38.508-1 to Proposal 1 content and capture in the revised TP (R4-2203078)

### Noc level derivation

Candidate option:

- Proposal 1: For FR2-2 RRM testing the Noc level is derived similar to the Noc level for UE demodulation test methods

Agreement: Approve Proposal 1

### Assessment of testable RRM DL SNR range

Candidate option:

- Proposal 1: Perform an informative assessment of testable RRM DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point

Agreement: Approve Proposal 1

## Test methodology for UE demodulation and CSI

### Multi-path fading channel model

Candidate option:

- For analysis of FR2-2 test methodology definition for multi-path fading channels consider TDL-A channel model with RMS delay spread is in range of 5-20ns and with 3km/h UE mobility.

Agreement: Consider TDL-A channel model with RMS delay spread as in range of 5-20ns and with 3km/h UE mobility as starting point for the analysis

### Path delay grid

Candidate option:

- For multi-path fading channel modelling use Fsample = 2000MHz for channel bandwidth up to 2000MHz as long the value is feasible from TE implementation perspective

Agreement:

- Max applicable channel bandwidth:

  - Option 1: 2GHz

  - Option 2: Smaller than 2GHz

- Sampling frequency:

  - Option 1: 2GHz

  - Option 2: 800/400MHz

### Noc level derivation

Agreement:

- Define Noc levels for FR2-2 UE demodulation testing only based on the following definition:

Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

- Consider parameters in table below for Noc level definition

| Parameter | Clarification/Value |
| --- | --- |
| REFSENSPCX, BandY | The REFSENS value in dBm specified for Power Class X UE in band Y |
| CBW for REFSENS | 100 MHz |
| SCSREFSENS | 120 kHz |
| PRBREFSENS | NRB associated with subcarrier spacing SCSREFSENS for channel bandwidth used for REFSENS calcualtion |
| SNRREFSENS | -1 dB |
| ∆thermal | An amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB |

- FFS on ∆thermal value revision to cope with potential low max DL SNR

### Assessment of testable Demodulation DL SNR range

Candidate option:

- Ask inputs from TE vendors on the following test system parameters to derive max testable SNR:

  - TE amplifier 1dB compression, dBm;

  - Backoff from 1dB compression, dB;

  - Cable loss, dB;

  - Free space path loss, dB;

  - TE DL absolute power setting uncertainty, dB;

  - Probe antenna gain, dB;

  - Beam peak search procedure/measurement error, dB.

Agreement:

- Prioritize IFF method for the testable DL SNR assessment

- Assume backoff from 1dB compression as [] following the latest conclusion in RAN5 for FR2-1 as starting point value for further assessment

- Assume reference range length as [0.725m] for IFF method.

- Provide the following remaining test system parameters next meeting: TE amplifier 1dB compression, cable loss, free space path loss, TE DL absolute power setting uncertainty, probe antenna gain.

- Add the parameter "Connector insertion loss," to the TR 38.810 DL SNR assessment methodology for FR2-2 assessment with the value to be further checked by the next meeting

# References

- R4-2202366, “WF on 60 GHz UE RF,” Qualcomm, RAN4 #101Bis-e, January 2022

- R4-2203120, “Email discussion summary for [101-bis-e][327] FR2_enhTestMethods,” Moderator (Intel), RAN4 #101Bis-e, January 2022