3GPP TSG-RAN WG4 Meeting # 101-bis-e 												R4-2203090

Electronic Meeting, 17th – 25th Jan, 2022

Agenda item:			6.19.4

Source:	Samsung

Title:	WF on general and CSI requirement for Rel-17 FeMIMO

Document for:	Approval

Background

This contribution is to capture the agreements and the directions for the further studies of performance requirements for  Rel-17 FeMIMO

The following sub-agenda items from e-mail thread [101-bis-e][320] NR_FeMIMO_Demod_NWM are considered:

- Sub-topic 1-1: Work Plan

- Sub-topic 2-1: Test scope for Enhancement on multi-beam operation

- Sub-topic 3-1: General scope for Multi-TRP

- Sub-topic 3-4: Test scope on beam management for multi-TRP

- Sub-topic 4-1: Test scope for Enhancement on SRS

- Topic 5: Enhancement on CSI reporting

The detailed discussions can be found in the e-mail discussion summary [R4-2202976]

Work Plan for Rel-17 FeMIMO performance requirement

- RAN4 #101bis: - Identify performance impact and endorse initial work scope for performance requirements RAN4#102 - Update work scope of performance requirements - Agree initial simulation assumption. - March 2022 RAN#95. - RAN4#103 - Further discuss and resolve remaining open issues for test set-up - Provide alignment simulation results. - June 2022 RAN#96. - RAN4#104 - Provide impairment results - Agree CRs to introduce performance requirements for Rel-17 FeMIMO WI. - Sep 2022 RAN#97.

Scope of Enhancement on multi-beam operation

- No performance requirements (demodulation and CSI) impact for Rel-17 FeMIMO objective “Enhancement on multi-beam operation”.

Scope of Enhancement on multi-TRP

General scope of Multi-TRP

- Postpone the discussion on m-TRP transmission UE demodulation/CSI requirements in FR2 till RAN4 RF and RRM core have conclusion on supporting FR2 UE with the capability of simultaneous reception and non-simultaneous (e.g., PDCCH TDM) with different QCL Type-D RSs. - With current testing methodology has limitation from OTA feasibility to support simultaneous 2 AOA for demodulation requirement test

Test Scope on beam management for multi-TRP

- No performance requirements (demodulation and CSI) on beam management for multi-TRP

Scope of Enhancement on SRS

- No performance requirements (demodulation and CSI) impact for Rel-17 FeMIMO objective “Enhancement on SRS”

Scope of Enhancement on CSI reporting

Test scope of CSI reporting for Multi-TRP transmission

- Introduce CSI requirements for CSI reporting enhancement for m-TRP NCJT transmission - FFS on CQI, PMI, and RI requirement for Single-DCI based Multi-TRP scheme - Option 1: CQI, PMI, RI - Option 2: CQI - Option 3: PMI only - FFS on CQI, PMI, and RI requirement for Multi-DCI based Multi-TRP scheme - Option 1: CQI

Test scope of PMI requirement for Rel-17 eType II port selection codebook for FDD

- FFS to define PMI requirement for Rel-17 eType II port selection

Test scope of PMI requirement for inter-cell interference scenario

- Option 1: Yes - Companies evaluate the impact of false PMI reporting on throughput - RAN4 defines PMI reporting requirement for inter-cell interference scenario - Option 2: No - Option 3: Further evaluation is needed

Test Setup for CSI reporting requirement for multi-TRP

CQI test setup for single-DCI based multi-TRP transmission SDM if introduced

- Option 1: - 2 TPs configured with fully overlapping Resource allocation - One CSI-RS resource set with Ks = 2 - TP1 associated with NZP-CSI-RS resource 1 - TP2 associated with NZP CSI-RS resource 2 - CSI reporting: One CSI associated with multi-TRP measurement hypothesis and X=0 CSI associated with single-TRP measurement hypothesis - CMR group 1 {CMR a} corresponding to NZP CSI-RS resource 1, K1=1 - CMR group 2 {CMR b} corresponding to NZP CSI-RS resource 2, K2=1 - CMR pair (N=1) : CMR {a,b} for M-TRP measurement hypothesis - Fix layer combination and precoding during test cases i.e. 1+1 for 2Rx, 2+2 for 4Rx - No time/frequency offset between two TPs - Other test parameters reusing existing Rel-16 PDSCH requirements with single-DCI M-TRP SDM scheme - Option 2: - Only consider the first reporting method with X=0 for CSI reporting requirement - Number of antenna port, reporting granularity, CSI-RS resource type (P/AP), CSI-RS reporting type (P/AP), test metric, etc. can be reused from the existing CSI reporting cases, i.e. configuration of 4+4/8+8/16+16 port case is corresponding to that of the existing 8/16/32 port case. - Other options are not precluded

Common setup for CSI reporting requirement for multi-TRP

- Option 1: - TDLA30-5 with statistically independent for each TRP - XP High for each TRP for correlation matrix and antenna configuration - Same Pc ratios for each TRP in defining requirement - The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP - Other options are not precluded

Number of CSI-RS Ports for PMI reporting test if introduced

- Option 1 - 8 for each TRP - Option 2 - 4 for each TRP - Other options are not precluded

Test Setup for PMI reporting requirement for eType II port selection if introduced

Scope of PMI reporting requirement for eType II port selection

- Option 1: - RAN4 to evaluate both SU-MIMO and MU-MIMO options for the propagation environment and/or interference setting, when determining the Rel-17 feType II PS performance requirements. - Option 2: SU-MIMO setup

Modeling BF CSI-RS Port

- Option 1 - Option 1a: Apply specific beamforming vector on each CSI-RS pair (polarization) - Option 1b: Apply power scaling factor on each CSI-RS pair (polarization) - Other options are not precluded:

Test Setup for PMI reporting requirement for inter-cell interference if introduced

Simulation Assumption for evaluation

- Option 1: - Consider the simulation assumption of RAN1 evaluation and CQI reporting test for inter-cell interference as a starting point

Possible Test Metric

- Throughput ratio between follow PMI with inter-cell interference and follow PMI without interference as a starting point

Reference

- R4-2202976, Email discussion summary for [101-bis-e][320] NR_FeMIMO_Demod_NWM, Samsung, RAN4#101-e meeting