3GPP TSG-RAN WG4 Meeting # 101-bis-e												R4-2203112

Electronic Meeting, 17 – 25 January, 2022

Agenda item:			6.13.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [101-bis-e][307] NTN_Solutions_Part2

Document for:	Information

Introduction

This summary document captures issues related to NR-NTN coexistence aspects. It contains a summary of the contributions under Agenda Item 6.13.2 at TSG-RAN WG4 #101-bis-e, together with identified key open issues, and recommends topics/questions to be handled via email discussions. The goal of this document is also to provide recommendations on prioritization of discussion and finalize this topic if agreed.

A total of 23 TDOCs have been received for this agenda (See Annex 2) and 4 topics are listed as below to cover proposals and contents in these documents as appropriate.

- Topic #1: Co-existence scenarios and assumptions

- Topic #2: Co-existence results handling

- Topic #3: ACLR and ACS

- Topic #4: HAPS coexistence scenarios and results

To progress the discussion, it is proposed that the meeting could:

- in 1st round: focus on Topic #1, 2 and 4 to finalize scenarios and assumptions for co-existence studies; to agree on the methodology and principle to conclude ACIR based on co-ex results; to make progress on Topic #3 and then assign the editor if needed;

- in 2nd round: focus discussion to determine and agree on ACLR and ACS conclusion on Topic#3; to discuss and agree on the draft TP to update TR 38.863.

Topic #1: Co-existence scenarios and assumptions

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200781 | Qualcomm Incorporated | Observation 1: Coexistence in case 1, i.e., TN DL to NTN DL, is challenging in urban scenario. This is due to the NTN DL being much weaker than TN DL. In the area outside of the TN clusters where UE could receive TN signals, that UE supporting both TN and NTN functionalities shall access TN rather than NTN Therefore, a minimum isolation distance as simulation assumption is required to evaluate required ACIR in urban scenario. Observation 8: Based on the simulation results from Case 4 and given that NTN UE should be placed with a minimum isolation distance in urban scenario, the current BS ACS requirement will allow to meet the coexistence criteria. The NTN UE ACLR requirement can be the same as current TN UE requirement. Observation 11: Based on the simulation results from case 6, lower elevation angles for GEO - e.g., 45 degrees - will require a tighter ACIR of about 6 to 7dB. Based on that, the NTN BS ACS requirement should be around 46 dB same as TN BS ACS requirement. Proposal 1: To use as simulation assumption the minimum isolation distance in urban scenario in Case 1 and Case 4. Proposal 2: To use free space path loss propagation model in Case 5 between NTN UE and TN UE because the BS antenna height range in 38.901 RMa or UMa model exceeds the NTN UE antenna heigh which is 1.5 m. Proposal 5: To consider the lower elevation angles for Case 6 since this represents the worst-case scenario for NTN BS ACS. |
| R4-2201842 | THALES | we recommend one or multiple resolutions from the following proposals: Proposal 1: RAN4 shall not perform coexistence analysis for Case 6 urban. Proposal 2: RAN4 shall perform only coexistence analysis for Case 6 rural. Proposal 3: RAN4 shall decrease the activity/active factor of 20% for Case 6 for NTN coexistence analysis. Proposal 4: RAN4 shall decrease the activity/active factor of 20% for Case 6 urban for NTN coexistence analysis. ----------------------------------------- Proposal 5: RAN4 shall correct the equations used to simplify the NTN coexistence simulations for Case 2 and Case 6 as follows: Proposal 6: RAN4 to continue discussion on scaling factor. Proposal 7: RAN4 to continue discussion on active factor/activity factor. Proposal 8: RAN4 to potential consider rural deployment in adjacent satellite beams (of the main beam serving urban deployment). |

Open issues summary

Sub-topic 1-1

Issue 1-1: Isolation distance consideration

- Proposals

  - Option 1: To use as simulation assumption the minimum isolation distance in urban scenario in Case 1 and Case 4.

- Recommended WF

  - TBA.

Issue 1-2: Case 5 propagation model between NTN UE and TN UE

- Proposals

  - Option 1: To use free space path loss propagation model in Case 5 between NTN UE and TN UE.

  - Option 2:

    - Agree on free space path loss is more appropriate for link between NTN UE and TN UE;

    - Ask the results contributor of Case 5 to indicate their propagation models;

    - Conclude the worst required ACLR from the results using FSL model;

    - Invite contributor to update their results in Case 5, but not mandate.

- Recommended WF

  - TBA

Issue 1-3: Case 6 Urban

- Proposals

  - Option 1: RAN4 shall perform only coexistence analysis for Case 6 rural.

- Recommended WF

  - TBA

Issue 1-4: Active/activity factor

- Proposals

  - Option 1: RAN4 to continue discussion on active factor

  - Option 2-1: RAN4 shall decrease the activity/active factor of 20% for Case 6 for NTN coexistence analysis

  - Option 2-2: RAN4 shall decrease the activity/active factor of 20% for Case 6 urban for NTN coexistence analysis

- Recommended WF

  - TBA

Issue 1-5: Scaling factor

- Proposals

  - Option 1: Continue discussion on scaling factor.

- Recommended WF

  - TBA

Issue 1-6: Case 2 and 6 simplification method

- Proposals

  - Option 1: To suggest following editorial changes

- Step 1: to drop NTN UE per beamprint randomly;

- Step 2: to drop N clusters consisting of 57 sites sectors per beamprint randomly which should be larger than the active TN cluster where the number of active TN clusters is calculated as following:

where active_factor = 20%

- Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:

- active_TN=active_factor*round(the area per beam/the area of 57 sectors)

- active_factor = 20% (or lower, particularly for urban scenarios)

- Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN:

- Recommended WF

  - Agree with Option 1, since 1) this has been offline discussed with the method contributor; and, 2) this was listed only as one of the possible method.

Issue 1-7: Rural deployment in adjacent beam while main beam serving urban deployment

- Proposals

  - Option 1: RAN4 to potential consider rural deployment in adjacent satellite beams (of the main beam serving urban deployment).

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 1-1: Isolation distance consideration

| Company | Comments |
| --- | --- |
| Samsung | Agree on Option 1 for Case 1 only. Since agreed 101-e meeting WF is to consider isolation for case 1 only, and considering timeline of this work and also to avoid further co-ex work after this meeting, we can support to use isolated option in Case 1, but not in Case 4. And also, the current Case 4 results already suggested TN UE ACLR is sufficient to be applied to NTN UE ACLR, which is highly agreeable among all the contributors. No need to complicate Case 4 discussion in such situation. |
| Ericsson | We could agree on considering an isolation distance for case 4 to avoid introducing too stringent ACS requirement but this shall be captured somehow in the relevant TS(s) to avoid any misunderstanding in the future. We agree with Samsung on case 4: we don’t think we should re-run simulations with this assumption for case 4. |
| THALES | Our understanding is that isolation distance could be further used. In any case, in our opinion the simulations results of different companies are still different (e.g. see for example in the case without isolation) because of the selection mechanism between Terrestrial (TN) and Non-Terrestrial Networks (NTN). |
| Huawei | From timeline perspective, we don’t another meeting to resubmit the co-existence results. Thus, we prefer not to change the assumption at this stage. |
| ZTE | For scenario 1, we are fine to have the isolation assumptions, otherwise in urban macro scenario, requirement might be too stringent. For scenario 4, based on the simulation result, the existing TN UE ACLR requirement applied for NTN UE should be sufficient enough. |
| CMCC | If we consider isolation discussion, this should be explicitly reflected into the spec to avoid any mis-understanding. |
| MediaTek | Just to point out that the main issue leading to isolation was the impact from TN network to NTN UE. To avoid performance degradation it seems natural that the NTN UE would naturally connect to the TN in such case, or it would possibly not have a DL NTN connection at all, and not transmit in UL either. We do not see a need to run more simulations, and agree with Samsung that there was actually no NTN UE to TN UL issue identified even with the scenario that the UE is right at the edge of the TN cell (with no isolation) and does maintain the NTN DL. |
| Inmarsat | We agree with MediaTek’s observation and in our view it doesn’t make a lot of sense to include this in the spec, because ultimately the UE will connect to NTN only if DL signal is strong enough and based on NTN/TN selection mechanism. |
| Qualcomm | We agree with companies’ views that we don’t need to re-run the simulation for case 4 since the current results somehow show that the 33dB ACS is enough (without no isolation). We are OK to consider case 1 for the co-ex study. Regarding how to capture the isolation in the TR, we agree with MTK’s comments. It is natural that NTN UE would connect to the TN within the isolation area. We want to highlight that this case 1 is considering that TN DL is interfering to NTN DL. So we suggest to capturing the statements like “If the isolation is not considered, the performance degradation for NTN DL might be expected”. |

Issue 1-2: Case 5 propagation model between NTN UE and TN UE

| Company | Comments |
| --- | --- |
| Samsung | We would like to propose a new Option 2, and support this new option. We also noticed this difference in offline discussion.Considering the results of Case 5 with free space path loss model still suggested smaller NTN UE ACLR than Case 4, we’d like to suggest following option for the meeting to consider: - Agree on free space path loss is more appropriate for link between NTN UE and TN UE; - Ask the results contributor of Case 5 to indicate their propagation models; - Conclude the worst required ACLR from the results using FSL model; - Invite contributor to update their results in Case 5, but not mandate. Then we can avoid further work for this issue. |
| Ericsson | It’s indeed very difficult to conclude on case 5 with the large spread of results between companies. To Samsung: your proposal would be to conclude on the ACIR for case 5 in this meeting based on the submitted results which used the FSL model and ignore the other ones, is that correct understanding? |
| Samsung | To Ericsson: yes, because NTN UE ACLR would highly possible be derived by results in Case 4. Then we can put notes in the TR for Case 5, (for example) explaining the different path losses were used, and the results is like this, and we observed and (maybe) can conclude that the TN ACLR is sufficient to cover Case 5. |
| Huawei | From timeline perspective, we don’t have another meeting to resubmit the co-existence results. Thus, we prefer not to change the assumption at this stage. However, if working group conclude case 5 is not the worst case even if we use the FSL model, it seems that we don’t need to consider case 5. |
| ZTE | We don’t have strong opinions on this issues since case 5 might be not most demanding cases to determining NTN UE ACLR requirement compared with case 4. |
| CMCC | In TR 37.885, there is V2V propagation model which could also be applicable for P2P scenario. This could be regarded as reasonable UE-UE propagation model. Of cause free space could contribute to severe interference and hence could be regarded as the baseline and we could accept option 2 proposed by Samsung. Compared with case 5, case 4 show more severe interference and hence requires more stringent NTN UE ACLR which will determine final ACLR definition. So maybe the propagation model between UE and UE doesn’t contribute too much for final results and we could capture all the simulation result in the TR with some explanation of propagation model. |
| MediaTek | Agree with Huawei. Also, there may be some UE-UE pairs that have objects/buildings between them in practice so not sure that free space is always the most realistic. |
| Inmarsat | We agree with the views that we should not spend additional effort in aligning the results of case 5 if it’s not going to be useful towards the definition of ACLR. |
| Qualcomm | Agree with option 2. We agree case 5 is not the case to determine the ACLR. But considering the simulation results will be captured in 3GPP TR, so it is ncouraged if companies can align the simulation results. |

Issue 1-3: Case 6 Urban

| Company | Comments |
| --- | --- |
| Ericsson | We don’t really understand the given rationale (size fo the cities) for this proposal, why it would be specific for this case only, it would also be applicable for other cases then… We don’t agree then with option 1. |
| THALES | We believe that the urban scenario is not very realistic to be used in the main beam and in the adjacent beams. Today such metropolitan area with such network density does not exist. For example, in the case of LEO@1200, the urban area has a diameter of approximatively 300km, and the satellite covers more than 10000 BS. This does not seem realistic in terms of simulation assumptions. Another option is to consider e.g. rural deployments in the adjacent beam from the main beam, but is still too much. |
| Samsung | Considering the results in Case 6 is spreading, and the objective of this case is to derive NTN BS ACS, we understand if the requirement is too high, then there’s no practical meaning. Since this case is for NTN BS ACS, we are wondering what’s the best effort ACS we can expect on satellite from the current technology and manufacturing. Can satellite companies or operators to give some reference? We think this can be helpful for this issue and also the issues 1-4, 1-5 and 1-7. |
| Huawei | No strong view on this proposal. |
| ZTE | It’s not good to revisit the certain deployment in certain scenarios which is also aligned in other scenarios. It’s better to follow the previous agreement. |
| CMCC | If we only consider rural scenario, we should emphasize into the spec that urban scenario is not considered. Our preference is to still include all the rural and urban scenario into the simulation. |
| Hughes/EchoStar | Agree with Option 1 for now. |
| Inmarsat | We don’t have strong views, but we tend to agree with Thales, moreover, it’s dubious how much additional information the urban scenario will yield vs the additional effort required to properly address it. It’s also unlikely that it will yield to stricter requirements for NTN so there is no need to restrict the deployment either. To answer Samsung: we need some more time to answer this question, but we don’t think today ACS is specified for satellite access nodes (TBC). |
| Qualcomm | We don’t agree with option 1. Considering the large beam coverage of satellite, it is not possible to restrict the interfering ONLY coming from Rural gNB. We think the worst interference scenarios for case 6 is Urban with low elevation angle, i.e., 45 deg. In summary, our proposal is to consider Urban with 45deg elevation angle for case 6. |

Issue 1-4: Active/activity factor

| Company | Comments |
| --- | --- |
| Ericsson | We think it’s too late to come back on the agreed assumptions without a very strong rationale to do so. |
| THALES | Please see above, it seems that the density of the BS is too high and over a very large region. Small question (if the proponents of this value could reply): which was the exact rationale for setting the initial active/activity factor of 20%? We tried to find it in TR 38.803 and TR 36.942, and in any case, for NTN it does not seem realistic because the covered area is very large.. |
| Huawei | Due to limited timeline, this factor may not have a good judgement from real deployment. However, we don’t have another meeting to resubmit the results. |
| ZTE | This active factor is based on ITU-R study and it’s too late to revisit close to the completion of core part. |
| CMCC | Again, if we change the activity factor we should emphasize this into the spec. |
| HughesEchoStar | If we do not have time to discuss activity factor, then prefer option 2-1. |
| Inmarsat | Agree with Thales and Hughes. |
| Qualcomm | No time to re-open the discussion. We can consider this impact when deciding the NTN BS ACS. |

Issue 1-5: Scaling factor

| Company | Comments |
| --- | --- |
| Ericsson | We could continue discussing the scaling factor but what’s the goal with this? See our comment on issue 1-4. |
| THALES | See above. The problem is with respect to current ACIR values (which we think are too high), especially for Case 6. At the same time, it also seems that the simulation model could be improved. |
| ZTE | Similar comment as Issue 1-4. |
| HughesEchoStar | See above |
| Inmarsat | Agree with Thales and Hughes. |
| Qualcomm | No time to re-open the discussion. We can consider this impact when deciding the NTN BS ACS. |

Issue 1-6: Case 2 and 6 simplification method

| Company | Comments |
| --- | --- |
| Samsung | We support the recommended WF. |
| Ericsson | Agree |
| THALES | We are fine, but we need to discuss which active factor we should consider for more accurate simulations better representing the real deployments, especially for Case 6. |
| Huawei | OK with recommended WF. |
| ZTE | Okay with the proposed WF. |
| HughesEchoStar | Agree with Thales |
| MediaTek | Ok with proposed WF. We also felt the activity factor could be discussed further as an input to this meeting, but no strong view on the outcome. |
| Inmarsat | Agree with Thales and Hughes, but no strong views. |
| Qualcomm | Agree |

Issue 1-7: Rural deployment in adjacent beam while main beam serving urban deployment

| Company | Comments |
| --- | --- |
| Ericsson | We would like to have further explanation from the company who proposed this to better understand what would be the goal with this new scenario. |
| THALES | Please see above. It is somehow not realistic to consider an urban scenario of e.g. 300 km diameter with the urban density. If in the adjacent beams we consider urban deployments, then the interference from adjacent beams is too high. On the other hand, we could simply decide to set the requirements only for Case 6 rural deployment, or provide requirements by TN type (FDD or TDD). |
| Omnispace | Agree with Thales views. |
| HughesEchoStar | Agree with Thales |
| Inmarsat | No strong views at this point. |
| Qualcomm | Agree with Thales’s view. But we’d like to point out that the current assumption is consider the case that interference only from one beam (adjacent beams are not taken into account ) in case 6. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1: Isolation distance consideration | With the consideration that: - RAN4 aims to agree on NTN SAN and UE ACLR/ACS by this meeting which means we have very limited time; - Isolation condition has already been adopted in Case 1; - Whether isolation would be adopted in Case 4 does not affect NTN UE ACLR which was agreed to reuse TN UE requirements; The moderator propose with following Tentative agreements: - There’s no need to conduct more simulations. - Do not consider isolation distance in Case 4. Candidate options: - Option 1: Capture the consideration of isolation distance in relevant TS(s). Recommendations for 2nd round: Further discuss the candidate option. |
| Issue 1-2: Case 5 propagation model between NTN UE and TN UE | Tentative agreements: - Do not use Case 5 to derive NTN UE ACLR - To better improve the contents in TR 38.863: - Results contributor of Case 5 are encouraged to indicate their propagation models; - Contributor are encouraged to update their results of Case 5, but not mandate. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-3: Case 6 Urban | Tentative agreements: - Do not re-visit current assumptions due to very limited time. Candidate options: N/A Recommendations for 2nd round: Further discuss in a new Issue 1-8 how to handle Case 6 results and SAN ACS taking into account factors in Issue 1-3, 1-4, 1-5 and 1-7, etc. |
| Issue 1-4: Active/activity factor | Tentative agreements: - Do not re-visit current assumptions due to very limited time. Candidate options: N/A Recommendations for 2nd round: Further discuss in a new Issue 1-8 how to handle Case 6 results and SAN ACS taking into account factors in Issue 1-3, 1-4, 1-5 and 1-7, etc. |
| Issue 1-5: Scaling factor | Tentative agreements: - Do not re-visit current assumptions due to very limited time. Candidate options: N/A Recommendations for 2nd round: Further discuss in a new Issue 1-8 how to handle Case 6 results and SAN ACS taking into account factors in Issue 1-3, 1-4, 1-5 and 1-7, etc. |
| Issue 1-6: Case 2 and 6 simplification method | Tentative agreements: Agree on Option 1 Step 1: to drop NTN UE per beamprint randomly; Step 2: to drop N clusters consisting of 57 sectors per beamprint randomly Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor: where: - active_TN=active_factor*round(the area per beam/the area of 57 sectors) - active_factor = 20% (or lower, particularly for urban scenarios) Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN: |
| Issue 1-7: Rural deployment in adjacent beam while main beam serving urban deployment | Tentative agreements: - Do not re-visit current assumptions due to very limited time. Candidate options: N/A Recommendations for 2nd round: Further discuss in a new Issue 1-8 how to handle Case 6 results and SAN ACS taking into account factors in Issue 1-3, 1-4, 1-5 and 1-7, etc. |

Discussion on 2nd round

Open issues and view collection

Issue 1-1: Isolation distance consideration

- Proposals

  - Option 1: Capture the consideration of isolation distance in relevant TS(s)

- Recommended WF

  - TBA.

| Company | Comments |
| --- | --- |
| Qualcomm | Suggest capturing the information in TR rather than TSs. Note that isolation is only considered for Case 1 and it is the co-ex scenario of TN is interfering DL to NTN DL. |
| Ericsson | Option 1. We should capture all assumptions in the TR but there are some key assumptions we should also mention in the TS. If not, after a while, reader of the TS will consider all the requirements specified in the TS will be applicable for any situation/deployment. |
| THALES | No, is relevant only for the TR. Agree with Qualcomm. We should not forget also about the network selection algorithms (TN-NTN) based on QoS, which seem to be (in some cases) neglected for the current coexistence study. The isolation distance does not make too much sense with respect to this assumption. Moreover, for the case 1 (NTN UE (victim) ACS analysis from aggressor TN gNB), the NTN UE shares the same characteristics as a TN UE, so is like a normal UE. And currently, as you know, there is no separation distance between different TN operators. |
| Hughes/EchoStar | Agree with Thales view |
| Inmarsat | We concur with Qualcomm, Thales, and Hughes for the reasons stated above. |

Issue 1-8: How to handle Case 6 results

- Proposals

  - Option 1: Discuss how to handle Case 6 results in a sub-email thread. Use TN BS ACS as the baseline if no agreements can be made.

- Recommended WF

  - TBA.

| Company | Comments |
| --- | --- |
| Qualcomm | Agree with option 3, i.e., 46dB for ACS. |
| Ericsson | Fine with option 1. We can’t ignore case 6 and have to find a way to address this case anyway. Still we understand satellite companies’ concern and are open for discussion to find an acceptable solution. |
| Huawei | Alternatively we can summarize this value by separating GEO and LEO. |
| THALES | Please see discussion from separate thread: nAAS results are worse than AAS there is too much difference between Qualcomm result (ACIR 50dBs) and Xiaomi (24dBs). How can we have a conclusion based on this? We already decided not to consider differences higher than 10dB with respect to the average of other companies. we need to consider values which are not too different from other companies Do not agree with 46 dB ACS. We should consider at this stage maximum 37-38dBs (see discussion on the dedicated thread). Template:. Case 6: Aggressor TN DL* to Victim NTN UL*. *NTN type (LEO1200) *TN BS type (non-AAS) *Deployment scenario (Rural). Required ACIR [dB]; Company; Interpolate ACIR; 0; 2; 4; 6; 8; 10; 12; 14; 16; 18; 20; 22; 24; 26; 28; 30; 32; 34; 36; 38; 40. Throughput Loss; Average; ZTE. THALES; 88,99; 84,04; 77,53; 69,13; 59,27; 48,66; 38,05; 28,43; 20,4; 14,2; 9,59; 6,27; 4,07; 2,64; 1,69. MTK; 60,75; 51,92; 42,79; 35,86; 29,54; 23,71; 18,40; 13,73; 9,85; 6,83; 4,61; 3,04; 1,98; 1,27; 0,81; 0,52; 0,33; 0,21; 0,13; 0,08; 0,05. 5%-tile; ZTE. THALES; 36.70; 90,57; 84,91; 79,25; 71,7; 62,26; 50,94; 41,51; 30,19; 22,64; 15,09; 11,32; 7,55; 5,66; 3,77; 1,89. MTK; 24.67(*); 100,00; 100,00; 85,11; 78,34; 69,64; 59,31; 48,09; 37,08; 27,24; 19,20; 13,08; 8,69; 5,68; 3,66; 2,35; 1,49; 0,95; 0,60; 0,38; 0,24; 0,15. (*) This value is not considered for average ACIR. Average ACIR for above results is: 36.70 dB. Above results suggested SAN ACS is: > 37.40 dB Template:. Case 6: Aggressor TN DL* to Victim NTN UL*. *NTN type (LEO600) *TN BS type (non-AAS) *Deployment scenario (Rural). Required ACIR [dB]; Company; Interpolate ACIR; 0; 2; 4; 6; 8; 10; 12; 14; 16; 18; 20; 22; 24; 26; 28; 30; 32; 34; 36; 38; 40. Throughput Loss; Average; ZTE. THALES; 88,92; 84,11; 77,78; 69,97; 60,88; 51,17; 41,28; 31,94; 23,87; 17,01; 11,87; 7,91; 5,26; 3,4; 2,22. MTK; 60,76; 52,08; 42,80; 35,88; 29,57; 23,75; 18,43; 13,76; 9,88; 6,86; 4,62; 3,05; 1,98; 1,28; 0,82; 0,52; 0,33; 0,21; 0,13; 0,08; 0,05. 5%-tile; ZTE. THALES; 36.70; 90,7; 86,05; 80,23; 72,09; 63,95; 53,49; 43,02; 33,72; 25,58; 17,44; 12,79; 8,14; 5,81; 3,49; 2,33. MTK; 24.76(*); 100,00; 100,00; 85,40; 78,72; 70,11; 59,83; 48,62; 37,56; 27,65; 19,50; 13,30; 8,85; 5,78; 3,73; 2,39; 1,52; 0,97; 0,61; 0,39; 0,24; 0,15. (*) This value is not considered for average ACIR. Average ACIR for above results is: 36.70 dB. Above results suggested SAN ACS is: > 37.40 dB |
| Hughes/EchoStar | Too much spread in ACIR, have to find a way to streamline. Option 1 seems fair but NTN BS ACS can be lower than TN |
| Inmarsat | We find Thales’s view persuasive, and believe that considering the required ACIR in light of regulatory standards for out-of-band emissions would be wise. A specification more stringent than that derived from what one can expect from regulation would seem dubious. |
| Omnispace | Agree with Thales view. |
| Moderator | A separate discussion was conducted in a dedicated sub-email thread under [307], which has been captured here. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1: Isolation distance consideration | Agreement: Capture the consideration of isolation distance in TR 38.863. Further discuss whether it should be captured in relevant TS(s). |
| Issue 1-8 How to handle Case 6 results | Way Forward: Case 6 should be considered, however further discussion is still needed. |

Topic #2: Co-existence results handling

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4- 2201072 | Samsung | Proposal 1: It is proposed to include the above co-ex results from our company to the TR 38.863, and also to consider the above results from our company when discussing the required ACIR and ACLR/ACS. |
| R4-2201126 | Samsung | Proposal 1: It is proposed to consider following methodology to determine the required ACIR for each case: Step 1: Discuss and agree on the worst case option for each case (Case 1, 2, 3…,6); Step 2: Discuss and determine the required ACIR from all calibrated results for each case; Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each case. Proposal 2: It is proposed to consider our derived interpolate required ACIR for Case 3 and Case 4. For Case 3, our results suggested 22.72 dB as required ACIR; for Case 4, our results suggested 28.73 dB as required ACIR. |
| R4-2201127 | Samsung | Proposal 2: It is proposed to consider the following principles when discussing the required ACIR for the worst case option: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference of >10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference of >10 dB with most others, this result can be not considered in the discussion. Proposal 3: It is proposed to focus discussion and results alignment on the following option for each Case. For case 1, focus on TN (AAS) interfering NTN (GEO) in Urban case; For case 2, focus on TN (AAS) interfering NTN (LEO600) in Urban case; For case 3, focus on NTN (LEO600) interfering TN (AAS) in Rural case; For case 4, focus on NTN (GEO) interfering TN (AAS) in Urban case. Case #; NTN station; Worst option (options with >1 contributions); ACIR ranges; Target. 1; GEO; *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Urban - Option 1 no isolation); 40~48+; NTN UE ACS. *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Urban - Option 2 1.5 km isolation); 24~30. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Urban - Option 1 no isolation); 30~38. *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Urban - Option 2 1.5 km isolation); 12~20. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Urban - Option 1 no isolation); 30~40+. *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Urban - Option 2 1.5km isolation); 10~18. 2; GEO; *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Urban); 24~30; NTN BS ACS. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Urban); 18~30. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Urban); 22~30. 3; GEO; *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Rural); 8~16; NTN BS ACLR. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Rural); 18~26. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Rural); 18~26. 4; GEO; *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Urban); 24~34; NTN UE ACLR. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Urban); 22~34. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Urban); 22~34. 5; GEO; *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Rural); 24~26; NTN UE ACLR. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Rural); 24~26. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Rural); 24~26. 6; GEO; *NTN type (GEO @ EL = 45 deg) *TN BS type (AAS) *Deployment scenario (Urban); 48~50 22~24; NTN BS ACS. LEO 1200; *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Rural); TBD. LEO 600; *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Rural); TBD. Proposal 4: It is proposed to agree to discuss Case 4 results to determine the NTN UE ACLR other than Case 5. |
| R4-2201255 | Huawei, HiSilicon | Observation 1: For case 1, the ACIR can be 32dB. Observation 2: For case 2, the ACIR can be 28~30dB. Observation 3: For case 3, the worst scenario is Aggressor LEO600 to Victim AAS Rural and ACIR can be 20dB. Observation 4: For case 4, the ACIR can be less than 30dB. Observation 5: For case 5, the ACIR can be less than 28dB. Observation 6: For case 6, the ACIR can be 36~38dB. |
| R4-2201256 | Huawei, HiSilicon | Observation 1: the simulation results for case 3(Aggressor NTN DL and victim TN DL) can be used to derive the ACLR requirement for satellite access. The worst scenarios are Aggressor LEO to Victim AAS Rural. Observation 2: case 2 is not suitable to specify the accurate ACS requirement for satellite access node. |
| R4-2201262 | MediaTek Inc. | Observation 3: For Case 2 and Case 6, with the simplified simulation process using the activity factor agreed in [1], we identified that there is quite strong interference impact from the TN to the NTN UL. We believe some further discussion is needed between companies for those cases. |
| R4-2201317 | Ericsson | Proposal2: RAN4 shall define some generic rules when selecting coexistence simulation results to derive ACLR and ACS values, e.g. : - If more than 4 companies have provided results and only one company has very different results (e.g. more than [6]dB spread), this company results should not be taken into account for the ACIR evaluation. - For each case, only the most stringent scenario(s) should be considered to evaluate the final ACIR value for that case. - When available and consistant, for each scenario, consider the most stringent results (highest ACIR values) from AAS BS and non-AAS BS sub-scenario. - When available and consistant, for each scenario, consider the most stringent results (highest ACIR values) from sub-scenarios at 90 and 45 degrees elevation angle. |
| R4-2201839 | THALES, Magister Solutions Ltd | Proposal 1: For NTN coexistence simulations in Case 1, an ACIR of 15 dBs seems sufficient. Proposal 2: For NTN coexistence simulations in Case 2, an ACIR of 25-30 dBs seems sufficient. Proposal 3: For NTN coexistence simulations in Case 3, an ACIR of 20 dBs seems sufficient. Proposal 4: For NTN coexistence simulations in Case 4, an ACIR of 10-15 dBs seems sufficient. Proposal 5: For NTN coexistence simulations in Case 5, an ACIR of 10-15 dBs seems sufficient. Proposal 6: For NTN coexistence simulations in Case 6, an ACIR of 35-40 dBs seems sufficient. |

Open issues summary

Sub-topic 2-1

Common issues that needs to be fixed first.

Issue 2-1: Working methods to proceed results analysis

- Proposals

  - Option 1:

    - Step 1: Discuss and agree on the worst case option for each case (Case 1, 2, 3…,6);

    - Step 2: Discuss and determine the required ACIR from all calibrated results for each case;

    - Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each case.

  - Option 2:

    - For each case, only the most stringent scenario(s) should be considered to evaluate the final ACIR value for that case.

    - When available and consistant, for each scenario, consider the most stringent results (highest ACIR values) from AAS BS and non-AAS BS sub-scenario.

    - When available and consistant, for each scenario, consider the most stringent results (highest ACIR values) from sub-scenarios at 90 and 45 degrees elevation angle.

- Recommended WF

  - Merge and agree on these options, since they are proposing similar working methods.

Issue 2-2: Working methods to handle large variation in results

- Proposals

  - Option 1: It is proposed to consider the following principles when discussing the required ACIR for the worst case option:

    - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference of >[10] dB with most others, this result can be not considered in the discussion.

    - If the required ACIR results, from one contributor, has a difference of >[10] dB with most others, this result can be not considered in the discussion.

  - Option 2: RAN4 shall define some generic rules when selecting coexistence simulation results

    - If more than 4 companies have provided results and only one company has very different results (e.g. more than [6]dB spread), this company results should not be taken into account for the ACIR evaluation..

- Recommended WF

  - Discuss and merge both options to agree on generic rule to handling results.

Sub-topic 2-2

Worst case and ACIR ranges.

Issue 2-3: Worst scenario for each case

- Proposals

  - Option 1:

    - For case 1, focus on TN (AAS) interfering NTN (GEO) in Urban case;

    - For case 2, focus on TN (AAS) interfering NTN (LEO600) in Urban case;

    - For case 3, focus on NTN (LEO600) interfering TN (AAS) in Rural case;

    - For case 4, focus on NTN (GEO) interfering TN (AAS) in Urban case.

  - Option 2:

    - For case 5, the results in this case is smaller than Case 4. Propose to use Case 4 results to determine NTN UE ACLR.

  - Option 3:

    - For case 6, consider lower elevation angle since this represents the worst-case scenario for NTN BS ACS

  - Option 4: TBA

[Moderator’s Note: The above proposals and submitted observations are summarized into the table below, if you propose new Option above, please help yourself to insert a new row to the table below.]

| Case # / Worst scenario | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Option 1&2 | TN (AAS) interfering NTN (GEO) in Urban case | TN (AAS) interfering NTN (LEO600) in Urban case | NTN (LEO600) interfering TN (AAS) in Rural case. Use this to derive NTN BS ACLR. | NTN (GEO) interfering TN (AAS) in Urban case. Use this to derive NTN UE ACLR. |  |  |
| Option 3 |  |  |  |  |  | Consider lower elevation angle. Use this to derive NTN BS ACS |
| Observations (Huawei/HiSilicon) |  | NOT suitable to specify NTN BS ACS. | LEO interfering AAS Rural. Use this to derive NTN BS ACLR. |  |  |  |

- Recommended WF

  - Encourage companies to first express views to lock down the worst scenario for each case, and then to determine which case should be selected for NTN BS/UE ACLR/ACS.

Issue 2-4: ACIR range for each case

- Proposals

  - Option 1:

    - For Case 3, our results suggested 22.72 dB as required ACIR.

    - For Case 4, our results suggested 28.73 dB as required ACIR.

  - Option 2:

    - For Case 1, collected results shows 40~48 dB (no isolation) or 24~30 dB (1.5km isolation) ACIR is required.

    - For Case 2, collected results shows 22~30 dB ACIR is required.

    - For Case 3, collected results shows 18~26 dB ACIR is required.

    - For Case 4, collected results shows 24~34 dB ACIR is required.

    - For Case 5, collected results shows 24~26 dB ACIR is required.

  - Option 3:

    - For Case 1, an ACIR of 15 dBs seems sufficient.

    - For Case 2, an ACIR of 25-30 dBs seems sufficient.

    - For Case 3, an ACIR of 20 dBs seems sufficient.

    - For Case 4, an ACIR of 10-15 dBs seems sufficient.

    - For Case 5, an ACIR of 10-15 dBs seems sufficient.

    - For Case 6, an ACIR of 35-40 dBs seems sufficient.

  - Option 4: TBA

[Moderator’s Note: The above proposals and submitted observations are summarized into the table below, if you propose new Option above, please help yourself to insert a new row to the table below.]

| Case # / ACIR | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Option 1 |  |  | 22.72 dB | 28.73 dB |  |  |
| Option 2 | 40~48 dB (no isolation) or 24~30 dB (1.5km isolation) | 22~30 dB | 18~26 dB | 24~34 dB | 24~26 dB |  |
| Option 3 | 15 dB | 25~30 dB | 20 dB | 10~15 dB | 10~15 dB | 35~40 dB |
| Observation (Huawei/HiSilicon) | 32 dB | 28~30 dB | 20 dB | 30 dB | 28 dB | 36~38 dB |

- Recommended WF

  - Express views to converge the ACIR ranges in 1st round. After Issue 2-3 is agreed, discuss and agree on the final ACIR methods and values.

Companies views’ collection for 1st round

Open issues

Issue 2-1: Working methods to proceed results analysis

| Company | Comments |
| --- | --- |
| Samsung | We support the recommended WF. We think these principles are common approaches in RAN4, and if these are agreeable to all, we can offline draft a merged version with the other contributor. |
| Ericsson | We are fine with Samsung proposal. It’s indeed based on common practice in RAN4, rules should be clarified in the TR to avoid questions in the future. |
| THALES | Should we separate (satellite) requirements depending on the TN AAS type (with AAS or with non-AAS)? In practice, the deployments can be mixed. Or should we consider an average/mean? |
| Huawei | We are OK with option 1. To Thales, we can’t separate requirements depending on the TN AAS type (with AAS or with non-AAS) as you said the deployments can be mixed in practice. Thus, we should consider the worst case to cover all scenario. |
| ZTE | We are also fine with samsung’r proposal, in addition, we also share similar concerns as Huawei on separate the requirement for AAS and non-AAS. |
| CMCC | The recommended WF is OK and we should focus on the most stringent results from AAS and non-AAS BS. |
| HughesEchoStar | Agree with WF (on the process) but have to be practical with assumptions especially in Case 6 |

Issue 2-2: Working methods to deal with large variation in results

| Company | Comments |
| --- | --- |
| Samsung | We support the recommended WF, and we believe we can merge option 2 to option 1 by putting 10 dB in square bracket as “[10]” dB. |
| Ericsson | We are also fine with Samsung proposal. |
| THALES | Not a clear opinion, because there are still differences which can be seen from the calibration phase (prior to the simulation phase). Moreover, each company may use/apply different TN-NTN selection mechanisms, or different resource allocation algorithms for the TN, and this differentiate even more the results. |
| Huawei | The specific threshold for excluding results can be discussed case by case. Anyway, it should be based on the consensus. |
| ZTE | It could be further discussed case by case. |
| CMCC | We support the recommended WF. |

Issue 2-3: Worst scenario for each case

| Company | Comments |
| --- | --- |
| Samsung | The current observations and proposals seems agreeable, and can be merged. We’d like to seek more views from all contributors, and then to check if we can have a common view. (FYI. The latest collected co-ex results can be found in the NTN results update folder.) |
| Ericsson | Option 1 and 2: In some cases, ACIR with non AAS TN might be higher than with AAS TS so we would be fine with the identified scenarios being the worst case ones but would prefer to remove “(AAS)” from TN or add “(AAS/non AAS)“ to TN. Difficult to make any conclusion for cases 5 and 6 with the high spread of results. Agree to consider a lower elevation angle with case 6. |
| THALES | We should also consider non-AAS, and maybe differentiate on the type of TN i.e. if FDD or TDD. |
| Samsung | To Ericsson, we are not against to take TN (AAS/non-AAS), the difficulties is that the non-AAS results is fewer than AAS results. And the other reason is that the AAS results are generally indeed a little bit worse (maybe not 100% all of them) than non-AAS. |
| Huawei | At least for case 3, companies have a common understanding. As we discussed in our paper, the ACS for satellite access node can be 32~43dB for 28~30dB ACIR range of case 2. Maybe we need a very accurate ACIR value to derive the ACS for SAN. Case 6 can be considered as reference if we can’t conclude an accurate ACIR value for case 2. |
| ZTE | More time to check its details. |
| CMCC | Option 1 and option 2 are both OK for us. |
| Hughes/EchoStar | Case 6 assumptions and results are still a major concern |
| Qualcomm | Agree with option 1&2&3. |

Issue 2-4: ACIR range for each case

| Company | Comments |
| --- | --- |
| Samsung | We believe most proposed and observed ACIR ranges are quite similar. And we’d like to seek more views from all contributors, and also to wait the agreements or WF from Issue 2.2. |
| Ericsson | We might not need to spend time on finding an agreement on ACIR value for each case, the most important is to agree on the ACRL/ACS values. It seems indeed all companies have the same proposal for UE ACLR/ACS, so we would propose to not try to further agree on the ACIR value for cases 1 (if we all agree on an isolation distance) and 4. In the TR, we could then capture a range of values from the companies’ results instead of one value for those cases at least. |
| THALES | We should probably consider the results with isolation. The results without isolation are high because of the TN-NTN selection mechanisms which (in our opinion) are not applied at all. In any case, all above options seem reasonable, except for the case 1 with no isolation. |
| Huawei | If we can’t conclude one value based on the coexistence, we can agree on a range for further discussion. |
| ZTE | To put the range in TR should be fine, in addition, as mentioned by Ericsson. Maybe we could agree on UE ACLR/ACS requirement firstly. |
| Hughes/EchoStar | Concur with Thales |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 2-1: Working methods to proceed results analysis | GTW Agreement (Jan. 18): combine option 1 and option 2 as generic approach, shown as below Step 1: Discuss and agree on the most stringent scenario(s) for each case (Case 1, 2, 3…,6); Step 2: Discuss and determine the required ACIR from results of the most stringent scenario(s) for each case; Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each case. |
| Issue 2-2: Working methods to handle large variation in results | GTW Agreement (Jan. 18): Option 1 adopted when discussing the required ACIR for the worst case option: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. |
| Issue 2-3: Worst scenario for each case | GTW Agreement (Jan. 18): Case 1; Case 2; Case 3; Case 4. TN interfering NTN (GEO) in Urban case; TN interfering NTN (LEO600) in Urban case; NTN (LEO600) interfering TN in Rural case.; NTN (GEO) interfering TN in Urban case. Use case 3 to decide NTN SAN (satellite access node) ACLR. RAN4 aims to agree NTN SAN ACLR/ACS by this meeting. |
| Issue 2-4: ACIR range for each case | Tentative agreements: Capture following ranges in TR 38.863. Case # / ACIR; Case 1; Case 4; Case 5. Option 2; 24~30 dB; 24~34 dB; 24~26 dB. Candidate options: Option 1: For NTN SAN Case # / ACIR; Case 2; Case 3; Case 6. Range 2; 22~30 dB; 18~26 dB. Range 3; 25~30 dB; 20 dB; 35~40 dB. Recommendations for 2nd round: Further discuss Option 1. |

Discussion on 2nd round (if applicable)

Open issues and view collection

Issue 2-4: ACIR range for each case

- Proposals:

| Case # / ACIR | Case 2 | Case 3 | Case 6 |
| --- | --- | --- | --- |
| Option 2 | 22~30 dB | 18~26 dB |  |
| Option 3 | 25~30 dB | 20 dB | 35~40 dB |
| Option 4 | 25~30 dB | 18~26 dB | 37~38 dB |

- Recommended WF

  - TBA.

| Company | Support Option 2 (Y/N) | Support Option 3 (Y/N) | Comments |
| --- | --- | --- | --- |
| Qualcomm | N | N | The range of Case 6 should consider the low elevation angle with urban. The upper value should be 46dB |
| Ericsson | N | N | The range for case 2 looks fine For case 3, it should be 20~26dB. For case 6, it goes up to ~49dB. |
| Huawei | Y | N |  |
| Hughes/EchoStar | Y | N | only one company shows ACIR range 40 dB |
| THALES | Y | Y (also fine with case 3 with 18~26 dB) | The case 6, based on current assumptions, does not go up to 49dBs, there is one company providing such result and there is too much difference from other values. Please play attention on how the average ACIR value is computed, and this is valid for all cases. - Consider only results which do not have much variance with respect to other companies’ results. - We cannot have a conclusion if only one company submitted results. We need at least 2 companies providing results for a Case. - Also, it is not clear for us how the average value between companies is computed. The correct methodology should be (see e.g. TR 36.942): - the average is done for throughput loss (based on different throughput loss between the companies), - a new curve representing the averaged throughput loss (between companies) is obtained, - then, a (new) ACIR value is obtained at 5%. We should not take the worst value (of a company) into account if the value at 5% throughput loss is 10dB higher (or lower) that the average of the other companies. For this case, the throughput loss shall not be used to compute the average throughput loss. Proposing Option 4 based on the following results: Case 2: Case 3: Case 6: |
| Omnnispace | Y | Y | Agree with Thales |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 2-4: ACIR range for each case | Way forward: Further discuss ACIR ranges based on following options. Case # / ACIR; Case 2; Case 3; Case 6. Option 2; 22~30 dB; 18~26 dB. Option 3; 25~30 dB; 20 dB; 35~40 dB. Option 4 (Qualcomm); 22~30; 20~26dB; 46 dB. Option 5 (Ericsson); 22~30 dB; 20~26dB; ?~49 dB. Option 6 (Thales); 25~30 dB; 18~26 dB; 37~38 dB. |

Topic #3: ACLR and ACS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200166 | CATT | Proposal 1: it is proposed to define satellite access node ACLR requirement as 38dB. Proposal 2: it is proposed to define satellite access node ACS requirement as 40dB. |
| R4-2200781 | Qualcomm Incorporated | Observation 2: Based on the simulation results from Case 1, the current BS ACLR requirement will be sufficient to meet the coexistence criteria. The NTN UE ACS requirement can be the same as current TN UE requirement. Observation 3: In case 2, TN UL and NTN UL can coexist. However, in this scenario, for the GEO and LEO 1200 cases, we observed that cell edge UEs are out of coverage since the UL SINR is less than -10 dB at 5% CDF. Observation 4: Based on the simulation results from Case 2, the current TN UE ACLR requirement will allow to meet the coexistence criteria. Observation 5: The interference from NTN to TN in DL is negligible since TN has much stronger link compared to the interference from NTN. Therefore, the NTN and TN in case 3 can coexist. Observation 6: Based on the simulation results from Case 3, the current TN UE ACS requirement will allow to meet the coexistence criteria. The NTN BS ACLR requirement should be around 26dB. Observation 7: In case 4, all the NTN UEs use full power so we can see similar performance from the NTN UEs in UL direction. Observation 8: Based on the simulation results from Case 4 and given that NTN UE should be placed with a minimum isolation distance in urban scenario, the current BS ACS requirement will allow to meet the coexistence criteria. The NTN UE ACLR requirement can be the same as current TN UE requirement. Observation 9: Based on the simulation results from Case 5, the current TN UE ACS requirement will allow to meet the coexistence criteria. Furthermore, the NTN UE ACLR requirement can be the same as current TN UE requirement. Observation 10: In case 6, NTN cell edge UEs are out of coverage for the urban case since the UL SINR is less than -10 dB at 5% CDF. Observation 11: Based on the simulation results from case 6, lower elevation angles for GEO – e.g., 45 degrees – will require a tighter ACIR of about 6 to 7dB. Based on that, the NTN BS ACS requirement should be around 46 dB same as TN BS ACS requirement. Proposal 3: To not change current requirements for TN BS and TN UE. Proposal 4: To reuse same requirement of TN UE with NTN UE. Proposal 6: Suggest using 26 dB as NTN BS ACLR. Proposal 7: Suggest using 46 dB as NTN BS ACS when low EL. i.e., 45 deg, is considered for case 6. Proposal 8: In summary, the following ACLR and ACS are proposed for NTN BS and UE: Table 3 Summary of NTN BS and UE ACLR/ACS BS ACLR (dB); 26. BS ACS (dB); 46. UE ACLR (dB); 30. UE ACS (dB); 33 (5/10MHz channel BW). |
| R4-2201126 | Samsung | Proposal 3: It is proposed to consider our derived ACLR and ACS for Case 3 and Case 4. For Case 3, our results suggested 23.15 dB as NTN BS ACLR; for Case 4, our results suggested 28.81 dB as ACLR1 and 41.81 dB as ACLR2 for NTN UE ACLR. |
| R4-2201256 | Huawei, HiSilicon | Proposal 1: To specify 20dB ACLR requirement for satellite access node. Proposal 2: To specify 36dB ACS requirement for satellite access node. Observation 3: the ACLR requirement for UE for satellite access can be less than 30dB. Observation 4: the ACS requirement for UE for satellite access can be 33dB. |
| R4-2201262 | MediaTek Inc. | Observation 1: For Case 1, we logically observe that the 1.5km isolation difference reduces the level of interference from TN DL to NTN DL. A UE ACS seems to be shown to be acceptable in such scenario. Observation 2: For Case 2, the level of interference observed from NTN UL to TN UL suggests that a UE ACLR of 30dB would be acceptable. However, it should be noted also that the NTN UE location at the geographical edge of the TN cluster may be a bit extreme considering that Case 1 shows that at such a location the UE is likely to be heavily blocked by the TN DL, suggesting that the UE may instead more likely be connected to the TN in such a location. |
| R4-2201317 | Ericsson | Proposal1: Based our simulation results, consider following limits for the NTN satellite access node and NTN UE ACLR and ACS: NTN Satellite access node; NTN UE. ACLR; 25; 21. ACS; 32; No isolation; With isolation. NA or at least >60; 20. |
| R4-2201467 | ZTE Corporation | Proposal 1: for NTN UE, reuse FR1 TN UE ACLR and ACS requirements; Proposal 2: for NTN BS, reuse FR1 TN BS ACLR and ACS requirements; |
| R4-2201839 | THALES, Magister Solutions Ltd | Observation 1: Scenario 1. In Urban scenario with TN DL aggressor and NTN DL victim, the average SINR of NTN UEs degrades up to 1.9 dB depending on NTN UE ACS which also decreases average throughput in the order of ~10% with low ACS. NTN cell edge may suffer additional losses with LEO satellite compared to average values. Observation 2: Scenario 2. Link budget from omnidirectional UE to the satellite is very modest, and with TN power control and limitations to the number of scheduled users the TN UEs do not pose a threat to the satellite UL. However, a great number of non-power controlled UEs under a large satellite beam could theoretically cause noticeable interference. Observation 3: Scenario 3. Path loss from satellite to UEs with omnidirectional antennas is high, so leaked interference power to adjacent channel becomes even less. In Rural scenario the TN UE link budget is smaller, so slightly increased interference can be seen in the SINR with low satellite ACLR. Observation 4: Scenario 4. The UEs that connect to the satellite have a very large path loss towards TN gNBs. When ACIR is reduced from the NTN UEs’ transmitted power, the NTN UL ACI cannot be heard at the TN gNBs. Observation 5: Scenario 5. Under used simulation assumptions there is a low probability that UEs are close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE. Observation 6: Scenario 6. The antenna directivity for TN gNBs is not low enough, and with high transmission power (and depending on the density of gNBs/number of gNBs in a beam) the TN gNBs will interfere with adjacent channel NTN UL transmissions with a low link budget. This situation is very particular, for a specific scenario, for a specific KPI, and the resulted requirement can be considered as for the worst case scenario. On the other hand, this could be a requirement integrated for an identified specific class of satellite access node, if further needed. Proposal 7. RAN4 shall consider for NTN UE same ACLR and ACS parameter values as for TN UE. Proposal 8. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR. Proposal 9. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS. Proposal 10. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR when TN FDD. Proposal 11. RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACS when TN FDD. Proposal 12. RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS when TN TDD. Proposal 13. RAN4 may consider to separate NTN requirements based on the TN type (FDD or TDD). Proposal 14. RAN4 could consider taking the average value of Satellite Access Node ACS between Case 2 and Case 6 (rural) scenarios (see R4-2201842). Proposal 15. RAN4 could consider taking the average value of NTN UE ACLR between Case 4 and Case 5 scenarios. |

Open issues summary

Sub-topic 3-1

Common issues that needs to be fixed first.

Issue 3-1: Separate NTN requirements based on TN type

- Proposals

  - Option 1: Separate NTN requirements based on the TN type (FDD or TDD)

    - Option 1-1: RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACLR when TN FDD.

    - Option 1-2: RAN4 shall consider for NTN Satellite node a maximum value of 15-20 dBs for ACS when TN FDD.

    - Option 1-3: RAN4 shall consider for NTN Satellite node a maximum value of 35-40 dBs for ACS when TN TDD.

  - Option 2: TBA

- Recommended WF

  - TBA.

Issue 3-2: Consideration of case 2 & 6 (for NTN SAN ACS) and case 4 & 5 (for NTN UE ACLR)

- Proposals

  - Option 1: RAN4 could consider taking the average value of Satellite Access Node ACS between Case 2 and Case 6 (rural) scenarios

  - Option 2: RAN4 could consider taking the average value of NTN UE ACLR between Case 4 and Case 5 scenarios.

  - Option 3: TBA

- Recommended WF

  - TBA.

Sub-topic 3-2

ACLR and ACS for NTN BS and UE.

Issue 3-3: NTN SAN ACLR

- Proposals

  - Option 1: to define satellite access node ACLR requirement as 38dB

  - Option 2: Suggest using 26 dB as NTN BS ACLR.

  - Option 3: our results suggested 23.15 dB as NTN BS ACLR.

  - Option 4: To specify 20dB ACLR requirement for satellite access node

  - Option 5: 25 dB

  - Option 6: Re-use FR1 TN BS ACLR. [Moderator note: 45 dB]

  - Option 7: maximum value of 15~20 dBs for ACLR.

- Recommended WF

  - Discuss the methods to converge the proposals in 1st round. Averaging over all agreed options can be one final option.

Issue 3-4: NTN SAN ACS

- Proposals

  - Option 1: to define satellite access node ACS requirement as 40dB

  - Option 2: Suggest using 46 dB as NTN BS ACS when low EL. i.e., 45 deg, is considered for case 6.

  - Option 3: To specify 36dB ACS requirement for satellite access node.

  - Option 4: 32 dB

  - Option 5: Re-use FR1 TN BS ACS. [Moderator note: 46 dB]

  - Option 6: a maximum value of 35-40 dBs for ACS

- Recommended WF

  - Discuss the methods to converge the proposals in 1st round. Averaging over all agreed options can be one final option.

Issue 3-5: NTN UE ACLR

- Proposals

  - Option 1: same requirement of TN UE for NTN UE, 30 dB (ACLR1), 43dB (ACLR2)

- Recommended WF

  - Agree on Option 1.

Issue 3-6: NTN UE ACS

- Proposals

  - Option 1: same requirement of TN UE for NTN UE, 33 dB

- Recommended WF

  - Agree on Option 1.

Companies views’ collection for 1st round

Open issues

Issue 3-1: Separate NTN requirements based on TN type

| Company | Comments |
| --- | --- |
| Ericsson | We don’t think it makes sense to consider different ACS/ACLR values depending on the TN type, TDD or FDD, especially with LEO satellite. This would complicate deployment, considering the type of TN covered by the satellite. |
| THALES | What all companies are currently experiencing is that results with TN TDD and TN FDD give very different satellite requirements, especially for case 6. |
| Huawei | Generally, RAN4 can’t specify the RF requirements based on the victim deployed scenario. For SAN ACLR, we just consider case 3. |
| ZTE | As mentioned by Huawei, we need to consider the worst case or most stringent requirement that satellite node needed. |
| CMCC | We suggest to define only the most stringent ACLR and ACS for NTN and independent on TN types. |
| Hughes/EchoStar | Same observation as Thales. The question is are there really large deployment of TN TDD in urban scenario |
| Qualcomm | Usually, RAN4 only considers the worst case in the co-ex study. |

Issue 3-2: Consideration of case 2 & 6 (for NTN BS ACS) and case 4 & 5 (for NTN UE ACLR)

| Company | Comments |
| --- | --- |
| Samsung | We believe the issue is whether to consider worst or averaged number between these cases for its targeted RF requirements. This can also be discussed in Issue 2-1 as one of the principles. |
| Ericsson | We could average the values between companies for one case, but not in between cases, that won’t make sense. RAN4 shall take the worst case value from the different cases when applicable. None of the options here are agreeable. |
| THALES | The proposed average value is based on the same parameters: ACS or ACLR, and different TN deployments: FDD and TDD. Is an alternative for the requirement issue, since we believe the requirement is currently too high. Also, it is not clear for us how the average value between companies is computed. The correct methodology should be (see TR 36.942): - the average is done for throughput loss (based on different throughput loss between the companies), - a new curve representing the averaged throughput loss (between companies) is obtained, - then, a (new) ACIR value is obtained at 5% from the averaged throughput loss between the companies. Also, we should not take the worst value (of a company) into account if the value at 5% throughput loss is 10dB higher that the average of the other companies. For this case, the specific throughput loss shall not be used to compute the average throughput loss. |
| Huawei | We share the similar view with Ericsson. But we are open to discuss the compromise if companies have strong view on it. |
| ZTE | Agree with Ericsson and Huawei, not reasonable to average results between different cases, we could pick up most stringent requirement among the cases. |
| CMCC | Share the similar view with Ericsson, we should focus on the worst case. |
| Hughes/EchoStar | Same observation as Thales. |
| Qualcomm | Usually, RAN4 only considers the worst case in the co-ex study. |

Issue 3-3: NTN SAN ACLR

| Company | Comments |
| --- | --- |
| Samsung | Option 6 may need further explanation. The current collected results suggested a maximum required ACIR as around 26 dB, then we believe it’s not necessary to ask for an ACLR of 45 dB as proposed in Option 6. And also it’s too far from other options. The rest options, we can support taking average among the agreed options in the end. |
| Ericsson | Agree with Samsung, based on the simulation results only, ~26 dB ACLR should be agreeable by everyone but still there are 2 proposals for more stringent values (38 and 45), It would be good to understand the rationale for those 2 proposals. |
| THALES | It should be between 15-25 dBs. Other values do not make sense. We should not reuse TN BS ACLR for the satellite. |
| Huawei | Share similar view with Ericsson. It seems that option 1 and 6 are not based on the co-existence results. |
| ZTE | Based on the simulation results in case 3, we are also fine with ACIR as 26dBc, frankly speaking, not sure why satellite gNB’s requirement is less stringent than NTN UE ACLR 30dBc. If possible, we still like to know the capability of satellite node so far. |
| Qualcomm | Agree with option 2 which is also the proposal from our paper. |

Issue 3-4: NTN SAN ACS

| Company | Comments |
| --- | --- |
| Samsung | The key issue is whether to use Case 2 or Case 6 to derive NTN BS ACS, or use average. We can first discuss in Issue 2-3 and 2-4, and then come back to this issue. Our view is that taking average from all agreed proposals in the end can be acceptable. |
| Ericsson | It’s not acceptable for us to take the average between cases 2 and 6, this would mean we will accept higher degradation in case 6, higher than the 5% throughout threshold. |
| THALES | Option 1, 3, 4, 6. We can also average. We can reuse current (TN) ACS BS requirements but we think the ACS requirements are too high for satellite, and applicable only for specific situations/specific regions. |
| Huawei | Maybe we need to bring more study on case 6. Since larger path loss is observed between satellite and UE, that means it’s very sensitive to the interference for SAN receiver. Only to improve the receiver linearity may not achieve more gain without considering coexistence results. |
| THALES | And to answer to Ericsson, simulation results show that it depends if we consider the 5% average degradation throughput loss or the 5%-tile throughput loss. Depending on the method the result may be quite different. We might want to check again the calibration assumptions of the BS if the results are very different between the companies. |
| ZTE | We would like to check the simulation results for case 2 and case 6 firstly and further discuss the acceptable values. |
| CMCC | Share the same view with Ericsson, we should take the most stringent requirement from case 2 and 6 not the average. |
| Hughes/EchoStar | Same observation as Thales. The question is are there really large deployment of TN TDD in urban scenario |
| Qualcomm | We support option 5 since with case 6, 45deg elevation angle, 46dB is needed to make sure the co-ex. 46dB is also the same as the legacy BS ACS. We think it should not be a problem for vendors to reach 46dB ACS requirements. |

Issue 3-5: NTN UE ACLR

| Company | Comments |
| --- | --- |
| Samsung | Support recommended WF. |
| Ericsson | Agree with the WF, option 1 |
| THALES | Agree with the WF and support. |
| Huawei | Generally, we just assume ACLR2 instead of specifying ACLR2 for UE. 43dB ACLR2 can be removed. |
| ZTE | Agree with the WF, option 1 |
| CMCC | We support the WF |
| Hughes/EchoStar | Agreed with WF |
| Qualcomm | Agree |

Issue 3-6: NTN UE ACS

| Company | Comments |
| --- | --- |
| Samsung | Support recommended WF. |
| Ericsson | Agree with the WF, option 1 |
| THALES | Agree with the WF and support |
| Huawei | OK with option 1 |
| ZTE | Agree with the WF, option 1 |
| CMCC | We support the WF |
| Hughes/EchoStar | Agreed with WF |
| Qualcomm | Agree. Note that 33dB is only for the CBW <10MHz for carrier <2.7GHz. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201127 | Moderator: please kindly be noted the TP in R4-2201127 covers Chapter 6.3, 6.4 and a new Annex of TR 38.863. However, it is proposed to treat the document under this topic for the convenience of editing of this summary. Also, please be noted that the “R4-2201124 Collected NR-NTN co-ex results.xlsx” file should contain all the submitted co-ex results offline before Jan.10, and the intention is to add this file to the TR 38.863 as annex. If you have any new updates in your T-Doc to this meeting, please join the email discussion to update your results to the R4-2201124. |
|  | Samsung: Support the draft TP. We’d like to suggest one editorial changes if Case 5 and Case 6 need further discussion and may complicate the issue. If Case 5 and Case 6 scenarios is too complicated to reach consensus, and at the same time if the meeting agreed to use Case 1~4 to derive the all targeted RF requirements for NTN, then we suggest: - To remove the dedicated sections in Chapter 6.3 for Case 5 and Case 6; - To add wordings to explain why Case 5 and Case 6 is only attached in the annexes for information; - To include all the results of Case 5 and Case 6 in the attached excel sheet as annex for information only; |
|  | Ericsson: We are fine with the proposed TP. We would prefer to keep cases 5 and 6. RAN4 should anyway make some conclusion on those cases, clarifying if they were considered to determine ACLR/ACS values or, if not, how these 2 cases should be handled. |
|  | Qualcomm: we should capture all the cases in the TR. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 3-1: Separate NTN requirements based on TN type | Tentative agreements: No more discussions Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-2: Consideration of case 2 & 6 (for NTN BS ACS) and case 4 & 5 (for NTN UE ACLR) | Tentative agreements: No more discussions Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-3: NTN SAN ACLR | Tentative agreements: Use the result of the agreed scenario in Case 3 to derive NTN SAN ACLR Candidate options: N/A Recommendations for 2nd round: Further discuss |
| Issue 3-4: NTN SAN ACS | Tentative agreements: N/A Candidate options: Option 1: Use the results of the agreed scenario in Case 2 to derive a candidate NTN SAN ACS value and further discuss the value taking into account the outcome of Issue 1-8 "How to handle Case 6 results". Recommendations for 2nd round: Further discuss Option 1 |
| Issue 3-5: NTN UE ACLR | GTW Agreement (Jan. 18): Same requirement of TN UE for NTN UE |
| Issue 3-6: NTN UE ACS | GTW Agreement (Jan. 18): Same requirement of TN UE for NTN UE |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201127 | to be revised |

Discussion on 2nd round

Open issues and view collection

Issue 3-3: NTN SAN ACLR

- Proposals

  - Option 1(CATT): 38dB

  - Option 2 (Qualcomm): 26dB

  - Option 3 (Samsung): 23.15dB

  - Option 4 (Huawei): 20dB

  - Option 5 (Ericsson): 25dB

  - Option 6 (ZTE): 45dB

  - Option 7 (Thales): maximum value of 15~20 dBs

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Samsung | We propose to adopt 24 dB (or larger) as SAN ACLR based on the summary of co-ex results as below. The co-ex results suggested an averaged ACIR of 23.32 is required from the worst option in Case (Scenario) 3. Considering the TN UE ACS is 33 dB, by equation, the required ACLR for SAN should be at least 23.8 dB. Thus, we propose to consider 24 dB or larger value as SAN ACLR. Note: Below content is part of our draft TP to TR 38.863 based on the contributions to this meeting and can be found in “Draft TP” folder. - Scenario 3: NTN DL interfering TN DL The meeting evaluated the co-ex results from all concerned options in this scenario, and agreed to select the NR-NTN LEO-600 DL interfering the NR DL that deployed in rural environment as the most stringent case. Table 6.4.3-1 Simulation results for average throughput loss ACIR[dB]; 10; 12; 14; 16; 18; 20; 22; 24; 26; 28. Qualcomm; 21.02; 16.68; 12.34; 8.94; 6.48; 4.03; 2.98; 1.93; 1.21; 0.84. Samsung; 20.06; 15.28; 11.24; 8.00; 5.52; 3.72; 2.45; 1.60; 1.03; 0.66. MTK; 25.22; 19.06; 13.86; 9.70; 6.66; 4.41; 2.92; 1.90; 1.22; 0.77. ZTE; 16.65; 12.34; 8.84; 6.15; 4.17; 2.77; 1.81; 1.17; 0.75; 0.48. Ericsson; 3.7; 2.4; 1.6; 1.0. Huawei; 5.94; 3.97; 2.52; 1.64; 1.24. CATT; 17.3; 12.2; 10.1; 8.3; 6.6; 4.7. Xiaomi; 30.71; 23.92; 17.92; 12.93; 9.01; 6.11; 4.05; 2.64; 1.70; 1.09. Figure 6.4.3-1 Simulation results for average throughput loss Table 6.4.3-2 Simulation results for 5%-tile throughput loss ACIR[dB]; 14; 16; 18; 20; 22; 24; 26; 28; 30; 32. Qualcomm; 37.78; 28.51; 21.12; 13.72; 10.16; 6.60; 4.18; 2.89. Samsung; 27.10; 19.19; 13.13; 8.76; 5.73; 3.71; 2.37; 1.51; 0.97; 0.62. MTK; 37.56; 27.47; 19.31; 13.11; 8.69; 5.67; 3.65; 2.33; 1.49; 0.94. ZTE; 16.27; 10.85; 7.06; 4.59; 2.82; 1.91; 1.31; 0.88; 0.60; 0.39. Ericsson; 9.8; 6.2; 4.5; 2.8. Huawei; 8.61; 5.01; 3.12; 1.94; 1.32. CATT; 42.07; 32.30; 22.43; 15.88; 10.74; 7.90; 4.40. Xiaomi; 38.87; 28.90; 20.59; 14.15; 9.47; 6.22; 4.03; 2.59. Figure 6.4.3-2 Simulation results for average throughput loss Table 6.4.3-3 Interpolated ACIR values for Scenario 3 to meet the 5% throughput loss criteria Source; Interpolated ACIR. Qualcomm; Average; 19.21. 5%-tile; 25.32. Samsung; Average; 18.58. 5%-tile; 22.72. MTK; Average; 19.48. 5%-tile; 24.66. ZTE; Average; 17.16. 5%-tile; 19.67. Ericsson; Average. 5%-tile; 23.41. Huawei; Average; 18.95. 5%-tile; 20.01. CATT; Average; 19.68. 5%-tile; 25.66. Xiaomi; Average; 21.01. 5%-tile; 25.11. Table 6.4.3-4 Average ACIR values in the above worse case for Scenario 3 Scenario 3. ACIR value [dB]; 23.32. |
| Xiaomi | We support the value 24 dB as SAN ACLR which are derived through company’s co-ex results. |
| Qualcomm | Support Samsung’s comments. |
| Ericsson | Ok with Samsung’s proposal. |
| Huawei | Support Samsung’s proposal. |
| THALES | We need to check again how do we compute the ACIR, based on which values and which is the exact equation to perform the average. We also need more companies to provide simulations for both AAS non-AAS values (only 2-3 values is not sufficient to determine the worst case is AAS). Moreover, our simulation results show the following worst case for Case 3: |

Issue 3-4: NTN SAN ACS

- Proposals

  - Option 1: Use the results of the agreed scenario in Case 2 to derive a candidate NTN SAN ACS value and further discuss the value taking into account the outcome of Issue 1-8 "How to handle Case 6 results".

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Samsung | We propose to consider 38 dB (or larger) as a CANDIDATE SAN ACS based on the summary of co-ex results for Case 2 as below. And Case 6 needs more discussion. The co-ex results suggested an averaged ACIR of 29.25 is required from the worst option in Case (Scenario) 2. Considering the TN UE ACLR is 30 dB, by equation, the required ACS for SAN should be at least 37.2 dB. Thus, we propose to consider 38 dB or larger value as SAN ACS. Note: Below content is part of our draft TP to TR 38.863 based on the contributions to this meeting and can be found in “Draft TP” folder. - Scenario 2: TN UL interfering NTN UL The meeting evaluated the co-ex results from all concerned options in this scenario, and agreed to select the NR UL interfering the NR-NTN GEO UL that deployed in urban environment as the most stringent case. Table 6.4.2-1 Simulation results for average throughput loss ACIR[dB]; 18; 20; 22; 24; 26; 28; 30; 32; 34; 36. Qualcomm; 36.46; 24.81; 18.43; 12.06; 7.70; 5.34; 2.98. ZTE; 16.46; 11.77; 8.26; 5.51; 3.55; 2.25; 1.48; 0.98; 0.63; 0.39. MTK; 38.61; 31.96; 25.57; 20.13; 15.27; 11.22; 7.96; 5.48; 3.68; 2.42. Ericsson; 15.6; 10.4; 7.2; 4.2; 2.9; 1.9. CATT; NA; NA; NA; NA; NA; NA; NA; NA; NA; NA. Xiaomi; 41.11; 30.58; 21.75; 14.92; 9.96; 6.53; 4.22; 2.70; 1.72; 1.09. Figure 6.4.2-1 Simulation results for average throughput loss Table 6.4.2-2 Simulation results for 5%-tile throughput loss ACIR[dB]; 22; 24; 26; 28; 30; 32; 34; 36; 38; 40. Qualcomm; NA; NA; NA; NA; NA. ZTE; NA; NA; NA; NA; NA; NA; NA; NA; NA; NA. MTK; 68.80; 58.62; 47.26; 36.43; 26.97; 19.13; 13.06; 8.68; 5.67; 3.67. Ericsson; NA; NA; NA; NA; NA. CATT; NA; NA; NA; NA; NA; NA; NA; NA; NA; NA. Xiaomi; NA; NA; NA; NA; NA; NA; NA; NA; NA. Figure 6.4.2-2 Simulation results for 5%-tile throughput loss Table 6.4.2-3 Interpolated ACIR values for Scenario 2 to meet the 5% throughput loss criteria Source; Interpolated ACIR. Qualcomm; Average; 28.29. 5%-tile. ZTE; Average; 24.52. 5%-tile. MTK; Average; 32.53. 5%-tile; 38.67. Ericsson; Average; 25.47. 5%-tile. CATT; Average. 5%-tile. Xiaomi; Average; 29.32. 5%-tile. Table 6.4.2-4 Average ACIR values in the above worse case for Scenario 2 Scenario 2. ACIR value [dB]; 29.25. |
| Xiaomi | We support the candidate value 38 dB as SAN ACS which are derived through company’s co-ex results for case 2. We can use this value for further check with case 6. |
| Qualcomm | Need to consider Case 6 as the worst case to derive SAN ACS. |
| Ericsson | We can’t just ignore case 6. If it can’t be done via SAN ACS, some other requirement(s) should take this into account, but we have no such proposal so, for the time being, we need to include case 6 in SAN ACS evaluation. |
| Huawei | After checking the email discussion, maybe we need to use the value for average throughput loss from the companies. |
| THALES | We need to check again how do we compute the ACIR, based on which values and which is the exact equation to perform the average. MKT value of 38.67dB should not be taken into account, since is the only result provided for 5%-tile and for this reason it might not be representative. We also need more companies to provide simulations for both AAS non-AAS values (only 2-3 values is not sufficient to determine the worst case is AAS). Our simulation results show the following worst case for Case 2: |
| Samsung | We’d like to support views that Case 6 should be taken into account when decide the SAN ACS. And at the same time, for Case 2, we would like to update the results based average throughput loss as follows. The updated average ACIR would be 28.03 dB, while the suggested SAN ACS from Case 2 would be 32.41 dB (or larger). Table 6.4.2-3 Interpolated ACIR values for Scenario 2 to meet the 5% throughput loss criteria Source; Interpolated ACIR. Qualcomm; Average; 28.29. 5%-tile. ZTE; Average; 24.52. 5%-tile. MTK; Average; 32.53. 5%-tile; 38.67. Ericsson; Average; 25.47. 5%-tile. CATT; Average. 5%-tile. Xiaomi; Average; 29.32. 5%-tile. Table 6.4.2-4 Average ACIR values in the above worse case for Scenario 2 Scenario 2. ACIR value [dB]; 28.03. |
| Omnispace | Agree with Thales |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2202988 (revised from R4-2201127) | Moderator: The draft R4-2202988 (revised from R4-2201127) has been uploaded to ftp in “Draft TP” folder for comments. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 3-3: NTN SAN ACLR | Agreements: NTN SAN ACLR is 24dB. |
| Issue 3-4: NTN SAN ACS | Further discuss following Options to conclude NTN SAN ACS. NTN SAN ACS; Case 2; Option 1-1: 38dB Option 1-2: >32.41dB. Case 6; Option 2-1: 46dB Option 2-2: >37.4 dB (Rural only). Propose to discuss the ACS value in GTW session. |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2202988 (revised from R4-2201127) | Agreeable |

Topic #4: HAPS coexistence scenarios and results

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200782 | Qualcomm Incorporated | Observation 1: The interference from HAPS to AAS TN is acceptable. Introducing HAPS will not impact current TN deployment. Observation 2: The edge performance of HAPS is vulnerable. It is difficult to measure the interference’s impact brought by other systems using 5-ile performance loss. The operators’ coordination mechanism is needed to enable the co-coverage of HAPS and TN, e.g., HAPS UE will handover/roam to TN network configured by network in this case. Proposal 1: The ACLR/ACS for TN UE is also applicable for HAPS UE. HAPS can support existing TN UE. Proposal 2: The frequency coordination measures are needed to enable HAPS and TN coexistence in the same coverage. The HAPS operator should plan its frequency deployment considering the ACI impact from TN but there is no need to specify the corresponding RAN4 requirements. |
| R4-2201077 | Nokia, Nokia Shanghai Bell | Observation 1: The operation scenarios of HAPS require only new ACLR/ACS requirements for HAPS and four different coexistence scenarios. Observation 2: A small part of HAPS UEs have a higher coupling loss than the maximum coupling loss in TN. Proposal 1: For HAPS coexistence, only simulate the four scenarios for HAPS ACLR/ACS requirements as shown in Table 1. Proposal 2: Adopt the revised HAPS cell layout in Figure 2 with a circular HAPS coverage area. Proposal 3: Add frequency reuse factor 1 to the HAPS system parameters. Proposal 4: Impose a maximum coupling loss limit of 140 dB for the HAPS system. HAPS UEs with a coupling loss >140 dB are excluded from simulations. Proposal 5: In HAPS UL, 9 UEs per cell are scheduled. Each UE is allocated 6 RBs. The allocated frequency resources are all aligned across the 7 cells of HAPS as shown in Figure 4. Proposal 6: Adopt the TR 38.942 ACIR model for HAPS coexistence simulations. Same ACIR value is given in a bandwidth equal to the aggressor UE’s transmission bandwidth. Proposal 7: Revise the uplink transmission power control parameter X according to UL scheduled bandwidth assumptions, X=6.3 for TN and X=1.08 for HAPS. |
| R4-2201079 | Nokia, Nokia Shanghai Bell | Observation 1: The required ACIR for the HAPS DL aggressor scenarios is 23.0 dB. Observation 2: HAPS DL coexistence simulation results indicate the required ACLR for HAPS is about 24 dB. Proposal 1: The required ACLR for HAPS shall be >24 dB. |
| R4-2201254 | Huawei, HiSilicon | Observation 1: Since current RF module of WA BS working in the IMT bands can be used for HAPS, the RF requirement for WA BS can be used for HAPS. Observation 2: Based on the simulation assumptions [2] for HAPS co-existence, all of the BS RF parameters for HAPS are same with Macro (WA) BS, e.g. output power, antenna parameter, Noise Figure. Observation 3: HAPS based on WA BS RF requirements has been deployed in the current field and worked well. Proposal 1: To reuse the current WA BS RF requirements for HAPS. |

Open issues summary

Sub-topic 4-1

General discussions.

Issue 4-1: Scenarios for simulation and discussion

- Proposals

  - Option 1: For HAPS coexistence, only simulate the four scenarios for HAPS ACLR/ACS requirements as table below

| Coexistence | Aggressor | Victim | New requirement |
| --- | --- | --- | --- |
| TN with HAPS | HAPS DL | TN DL | HAPS ACLR |
| HAPS with HAPS | HAPS DL | HAPS DL | HAPS ACLR |
| TN with HAPS | TN UL | HAPS UL | HAPS ACS |
| HAPS with HAPS | HAPS UL | HAPS UL | HAPS ACS |

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-2: Cell layout

- Proposals

  - Option 1: Adopt the revised HAPS cell layout in below figure with a circular HAPS coverage area

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-3: Frequency reuse factor

- Proposals

  - Option 1: Add frequency reuse factor 1 to the HAPS system parameters

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-4: Coupling loss limit

- Proposals

  - Option 1: Impose a maximum coupling loss limit of 140 dB for the HAPS system. HAPS UEs with a coupling loss >140 dB are excluded from simulations.

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-5: UL UE scheduling

- Proposals

  - Option 1: In HAPS UL, 9 UEs per cell are scheduled. Each UE is allocated 6 RBs. The allocated frequency resources are all aligned across the 7 cells of HAPS as shown in below figure.

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-6: ACIR model

- Proposals

  - Option 1: Adopt the TR 38.942 ACIR model for HAPS coexistence simulations. Same ACIR value is given in a bandwidth equal to the aggressor UE’s transmission bandwidth.

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-7: UL power control parameter X

- Proposals

  - Option 1: Revise the uplink transmission power control parameter X according to UL scheduled bandwidth assumptions, X=6.3 for TN and X=1.08 for HAPS.

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 4-8: Frequency coordination measures

- Proposals

  - Option 1: The frequency coordination measures are needed to enable HAPS and TN coexistence in the same coverage. The HAPS operator should plan its frequency deployment considering the ACI impact from TN but there is no need to specify the corresponding RAN4 requirements.

  - Option 2: TBA

- Recommended WF

  - TBA

Sub-topic 4-2

Results discussions.

Issue 4-9: ACLR and ACS for HAPS BS

- Proposals

  - Option 1: The required ACLR for HAPS shall be >24 dB.

  - Option 2: To reuse the current WA BS RF requirements for HAPS

- Recommended WF

  - TBA

Issue 4-10: ACLR and ACS for HAPS UE

- Proposals

  - Option 1: The ACLR/ACS for TN UE is also applicable for HAPS UE. HAPS can support existing TN UE.

  - Option 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 4-1: Scenarios for simulation and discussion

| Company | Comments |
| --- | --- |
| Ericsson | We don’t agree with option 1. We understand the assumption is to reuse a TN UE for HAPS but still, RAN4 should verify this assumption is correct, making sure UE TN ACLR and ACS are sufficient to not degrade adjacent TN. This is similar approach RAN4 took for satellite. |
| Huawei | We have similar view with Ericsson. |
| Nokia | Support Option 1. Given that the UE connected to HAPS is the same type of UE that connect with TN. There is no new ACLR/ACS requirements for the HAPS UE. With this understanding, we only need these four scenarios to derive the ACLR/ACS requirements for HAPS. |

Issue 4-2: Cell layout

| Company | Comments |
| --- | --- |
| Nokia | Support option 1. This cell layout is simple to implement and consistent with the assumption of 100 km HAPS coverage radius and the HAPS antenna model. We also noticed that an alternative layout is proposed by Qualcomm in R4-2200782. The difference between these two topologies is small and the calibration results between the two companies are similar. Perhaps we can compromise to option 1. |
| Qualcomm | We prefer to use hexagon layout which is the typical simulation assumptions in RAN4. As commented by Nokia, we see the difference is small between two topologies. We are OK to keep the two options in the simulation. |

Issue 4-3: Frequency reuse factor

| Company | Comments |
| --- | --- |
| Nokia | Support Option 1. This is to make up an assumption we overlooked previously. |
| Qualcomm | Agree with option 1 |

Issue 4-4: Coupling loss limit

| Company | Comments |
| --- | --- |
| Ericsson | This is not common practice in RAN4 coexistence simulations. This would also ignore the UEs might be able to connect and have som UL throughput (#RBs<6). |
| Nokia | Support Option 1. Within the 100 km coverage radius, the elevation angle for a HAPS UE can vary from 11⁰ to 90⁰. And NLOS probability is high for the low elevation angle Ues. For those NLOS Ues, path loss can be very large (>25 dB compared to LOS). When coupling loss is >140 dB for some NLOS cases, the link cannot be closed. There should be a maximum CL for those Ues served by HAPS. With this CL limit, we can evaluate the 5%-tile throughput degradation without running to the problem of 0 throughput. |
| Qualcomm | Agree with Ericsson. |

Issue 4-5: UL UE scheduling

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Nokia | Option 1 |
| Qualcomm | Option 1 |

Issue 4-6: ACIR model

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | TR 36.942. Is it a type? |
| Nokia | Support Option 1. The ACIR model is needed for UL simulations since one UE only uses part of the channel bandwidth. |
| Qualcomm | Agree option 1 (TR36.942) |

Issue 4-7: UL power control parameter X

| Company | Comments |
| --- | --- |
| Ericsson | X depends on the #RBs scheduled per UE anyway |
| Nokia | Support Option 1. This is based on the UL scheduling bandwidth (option 1) proposal in Issue 4-5 |

Issue 4-8: Frequency coordination measures

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Nokia | We think there is no need to specify UE requirements for HAPS and TN coexistence. The same UE used in TN can connect to HAPS. But there needs to be HAPS requirements to protect TN and other HAPS from the ACI of the aggressor HAPS. If HAPS is to be deployed in a TN coverage area, frequency planning is needed to mitigate the impact of TN DL ACI on HAPS DL. (But TN is protected by the ACI coexistence requirements of HAPS and UE.) |
| Qualcomm | Option 1 |

Issue 4-9: ACLR and ACS for HAPS BS

| Company | Comments |
| --- | --- |
| Ericsson | Both options are not contradictory |
| Huawei | We support option 2. |
| Nokia | Support Option 1. Simulation results show the ACLR requirement for HAPS is 24 dB. This applies to the HAPS specific operating scenarios with its 20 km height and 100 km coverage radius. We should consider the coexistence scenarios for HAPS ACLR and ACS requirements, and the HAPS scenarios are very different from terrestrial BS scenarios. The existing 45 dB ACLR for TN BS is overly stringent for HAPS. Other than ACLR/ACS, we think the current WA BS RF requirements, e.g., output power and receiver sensitivity, are applicable to HAPS BS. By that we could be okay with option 2 if we further discuss ACLR/ACS. |

Issue 4-10: ACLR and ACS for HAPS UE

| Company | Comments |
| --- | --- |
| Ericsson | Before we agree on this, that should be confirmed by the simulation results, see issue 4-1 |
| Nokia | Support Option 1. Our view is that HAPS can support existing TN UEs and this is already supported by the provided simulation results. |
| Qualcomm | Option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201078 | Moderator: please kindly be noted the TP in R4-2201078 covers Chapter 6.1 and 6.2 of TR 38.863. However, it is proposed to treat the document under this topic for the convenience of editing of this summary. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 4-1: Scenarios for simulation and discussion | Tentative agreements: N/A Candidate options: Option 1: For HAPS coexistence, only simulate the four scenarios for HAPS ACLR/ACS requirements as table below Coexistence; Aggressor; Victim; New requirement. TN with HAPS; HAPS DL; TN DL; HAPS ACLR. HAPS with HAPS; HAPS DL; HAPS DL; HAPS ACLR. TN with HAPS; TN UL; HAPS UL; HAPS ACS. HAPS with HAPS; HAPS UL; HAPS UL; HAPS ACS. Recommendations for 2nd round: Further discuss Option 1. |
| Issue 4-2: Cell layout | Tentative agreements: Adopt the revised HAPS cell layout in below figure with a circular HAPS coverage area as a new Option. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-3: Frequency reuse factor | Tentative agreements: Add frequency reuse factor 1 to the HAPS system parameters Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-4: Coupling loss limit | Tentative agreements: N/A Candidate options: Option 1: Impose a maximum coupling loss limit of 140 dB for the HAPS system. HAPS UEs with a coupling loss >140 dB are excluded from simulations. Recommendations for 2nd round: Further discuss Option 1. |
| Issue 4-5: UL UE scheduling | Tentative agreements: In HAPS UL, 9 UEs per cell are scheduled. Each UE is allocated 6 RBs. The allocated frequency resources are all aligned across the 7 cells of HAPS as shown in below figure. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-6: ACIR model | Tentative agreements: Adopt the TR 36.942 ACIR model for HAPS coexistence simulations. Same ACIR value is given in a bandwidth equal to the aggressor UE’s transmission bandwidth. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-7: UL power control parameter X | Tentative agreements: Revise the uplink transmission power control parameter X according to UL scheduled bandwidth assumptions, X=6.3 for TN and X=1.08 for HAPS. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-8: Frequency coordination measures | Tentative agreements: The frequency coordination measures are needed to enable HAPS and TN coexistence in the same coverage. The HAPS operator should plan its frequency deployment considering the ACI impact from TN but there is no need to specify the corresponding RAN4 requirements. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-9: ACLR and ACS for HAPS BS | Tentative agreements: N/A Candidate options: - Option 1: The required ACLR for HAPS shall be >24 dB. - Option 2: To reuse the current WA BS RF requirements for HAPS Recommendations for 2nd round: Try to merge Option 1 & 2 |
| Issue 4-10: ACLR and ACS for HAPS UE | Tentative agreements: To capture following wording in TR 38.863. “It is the baseline assumption that the ACLR/ACS for TN UE is also applicable for HAPS UE and HAPS can support existing TN UE. However such assumption should be verified by co-existence studies.” Candidate options: N/A Recommendations for 2nd round: N/A |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2201078 | Agreeable |

Discussion on 2nd round

Open issues and view collection

Issue 4-1: Scenarios for simulation and discussion

- Proposals

  - Option 1: For HAPS coexistence, only simulate the four scenarios for HAPS ACLR/ACS requirements as table below

| Coexistence | Aggressor | Victim | New requirement |
| --- | --- | --- | --- |
| TN with HAPS | HAPS DL | TN DL | HAPS ACLR |
| HAPS with HAPS | HAPS DL | HAPS DL | HAPS ACLR |
| TN with HAPS | TN UL | HAPS UL | HAPS ACS |
| HAPS with HAPS | HAPS UL | HAPS UL | HAPS ACS |

- Recommended WF

  - As agreed in Issue 4-10, simulation for HAPS UE ACLR/ACS is still needed.

| Company | Agree with WF? (Y/N) | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| Ericsson | Yes |  |
| Nokia | Yes | The four scenarios in Option 1 are what we think necessary to derive HAPS BS ACLR/ACS requirements. For the purpose of coexistence study, we see two additional scenarios: (1) TN DL interfering HAPS DL, and (2) HAPS UL interfering TN UL. These two scenarios are not related to HAPS BS requirements, but for a better understanding of HAPS coexistence, we will try to provide simulation results of these two scenarios. |

Issue 4-4: Coupling loss limit

- Proposals

  - Option 1: Impose a maximum coupling loss limit of 140 dB for the HAPS system. HAPS UEs with a coupling loss >140 dB are excluded from simulations.

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2. Define a maximum CL could not reflect the real co-ex scenarios. |
| Ericsson | Option 2, agree with Qualcomm |
| Huawei | Option 2. Agree with Qualcomm. |
| Nokia | Option 1 would allow us to evaluate the ACI impact on 5%-tile throughput. Without that assumption, the 5%-tile throughput will be 0 in many cases. In light of the input from round 1, we would like to make proposal for option 2. “Option 2: Whenever evaluation of 5%-tile throughput degradation is impossible due to zero throughput, adjacent channel coexistence requirements will be derived only from the mean throughput loss.” To derive ACLR/ACS requirements, we need to select one of the two options. We are fine with either option. |

Issue 4-9: ACLR and ACS for HAPS BS

- Proposals

  - Option 1: The required ACLR for HAPS shall be >24 dB.

  - Option 2: To reuse the current WA BS RF requirements for HAPS.

- Recommended WF

  - Reuse the current WA BS RF requirements for HAPS, except ACLR/ACS which need further discussion.

| Company | Agree with WF? (Y/N) | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| China Unicom |  | We prefer to reuse the current WA BS RF requirements for HAPS, including ACLR/ACS. |
| Huawei |  | We prefer option 2. |
| Nokia | Yes | We have no objection of reusing current WA BS RF requirements except ACLR/ACS. The ACLR/ACS requirements should be based on the coexistence study outcome, considering HAPS specific operating scenarios. Nokia’s results show the required ACIR for HAPS DL aggressor is 23 dB (R4-2201079), corresponding to HAPS ACLR 23.5 dB (assuming UE ACS 33 dB). Qualcomm’s results show the required ACIR for HAPS DL aggressor is 26 dB (R4-2200782), corresponding to HAPS ACLR 27.0 dB. Based on these results, the ACLR requirement for HAPS BS should be in the range of 24-27 dB. So we would like to propose adopting an ACLR requirement for HAPS BS in this range, 24-27 dB. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 4-1: Scenarios for simulation and discussion | Agreement: Simulation for HAPS UE ACLR/ACS is still needed. Note: Two additional scenarios: (1) TN DL interfering HAPS DL, and (2) HAPS UL interfering TN UL have been proposed for further consideration |
| Issue 4-4: Coupling loss limit | Further discuss this issue in next meeting, with two options considered: - Option 1: Impose a maximum coupling loss limit of 140 dB for the HAPS system. HAPS UEs with a coupling loss >140 dB are excluded from simulations. - Option 2: Whenever evaluation of 5%-tile throughput degradation is impossible due to zero throughput, adjacent channel coexistence requirements will be derived only from the mean throughput loss. |
| Issue 4-9: ACLR and ACS for HAPS BS | Agreement: Reuse the current WA BS RF requirements for HAPS, except ACLR/ACS which need further discussion based on co-existence study results, noting that this does not preclude the possibility to reuse ACLR/ACS of current WA BS RF requirements for HAPS. |

Topic #5 Work on TR 38.863

Discussion on 2nd round

Open issues and views collection

Issue 5-1 Update of TR 38.863 skeleton

- Proposal

  - Option 1: To update sub-sections of Chapter 7 as following so as to ensure the future proof.

| 7.3.2 | Conducted transmission characteristics |
| --- | --- |
| 7.3.3 | Conducted receiver characteristics |
| 7.3.4 | Reserved (for Radiated transmission characteristics) |
| 7.3.5 | Reserved (for Radiated recerver characteristics) |
| 7.3.6 | Others |
| 7.4 | NTN UE requirements |
| 7.4.1 | General |
| 7.4.2 | NTN UE conducted transmission characteristics |
| 7.4.3 | NTN UE conducted receiver characteristics |
| 7.4.4 | Reserved (for Radiated transmission characteristics) |
| 7.4.5 | Reserved (for Radiated receiver characteristics) |
| 7.4.6 | Others |

- Recommended WF

  - Agree on Option 1

| Company | Comments |
| --- | --- |

Issue 5-2 Work split of TR 38.863

- Proposal

  - To update work split. Please put your name in the column after corresponding sections your wish to lead.

| Section | Title | Company |  |
| --- | --- | --- | --- |
| 1 | Scope | Samsung | Samsung |
| 2 | References |  |  |
| 3 | Definition of terms, symbols and abbreviations |  |  |
| 3.1 | Terms |  |  |
| 3.2 | Symbos |  |  |
| 3.3 | Abbreviations |  |  |
| 4 | General aspects | Nokia | Nokia |
| 4.1 | Work item objective |  | Nokia |
| 5 | Regulatory aspects | Ericsson, Hughes/EchoStar | Ericsson, Hughes/EchoStar |
| 5.1 | ITU-R |  |  |
| 5.4 | Regulatory aspects for HAPS | Nokia | Nokia |
| 6 | Co-existence study | Samsung, Nokia | Samsung, Nokia |
| 6.1 | Co-existence simulation scenario |  |  |
| 6.2 | Co-existence simulation assumption |  |  |
| 6.3 | Co-existence simulation methodology |  |  |
| 6.4 | Co-existence simulation results |  |  |
| 6.5 | Summary of co-existence study |  |  |
| 7 | RF requirements |  |  |
| 7.1 | Reference points for RF requirements | Ericsson | Ericsson |
| 7.2 | Common issues for satellite node and NTN UE | THALES | THALES |
| 7.2.1 | Operating bands and channel arrangements | ZTE | ZTE |
| 7.2.2 | Channel bandwidth, SCS and spectral utilization | ZTE | ZTE |
| 7.2.3 | Channel raster and sync raster | CATT | CATT |
| 7.31 | Satellite access node requirements | THALES [THALES, CATT, Ericsson, Huawei, ZTE]1 | THALES |
| 7.3.1 | General |  | THALES |
| 7.3.1.11 | Satellite access node class |  |  |
| 7.3.2 | Conducted transmission characteristics | CATT | CATT |
| 7.3.3 | Conducted receiver characteristics |  | Huawei |
| 7.3.4 | Reserved (for Radiated transmission characteristics) |  | To be decided later |
| 7.3.5 | Reserved (for Radiated recerver characteristics) |  | To be decided later |
| 7.3.6 | Others | ZTE | ZTE |
| 7.4 | NTN UE requirements | [Huawei, Qualcomm, Xiaomi, Hughes/EchoStar]1 | Huawei |
| 7.4.1 | General |  | Hughes/EchoStar |
| 7.4.2 | NTN UE conducted transmission characteristics | Qualcomm | Qualcomm |
| 7.4.3 | NTN UE conducted receiver characteristics |  | Xiaomi |
| 7.4.4 | Reserved (for Radiated transmission characteristics) |  | To be decided later |
| 7.4.5 | Reserved (for Radiated recerver characteristics) |  | To be decided later |
| 7.4.6 | Others |  | Huawei |
| Annex A | Simulation results of NTN components | Samsung | Samsung |
| Annex B | Simulation results of TN components | Samsung | Samsung, |
| Note 1: Structure and contents of Section 7.3 and 7.4 will be further discussed and determined taking due consideration of the development of TS 38.108, 38.181 and a potential TS of NTN UE. Therefore, lead of each section or sub-section is still open for discussion. |  |  |  |

- Recommended WF

  - TBA

Summary for 2nd round

Open issues

Issue 5-1 Update of TR 38.863 skeleton

Agreement: To update sub-sections of Chapter 7 as following with updated parts marked in yellow.

| Section | Title |
| --- | --- |
| 7.3.2 | Conducted transmission characteristics |
| 7.3.3 | Conducted receiver characteristics |
| 7.3.4 | Reserved (for Radiated transmission characteristics) |
| 7.3.5 | Reserved (for Radiated receiver characteristics) |
| 7.3.6 | Others |
| 7.4 | NTN UE requirements |
| 7.4.1 | General |
| 7.4.2 | NTN UE conducted transmission characteristics |
| 7.4.3 | NTN UE conducted receiver characteristics |
| 7.4.4 | Reserved (for Radiated transmission characteristics) |
| 7.4.5 | Reserved (for Radiated receiver characteristics) |
| 7.4.6 | Others |

Issue 5-2 Work split of TR 38.863

Agreement: Work split of TR 38.863 has been updated and agreed as below.

| Section | Title | Company |
| --- | --- | --- |
| 1 | Scope | Samsung |
| 2 | References |  |
| 3 | Definition of terms, symbols and abbreviations |  |
| 3.1 | Terms |  |
| 3.2 | Symbos |  |
| 3.3 | Abbreviations |  |
| 4 | General aspects | Nokia |
| 4.1 | Work item objective | Nokia |
| 5 | Regulatory aspects | Ericsson, Hughes/EchoStar |
| 5.1 | ITU-R |  |
| 5.4 | Regulatory aspects for HAPS | Nokia |
| 6 | Co-existence study | Samsung, Nokia |
| 6.1 | Co-existence simulation scenario |  |
| 6.2 | Co-existence simulation assumption |  |
| 6.3 | Co-existence simulation methodology |  |
| 6.4 | Co-existence simulation results |  |
| 6.5 | Summary of co-existence study |  |
| 7 | RF requirements |  |
| 7.1 | Reference points for RF requirements | Ericsson |
| 7.2 | Common issues for satellite node and NTN UE | THALES |
| 7.2.1 | Operating bands and channel arrangements | ZTE |
| 7.2.2 | Channel bandwidth, SCS and spectral utilization | ZTE |
| 7.2.3 | Channel raster and sync raster | CATT |
| 7.31 | Satellite access node requirements | THALES |
| 7.3.1 | General | THALES |
| 7.3.1.11 | Satellite access node class |  |
| 7.3.2 | Conducted transmission characteristics | CATT |
| 7.3.3 | Conducted receiver characteristics | Huawei |
| 7.3.4 | Reserved (for Radiated transmission characteristics) | To be decided later |
| 7.3.5 | Reserved (for Radiated recerver characteristics) | To be decided later |
| 7.3.6 | Others | ZTE |
| 7.4 | NTN UE requirements | Huawei |
| 7.4.1 | General | Hughes/EchoStar |
| 7.4.2 | NTN UE conducted transmission characteristics | Qualcomm |
| 7.4.3 | NTN UE conducted receiver characteristics | Xiaomi |
| 7.4.4 | Reserved (for Radiated transmission characteristics) | To be decided later |
| 7.4.5 | Reserved (for Radiated recerver characteristics) | To be decided later |
| 7.4.6 | Others | Huawei |
| Annex A | Simulation results of NTN components | Samsung |
| Annex B | Simulation results of TN components | Samsung |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on [307] NTN_Solutions_Part2 | Samsung |  |
| Simulation assumptions for NTN co-existence | Samsung, CATT |  |
| Simulation assumptions for HAPS co-existence | Nokia |  |
| Summary of NTN co-existence study | Samsung |  |
| Summary of HAPS co-existence study | Nokia |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2201127 | Draft Text Proposal for TR 38.863 Chapter 6.3 and 6.4 | Samsung | Revised | Revised to R4-2202988 |
| R4-2201078 | TP to TR 38.863 on HAPS coexistence study | Nokia, Nokia Shanghai Bell | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2202988 | Draft Text Proposal for TR 38.863 | Samsung | Agreeable |  |
| R4-2202990 | WF on [307] NTN _Solutions_Part2 | Samsung | Return to | Decision from Session Chair: The WF agreeable, we will discuss the WF in final round GTW to make decision on NTN SAN ACLR/ACS. |
| R4-2202991 | Simulation assumptions for NTN co-existence | Samsung, CATT | Agreeable |  |
| R4-2202992 | Simulation assumptions for HAPS co-existence | Nokia | Agreeable |  |
| R4-2202993 | Summary of NTN co-existence study | Samsung | Agreeable |  |
| R4-2202994 | Summary of HAPS co-existence study | Nokia | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex 1 Contact information

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Runsen TANG | runsen.tang@samsung.com |
| Ericsson | Dominique Everaere | dominique.everaere@ericsson.com |
| Nokia | Johannes Hejselbaek | Johannes.hejselbaek@nokia.com |
| Qualcomm | Bin Han | binhan@qti.qualcomm.com |
| THALES | Dorin Panaitopol |  |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

- Annex 2 TDOC list for Agenda Item 6.13.2

A total of 24 TDOCs have been reserved, while 23 of them are received for this agenda and listed as below.

| TDoc No. | Title | Source | Type | For | Agenda Item | Status |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2200164 | NTN coexistence simulations | CATT | discussion | Discussion | 6.13.2.1 | reserved |
| R4-2200166 | ACLR/ACS proposal | CATT | discussion | Discussion | 6.13.2.3 | available |
| R4-2200781 | Coexistence simulation results for TN-NTN | Qualcomm Incorporated | discussion |  | 6.13.2.1 | available |
| R4-2200782 | Coexistence simulation restuls for HAPS | Qualcomm Incorporated | discussion |  | 6.13.2.2 | available |
| R4-2201072 | NR-NTN co-ex study results | Samsung | discussion | Approval | 6.13.2.1 | available |
| R4-2201077 | HAPS simulation assumptions for coexistence study | Nokia, Nokia Shanghai Bell | discussion | Approval | 6.13.2.2 | available |
| R4-2201078 | TP to TR 38.863 on HAPS coexistence study | Nokia, Nokia Shanghai Bell | pCR | Approval | 6.13.2.2 | available |
| R4-2201079 | ACLR and ACS proposal for HAPS | Nokia, Nokia Shanghai Bel | discussion | Approval | 6.13.2.3 | available |
| R4-2201124 | Collected NR-NTN co-ex results | Samsung | discussion | Discussion | 6.13.2.1 | available |
| R4-2201126 | NR-NTN co-ex results analysis and ACLR ACS proposal | Samsung | discussion | Approval | 6.13.2.3 | available |
| R4-2201127 | Draft Text Proposal for TR 38.863 Chapter 6.3 and 6.4 | Samsung | discussion | Approval | 6.13.2.3 | available |
| R4-2201222 | Simulation result for coexistence study on NR to support non-terrestrial networks | Xiaomi | discussion | Information | 6.13.2.1 | available |
| R4-2201254 | Discussion on HAPS requirements | Huawei, HiSilicon | other | Approval | 6.13.2.2 | available |
| R4-2201255 | NR NTN co-existence simulation Results | Huawei, HiSilicon | discussion | Discussion | 6.13.2.1 | available |
| R4-2201256 | Discussion on ACLR and ACS for NR NTN | Huawei, HiSilicon | other | Approval | 6.13.2.3 | available |
| R4-2201262 | NTN coexistence results and observations | MediaTek (Chengdu) Inc. | discussion | Discussion | 6.13.2.1 | available |
| R4-2201316 | NTN - Coexistence simulation results | Ericsson | discussion | Discussion | 6.13.2.1 | available |
| R4-2201317 | NTN - ACLR/ACS proposals | Ericsson | other | Approval | 6.13.2.3 | available |
| R4-2201466 | Simulation results for NTN coexistence study | ZTE Corporation | other | Approval | 6.13.2.1 | available |
| R4-2201467 | Discussion on ACLR and ACS requirements for NTN | ZTE Corporation | other | Approval | 6.13.2.3 | available |
| R4-2201839 | TN-NTN Coexistence Results Updates | THALES, Magister Solutions Ltd | discussion | Agreement | 6.13.2.3 | available |
| R4-2201842 | NTN Simulation Assumptions | THALES | discussion | Discussion | 6.13.2.1 | available |
| R4-2201843 | NTN coexistence calibration data - THALES 28_09_2021 | THALES | discussion | Information | 6.13.2.1 | available |
| R4-2201852 | NR-NTN coexistence results - THALES updates 10_01_2022 | THALES | discussion | Information | 6.13.2.1 | available |