3GPP TSG-RAN WG4 Meeting # 101-e-bis	R4-2201151

# Electronic Meeting, Jan.17-25, 2022

Agenda item:	6.22.2.3

Source: 	OPPO

Title: 	Discussion on conditional PSCell change and addition

Document for:	Discussion and decision

- Introduction

In RAN4#101 e-meeting, RAN4 related R17 MR-DC enhancement was discussed, with agreement captured in [1]. There are still some open issues left on conditional PSCell change and addition. In this contribution, we will discuss the remaining issues captured in [1].

- Discussion

Issue 3-2-1: Tprocessing in conditional PSCell addition delay - Option 1(Apple, ZTE, MTK, Huawei, vivo, Nokia): - For ENDC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2; - For NRDC Tprocessing = 40 ms. - Option 2(Qualcomm, Ericsson): Tprocessing = 40ms is used regardless of FR.

Regarding TUE_preparation in Conditional PSCell change delay, we prefer to reuse existing requirements: Tprocessing = 20 ms when source and target cells are in the same FR, and Tprocessing = 40 ms when source and target cells are in different FRs. It was agreed in the last meeting that CPAC related requirements are developed for

- ENDC: FR1+FR1, FR1+FR2

- NRDC: PCell on FR1+ PSCell FR2

Therefore, for EN-DC FR1+FR1, Tprocessing = 20 ms. For EN-DC FR1+FR2 and NR-DC FR1+FR2, Tprocessing = 40 ms.

Proposal 1: For EN-DC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2;

For NR-DC FR1+FR2, Tprocessing = 40 ms.

Issue 3-1-2: TUE_preparation in Conditional PSCell change delay For information (In clause 8.11B) TUE_preparation is the UE preparation time for conditional PSCell change, and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined. TUE_preparation is up to 10ms.. - Option 1(Ericsson): preparation time TUE_preparation is changed from 10 to 8ms. - Option 2 (Qualcomm, Apple, MTK, Huawei, vivo): preparation time TUE_preparation shall be 10ms. Issue 3-2-3: TUE_preparation in conditional PSCell addition delay - Option 1(Ericsson): preparation time TUE_preparation is changed from 10ms to 8ms. - Option 2 (Qualcomm, Apple, MTK, Huawei, vivo): preparation time TUE_preparation shall be 10ms.

TUE_preparation is the UE preparation time for conditional PSCell change, and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined. TUE_preparation is up to 10ms.

As agreed the existing 2ms margin shall not be removed, we do not see the urgency to reduce the existing UE preparation time as well. We suggest to keep the current TUE_preparation up to 10ms.

Proposal 2: Keep the current TUE_preparation up to 10ms.

- Conclusion

In this contribution, we further discuss the remaining issues on conditional PSCell change and addition. After discussion the following conclusions are provided:

Proposal 1: For EN-DC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2;

For NR-DC FR1+FR2, Tprocessing = 40 ms.

Proposal 2: Keep the current TUE_preparation up to 10ms.

- References

- R4-2120344, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei, HiSilicon