TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2117058

Electronic Meeting, November 01-12, 2021

Agenda Item:	10.8.1

Source: 	Puloli, Baicells

Title: 	TP to TR 36.779: Channel numbering

Document for:	Approval

# Introduction

This document provides text proposal for section 9 of TR 36.779 on channel numbering.

# Test proposal

# <<< start of TP>>>

# 9	Channel numbering for E-UTRA, MSR

The E-UTRA Absolute Radio Frequency Channel Numbers (EARFCNs) allocated for Band [100] are outlined in Table 9-1.

Table 9-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 88 | 422 | 70596 | 70596 - 70645 | 412 | 134232 | 134232 - 134281 |
| [100] | 757 | 70646 | 70646 - 70655 | 787 | 134282 | 134282 - 134291 |

<<< end of TP>>>