3GPP TSG-RAN WG4 Meeting #101-e	R4-2117180

Electronic Meeting, 01st -12th November 2021

Source:	Verizon, Samsung

Title:	TP to TR 37.717-11-21: Addition of DC configurations for DC_2-66_n66-n77

Agenda item:	x.x.x

Document for:	Approval

# Background

This text proposal is a revision of R4-2016316 which was approved for TR 37.717-11-21 to add DC_2-66_n66-n77 configurations as defined in WID [1] and the modified text proposal [2]. The proposal introduces an uplink DC_2A_n66A and an additional downlink DC_2-2-66_n66-n77 configurations into the existing configuration.

Reference

[1] RP-211750:	 Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

[2] R4-2117177:	 A modified (R42016315) TP for TR 37.717-11-21 to include additional configurations

# Text Proposal

##### ---Start of changes---

## 7.74	DC_2-66_n66-n77

### 7.74.1	Operating bands for DC

Table 7.74.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.74.2		Configuration for DC

Table 7.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n66A-n77A DC_2A-66A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_66A_n77A | CA_2A-66A | CA_n66A-n77A |
| DC_2A-2A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_66A_n77A | CA_2A-2A-66A | CA_n66A-n77A |

### 7.74.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.74.4	∆TIB and ∆RIB values

For DC_2-66_n66-n77, the TIB,c and RIB,c values are derived from DC_2_n66-n78 and are given in the tables below.

Table 7.74.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.74.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.74.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

##### ---End of changes---