3GPP TSG-RAN WG4 Meeting #101e	R4-2117184

Electronic Meeting, November 01-12, 2021

Source:	China Telecom

Title:	Updated RAN4 RF work plan for NR coverage enhancements WI

Agenda Item:	8.18.1

Document for:	Information

Introduction

The RAN4 RF work on NR coverage enhancements WI was started from RAN4 #98e meeting in January 2021. Based on the open issues identified, this contribution provides a work plan for the future meetings.

In addition, there are several LS exchange between RAN1 and RAN4, and many RAN4 internal agreements captured in the WF. To better track those agreements, the RAN1/4 LS and WF approved in previous meetings are summarized in the Annex of this contribution. The work plan has been updated to capture the content of the RAN1/4 LS and RAN4 WF approved in the recent meetings.

Work Plan

- RAN4 #100e (Aug 2021)

  - Non-zero un-scheduled gap in-between repetitions

    - Conclude on whether maximum X can be 14OS or 1ms for different SCS

    - Further discuss whether new RF requirements for UE during the gap are needed

  - Non-zero gap with other uplink transmissions

    - Further discuss and provide final view to RAN1

  - TA adjustment impact on phase continuity

    - Conclude and provide final view to RAN1

  - Tolerance of phase continuity and amplitude consistency

    - List the candidate values for the tolerances

  - DL slot(s) in-between repetition

    - Further discuss the feasibility

  - Maximum duration for joint channel estimation

    - Discuss and provide feedback to RAN1

- RAN4 #101e (Nov 2021)

  - Non-zero gap in-between repetitions

    - Further discuss and conclude on whether new RF requirements for UE during the gap are needed

  - TA adjustment impact on phase continuity

    - TBD pending on whether further work is needed based on RAN4 #100e agreement

  - Tolerance of phase continuity and amplitude consistency

    - Down-select the candidate values for the tolerances

  - DL slot(s) in-between repetition

    - Conclude on the feasibility

  - Maximum duration for joint channel estimation

    - Discuss and provide final view to RAN1

  - RAN4 spec impact and CR structure

    - Discuss and initial agreement

Note: RAN1 Rel-17 is scheduled to be frozen at Dec 2021.

- RAN4 #101(e)-bis (Jan 2022)

  - Tolerance of phase continuity and amplitude consistency

    - Conclude on the tolerances for the applicable scenarios

  - Further discuss all the remaining open issues

  - Endorse RAN4 CRs

- RAN4 #102(e) (Feb - Mar 2022)

  - Finalize all the technical issues

  - Agree on RAN4 CRs

References

- R4-2103341, Email discussion summary for [98e][155] NR_reply_LS_Part_2, RAN4 #98e, Jan - Feb 2021.

- R4-2105477, Email discussion summary for [98-bis-e][140] NR_cov_enh, RAN4 #98e-bis, Apr 2021.

- R4-2107957, Email discussion summary for [99-e][147] NR_cov_enh, RAN4 #99e, May 2021.

- Annex A: Summary of agreements in previous meetings

- A.1		List of RAN1 LS to RAN4

A.1.1  R1-2009784, LS on PUCCH and PUSCH repetition (RAN1 #103e, Oct - Nov 2020)

1. Overall Description:

In the study of coverage enhancement in Rel-17, DMRS bundling cross PUCCH or PUSCH is identified as a potential scheme to enhance PUCCH or PUSCH coverage. For RAN1 to further study the DMRS bundling scheme, RAN1 made the following agreement to ask feedback from RAN4 on the conditions for UE to keep phase continuity and same power level (with certain tolerance level) cross PUCCH or PUSCH repetitions.

Agreements: For DMRS bundling cross PUCCH or PUSCH repetitions, send an LS to RAN4 to ask the following

- Under what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions

  - Whether back-to-back PUCCH or PUSCH repetitions is one of the conditions required to keep phase continuity cross the repetitions

- Power control tolerance level cross PUCCH or PUSCH repetitions

2. Actions:

RAN1 respectfully asks RAN4 to provide answers to the following questions

- Question 1: Under what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions

- Question 2: Whether back-to-back PUCCH or PUSCH repetitions is one of the conditions required to keep phase continuity cross the repetitions

- Question 3: Under what conditions UE can meet the power control tolerance level cross PUCCH or PUSCH repetitions

A.1.2  R1-2104119, Reply on LS on PUCCH and PUSCH repetition (RAN1 #104e-bis, Apr 2021)

1. Overall Description:

In LS R1-2102298 (R4-2103393), RAN1 received the following question from RAN4.

Question from RAN4 to RAN1: For analysis for the amount of tolerable phase change between repetitions, RAN4 respectably asks RAN1 if RAN1 has specific scenario what RAN4 should focus in their study? (e.g contiguous/non-contiguous transmission, within one time slot or multiple time slots, TDD band or FDD band etc)

RAN1 has discussed the issues raised in this question and would like to provide the following answers to RAN4.

For PUSCH transmission, the following use cases are considered in RAN1. Among the following cases, RAN1 suggest RAN4 to deprioritize use case 5a for PUSCH transmission.

   Use case 1: back-to-back PUSCH transmissions within one slot.

   Use case 2: non-back-to-back PUSCH transmissions within one slot.

‐   Use case 2a: no uplink transmission in the middle of two PUSCH transmissions

‐   Use case 2b: other uplink transmissions in the middle of two PUSCH transmissions

   Use case 3: back-to-back PUSCH transmissions across consecutive slots.

   Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

‐   Use case 4a: no uplink transmission in the middle of two PUSCH transmissions

‐   Use case 4b: other uplink transmissions in the middle of two PUSCH transmissions

   Use case 5: PUSCH transmissions across non-consecutive slots.

‐   Use case 5a: no uplink transmission in the middle of two PUSCH transmissions

‐   Use case 5b: other uplink transmissions in the middle of two PUSCH transmissions

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

For PUCCH repetitions, the following use cases are considered in RAN1. Among the following cases, RAN1 suggest RAN4 to prioritize the study on use case 3, 4a, 4b, and 5b for PUCCH repetitions.

   Use case 1: back-to-back PUCCH repetitions within one slot.

   Use case 2: non-back-to-back PUCCH repetitions within one slot.

‐   Use case 2a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions

   Use case 3: back-to-back PUCCH repetitions across consecutive slots.

   Use case 4: non-back-to-back PUCCH repetitions across consecutive slots.

‐   Use 4a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use 4b: other uplink transmissions in the middle of two PUCCH repetitions

   Use case 5: PUCCH repetitions across non-consecutive slots.

‐   Use case 5a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use case 5b: other uplink transmissions in the middle of two PUCCH repetitions

Note: RAN1 assumes “back-to-back PUCCH repetitions” has zero gap in-between adjacent PUCCH repetitions.

Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

In additional, RAN1 also discussed a few more related aspects on PUCCH repetitions and PUSCH transmissions. RAN1 respectfully asks RAN4 to provide answers to the following questions.

Question 1: In addition to the conditions provided in R4-2103393, can RAN4 please confirm that “Applying the same TPMI precoder across PUSCH transmissions” is also a necessary condition to keep phase continuity across PUSCH transmissions?

Question 2: Whether “no TA adjustment in between PUCCH transmissions or PUSCH transmissions” is another necessary condition to keep phase continuity across PUCCH repetitions or PUSCH transmissions?

Question 3: There are two different interpretation in RAN1 regarding the “downlink reception” in “No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case” (in R4-2103393)

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE.

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE and/or downlink symbols without actual DL transmission from gNB to UE and/or no DL monitoring occasions configured.

Can RAN4 please confirm which interpretation is correct?

2. Actions:

RAN1 respectfully asks RAN4 to take the above information into account in their work on NR coverage enhancement.

RAN1 respectfully asks RAN4 to provide answers to the following questions.

Question 1: In addition to the conditions provided in R4-2103393, can RAN4 please confirm that “Applying the same TPMI precoder across PUSCH transmissions” is also a necessary condition to keep phase continuity across PUSCH transmissions?

Question 2: Whether “no TA adjustment in between PUCCH transmissions or PUSCH transmissions” is another necessary condition to keep phase continuity across PUCCH repetitions or PUSCH transmissions?

Question 3: There are two different interpretation in RAN1 regarding the “downlink reception” in “No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case” (in R4-2103393)

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE.

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE and/or downlink symbols without actual DL transmission from gNB to UE and/or no DL monitoring occasions configured.

Can RAN4 please confirm which interpretation is correct?

A.1.3  R1-2106212, LS on joint channel estimation for PUSCH and PUCCH (RAN1 #105e, May 2021)

1. Overall Description:

RAN1 has discussed the maximum duration for joint channel estimation for PUSCH and PUCCH in RAN1#105-e meeting. RAN1 respectfully asks RAN4 to provide answers to the following questions.

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

2. Actions:

To RAN WG4:

ACTION:

RAN1 respectfully asks RAN4 to provide answers to the following questions.

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

A.1.4  R1-2108458, Reply LS on PUCCH and PUSCH repetition (RAN1 #106-e, Aug 2021)

1. Overall Description:

In LS R1-2106423 (R4-2107880), RAN1 received the following question from RAN4.

RAN4 also asks RAN1 to provide more information on the scenario when “downlink reception” from UE point of view includes downlink symbols without actual DL transmission from gNB to UE and UE is not assumed to do DL monitoring?

RAN1 has discussed the issue raised in this question and would like to provide the following answers to RAN4.

In RAN1 understanding, regarding to the “downlink reception”, there are actually three scenarios:

- Scenario 1: downlink or flexible symbols with actual DL transmission from gNB to UE, with/without DL monitoring occasion configured

- Scenario 2: downlink or flexible symbols without actual DL transmission from gNB to UE, but with DL monitoring occasion configured

- Scenario 3: downlink or flexible symbols without DL monitoring occasion configured

For scenario 1, one example is that PDSCH is actually transmitted on DL symbols. For scenario 2, one example is that a PDCCH monitoring occasion is configured, but gNB does not send PDCCH actually on the PDCCH monitoring occasion. For scenario 3, one example is that some symbols are indicated (e.g., by tdd-UL-DL-ConfigurationCommon) as DL symbols, but neither PDCCH monitoring occasion is configured nor PDSCH is transmitted on those DL symbols.

RAN1 understand that, the “downlink reception” in RAN4 reply LS R4-2103393 covers scenario 1 and scenario 2 already. The question “whether ‘downlink reception’ include downlink symbols without actual DL transmission from gNB to UE and without DL monitoring” that RAN1 asking to RAN4 in R1-2104119 simply means the following

- In additional to scenario 1 and 2, does the “downlink reception” in RAN4 reply LS R4-2103393 (“No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case”) further include scenario 3?

Besides the above, RAN1 also like to clarify that any DL measurement that a UE needs to perform is also included in “downlink reception”.

2. Actions:

RAN1 respectfully asks RAN4 to take the above information into account in their work on NR coverage enhancement.

RAN1 respectfully asks RAN4 to provide answer to the following question.

- Question 1: In additional to scenario 1 and 2, does the “downlink reception” in RAN4 reply LS R4-2103393 (“No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case”) further include scenario 3?

A.1.5  R1-2110642, Reply LS on PUCCH and PUSCH repetition (RAN1 #106e-bis, Oct 2021)

1. Overall Description:

In R1-2108703(R4-2114991) “Reply LS on PUCCH and PUSCH transmissions”, RAN 4 asked the following question to RAN1:

- What are the consequences if phase continuity cannot be maintained in the case of UL transmissions from other signals/channels in the repetition gap?

The following is the answer from RAN 1 to RAN 4, regarding the above question,

- If phase continuity cannot be maintained in the case of UL transmissions of other signals/channels in the repetition gap, then DM-RS symbols transmitted before and after the transmission of such other signals/channels cannot be part of the same bundle from UE perspective. A new DMRS bundle may or may not start after the other UL signals/channel transmission in the repetition gap. Details are still under discussion in RAN1.

2. Actions:

RAN1 respectfully asks RAN4 to take the above information into account in their work on NR coverage enhancement.

- A.2		List of RAN4 reply LS to RAN1

A.2.1  R4-2103393, Reply on LS on PUCCH and PUSCH repetition (RAN4 #98e, Jan - Feb 2021)

1. Overall Description:

Ran4 has discussed the phase continuity and concluded that the cases where continuity is lost in repetitions.

Questions from RAN1 with answers from RAN4

- Question 1: Under what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions

- RAN4 Answer for question 1: If the following conditions are met

  - Modulation order does not change.

  - RB allocation in terms of length and frequency position should not be changed, and intra-slot and inter-slot frequency hopping is not enabled within a repetition bundle.

  - No change on transmission power level of its own CC, i.e., no change on the power control parameters specified in TS 38.213, and also when own CC is not impacted by other concurrent CC(s) that are configured for inter-band CA or DC for same UE with dynamic power sharing and no change in any configured CC s that are part of configured intra-band uplink CA or DC.

  - No UL beam switching for FR2 UE occurs

- Question 2: Whether back-to-back PUCCH or PUSCH repetitions is one of the conditions required to keep phase continuity cross the repetitions.

- RAN4 Answer for question 2:

  - For back-to-back transmissions with zero gap in-between adjacent transmissions, the conditions under Q1 need to be met to maintain phase continuity.

  - For non-back-to-back transmission with non-zero gap in-between adjacent transmissions, RAN4 concluded that at least following additional condition also need to be met in addition to the conditions under Q1:

    - No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case

  - In scenario of no more than X un-scheduled OFDM symbols in-between the PUSCH or PUCCH repetition (e.g., X = 0, 1, 2, …, 14), and scenario of other physical signals/channels in-between PUCCH or PUSCH repetitions from the UE perspective, e.g., SRS or PUCCH transmission in-between the PUSCH repetition for the UE, RAN4 is still discussing if X can be non-zero value and UE can maintain phase continuity

- Question 3: Under what conditions UE can meet the power control tolerance level cross PUCCH or PUSCH repetitions

- RAN4 answer 3: If the conditions for phase continuity cross PUSCH or PUCCH repetitions are fulfilled, the same power level (with certain tolerance level) can also be achieved. The certain tolerance level is still under discussion in RAN4.

RAN4 has also discussed that in order to quantify the phase discontinuity tolerance, more understanding is needed how much phase can change between two transmissions and how long gap in time between two repetitions is possible. For this issue RAN4 would like to ask RAN1

Question from RAN4 to RAN1: For analysis for the amount of tolerable phase change between repetitions, RAN4 respectably asks RAN1 if RAN1 has specific scenario what RAN4 should focus in their study? (e.g contiguous/non-contiguous transmission, within one time slot or multiple time slots, TDD band or FDD band etc)

2. Actions:

RAN4 respectfully asks RAN1 to consider the answers in their work and RAN4 asks if RAN1 could reply to the one additional question related to the amount if phase change study:

Question 4: For analysis for the amount of tolerable phase change between repetitions, RAN4 respectably asks RAN1 if RAN1 has specific scenario what RAN4 should focus in their study?

A.2.2  R4-2105417, Reply LS on PUCCH and PUSCH repetition (RAN4 #98e-bis, Apr 2021)

1	Overall description

RAN4 has continued the discussion about the open items in previous reply on the same topic as highlighted below:

- In scenario of no more than X un-scheduled OFDM symbols in-between the PUSCH or PUCCH repetition (e.g., X = 0, 1, 2, …, 14), and scenario of other physical signals/channels in-between PUCCH or PUSCH repetitions from the UE perspective, e.g., SRS or PUCCH transmission in-between the PUSCH repetition for the UE, RAN4 is still discussing if X can be non-zero value and UE can maintain phase continuity

- RAN4 answer 3: If the conditions for phase continuity cross PUSCH or PUCCH repetitions are fulfilled, the same power level (with certain tolerance level) can also be achieved. The certain tolerance level is still under discussion in RAN4.

RAN4 confirms the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap less than 14 symbols when UE is not required to meet the existing off power requirements. Whether new or existing off power requirements for shorter duration than 1 msec as well as the maximum value of X un-scheduled symbols will be introduced are pending on further RAN4 discussions. For the case with other UL channels in between repetitions, at least if the other scheduled signals/channels during the non-zero gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels, it is feasible to maintain the phase continuity and power consistency across the repetitions.

For the phase tolerance level, RAN4 is planning to perform further studies in following meetings.

2	Actions

To RAN1

ACTION: 	To RAN1

RAN4 respectfully asks RAN1 to the above information in account in their work

A.2.3  R4-2107880, Reply LS on PUCCH and PUSCH repetition (RAN4 #99e, May 2021)

1	Overall description

RAN4 has continued discussing the un-scheduled gap consisting of unscheduled symbols between two PUCCH repetitions or PUSCH transmissions and reached a conclusion that it is feasible for UE to maintain phase continuity when the gap is 13 symbols or less. RAN4 is still discussing the feasibility of 14 symbols or 1 ms for different SCSs for the un-scheduled gap. Main drawback RAN4 sees with long gaps is UE energy efficiency since it needs to maintain TX parts active but UE is not transmitting and the issue of existing OFF power requirements not being satisfied for less 1ms duration.

If new RF requirements for UE during the gap are needed, is under discussion in RAN4.

Regarding questions from RAN1 in R1-2104119

- RAN1 Question 1:

  - In addition to the conditions provided in R4-2103393, can RAN4 please confirm that “Applying the same TPMI precoder across PUSCH transmissions” is also a necessary condition to keep phase continuity across PUSCH transmissions?

RAN4 Answer is that applying the same TPMI precoder across PUSCH transmissions is necessary condition to apply joint channel estimation.

- RAN1 Question 2:

  - Whether “no TA adjustment in between PUCCH transmissions or PUSCH transmissions” is another necessary condition to keep phase continuity across PUCCH repetitions or PUSCH transmissions?

RAN4 Answer is that TA adjustment and UE uplink timing autonomous adjustments cause the phase to change. RAN4 is still investigating the full impacts of the detailed scenarios, and will provide a final view about this at the next RAN4 meeting.

- RAN1 Question 3:

  - There are two different interpretation in RAN1 regarding the “downlink reception” in “No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case” (in R4-2103393)

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE.

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE and/or downlink symbols without actual DL transmission from gNB to UE and/or no DL monitoring occasions configured.

Can RAN4 please confirm which interpretation is correct?

RAN4 Answer is that

- The “downlink reception” means downlink symbols with actual DL transmission from gNB to UE and/or DL monitoring with the assumption that UE is receiving information.

- Regarding whether “downlink reception” include downlink symbols without actual DL transmission from gNB to UE and without DL monitoring, it would be helpful if RAN1 could provide more information on the exact scenario.

- Phase discontinuity tolerance LLS is ongoing in RAN4 study and conditions of whether the phase continuity can be maintained in TDD case that has downlink reception in-between the PUSCH or PUCCH repetition could be revisited in future meeting with consideration of phase discontinuity tolerance.  RAN4 is also still checking whether there are any optional UE antenna configurations where a UE could overcome this problem and still gain from using the feature

2	Actions

To RAN1

ACTION:

RAN4 asks RAN1 to take above information in to account in their work.

RAN4 also asks RAN1 to provide more information on the scenario when “downlink reception” from UE point of view includes downlink symbols without actual DL transmission from gNB to UE and UE is not assumed to do DL monitoring?

A.2.4  R4-2114991, Reply LS on PUCCH and PUSCH transmissions (RAN4 #100e, Aug 2021)

1. Overall Description:

RAN1 question: For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

RAN4 answer: Yes, there is a maximum duration but RAN4 has not agreed how many slots it is.

- RAN1 question: What factors determine the maximum duration?

  - RAN4 answer: RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation. RAN4 is still investigating other factors impact  in more detail.

- RAN1 question: Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - RAN4 answer: Yes

- RAN1 question: Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - RAN4 answer: Considering the scenario of coverage extension, RAN4 recommends to only focus on modulation orders not higher than QPSK, i.e., focus on QPSK (PUCCH and PUSCH), Pi/2 BPSK (PUCCH and PUSCH), BPSK (PUCCH). RAN4 is still discussing whether maximum duration depends on modulation order for the above modulation schemes.

- RAN1 question: Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - RAN4 answer: No

- RAN1 question: Whether the maximum duration is band specific?

  - RAN4 answer: It may be FR dependent, and RAN4 is still discussing whether it is band dependent as well.

- RAN1 question: Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

  - RAN4 answer: Still under discussion in RAN4.

RAN4 has further agreed for the gap between PUSCH/PUCCH transmissions, that the 13-symbol is the maximum length for the gap for all SCS, and that the 14-symbol or 1ms will not be discussed in RAN4 anymore for un-scheduled gap in Rel-17.

RAN4 has agreed for the case of other signals/channels in the gap between repetitions, it is not considered for UE to transmit other channels in the gap with different settings.

For the case of other UL signals/channels in the gap between repetitions with same settings, as communicated in R4-2105417, RAN4 has further refined the conditions when phase continuity can be met as follows:

- Signals/channels with repetitions and other UL signals/channels in the gap have the same:

  - PAPR and average power, e.g., PUSCH/PUCCH part of repetitions and SRS has same PAPR and average power.

  - Allocated number and locations of PRBs transmitted

  - Antenna port settings

RAN4 has not agreed detailed requirement for phase continuity and plans to revisit the above agreement in the scenario of other UL signals/channels in the gap once the requirement is defined. Therefore, RAN4 would like to ask RAN1 what are the consequences if phase continuity cannot be maintained in that scenario?

2. Actions:

To RAN1 group.

ACTION:   RAN4 would like to ask RAN1 what are the consequences if phase continuity cannot be maintained in the case of UL transmissions from other signals/channels in the repetition gap?

- A.3		List of RAN4 WF

A.3.1  R4-2103363, WF on phase continuity and power consistency for PUCCH and PUSCH repetition (RAN4 #98e, Jan - Feb 2021)

A.3.2  R4-2105418, WF on phase continuity and power consistency for PUCCH and PUSCH repetition (RAN4 #98e-bis, Apr 2021)

A.3.3  R4-2107881, WF on phase continuity and power consistency for PUCCH and PUSCH repetition (RAN4 #99e, May 2021)

A.3.4  R4-2114992, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions (RAN4 #100e, Aug 2021)

Sub-topic 1-1: Non-zero un-scheduled gap in-between transmissions

Issue 1-1-2: RF requirements for the non-scheduled gap

Agreement in GTW session:

Send LS to RAN1 to explain that the 13-symbol is the maximum length and that the 14-symbol or 1ms will not be discussed in RAN4 anymore for un-scheduled gap in Rel-17.

- To clarify work in RAN4, off-power requirement can be further discussed for up to 13 symbols.

- On off power less than 1ms gap

  - Option 1: Define new transmit off power for gap symbols explicitly for Rel-17 coverage enhancement case

    - Alt.1: Define a relaxed transmit off power due to shorter measurement period

    - Alt.2: Carrier leakage requirement applies for the non-scheduled gap

      - FFS how to define carrier leakage requirement with no signal transmitted during the gap

  - Option 2: RAN4 do not introduce new transmit off power

    - i.e. no requirement applies during the gap

  - Option 3: The existing OFF power level of -50dBm apply for less than 1 ms

    - FFS whether to and how to introduce measurement uncertainty

Sub-topic 1-2: Non-zero gap with other uplink transmissions

Issue 1-2-1: Non-zero gap with other uplink transmissions for the UE

Agreement in GTW:

- Scenario 2 is not considered.

- For scenario 1, there is no guard period on the condition that

  - Signals/channels with repetitions and other signals/channels in the gap have the same PAPR and AVG power, e.g., PUSCH/PUCCH part of repetitions and SRS has same PAPR and AVG power.

  - The same PRB location and PRB size for signals/channels with repetitions and other signals/channels in the gap

  - Signals/channels with repetitions and other signals/channels in the gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels

- Re-visit the above conclusions after RAN4 finalizes the phase continuity tolerance requirement.

Sub-topic 1-3: TA adjustment impact on phase continuity

Issue 1-3-1: For network commanded TA adjustment

Agreement in GTW: TA adjustments should be avoided across the PUSCH/PUCCH transmissions (i.e., from starting the first transmission until the end of repetition) for joint channel estimation.

Issue 1-3-2: For UE autonomous adjustment

- Option 1: UE autonomous adjustment is not expected across the repetitions

  - Check with RAN1 if there is RAN1 spec impact before concluded with option 1

- Option 2: Up to UE implementation

  - UE should maintain within phase tolerance even when UE autonomous adjustment is allowed but not (pre-)compensated

- Option 3: The corresponding phase change can be pre-compensated at UE baseband processing, or estimated and compensated at BS baseband processing.

  - Check feasibility of (pre-)compensation at UE/BS baseband processing

- Down select among above options

Sub-topic 1-4: Phase continuity and power consistency tolerance

Issue 1-4-1: Phase continuity tolerance

- On phase continuity tolerance initial thoughts based on simulation results

  - 1: within 40 degrees based on simulation for PUCCH

  - 2: within 30 degrees based on simulation for PUSCH QPSK

  - 3:

    - For PUSCH QPSK, in the order of 20 degrees for 8 repetitions

    - For PUCCH format 3, 40 degree’s STD for 2 repetition and 20 degree’s STD for 8 repetition

  - Align on simulation assumption, decide the phase tolerance in next RAN4 meeting, key factors for alignment are as follow

    - Physical channel: PUSCH and PUCCH

    - Repetition number: 2/8

    - Model of phase variation: in issue 1-4-2

    - DMRS configuration per slot

    - Modulation order: focus on Pi/2 BPSK (PUCCH/PUSCH), QPSK (PUCCH/PUSCH), BPSK (PUCCH)

Issue 1-4-2: Model of phase variation

- Down select Model of explicit phase offset between option 1 and option 2

  - Option 1: Gaussian, std deviation

  - Option 2: uniform distribution

- Definition of the offset

  - For each individual slot (0…n) within the bundle, an independent offset is generated and applied.

Issue 1-4-3: Definition of phase continuity tolerance

- Issue A: Definition for the requirements and reference point

- Issue B: Definition of measurement point

- For Issue A and Issue B, encourage companies to further discuss in the next meeting, taking into account the inputs in this meeting.

Issue 1-4-4: Power consistency tolerance

- Proposed power variation based on simulation results

  - Option 1: 2 dB power variation with uniform distribution

  - Option 2: 1 dB fixed power offset (0.5 dB amplitude offset)

  - Option 3: Further study

  - Option 4: Reuse existing power consistency requirements defined in RAN4 spec.

- Focus on Pi/2 BPSK (PUCCH/PUSCH), QPSK (PUCCH/PUSCH), BPSK (PUCCH).

- Further discuss the model of power variation if existing power consistency requirements cannot be reused :

  - Option 1: uniform distribution

  - Option 2: fixed offset

Issue 1-4-5: Impact from frequency offset

- CFO/frequency offset compensation at the receiver is considered assuming

  - 1: constant CFO between transmission is assumed at UE

  - 2: full compensation of CFO assumed or leftover phase offset after compensation is assumed

Needs further discussion on whether the above points 1 and 2 can actually be assumed.

Sub-topic 1-5: Maximum duration for joint channel estimation

Issue 1-5-1: For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level?

Agreement in GTW: For joint channel estimation, there is a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level

Issue 1-5-2: If there is a maximum duration, how long is it?

- Option 1: The maximum duration should depend on the interval where the UE does not make frequency adjustment with DL signal

- Alt 1: at least smaller/equal than the configured SSB periodicity

- Alt 2: other DL RS periodicity

- Option 2: Depends on JCE performance considering the phase tolerance and/or gNB frequency offset compensation accuracy during the duration even within a sync periodicity.

- Option 3: Maximum duration length depends on both option 1 and option 2

Issue 1-5-3: What factors determine the maximum duration?

Further discuss on following proposals, and other factor is not precluded

- Proposal 1: Energy efficiency and thermal changes

- Proposal 2: The maximum time the UE not adjusting its frequency/time.

- Proposal 3: Phase tolerance within the duration

- Proposal 4: Factors related to the leftover frequency offset across slots, e.g., channel BW

- Proposal 5: Whether to configure the PT-RS

- Proposal 6: Clock stability and PA behavior

Issue 1-5-4: Whether the maximum duration is dependent on the modulation order of transmission?

For modulation orders not higher than QPSK, further discuss whether the maximum duration is dependent on the modulation order of transmission or not?

Issue 1-5-6: Whether the maximum duration is band specific?

FFS on whether maximum duration is FR(frequency range) specific, and/or band specific

Issue 1-5-7: Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

FFS with following options:

- Option 1: Yes

- Option 2: Subject to a single maximum duration

- Option 3: Needs further discussion

Sub-topic 1-6: DL slot(s) in-between repetition

Issue 1-6: DL slot(s) in-between repetition

- For the case of “with DL reception (including monitoring and/or measurements)”:

- Alt 1: This case will not be discussed in RAN4 anymore in Rel-17, FFS for future release

- Alt 2: other method

  - UE can retune the phase with a gap period to maintain the phase continuity (R4-2112889)

  - UE can use separate Tx/Rx antennas to maintain the phase continuity (R4-2112889)

  - UE can meet the to-be-defined phase/amplitude discontinuity tolerance requirement with some design

- For the case of “without actual DL transmission from gNB to UE and without DL monitoring”:

- Hold on the discussion till we receive the response from RAN1.