3GPP TSG-RAN WG4 Meeting #101-e	R4-2117268

Electronic Meeting, 1st – 12th November 2021

Source:	AT&T

Title:	TP for TR 37.827 Addition of DC_12-30_n77

Agenda item:			7.32.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.827 to add DC_12-30_n77.

# 2. Text Proposal

---Start of changes---

## 5.X	DC_12-30_n77

### 5.X.1	Transmitter Characteristics

#### 5.X.1.1	Maximum Output Power

Table 5.X.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n77A | 266 | +2/-31 |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.X.1.2	Configurations for EN-DC

Table 5.X.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-30A_n77A | DC_12A_n77A DC_30A_n77A |

#### 5.X.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 12 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 might fall in Rx of band 12.

### 5.X.2	Receiver Characteristics

#### 5.X.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.X.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_12A-30A_n77A are defined in Table 5.X.2.1.1-1.

Table 5.X.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-30A_n77A | 12 | 710 | 5 | 25 | 740 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.X.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_12A-30A_n77A are the same as the Case A defined in Table 5.X.2.1.1-1.

---End of changes---

# References