3GPP TSG-RAN WG4 Meeting #101-e	R4-2117303

Electronic Meeting, Nov. 1 – Nov. 12, 2021

Agenda Item:	8.9.4.5

Source:	Apple

Title:	Discussion on the signaling characteristic requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 100-e.

In this paper, we discuss the signaling characteristic requirement.

# 2	Discussion

RLM/BFD and link recovery

In RAN4 100e, the following agreements have been captured in [3] on RLM/BFD and link recovery.

- RLM/BFD:

- The sharing factor P and PCBD, current specification can be reused for FR2 HST

- RAN4 will further study the time instances for measurement for RLM/CBD evaluation period for short DRX case

RLM/BFD requirement depends on whether SSB or CSI-RS is used for RLM/BFD measurement. For CSI-RS based RLM/BFD, current R15/R16 requirement can be reused for FR2 HST. For SSB based RLM/BFD, depends on the number of Rx beam discussion, the requirement can be enhanced correspondingly.

For CBD/Link recovery: To enable faster CBD and link recovery, similar to RLM/BFD, the number of Rx beam scaling factor can be used for SSB or CSI-RS based CBD requirement. The scaling factor N can follow the general discussion on Rx beam value based on deployment scenario evaluation.

Proposal 1: Enhanced RLM/BFD/CBD requirement with scaling Rx beam factor, where the number of Rx beam in general RRM requirement discussion.

TCI state switching delay

The following TCI state switching delay discussion have been captured in [3].

- RAN4 will further study 1280ms duration for known condition

- RAN4 will further study the TCI switching delay requirements

  - Option 1: Reuse the existing TCI switching delay requirements for known condition

  - Option 2: NW triggered TCI switching to avoid sharp SNR drop

- Further enhancement on TCI switching delay based on Rel-17 TCI design can be discussed in FeMIMO WI.

Current known state is defined as “TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement”. With 350km/hour, within 1280ms, the movement can be 124.4m. Beam overlapping range highly depends on the choice of uni-directional or bi-directional deployment.

In deployment evaluation study, for bi-directional deployment, it has been agreed in [4] that “Introducing performance requirements for both uni-directional and bi-directional deployment in scenario B”. For scenario B, scheme 1 under bi-directional scenario is feasible without coverage hole issue, and no propagation delay jump between switching points. However in scheme-2, both large delay jump and coverage hole issue has been identified and the beam overlapping in scheme-2 is quite limited, requires much smaller value than 1280ms to define known state.

Proposal 2: 1280ms duration for TCI known condition can be reduced.

Proposal 3: Reuse the existing TCI switching delay requirements for known condition.

# 3	Summary

In this paper, we provide our view on signalling aspect of RRM enhancement for HST FR2.

Proposal 1: Enhanced RLM/BFD/CBD requirement with scaling Rx beam factor, where the number of Rx beam in general RRM requirement discussion.

Proposal 2: 1280ms duration for TCI known condition can be reduced.

Proposal 3: Reuse the existing TCI switching delay requirements for known condition.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2115334, Way forward for FR2 HST RRM (part 1), Nokia, Aug 2021

- R4-2115335, Way forward for FR2 HST RRM part 2, Samsung, Aug 2021

- R4-2115725, WF on remaining issues on FR2 HST deployment scenario and channel modeling, Samsung, Aug