3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117319

Electronic Meeting, November 01-12, 2021

Title: 	Discussion on NR repeater radiated output power requirement

Source: 	CATT

Agenda item:	8.5.3.1

Document for:	Approval

- Introduction

This contribution provides our views on the open issues for NR repeater radiated output power requirement including UL output power level, UL power accuracy and ALC.

- Discussion

  - UL output power level

The last meeting’s status is as following,

Radiated UL transmission there are 2 power classes one without a power limit and 1 with a power limit. With the following options on the power limit

- The power limit uses the same limit as PC1.

- Other options not precluded.

For the power limit, our understanding is that using UE PC1 may be a straightforward solution. There’s another issue should be clarified that if all of the UE PC1 requirement should be reused. In our understanding, EIRP and TRP upper limit can be reused but there’s no spherical coverage requirement for repeater.

Proposal 1: UE mmWave PC1 EIRP and TRP upper limit are reused for mmWave NR repeater. There’s no spherical coverage requirement for repeater.

  - UL power accuracy

For UL power accuracy, we have the same proposal as conducted requirement that

Proposal 2: UL use the same power accuracy as DL.

  - ALC

For ALC, we think 10 dB as conducted may be ok. For the requirement, we proposed output power and emission requirement for conducted but we slightly prefer only output power is tested for mmWave repeater to consider is the test burden for the mmWave equipment.

Proposal 3: one 10 dB higher level for ALC is defined and only output power is tested for mmWave ALC requirement.

- Conclusion

This contribution provides our views on the open issues for NR repeater radiated output power requirement. We have the following proposal,

Proposal 1: UE mmWave PC1 EIRP and TRP upper limit are reused for mmWave NR repeater. There’s no spherical coverage requirement for repeater.

Proposal 2: UL use the same power accuracy as DL.

Proposal 3: one 10 dB higher level for ALC is defined and only output power is tested for mmWave ALC requirement.

Reference

[1] R4-2115782, “Email discussion summary for [100-e][310] NR_Repeater_RF_Part2”, Huawei

[2] R4-2115722, “WF on NR Repeater FR2 RF”, Huawei