3GPP TSG- RAN WG4 Meeting #101-e	R4-2117331

# Electronic Meeting, Nov. 1-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for PUCCH SCell activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements for PUCCH SCell activation/deactivation should be introduced in RRM specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the requirements for PUCCH SCell activation/deactivation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for PUCCH SCell activation/deactivation will be not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 8.3.12, new 8.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

8.3.12	PUCCH SCell activation delay Requirement

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

If the UE has a valid TA for transmitting on the SCell then the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+Tactivate_basic: upon receiving SCell activation command in slot n.

where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	Tactivate_basic = THARQ+Tactivation_time+TCSI_ReportingNR slot length is the SCell activation delay as defined in section 8.3.2.

If the UE does not have a valid TA for transmitting on the SCell, then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in subframe n+THARQ+Tactivation_time and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell and shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell, where:

Tdelay_PUCCH SCell = THARQ+Tactivation_time+TPDCCH_order+ T1 + T2 + T3+ TCSI_Reporting

Where:

-	TPDCCH_order is delay for PDCCH order sent by network to trigger PRACH from n+THARQ+Tactivation_time.

-	T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to 10 ms and the actual value of T1 shall depend upon the PRACH configuration used in the PUCCH SCell.

-	T2 is the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs. T2 is up to 13 ms.

-	T3 is the delay for applying the received TA for upling transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

-	TCSI_Reporting is UE wait the uplink resource for CSI report after Msg3.

The above delay requirement (Tdelay_PUCCH SCell) shall apply provided that:

-	The SCell being activated is a known cell, or a unknown cell and there is at least one contiguous active serving cell on same FR1 or FR2 band; and

-	The PDCCH order to initiate RA procedure on the PUCCH SCell is not earler than n+THARQ+Tactivation_time; and

-	The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay (Tdelay_PUCCH SCell) can be extended.

-	The target PUCCH Scell RACH has same SCS from spCell data/control channel, or has different SCS from spCell data/control channel and UE supports diffNumerologyAcrossPUCCH-Group.

-	no interruption occurs on the PUCCH SCell activation procedure made by action on other cell(s) if UE does not support per-FR MG, or by action on other cell(s) in same FR as the target PUCCH SCell if UE support per-FR MG, otherwise the delay requirements will be extended.

The interruption on the PCell specified in section 8.2.2 shall meet all applicable requirements in clause 8.3.2.

8.3.13	PUCCH SCell deactivation delay Requirement

The requirements in this section shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

The UE shall deactivate a SCell configured with PUCCH and meet the SCell deactivation delay requirements specified in section 8.3.3.

The interruption on the PCell specified in section 8.2.2 shall meet all applicable requirements in clause 8.3.3.