3GPP TSG-RAN WG4 Meeting #101-e	R4-2117333

Electronic Meeting, Nov. 1-12, 2021

Title: 	Discussion on UE Rx RF requirements for NTN

Source: 	CATT

Agenda item:	8.13.4.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the Rx requirements for NTN UE were discussed, and a way forward was approved [1]. The following points were agreed:

- Current requirement for out-of band blocking can be reused.

- Current requirement for spurious response can be reused.

- 补充一下open issue的bullet

This document will further discuss the open issue for NTN UE Rx RF requirements.

- Discussion

For NTN UE Rx RF requirements, the most important work is to determine reference sensitivity requirement and the reference measurement channel at first. The reference measurement channel will be defined based on channel coding and TBS which will be reused from Rel-15 design in RAN1. The problem is whether lower SNR operating point needs to be defined for NTN due to very weak receiving signal. If yes, then a lower MCS and new FRC need to be defined. As a general principle, the option 2 in WF [1] can be agreed as starting point. We think diversity gain, NF and IM can be reused for S band.

Proposal 1: The option 2 for reference sensitivity in WF [1] can be agreed as start point, i.e. Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain can be reused to specify the REFSENS.

- Diversity gain, NF and IM can be reused for S band. FFS on FRC and SNR

For maximum input level requirement, it is defined up to -25dBm for TN. This value is determined by the minimum coupling loss between BS and UE. In NTN system, The MCL between satellite and UE will be very large than TN system. So maximum input level requirements for NTN UE should be re-evaluated. Of course, reusing the current requirement is ok for us.

Proposal 2: The maximum input level requirements for NTN UE can be relaxed XdB based MCL increase, where X needs further discussion to decide.

It is FFS for intermodulation characteristics requirements in WF [1]. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to and close to the wanted signal frequency. The interfering signals will also be about 40dB bigger than wanted signal in NR UE requirements. In NTN network, these scenarios are very rare. So we think this requirement is meaningless for NTN system, and can be omitted.

Proposal 3: Intermodulation characteristics requirements can be omitted for NTN.

For RX spurious emissions, it is agreed in WF [1] that use the 38.101-1 requirement as the starting point , and further check whether NTN UE would be a satellite device and the compliance to regulatory, e.g. DRC 74-01. The FR1 NTN UE transmits signals with omnidirectional antenna. Its transmission will interfere other device on aground same as NR UE. The requirement of spurious emission should also be met by NTN UE for defend other system. So we think spurious emission requirement for NR UE should be reused for NTN FR1 UE.

Proposal 4: The spurious emission requirement for NR UE should be reused for NTN FR1 UE.

- Conclusion

This document discussed Rx RF requirements for NTN UE and concluded the following.

Proposal 1: The option 2 for reference sensitivity in WF [1] can be agreed as start point, i.e. Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain can be reused to specify the REFSENS.

- Diversity gain, NF and IM can be reused for S band. FFS on FRC and SNR

Proposal 2: The maximum input level requirements for NTN UE can be relaxed XdB based MCL increase,

where X needs further discussion to decide.

Proposal 3: Intermodulation characteristics requirements can be omitted for NTN.

Proposal 4: The spurious emission requirement for NR UE should be reused for NTN FR1 UE.

- Reference

[1]	R4-2115642, WF on NTN UE requirements, Huawei