3GPP TSG-RAN WG4 Meeting # 101-e		R4-2117367

Electronic Meeting, November 1-12, 2021

Agenda item:	8.9.4.4

Source: 	CATT

Title: 	Further discussion on timing requirements for FR2 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR2 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR2 HST was approved [1] in RAN4#100-e meeting. Although some topics were agreed, there are still some open issues.

In this contribution, we provide our consideration of several open issues for timing requirements and give our proposals.

Discussion

- Scheduling restriction

For scheduling restriction, it is listed as below in the WF [1]:

Scheduling restriction - Study the impact of timing different between beams on measurement and demod to decide whether scheduling restrictions shall apply for one symbol before and one symbol after resources (SSB, CSI-RS etc) used for L1-RSRP measurements.

When L1-RSRP measurement based on SSB or CSI-RS is applying, there is large propagation larger than CP in 120 kHz or 240 kHz SCS. For better performance of L1-RSRP measurement with no ISI, it should apply the scheduling restriction for one symbol before and one symbol after resources used for L1-RSRP measurements.

Proposal 1: Define scheduling restriction for one symbol before and one symbol after resources used for L1-RSRP measurements.

Conclusion

In this contribution, we provide the further consideration of several open issues for timing requirements and present our proposals as below:

Proposal 1: Define scheduling restriction for one symbol before and one symbol after resources used for L1-RSRP measurements.

Reference

[1] R4-2115335, WF on FR2 HST RRM (part 2), Samsung