3GPP TSG-RAN WG4 Meeting #101-e                                                             R4-2117410

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	Discussion on UL TA demodulation requirements for FR2 HST

Agenda item:	8.9.5.3.2

Document for:	Approval

- Introduction

In RAN4#100-e, a way forward on FR2 HST demodulation was approved [1]. The remaining issues in WF concerning UL TA demodulation such as test Parameters for timing offset need to be further discussed.

In this contribution, we provide our views on these issues for UL TA demodulation FR2 HST.

- Discussion

Test Parameters for timing offset

As per the WF [1], the remaining issues concerning test parameters for timing offset are shown as below:

- Test Parameters for timing offset - Option 1: Use A= 1.25 us, Δω = 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements - Other options are not preluded

For FR1 HST, the timing offsets for scenario X, Y and Z defined in TS 38.104 are shown bellows:

(G.4-1) Table G.4-1: Parameters for UL timing adjustment Parameter; Scenario X; Scenario Y; Scenario Z. Channel model; Stationary UE: AWGN Moving UE: TDLC300-400; Stationary UE: AWGN Moving UE: AWGN; Stationary UE: AWGN Moving UE: AWGN. UE speed; 120 km/h; 350 km/h; 500 km/h. CP length; Normal; Normal; Normal. A; 15 kHz: 10 s 30 kHz: 5 s; 15 kHz: 10 s 30 kHz: 5 s; 15 kHz: 10 s 30 kHz: 5 s. ; 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1; 15 kHz: 0.13 s-1 30 kHz: 0.26 s-1; 15 kHz: 0.18 s-1 30 kHz: 0.36 s-1. NOTE 1: Doppler shift is not taken into account in UL TA scenario Y and Z.

As seen from above parameters for FR1 HST, the A is dependent on SCS. So for 120 kHz SCS, A equals 10us/8 =1,25us. And is dependent on UE speed and SCS, for 350km/h, the for 120 kHz SCS equals 0.13 s-1*8 =1.04 s-1. So we think option 1 is reasonable.

Proposal 1: To adopt A= 1.25 us, Δω = 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements (Option 1).

- Conclusion

This contribution provides analysis on UL TA demodulation requirements for FR2 HST. The following proposals are concluded:

Proposal 1: To adopt A= 1.25 us, Δω = 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements (Option 1).

- References

- R4-2108667, Way forward on FR2 HST demodulation, Samsung, RAN4#100-e