3GPP TSG-RAN4 Meeting #101-e	R4-2117418

# Electronic Meeting, Nov. 1-12, 2021

Agenda item:	8.16.7.2

Source:	Apple

Title:   	UE transmit timing for NR operation in 52.6GHz - 71GHz

Document for:	Discussion

# Introduction

RAN4 RRM on the WI “Extending current NR operation to 71GHz” has started. In particular, the following RAN4 impact is identified in the WID [1]:

- Core specifications for UE, gNB and RRM requirements [RAN4]:

  - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range.

  - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1).

  - Specify RRM/RLM/BM core requirements.

At RAN#92-e, further updates were made to the WID. As a result, the following SCS is supported in the WI:

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz for data and control channels and reference signals.

- In addition to 120kHz, 480 kHz SSB is supported for initial access.

- Specify 480kHz and 960kHz SCS for SSB for cases other than initial access.

In RAN4 meeting#100-e, UE transmit timing was discussed and the agreements were captured in the WF [2], including defining new RRM requirements due to higher data/SSB SCS. In this contribution, we focus on UE transmit timing.

# Discussion

  - Initial transmit timing error

As discussed, UE initial transmit timing error Te mainly depends on the following considerations [3]:

- DL timing estimation error, which depends on the DL bandwidth of reference signal that is used for DL timing estimation

- UL timing granularity, depending on UL channel bandwidth (CBW)

- The UL SCS used. More specifically, the timing error should not take too much portion of CP length, considering other impairments such as channel delay spread, UL TA command granularity, etc.

Based on the technical points raised in [3], the following was captured in the WF [2]:

Below we discuss those points in order.

Percentage of UL CP length

As different UEs may have different timing errors, the worst-case scenario is that there are two UEs that have the same amount of Te in plus and minus direction. In this case, if Te is more than 50% of CP, no matter how BS opens its FFT window, the BS demodulation performance of one UE will suffer. Note that in this analysis, other impairments such as delay spread are not even considered. Therefore, 50% of CP is the upper limit of Te.

UE implementation

The other factor to consider is UE implementation. Some timing error components in implementation are bandwidth-dependent and others are not. In other words, one cannot expect timing error will decrease proportionately as the SCS scales up. In existing requirements, when SSB SCS is 120kHz and UL SCS is 120kHz, the implementation margin is about 2.5*64*Tc, which is more than half CP for 480kHz SCS or the entire CR for 960kHz SCS. It can be seen that even though we use 50% of CP as the upper limit of Te, the proposed Te values for 480/960kHz SCS as shown in Table 1 is smaller than the implementation margin of 2.5*64*Tc, highlighting the challenges in implementing the higher SCS.

Combinations of SSB SCS and UL signal SCS

While it is always difficult to discuss the use cases, which may lead to the different combinations of SSB SCS and UL signal SCS, we believe RAN4 can prioritize the cases with the same SCS for SSB and UL signal, for the following reasons:

- The support of mixed numerology at the UE is a UE capability and many UEs choose not to support it because of implementation complexity. That is the reason that 480kHz SCS for SSB was added in addition to the 120kHz SCS for initial access.

- As discussed at RAN4#100-e, the combination of an SSB SCS with a larger UL signal SCS proves to be very challenging from implementation’s point of view.

Proposal 1: When considering the upper limit of Te, 50% of UL CP length is used.

Proposal 2: For R17, RAN4 specifies Te requirements for the combinations of same SCS for SSB and UL signal, i.e., (120kHz, 120kHz), (480kHz, 480kHz), (960kHz, 960kHz).

Based on the above proposals, the initial timing error Te is proposed in Table 1 below:

Table 1: Proposed Te for 120/480/960kHz SCS

| SCS of SSB (kHz) | SCS of UL signal (kHz) | Proposed Te | Percent of UL CP length |
| --- | --- | --- | --- |
| 120 | 120 | 3.5*64*Tc (reused from existing specs) | 19.7% |
| 480 | 480 | [2.25*64*Tc] | 50% |
| 960 | 960 | [1.125*64*Tc] | 50% |

Proposal 3: The initial timing error Te is defined as shown in Table 1.

In addition, currently the initial transmit timing accuracy test is defined in a deterministic manner. To further ease the burden on implementation, perhaps it is reasonable to define the initial transmit timing accuracy test for this band in a statistical manner, just like the test for timing advance adjustment accuracy, for which it is stated “The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%”.

Proposal 4: RAN4 considers defining the initial transmit timing accuracy test for 480/960kHz SCS in a statistical manner to ease the implementation burden.

  - Timing advance adjustment accuracy

RAN4#100-e, the timing advance adjustment accuracy was agreed in the table below, with a note “Revisit if certain implementation issues are identified.”

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±8 Tc] | [±4 Tc] |

In R15, the timing advance adjustment accuracy was decided based on the UL sampling rate corresponding to the minimum CBW for an SCS. And the values in the above table for 480/960kHz SCS are largely derived following this methodology. However, as the SCS increases, the limitations from implementation become dominant and must be considered as we are dealing with extremely tight accuracies.

Let us examine the proposed TA adjustment accuracy of 4*Tc for 960kHz, i.e., about 2ns, and understand what it entails for implementation. Since the TA adjustment accuracy is a relative accuracy, all the static timing error component are removed from consideration. Then the single biggest factor affecting the accuracy is time jitter of transmission, which results from clock crossings in the entire transmission path covering BBIC and RFIC. +/-2ns accuracy means that the allowed jitter of each transmission, i.e., before and after UE receives a TA command, is approximately in the order of +/-1ns. Clearly, the accuracy is very stringent and will have a huge impact on implementation.

Therefore, we propose the following values:

Table 2: Proposed UE timing advance adjustment accuracy for 480/960kHz SCS

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±16 Tc] | [±12 Tc] |

Note even for 12Tc, the accuracy is about 6ns, meaning the allowed jitter of each transmission is in the order of +/-3ns.

Proposal 5: The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in Table 2.

# Conclusion

In this contribution, we further discuss the UE UL transmit timing and timing advance adjustment accuracy for the 52.6GHz - 71GHz band. Based on our analysis, it is proposed:

Proposal 1: When considering the upper limit of Te, 50% of UL CP length is used.

Proposal 2: For R17, RAN4 specifies Te requirements for the combinations of same SCS for SSB and UL signal, i.e., (120kHz, 120kHz), (480kHz, 480kHz), (960kHz, 960kHz).

Proposal 3: The initial timing error Te is defined as shown in Table 1.

Proposal 4: RAN4 considers defining the initial transmit timing accuracy test for 480/960kHz SCS in a statistical manner to ease the implementation burden.

Proposal 5: The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in Table 2.

# References

- RP-211584, “Revised WID:  Extending current NR operation to 71 GHz”

- R4-2115351, “WF on R17 NR_ext_to_71GHz_RRM_1”, Qualcomm

- R4-2112135, “UE transmit timing for NR operation in 52.6GHz - 71GHz”, Apple