3GPP TSG-RAN4 Meeting #101-e	R4-2117449

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.20.3.1.2

Source:	Apple

Title:	Discussion on mobility requirement for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement impact for RedCap capability reduction has been discussed and the conclusions were captured in the WF[1].

In addition to the specification impact identified in [R4-2108359], following additional impact were identified: - Handover (section 6.1) - RRC re-establishment (6.2.1) - RRC Connection release with redirection (6.2.3) RRM impact due to maximum number of DL MIMO layers - No RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order.

Based on the last meeting agreements, there are three key features for complexity reduction which would impact the RAN4 RRM requirements,

- Reduced maximum UE bandwidth

- Reduced minimum number of Rx branches

- Duplex operation

In this contribution, we discuss the RRM mobility requirement for RedCap reduced capability.

# Mobility requirement for RedCap reduced capability

- Reduced maximum UE bandwidth

Reduced maximum UE bandwidth has no impact on UE handover requirement since HO mainly relies on SSB.

| Handover | No impact |
| --- | --- |
| Reestablishment/redirect | No impact |
| RACH | No impact |

- Reduced minimum number of Rx branches

The legacy NR RRM requirement is designed based on 2Rx assumption at UE, and when 1Rx is used for RedCap UE, many mobility requirements need to be revisited.

| Handover | HO requirement delay needs to be revisited due to 1Rx |
| --- | --- |
| Reestablishment/redirect | Reestablishment/redirect delay requirement needs to be revisited due to 1Rx |
| RACH | No impact |

For HO requirement with 1Rx, we need to evaluation Tsearch relaxation in HO delay by using 1Rx. The current HO requirement is defined as below (e.g., in section 6.1.1.2.2 TS38.133),

<other parts omitted> When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. <other parts omitted>

We did simulation evaluation for some examples of FR1 15kHz cell searching and FR2 120kHz cell searching under -2dB SINR (simulation assumption is based on agreed R4-2115359).

| TC index | TC of PSS/SSS detection | Sample number | 1 sample detection rate | 2 sample detection rate | 3 sample detection rate |
| --- | --- | --- | --- | --- | --- |
| 1 | TDLA-15KHz | 2 | 0.86 | 0.962481 | 0.987988 |
| 2 | TDLB-15KHz | 2 | 0.867 | 0.969485 | 0.991992 |
| 3 | TDLC-15KHz | 2 | 0.8855 | 0.971486 | 0.991491 |
| 4 | TDLA-30KHz | 2 | 0.857 | 0.982991 | 0.997497 |
| 5 | TDLB-30KHz | 2 | 0.892 | 0.982996 | 0.996496 |
| 6 | TDLC-30KHz | 1 | 0.912 | 0.994497 | 0.999499 |
| 7 | TDLA-120KHz | 2 | 0.8905 | 0.988994 | 0.998999 |
| 8 | TDLC-120KHz | 2 | 0.8965 | 0.987494 | 0.998498 |

Observation 1: Tsearch for intra-frequency HO needs 2 samples when SINR=-2dB and 1Rx is used.

But for inter-frequency HO the AGC estimation time shall not be changed. So we propose,

Proposal 1: RAN4 to relax the Tsearch for RedCap HO delay requirement as followings when SINR=-2dB and 1Rx is used

- For HO to FR1, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 4* Trs for inter-frequency HO

- For HO to FR2, Tsearch = 8*2*Trs for intra-frequency HO and Tsearch = 8*4* Trs for inter-frequency HO

For RRC reestablishment requirement with 1Rx, we need to evaluation Tidentify relaxation in reestablishment delay by using 1Rx. The current reestablishment requirement is defined as below (e.g., in section 6.2.1.2.1 TS38.133),

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; N/A; MAX (1000 ms, 80 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2; N/A; 3520Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2; N/A; MAX (1000 ms, 104 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2; N/A; 4000Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB..

Even though the side condition is ≥ -8dB, we think the Tidentify in known NR cell case could refer to SSB measurement requirement with 1Rx while the Tidentify in unknown NR cell case could refer to PSS/SSS detection + SSB measurement requirement with 1Rx. In our simulation, for PSS/SSS detection, we observed the relaxation of 2 more samples with 1Rx for FR1 and 1 more sample with 1Rx for FR2 compared with 2Rx case; and for SSB measurement, we observed that same number sample could be used but the accuracy could relaxed with 1dB like in LTE cat-1bis requirement.

Table 1. PSS/SSS detection with SINR=-8dB 1Rx

| TC index | TC of PSS/SSS detection | Sample number | 1 sample detection rate | 2 sample detection rate | 3 sample detection rate | 4 sample detection rate | 4 sample detection rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TDLA-15KHz | 4 | 0.607 | 0.768384 | 0.873874 | 0.922884 | 0.951403 |
| 2 | TDLB-15KHz | 4 | 0.6045 | 0.773387 | 0.875375 | 0.927391 | 0.956914 |
| 3 | TDLC-15KHz | 4 | 0.583 | 0.754377 | 0.864865 | 0.922384 | 0.957916 |
| 4 | TDLA-120KHz | 3 | 0.58 | 0.828914 | 0.926927 | 0.967952 | 0.987475 |
| 5 | TDLC-120KHz | 3 | 0.5565 | 0.817409 | 0.927928 | 0.971457 | 0.987421 |

Since RRC reestablishment is a procedure to recover RRC connection and the power consumption could somehow sacrifice a little compared with normal neighbor cell measurement, we don’t think it’s necessary to relax the lower boundary in the requirement for duty cycle.

So we propose:

Proposal 2: the lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx.

Proposal 3: RAN4 to relax the Tidentify  for RedCap reestablishment delay requirement with 1Rx by:

- 2 more samples for FR1

- 1 more sample for FR2

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 12 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 88 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 15 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 112 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2; MAX (880 ms, 8x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell..

For RRC redirection requirement with 1Rx, we need to evaluation Tidentify-NR relaxation in reestablishment delay by using 1Rx. The current reestablishment requirement is defined as below (e.g., in section 6.2.3.2.1 TS38.133),

Similar as for reestablishment, we need to consider the PSS/SS detection performance and measurement performance with 1Rx. With our simulation in PSS/SSS detection and SSB based measurement, we propose to extend 2 more samples for Tidentify-NR for RedCap with 1Rx for FR1 and 1 more sample for FR2. RRC redirection needs to release the existing RRC connection and build a new one, and therefore it’s also time sensitive to redirect to new RRC connection. So we propose to not change the lower boundary in Max function for RRC redirection delay requirement

Proposal 4: the lower boundary in Max function for RRC redirection delay requirement shall not be changed for RedCap UE with 1Rx.

Proposal 5: RAN4 to relax the Tidentify-NR  for RedCap RRC redirection delay requirement with 1Rx by:

- 2 more samples for FR1

- 1 more sample for FR2

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 13 x Trs) |
| FR2 | MAX (880 ms, 8x12 x Trs) |
| Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

- Duplex operation

Regarding the HD-FDD, RAN1 had following working assumption in last meeting,

Agreements: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreements: - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreements: - For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid, and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH - No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs Agreements: - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreements: - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) - Option 4: Valid RO is prioritized over dynamic DL reception

RAN4 needs to discuss, when RRM DL measurement is colliding with UL transmission due to HD-FDD, which one shall be prioritized. Since RAN1 was only considering the serving cell RS reception v.s. HD-FDD UL, RAN4 could decide the DL RS measurement for mobility purpose. In our understanding, the same principle for scheduling restriction in TDD band on FR1 could be reused for HD-FDD case (as following), i.e., RRM DL measurement is prioritized over the UL transmission of HD-FDD. For reestablishment, the UL connection is already lost and therefore the HD-FDD would not impact this case.

Proposal 6: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO/RRC-Redirection requirement.

# Conclusion

In this contribution, we discuss the RRM mobility requirement for RedCap reduced capability.

Observation 1: Tsearch for intra-frequency HO needs 2 samples when SINR=-2dB and 1Rx is used.

Proposal 1: RAN4 to relax the Tsearch for RedCap HO delay requirement as followings when SINR=-2dB and 1Rx is used

- For HO to FR1, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 4* Trs for inter-frequency HO

- For HO to FR2, Tsearch = 8*2*Trs for intra-frequency HO and Tsearch = 8*4* Trs for inter-frequency HO

Proposal 2: the lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx.

Proposal 3: RAN4 to relax the Tidentify  for RedCap reestablishment delay requirement with 1Rx by:

- 2 more samples for FR1

- 1 more sample for FR2

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 12 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 88 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 15 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 112 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 4: the lower boundary in Max function for RRC redirection delay requirement shall not be changed for RedCap UE with 1Rx.

Proposal 5: RAN4 to relax the Tidentify-NR  for RedCap RRC redirection delay requirement with 1Rx by:

- 2 more samples for FR1

- 1 more sample for FR2

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 13 x Trs) |
| FR2 | MAX (880 ms, 8x12 x Trs) |
| Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

Proposal 6: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO/RRC-Redirection requirement.

# References

[1] R4-2115358, WF on RedCap RRM requirements, Ericsson, RAN4 #100e