3GPP TSG-RAN4 Meeting #101-e	R4-2117450

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.20.3.1.4

Source:	Apple

Title:	Discussion on signalling characteristics for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement impact for RedCap capability reduction has been discussed and the conclusions were captured in the WF[1].

## RLM requirements Impact on RLM-RS resources The number of RLM-RS is determined by RAN1 in TS38.213. RAN4 shall follow the RAN1 conclusion on the number of RLM -RS. Impact on BW used in RLM Impact on bandwidth of CSI-RS for RLM with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded. Impact on parameters for IN/OOS Impact on parameters for IN/OOS due to BW reduction and 1 Rx is expected. Impact on in-sync/out-of-sync thresholds SNR thresholds for OOS/IS impacted due to 1 rx. The detailed SINR threshold to be discussed under performance part. ## Link recovery requirements Impact on BW used in BFD Impact on bandwidth of CSI-RS for BFD with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded. Impact on SINR thresholds used in BFD Impact on SINR thresholds setup in test cases can be discussed in the performance part of the WI at later stage. Impact on L1 RSRP measurement accuracy requirement L1 RSRP measurement accuracy is to be discussed under performance part of the WI. Impact on L1 RSRP measurement delay requirement - RAN4 to study the impact using the simulation assumptions to decide whether existing requirements can be reused.

Based on the last meeting agreements, there are three key features for complexity reduction which would impact the RAN4 RRM requirements,

- Reduced maximum UE bandwidth

- Reduced minimum number of Rx branches

- Duplex operation

In this contribution, we further discuss the signalling characteristics requirement for RedCap reduced capability.

# Signalling characteristics requirement for RedCap reduced capability

- Reduced maximum UE bandwidth

Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz, and maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. In some CSI-RS based requirements (CSI-RS based RLM, BFD and etc), RAN4 used 48PRBs for CSI-RS BW. If 60kHz CSI-RS is used, the CSI-RS BW would be 34.56MHz, and therefore the corresponding requirements need to be modified.

| RLM | Hypothetical PDCCH transmission parameter for RLM needs to be revised (Current hypothetical BW for CSI-RS is larger than 20MHz with 60kHz SCS for FR1). |
| --- | --- |
| Beam management (BFD/CBD) | Hypothetical PDCCH transmission parameter for BFD needs to be revisited (Current hypothetical BW for CSI-RS is larger than 20MHz with 60kHz SCS for FR1). |

Based on the analysis, the bandwidth in hypothetical PDCCH transmission parameter table for CSI-RS based RLM and CSI-RS based BFD shall be changed to 24PRBs for 60kHz SCS case in FR1. Regarding the CSI-RS based L1-RSRP, we think one shot is a baseline assumption for such beam measurement requirement in R15, so we propose to not change the CSI-RS based L1-RSRP measurement delay requirement. However, the CSI-RS based L1-RSRP measurement accuracy needs to be revisited since the CSI-RS BW configuration of 48PRBs is not applicable for RedCap UE with 60kHz SCS in FR1. Thus, CSI-RS BW for L1-RSRP measurement accuracy requirement with 60kHz SCS in FR1 could be changed to 24PRBs and the corresponding accuracy requirement shall be re-evaluated

Proposal 1: Regarding the reduced maximum UE bandwidth for RedCap,

- for CSI-RS based RLM/BFD, the bandwidth in hypothetical PDCCH transmission parameter tables shall be changed to 24PRBs for 60kHz SCS case in FR1.

- for CSI-RS based L1-RSRP, CSI-RS BW for L1-RSRP measurement accuracy requirement with 60kHz SCS in FR1 could be changed to 24PRBs and the corresponding accuracy requirement shall be re-evaluated.

  - No change is needed for CSI-RS based L1-RSRP measurement delay requirement.

- Reduced minimum number of Rx branches

The legacy NR RRM requirement is designed based on 2Rx assumption at UE, and when 1Rx is used for RedCap UE, many RRM requirements need to be revisited.

| RLM | - Hypothetical PDCCH transmission parameter: Ratio of hypothetical PDCCH RE/DMRS energy to average SSS/CSI-RS RE energy need to be revisited due to 1Rx. - the SNR1/2/3/4/5 threshold setup in TCs need to be revisited due to 1Rx. |
| --- | --- |
| Beam management (BFD/CBD) | BFD: - Hypothetical PDCCH transmission parameter: Ratio of hypothetical PDCCH RE/DMRS energy to average SSS/CSI-RS RE energy need to be revisited due to 1Rx. - the SNR1/2/3 threshold setup in TCs need to be revisited due to 1Rx CBD: - No need to revisit evaluation period due to 1Rx. (Using 1Rx is to save power for redcap, but extension of evolution period would defeat such power saving benefit.) - The margin of L1-RSRP setup in CBD TC shall be revisited due to 1Rx |

However, as we assumed for LTE cat-1bis UE, the OOS/IS evaluation period shall not be extended further for 1Rx case. In SSB based SINR accuracy simulation with 10/15/20 samples for 15kHz as below (simulation assumption R4-2115360), we didn’t see big difference (≤1dB) to extend the sample number even with low SINR condition as -15dB. The CSI-RS case is also similar as SSB case based on previous evaluation in R15.

Figure 1. SSB based SINR accuracy with 10 samples under -15dB (1Rx)

Figure 2. SSB based SINR accuracy with 15 samples under -15dB (1Rx)

Figure 3. SSB based SINR accuracy with 20 samples under -15dB (1Rx)

So, based on the above analysis, we propose,

Proposal 2: No need to extend the evaluation period for RLM/BFD/CBD.

- Duplex operation

Regarding the HD-FDD, RAN1 had following working assumption in last meeting,

Agreements: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreements: - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreements: - For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid, and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH - No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs Agreements: - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreements: - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) - Option 4: Valid RO is prioritized over dynamic DL reception

RAN4 needs to discuss, when RRM DL measurement is colliding with UL transmission due to HD-FDD, which one shall be prioritized. RAN1 agreed that “For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission”. In our understanding, the same principle for scheduling restriction in TDD band on FR1 could be reused for HD-FDD case (as following), i.e., RRM DL measurement is prioritized over the UL transmission of HD-FDD.

The discussion in last meeting was,

Conditions for performing RLM for HD-FDD UE - Proposal 1 (MTK, Qualcomm): Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. - Proposal 1a (Huawei): Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec.

In order to consider the solutions from other companies, we can compromise to define such side condition for RLM/BFD requirement applicability. However, for CBD, probably the BF has already occurred, we think it’s more important to recover the beam as soon as possible rather than UL transmission, thus, we propose to prioritize CBD-RS always over UL in case of HD-FDD.

But options in last meeting are not very clear: 1 RLM-RS could be understood as 1 sample of a specific RLM-RS or could be 1 RLM-RS among multiple RLM-RSs associated with multiple OOS/IS evaluation processes. Our understanding is: for each OOS/IS evaluation process based on one specific RLM-RS, at least 1 sample must fall with DL occasion within an indication period.

Proposal 3: CBD evaluation is always prioritized over the UL transmission of HD-FDD for RedCap.

Proposal 4: Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: for each RLM-RS in RLM requirement, at least 1 sample must fall with DL occasion within an indication period.

Proposal 5: Support adding the following condition for the evaluation period of 5G NR BFD requirement for HD-FDD: for each BFD-RS in BFD requirement, at least 1 sample must fall with DL occasion within an indication period.

# Conclusion

In this contribution, we further discuss the signalling characteristics requirement for RedCap reduced capability.

Proposal 1: Regarding the reduced maximum UE bandwidth for RedCap,

- for CSI-RS based RLM/BFD, the bandwidth in hypothetical PDCCH transmission parameter tables shall be changed to 24PRBs for 60kHz SCS case in FR1.

- for CSI-RS based L1-RSRP, CSI-RS BW for L1-RSRP measurement accuracy requirement with 60kHz SCS in FR1 could be changed to 24PRBs and the corresponding accuracy requirement shall be re-evaluated.

  - No change is needed for CSI-RS based L1-RSRP measurement delay requirement.

Proposal 2: No need to extend the evaluation period for RLM/BFD/CBD.

Proposal 3: CBD evaluation is always prioritized over the UL transmission of HD-FDD for RedCap.

Proposal 4: Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: for each RLM-RS in RLM requirement, at least 1 sample must fall with DL occasion within an indication period.

Proposal 5: Support adding the following condition for the evaluation period of 5G NR BFD requirement for HD-FDD: for each BFD-RS in BFD requirement, at least 1 sample must fall with DL occasion within an indication period.

# References

[1] R4-2115358, WF on RedCap RRM requirements, Ericsson, RAN4 #100e