3GPP TSG-RAN4 Meeting #101-e	R4-2117451

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.20.3.1.5

Source:	Apple

Title:	Discussion on cell identification and measurement for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement impact for RedCap capability reduction has been discussed and the conclusions were captured in the WF[1].

## Measurement capability requirements Whether measurement capability may have to be reduced FFS whether to reduce the number cells/layers/frequencies/SSBs to be monitored/measured by RedCap UEs.

And the simulation assumption have been agreed as the following list:

R4-2115359	Simulation assumptions for RedCap cell detection performance

R4-2115360	Simulation assumptions for RedCap SSB based RRM measurement performance

R4-2115361	Simulation assumptions for RedCap RLM and BFD performance

R4-2115362	Simulation assumptions for RedCap L1 RSRP measurement performance

R4-2115363	Simulation assumptions for RedCap PBCH detection

Based on the last meeting agreements, there are three key features for complexity reduction which would impact the RAN4 RRM requirements,

- Reduced maximum UE bandwidth

- Reduced minimum number of Rx branches

- Duplex operation

In this contribution, we discuss the cell identification and measurement requirement for RedCap reduced capability.

# Cell identification and measurement requirement for RedCap reduced capability

- Reduced maximum UE bandwidth

Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz, and maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. Reduced maximum UE bandwidth has impact on L3 measurement only if CSI-RS based L3 measurement is used.

| Measurement capability and cell identification/measurement delay in RRC connected mode | - CSI-RS based L3 measurement delay needs to be revisited (CSI-RS L3 minimum measurement BW assumption is larger than 20MHz with 60kHz SCS for FR1) if R16 CSI-RS L3 feature is considered for RedCap. |
| --- | --- |
| Measurement accuracy | - CSI-RS based L3 measurement accuracy need to be revisited for 60kHz SCS cases in FR1 if R16 CSI-RS L3 feature is considered for RedCap (current requirement assumes 48PRBs for CSI-RS). |

In last meeting RAN4 agreed that:

When discussing possible combinations of Rel-16 features and RedCap, we should by default assume that the features are not applicable and then identify which features (such as R16 CSI-RS based L3 measurement, L1-SINR measurement, SFTD measurement, CGI reading, 2-step RACH, and PL-RS change, etc.) can be combined with RedCap case by case based on justification.

So we could postpone the CSI-RS L3 measurement requirement for RedCap.

Proposal 1: postpone the CSI-RS L3 measurement requirement for RedCap.

- Reduced minimum number of Rx branches

The legacy NR RRM requirement is designed based on 2Rx assumption at UE, and when 1Rx is used for RedCap UE, cell identification and measurement requirements need to be revisited.

| Measurement capability and cell identification/measurement delay in RRC connected mode, SFTD measurement, L1-RSRP | - SSB based Cell identification/measurement delay needs to be revisited due to 1Rx. - CSI-RS based L3 measurement delay needs to be revisited due to 1Rx if R16 CSI-RS L3 feature is considered for RedCap. |
| --- | --- |
| Measurement accuracy | - SSB based L3 measurement accuracy requirement needs to be revisited due to 1Rx. - CSI-RS based L3 measurement accuracy requirement needs to be revisited due to 1Rx if R16 CSI-RS L3 feature is considered for RedCap. |

According to proposal 1, we would focus on SSB based cell identification and measurement in this contribution.

Regarding measurement capability, we could refer to the similar requirement applicability in LTE for cat-1bis UE. In TS36.133 section 3.6.1, the applicability requirement is defined as,

- The requirements for a UE category 1bis are derived assuming UE category 1bis [31] and a single antenna receiver. Following requirements are applicable to UE category 1bis. - Cell re-selection requirements in section 4.2.2.1 to 4.2.2.10 - Handover requirements in section 5.1, 5.2, 5.3 and 5.4 - RRC re-establishment requirements in section 6.1 - Random access requirements in section 6.2 - RRC connection release with redirection requirements in section 6.3 - UE transmit timing requirements in section 7.1 - UE timer accuracy requirements in section 7.2 - Timing advance requirements in section 7.3 - Radio link monitoring requirements in section 7.11 - UE measurement capability in section 8.1.2.1 - E-UTRAN intra frequency measurement requirements in section 8.5.2.1.1 and 8.5.2.1.3 - E-UTRAN inter frequency measurement requirements in section 8.1.2.3.1, 8.1.2.3.2, 8.1.2.3.3 and 8.1.2.3.4

The legacy UE’s measurement capability requirement is still applicable to cat-1bis UE. In last meeting RAN4 agreed that,

  - Define inter-RAT LTE RRM requirements in IDLE/INACTIVE and CONNECTED states

    - For 2RX capable RedCap UEs

      - Use 2RX inter-RAT LTE requirements defined in TS 38.133 as baseline

    - For 1RX capable RedCap UEs

      - Use LTE Cat1bis requirements in TS 36.133 as baseline

So, in RedCap we propose to handle the measurement capability in a same way as cat-1bis UE in LTE.

Proposal 2: The legacy NR requirements of number cells/layers/frequencies/SSBs to be monitored/measured is applied to RedCap UEs.

For PSS/SSS detection, PBCH DMRS acquisition and SSB based RSRP measurement, the simulation results are summarized as in the following tables,

Table 1. PSS/SSS detection performance for 15kHz with 1Rx (SINR=-6dB)

| TC index | TC of PSS/SSS detection | Sample number |
| --- | --- | --- |
| 1 | AWGN-15KHz-case1 | 1 |
| 2 | AWGN -15KHz-case2 | 1 |
| 3 | TDLA-15KHz-case1 | 3 |
| 4 | TDLA-15KHz-case2 | 3 |
| 5 | TDLB-15KHz-case1 | 3 |
| 6 | TDLB-15KHz-case2 | 3 |
| 7 | TDLC-15KHz-case1 | 3 |
| 8 | TDLC-15KHz-case2 | 3 |

Table 2. PSS/SSS detection performance for 30kHz with 1Rx (SINR=-6dB)

| TC index | TC of PSS/SSS detection | Sample number |
| --- | --- | --- |
| 11 | Static-30KHz-case1 | 1 |
| 12 | Static-30KHz-case2 | 1 |
| 13 | TDLA-30KHz-case1 | 3 |
| 14 | TDLA-30KHz-case2 | 3 |
| 15 | TDLB-30KHz-case1 | 4 |
| 16 | TDLB-30KHz-case2 | 3 |
| 17 | TDLC-30KHz-case1 | 4 |
| 18 | TDLC-30KHz-case2 | 3 |

Table 3. PSS/SSS detection performance for 120kHz with 1Rx (SINR=-6dB)

| TC index | TC of PSS/SSS detection | Sample number |
| --- | --- | --- |
| 21 | Static-120KHz-case1 | 1 |
| 22 | Static-120KHz-case2 | 1 |
| 23 | TDLA-120KHz-case1 | 2 |
| 24 | TDLA-120KHz-case2 | 2 |
| 25 | TDLC-120KHz-case1 | 2 |
| 26 | TDLC-120KHz-case2 | 3 |

Compared with simulation results in R4-1711691, for FR1 2 more samples are needed in the worst case, and for FR2 1 more sample is needed in the worst case.

Observation 1:

In FR1, 2 more samples are needed for RedCap 1Rx PSS/SSS detection compared with legacy UE in the worst case.

In FR2, 1 more sample is needed for RedCap 1Rx PSS/SSS detection compared with legacy UE in the worst case.

Since the lower bound 600ms was from the baseline cell detection of LTE, like in LTE cat-1bis case, we don’t think it shall be changed for RedCap 1Rx case.

Proposal 3: the baseline intra-frequency PSS/SSS detection requirement without DRX shall be defined:

- In FR1, extend the PSS/SSS detection delay by 2 SMTC without changing the lower boundary ‘600ms’

  - non-DRX delay requirement: max( 600ms, ceil( 7 x Kp) x SMTC period ) x CSSFintra

- In FR2, extend the PSS/SSS detection delay by 1*8 SMTC without changing the lower boundary ‘600ms’ (8 is the beam sweeping factor):

  - For a UE supporting power class 3, Mmeas_period_w/o_gaps = 32.

The PBCH DMRS detection simulation results are as below,

Table 4. FR1 Time index detection performance with 1Rx (SINR=-6dB)

| TC index | PBCH DMRS identification TC | Sample number |
| --- | --- | --- |
| 1 | TDLA30-30 for 1 Rx with -6dB and 15kHz | 3 |
| 2 | TDLB100-30 for 1 Rx with -6dB and 15kHz | 3 |
| 3 | TDLC300-30 for 1 Rx with -6dB and 15kHz | 3 |
| 4 | AWGN for 1 Rx with -6dB and 15kHz | 1 |
| 5 | TDLA30-30 for 1 Rx with -6dB and 30kHz | 3 |
| 6 | TDLB100-30 for 1 Rx with -6dB and 30kHz | 3 |
| 7 | TDLC300-30 for 1 Rx with -6dB and 30kHz | 2 |
| 8 | AWGN for 1 Rx with -6dB and 30kHz | 1 |

Table 5. FR1 Time index detection performance with 1Rx (SINR=-8dB)

| TC index | PBCH DMRS identification TC | Sample number |
| --- | --- | --- |
| 1 | TDLA30-30 for 1 Rx with -8dB and 15kHz | 4 |
| 2 | TDLB100-30 for 1 Rx with -8dB and 15kHz | 4 |
| 3 | TDLC300-30 for 1 Rx with -8dB and 15kHz | 3 |
| 4 | AWGN for 1 Rx with -8dB and 15kHz | 2 |
| 5 | TDLA30-30 for 1 Rx with -8dB and 30kHz | 4 |
| 6 | TDLB100-30 for 1 Rx with -8dB and 30kHz | 4 |
| 7 | TDLC300-30 for 1 Rx with -8dB and 30kHz | 3 |
| 8 | AWGN for 1 Rx with -8dB and 30kHz | 2 |

Compared with simulation results in R4-1711691, for FR1 1 more samples are needed in the worst case.

Observation 2:

In FR1, 1 more sample is needed for RedCap 1Rx time index detection compared with legacy UE in the worst case.

Since the lower bound 120ms for FR1 time index reading requirement was based on 3 samples with typical SMTC periodicity =40ms, we think it shall be revised for the RedCap time index detection with 1Rx (since sample number of RedCap 1Rx is creased to 4 based on simulation and margin), i.e., revised to 4*40=160ms.

Proposal 4: the baseline FR1 intra-frequency time index detection requirement without DRX shall be defined:

- In FR1, extend the time index detection delay by 1 SMTC with changing the lower boundary to ‘160 ms’

  - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra

The SSB based RSRP measurement simulation results are as below,

Table 6. SSB based RSRP measurement accuracy for 15kHz with 1Rx (SINR=-6dB)

Table 7. SSB based RSRP measurement accuracy for 30kHz with 1Rx (SINR=-6dB) (5 sample only)

Table 8. SSB based RSRP measurement accuracy for 120kHz with 1Rx (SINR=-6dB) (5 and 8 samples)

Compared with previous simulation results of RSRP measurement with 2Rx, for FR1 and FR2, the degradation of measurement accuracy is 1dB with 5 samples in the worst case.

Observation 3:

- For both FR1 and FR2 RSRP measurement error, there is no big difference between 5 samples and 8 samples (≤0.5dB).

- For both FR1 and FR2, the degradation of measurement accuracy is 1dB with 5 samples in the worst case.

In LTE Cat-1bis or Cat-0 UE measurement requirement, even though the sample number is still 5, the total measurement period is doubled from 200ms to 400ms with consideration of longer duty cycle for those UEs. We think it also makes sense to save power for RedCap UE with longer duty cycle. So, we propose:

Proposal 5: the baseline FR1 SSB based intra-frequency RSRP measurement requirement without DRX shall be defined:

- Change the lower bound of measurement delay to 400ms for FR1 and 800ms for FR2 for longer duty cycle (like in LTE cat1-bis) without increasing the sample number

  - Delay is max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra for FR1

  - Delay is max(800ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period) x CSSFintra

In legacy FR1 intra-frequency absolute RSRP measurement accuracy, 1.5dB RF margin reduction has been considered to achieve +/-4.5dB accuracy, but this enhancement gain shall not be applied to RedCap UE with 1Rx, like in LTE cat-1bis.

Proposal 6: the baseline FR1 SSB based intra-frequency RSRP measurement accuracy requirement without DRX shall be defined:

- For FR1 RedCap with 1Rx

  - Relax the current absolute accuracy of +/-4.5dB to +/-7dB, and relax the absolute accuracy for other Io and side conditions by 1dB

  - Relax the relative RSRP accuracy by 1dB

- For FR2 RedCap with 1Rx

  - Relax the current absolute and relative accuracy by 1dB

- Duplex operation

Regarding the HD-FDD, RAN1 had following working assumption in last meeting,

Agreements: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreements: - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreements: - For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid, and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH - No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs Agreements: - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreements: - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) - Option 4: Valid RO is prioritized over dynamic DL reception

RAN4 needs to discuss, when RRM DL measurement is colliding with UL transmission due to HD-FDD, which one shall be prioritized. Since RAN1 was only considering the serving cell RS reception v.s. HD-FDD UL, RAN4 could decide the DL RS measurement for mobility purpose. In our understanding, the same principle for scheduling restriction in TDD band on FR1 could be reused for HD-FDD case (as following), i.e., RRM DL measurement is prioritized over the UL transmission of HD-FDD. Moreover, if MG is used for RRM measurement, MG shall be also prioritized over UL of HD-FDD for RedCap, i.e., no UL transmission due to HD-FDD for RedCap is allowed during MG duration.

Proposal 7: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement.

Proposal 8: Measurement gap is prioritized over the UL transmission of HD-FDD for RedCap UE, i.e., no UL transmission due to HD-FDD is allowed during MG duration.

There was another issue from last meeting:

Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ - Proposal 1 (Ericsson, Apple, Vivo, Nokia): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. - Proposal 2 (Huawei, CMCC): Depends on network deployment. - Proposal 3 (Xiaomi, Oppo, MTK): FFS

As RAN4 agreed that,

- When discussing possible combinations of Rel-16 features and RedCap, we should by default assume that the features are not applicable and then identify which features (such as R16 CSI-RS based L3 measurement, L1-SINR measurement, SFTD measurement, CGI reading, 2-step RACH, and PL-RS change, etc.) can be combined with RedCap case by case based on justification.

Since inter-frequency without gap (when inter-frequency SSB is inside active BWP) is a R16 introduced feature, we propose to not consider it in RedCap for now. Moreover, RedCap UE in FR1 only has 20MHz BW, it would be less possible to contain other inter-frequency SSB in its active BWP compared with legacy UEs.

Proposal 9: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

# Conclusion

In this contribution, we discuss the cell identification and measurement requirement for RedCap reduced capability.

Proposal 1: postpone the CSI-RS L3 measurement requirement for RedCap.

Proposal 2: The legacy NR requirements of number cells/layers/frequencies/SSBs to be monitored/measured is applied to RedCap UEs.

Observation 1:

In FR1, 2 more samples are needed for RedCap 1Rx PSS/SSS detection compared with legacy UE in the worst case.

In FR2, 1 more sample is needed for RedCap 1Rx PSS/SSS detection compared with legacy UE in the worst case.

Proposal 3: the baseline intra-frequency PSS/SSS detection requirement without DRX shall be defined:

- In FR1, extend the PSS/SSS detection delay by 2 SMTC without changing the lower boundary ‘600ms’

  - non-DRX delay requirement: max( 600ms, ceil( 7 x Kp) x SMTC period ) x CSSFintra

- In FR2, extend the PSS/SSS detection delay by 1*8 SMTC without changing the lower boundary ‘600ms’ (8 is the beam sweeping factor):

  - For a UE supporting power class 3, Mmeas_period_w/o_gaps = 32.

Observation 2:

In FR1, 1 more sample is needed for RedCap 1Rx time index detection compared with legacy UE in the worst case.

Proposal 4: the baseline FR1 intra-frequency time index detection requirement without DRX shall be defined:

- In FR1, extend the time index detection delay by 1 SMTC with changing the lower boundary to ‘160 ms’

  - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra

Observation 3:

- For both FR1 and FR2 RSRP measurement error, there is no big difference between 5 samples and 8 samples (≤0.5dB).

- For both FR1 and FR2, the degradation of measurement accuracy is 1dB with 5 samples in the worst case.

Proposal 5: the baseline FR1 SSB based intra-frequency RSRP measurement requirement without DRX shall be defined:

- Change the lower bound of measurement delay to 400ms for FR1 and 800ms for FR2 for longer duty cycle (like in LTE cat1-bis) without increasing the sample number

  - Delay is max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra for FR1

  - Delay is max(800ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period) x CSSFintra

Proposal 6: the baseline FR1 SSB based intra-frequency RSRP measurement accuracy requirement without DRX shall be defined:

- For FR1 RedCap with 1Rx

  - Relax the current absolute accuracy of +/-4.5dB to +/-7dB, and relax the absolute accuracy for other Io and side conditions by 1dB

  - Relax the relative RSRP accuracy by 1dB

- For FR2 RedCap with 1Rx

  - Relax the current absolute and relative accuracy by 1dB

Proposal 7: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement.

Proposal 8: Measurement gap is prioritized over the UL transmission of HD-FDD for RedCap UE, i.e., no UL transmission due to HD-FDD is allowed during MG duration.

Proposal 9: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

# References

[1] R4-2115358, WF on RedCap RRM requirements, Ericsson, RAN4 #100e