3GPP TSG-RAN4 Meeting #101-e	R4-2117454

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.13.5.3

Source:	Apple

Title:	Discussion on mobility for NR NTN

Document for:	Discussion

# Introduction

In last RAN4 meeting, the mobility requirement for NR NTN has been discussed and the agreements were captured in the WF[1], duplicated as below.

Issue 1-2-2 Neighbour/Target Cell Information Acquisition Agreement: - If not violates RAN1/2 agreements, RAN4 further discuss and determine the following in RAN4#101 e-meeting. - RAN4 to develop RRM requirements assuming parameters to derive timing relation between the UE and non-serving cell(s) for measurement and/or handover will be provided by serving cell, e.g. K_offset, N_TA,common, ephemeris information, etc. If UE doesn’t obtain those parameters from the serving cell, additional delay is expected.

In this contribution, we further discuss the assumption of K_offset, N_TA,common, and ephemeris information in mobility requirement.

# RAN4 assumption for NTN mobility

In legacy TN CHO requirement, we have the delay requirement in TS38.133 as below,

SI (e.g., SIB reading) is not required in CHO procedure, and RO information would be configured by serving cell for the handover. We think the same assumption shall be applied for CHO in NTN as well, otherwise the CHO delay would be much longer than the legacy TN case.

In RAN1, K_offset, N_TA,common, and ephemeris information were introduced in the system information for UE to access to new NTN cell, and those parameters are essential for cell accessing and communication.

Even though the ending point of CHO requirement is the msg1 transmission timing, the UE still needs to know the K_offset and ephemeris information before msg1 transmission for msg3/msgB to complete a handover procedure in field. Since we don’t think this SI reading shall be conducted during the CHO procedure, it makes sense to let source serving cell provide such K_offset and ephemeris information info of target serving cell to UE for NTN CHO.

The common TA would also be applied to the UE msg1/msgA transmission to the target cell in CHO case based on the above definition. However, we don’t think UE shall be required to acquire common TA in SI of target cell during the CHO procedure, and therefore we also propose to let source serving cell provide such common TA info of target serving cell to UE for NTN CHO.

For NTN neighbor cell synchronization/measurement, it’s also important if serving cell could pass such ephemeris information to UE to improve the UE side compensation on high doppler shift when needed.

And also, for polarization information, UE shall not be required to read it in SIB during handover. RAN1 had following agreements that,

Agreement: When polarization signalling is present in SIB

- SIB indicates DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear

- FFS: whether polarization signalling is per SSB

Agreement in RAN1#106bis-e:

- Support polarization signalling for target serving cell in handover command message.

- Support polarization signalling for non-serving cell in RRM measurement configuration.

Proposal 1:

RAN4 to develop RRM requirements assuming parameters to derive timing/spatial relation between the UE and non-serving cell(s) for measurement and/or handover will be provided by serving cell, e.g. K_offset, N_TA,common, ephemeris information, polarization information, etc.

If UE doesn’t obtain those parameters from the serving cell, additional delay is expected in the corresponding requirement.

# Conclusion

In this contribution, we discuss timing requirements for NR NTN.

Proposal 1:

RAN4 to develop RRM requirements assuming parameters to derive timing/spatial relation between the UE and non-serving cell(s) for measurement and/or handover will be provided by serving cell, e.g. K_offset, N_TA,common, ephemeris information, polarization information, etc.

If UE doesn’t obtain those parameters from the serving cell, additional delay is expected in the corresponding requirement.

# References

[1] R4-2115345, WF on RRM requirements for NTN measurement and mobility, Qualcomm, RAN4 #100-e