3GPP TSG-RAN WG4 Meeting #101-e     	R4-2117477

e-Meeting, November 1 – November 12, 2021

Title:	Discussion on maximum duration for joint channel estimation

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.18.2.3

Document for:	Approval

# Introduction

The following can be noted from the work item description (WID) for Rel-17 coverage enhancement [1]:

  - “Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary, by RAN4 [RAN1, RAN4]

      - Potential optimization of DM-RS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]”

The remaining open issues concerning the phase continuity and power consistency conditions for PUSCH and PUCCH repetitions to enable joint channel estimation as exchanged in RAN4 [2]-[5] and RAN1 LSs [6]-[9] will be discussed in this contribution.

# Discussion

The following can be noted from the WF made in RAN4#100-e [10]:

- Option 1: The maximum duration should depend on the interval where the UE does not make frequency adjustment with DL signal - Alt 1: at least smaller/equal than the configured SSB periodicity - Alt 2: other DL RS periodicity - Option 2: Depends on JCE performance considering the phase tolerance and/or gNB frequency offset compensation accuracy during the duration even within a sync periodicity. - Option 3: Maximum duration length depends on both option 1 and option 2

For Option 2, the discussion on JCE performance may be complicated. Indeed, for defining maximum duration based on performance, one may need to identify a threshold related to duration for which the performance saturates. This task is hardly feasible. On the one hand, how to apply JCE is up to gNB’s implementation and not specified. On the other hand, even if a maximum duration can be identified from a possible JCE performance evaluation, it may not be applicable to all UEs. Therefore, the above Option 1 is more suitable for defining the length of maximum duration.

Proposal 1. The maximum duration should depend on the interval where the UE does not make frequency adjustment with DL signal. At least the maximum duration is equal to the minimum configured SSB periodicity.

# 3	Conclusion

In this contribution, we proposed the following for phase continuity and power consistency for PUSCH and PUCCH repetitions:

Proposal 1. The maximum duration should depend on the interval where the UE does not make frequency adjustment with DL signal. At least the maximum duration is equal to the minimum configured SSB periodicity.

# 4 	References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R4-2103393	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), February 2021.

- R4-2105417	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), April 2021.

- R4-2107880	Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), May 2021.

- R4-2114991 Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), August 2021.

- R1-2009784	LS on PUCCH and PUSCH repetition, Qualcomm (moderator), November 2020.

- R1-2104119	Reply on LS on PUCCH and PUSCH repetition, Qualcomm (moderator), April 2021.

- R1-2106212	LS on joint channel estimation for PUSCH and PUCCH, China Telecom (moderator), May 2021

- R1-2108458	Reply on LS on PUCCH and PUSCH repetition, Qualcomm (moderator), August 2021.

- R4-2114992	WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, August, 2021.