3GPP TSG-RAN WG4 Meeting #101-e 		R4-2117479

# Electronic Meeting, November 1-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	RedCap UE Tx requirements for FR1

Agenda item:	8.20.2.1.1

Document for:	Discussion

- Introduction

In RAN4 #100-e, the RedCap UE Tx requirements have been discussed, and the following agreement about the RedCap UE Power Class in FR1 is made [1].

GTW session agreement: - PC3 should be specified for Redcap in FR1 - No PC1 and PC1.5 for RedCap UE in FR1 - PC2 for RedCap UE depending on operator request

This contribution discusses the RedCap UE Tx requirements for FR1 and makes a proposal.

- Discussion

In RAN4 #100-e [1], RAN4 agreed on the power classes supported for RedCap UEs in the licensed bands. While confirming the agreement on UE power class in GTW session of RAN4 #100-e, this contribution further discusses the UE power class for NR unlicensed bands.

On the basis of the conclusion from an e-mail discussion in RAN #93-e [2], RedCap-type devices are not precluded from operating in unlicensed bands. Therefore, NR unlicensed bands can be potentially used for RedCap UE in FR1. Considering the use cases of NR bands such as n46 and n96, NR UE Tx requirements could be reused for RedCap UE based on the initial discussions in RAN4 and the approved WF [3]. Also, in [4], the NR UE maximum output power is specified for each power class supported in the different bands. In particular, Clauses 6.2F.1 of TS38.101 defines UE maximum output power of shared spectrum channel access carrier where Table 6.2F.1-1 UE Power Class has a note stating that Power Class 5 is default power class. Therefore, RedCap UE operating in n46 and n96 needs to follow the requirement for maximum output power in Table 6.2F.1-1.

Proposal 1: Power Class 5 is supported for RedCap UE using the unlicensed bands n46 and n97 in FR1.

- Conclusion

This contribution discussed the RedCap UE Tx requirements for FR1. The following proposal was made.

Proposal 1: Power Class 5 is supported for RedCap UE using the unlicensed bands n46 and n97 in FR1.

- References

- R4-2115096, “WF on the RedCap RF,” Ericsson, RAN4 #100-e.

- RP-212634	, “Moderator's summary for discussion [93e-16-RedCap-WI],” Intel, RAN #93-e.

- R4-2108005, “WF on RedCap,” Ericsson, RAN4 #99-e.

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone,” v17.2.0.